3GPP TSG RAN WG1 #108-e		R1-2201121

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Discussion on UE features for NR operation from 52.6-71GHz

Agenda Item:	8.16.2

Document for:	Discussion and Decision

- Introduction

After RAN1 #107bis-e meeting, the Rel-17 UE features are provided in [1]. In this contribution, our views on NR 52.6GHz part are provided.

- Discussion

Wideband PRACH:

After RAN1#107bis-e, it is not decided yet applicable spectrum type of wideband PRACH, i.e., unlicensed band only or not. The motivation of wideband PRACH is mainly from PSD limitation on unlicensed band. Besides, wideband PRACH (10-27 in [2]) in NRU is also limited to unlicensed band. Therefore, there is no need to extend them to licensed band.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- |
| 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Per band | [A UE that supports FG 24-2 must indicate this FG is supported] [Note: This FG is only supported in bands for shared spectrum operation] |
| 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Per band | [Note: This FG is only supported in bands for shared spectrum operation] |

Proposal 1: FG 24-1b and 24-4b are only supported in bands for shared spectrum operation.

On 24-1b, another issue is whether “A UE that supports FG 24-2 must indicate this FG is supported”, the same handling as NRU should be adopted, i.e. 10-27 in [2]. It is clearly that wideband PRACH is not a basic FG for any scenario. Therefore, there is no need to bundle 24-2 and 24-1b together.

Proposal 2: For 24-1b, remove “A UE that supports FG 24-2 must indicate this FG is supported”.

Multi-RB PUCCH:

Similar with wideband PRACH, there is no need to bundle the following FG with any other FG.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- |
| 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Per band | [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation |

Proposal 3: For 24-1c, remove “A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported”.

Multi-PDSCH scheduling:

On 24-1d for 120KHz multi-PDSCH scheduling, it is not decided yet whether it can be extended to other frequency ranges. First of all, this FG may only be extended to FR2-2 since there is no 120KHz SCS in FR1. Besides, it seems that there is no strong motivation and use case to extend this FG to other frequency ranges.

Proposal 4: For 24-1d, remove “FFS: to extend this FG to other frequency ranges”.

On 24-4/5 for 480/960KHz multi-PDSCH scheduling, it is not decided yet whether it is a separate FG or a component of 480/960 kHz support. In our view, there is no critical requirement for multi-PDSCH scheduling even when multi-slot PDCCH monitoring is used for 480/960 kHz. The only drawback is data rate loss brought by multi-slot PDCCH monitoring but the system still works. Therefore, separate optional multi-PDSCH scheduling capability is more flexible. In this way, to enhance data rate further, UE has flexibility to increase PDCCH monitoring complexity with smaller X value or support multi-PDSCH scheduling.

| Index | Feature group | Components | Prerequisite feature groups | Type | Note |
| --- | --- | --- | --- | --- | --- |
| 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements | 24-1 |  | FFS: to extend this FG to other frequency ranges |
| 24-4 | 480KHz SCS support for DL | 1. 480KH SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) FFS: 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot with set2 = (4, 3) and (7, 3) symbols where set2 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) | 24-1 |  |  |
| 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with X=8 slots FFS: 3. Multi-PDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 3. Within the Ys = 1 slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with set1 = (7, 3) symbols where set1 is defined in FG3-5b (FFS: Monitoring capability within slots of type 1 CSS without dedicated RRC configuration and type0, 0A, and 2 CSS) 4. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD (This supersedes corresponding component of FG 3-5b) 5. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD (This supersedes corresponding component of FG 3-5b) | 24-1 |  |  |

Proposal 5: List multi-PDSCH scheduling by single DCI as a separate FG from 24-4 and 24-5.

- Conclusion

In this contribution, we focus on UE feature discussion for operation from 52.6-71GHz, and have the following proposals:

Proposal 1: FG 24-1b and 24-4b are only supported in bands for shared spectrum operation.

Proposal 2: For 24-1b, remove “A UE that supports FG 24-2 must indicate this FG is supported”.

Proposal 3: For 24-1c, remove “A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported”.

Proposal 4: For 24-1d, remove “FFS: to extend this FG to other frequency ranges”.

Proposal 5: List multi-PDSCH scheduling by single DCI as a separate FG from 24-4 and 24-5.

- References

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, Moderators (AT&T, NTT DOCOMO, INC.)

- TR 38.822 v16.0.0, User Equipment (UE) feature list, 2021-06.