3GPP TSG RAN WG1 #108-e		R1-2201128

E-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Discussion on UE features for MBS

Agenda Item:	8.16.12

Document for:	Discussion and Decision

- Introduction

In last meeting, Rel-17 NR UE features list has been discussed and updated in section 11 [1]. In this contribution, our views on MBS part are further provided based on newly updated outcome [1].

- Discussion

For 33-1, higher layer configured slot-level repetition has been agreed for MTCH, and up to 8 repetitions is supported. Considering it is not necessary to support a flexible number of repetitions from 1 to 8 for broadcast, it is better to support fixed values, i.e., {2, 4, 8} times repetitions.

Regarding DCI indicated slot-level repetition for MTCH, it should be a separated FG as agreed for multicast for better commonality.

Furthermore, multiple G-RNTIs facilitate different services, and thus, support of  G-RNTIs should be included in 33-1.

Proposal 1 For higher layer configured slot-level repetition in FG 33-1, UE is mandated to support {2, 4, 8} times repetitions.

Proposal 2 DCI indicated slot-level repletion is not included in FG33-1.

Proposal 3 ‘Support of  G-RNTIs’ is included in FG 33-1.

| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - Support of higher layer configured slot-level repetition for MTCH：{2,4,8} - Support of G-RNTIs for broadcast. FFS details of N |  | Yes | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- |

It has been agreed that components 6/7/8 are separated from 33-2 and added as new FGs by leaving how to merge PTM and/or PTP retransmission to HAR-ACK feedback as FFS. Considering there exists ambiguity between  unicast re-transmission and PTP retransmission for multicast when the same HPID is used, it is better to merge PTM retransmission only to HARQ-ACK feedback, but leave PTP retransmission as a separate UE capability.

Proposal 4 Support to merge 33-2c to 33-a.

| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling. - Support of PTM retransmission for multicast | 33-2 | Yes |  |  | Per UE | No | No |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast FFS whether to merge with 33-2a | 33-2a | Yes |  |  | Per UE | No | No |

For FG33-3-2 and FG33-3-3, the HARQ-ACK codebook should be separated from multiplexing of unicast PDSCH and group-common PDSCH, and similar as that in NR Rel-15, support of type 1 and type 2 codebook should be separated as different UE capabilities. Furthermore, as there is no difference between FDMed and TDMed type-2 codebok, it is not necessary to differentiate them, and thus, option 1 in [1] should be supported.

Proposal 5 Regarding HARQ-ACK codebook

- Add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

- Add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast

- Add an FG for Type-2 HARQ-ACK codebook for multicast

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1 or 33-2 | Yes | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2a | FDM-ed Type-1 HARQ-ACK codebook for multicast | - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. | 33-2, 33-3-2 | Yes | Optional with capability signalling |

| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1 or 33-2 | Yes | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-3a | TDM-ed Type-1 HARQ-ACK codebook for multicast | - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. | 33-2, 33-3-3, | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Type 2 HARQ-ACK codebook for multicast | - Support Type-2 HARQ-ACK codebook for multicast. | 33-2, 33-3-2 or 33-3-3 | Yes | Optional with capability signalling |

In RAN 1 #107bis meeting, it has been agreed that support of MCS/NDI/RV for TB2 is subject to UE capability, and thus, an FG can be added to reflect this.

Proposal 6 Add an FG for support of MCS/NDI/RV for TB2.

| 33. NR_MBS | 33-3-2e | Support two TBs for multicast | - Support of MCS/NDI/RV for TB2 | 33-2 | Yes | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- |

- Conclusion

In this contribution, we provide our further considerations on MBS part in the following proposals:

Proposal 1 For higher layer configured slot-level repetition in FG 33-1, UE is mandated to support {2, 4, 8} times repetitions.

Proposal 2 DCI indicated slot-level repletion is not included in FG33-1.

Proposal 3 ‘Support of  G-RNTIs’ is included in FG 33-1.

Proposal 4 Support to merge 33-2c to 33-a.

Proposal 5 Regarding HARQ-ACK codebook

- Add an FG for TDM-ed Type-1 HARQ-ACK codebook for multicast

- Add an FG for FDM-ed Type-1 HARQ-ACK codebook for multicast

- Add an FG for Type-2 HARQ-ACK codebook for multicast

Proposal 6 Add an FG for support of MCS/NDI/RV for TB2.

- Appendix

Agreement DCI format 4_2 doesn’t include the following fields: - Scell dormancy indication - BWP indicator DCI format 4_2 includes the following field (configurable): - MCS/NDI/RV for TB2 - Support of this field is subject to UE capability Agreement - A new component for support of higher layer configured slot-level repetition up to 8 for MTCH is added into FG 33-1 - FFS DCI indicated slot-level repetition up to [8 or 16] for MTCH is defined as another FG Agreement - Slot level repetition (FG 33-3-1) is merged with FG 33-2 - FFS whether to include semi-static repetition only or both semi-static and dynamic repetitions - FFS max value of dynamic repetition - FFS max value of semi-static repetition - Components 6/7/8 are separated from FG 33-2, FFS how to separate Agreement - Only the capability for semi-static repetition is included in FG 33-2 - For semi-static repetition, UE is mandated to support {2, 4, 8} times repetitions - For dynamic repetition, a separate FG is introduced; UE reports one of the maximum values from {8, 16} Agreement - Add following FGs - FG 33-2a: Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast - [FG 33-2c: PTM retransmission for multicast] - [FG 33-2d: PTP retransmission for multicast] 33. NR_MBS; 33-2a; Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling. At least 33-2c or 33-2d is merged, FFS which one or both; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2c; PTM retransmission for multicast; Support of PTM retransmission for multicast FFS whether to merge with 33-2a; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-2d; PTP retransmission for multicast; Support of PTP retransmission for multicast FFS whether to merge with 33-2a; 33-2a; Yes; Per UE; No; No; Optional with capability signalling. Agreement - Add an FG for support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot as follows 33-3-5; Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type; Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot; 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. Agreement - Add an FG for support of mode 1 for type1 codebook generation as follows 33. NR_MBS; 33-3-4; Mode 1 for type1 codebook generation; Supports type1-Codebook-Generation-Mode configured as mode 1; TBD; Yes; Per UE; No; No; Optional with capability signalling. Agreement - Add an FG for support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling as follows 33-4-1; DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-4; Yes; Per UE; Optional with capability signalling. Agreement - FG33-5-2 is not separated into multiple FGs - Supported maximum number M of activated SPS group-common PDSCH configurations is reported, FFS candidate values - UE can report one value from the candidate values. Agreement - Add an FG for supporting unicast PDCCH to release SPS group-common PDSCH as follows: 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling.

- References

- R1-2200258, Summary on UE features for NR MBS, Moderators (NTT DOCOMO, INC.)

- Chair’s notes, RAN1 #107bis