# 3GPP TSG RAN WG1 #108-e			R1-2201142

# e-Meeting, February 21st – March 3rd, 2022

Title:	Draft reply LS on PDCCH Skipping in RRC_CONNECTED

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:

E-mail Address:

- 1. Overall Description

RAN1 would like to thank RAN2 for the LS on PDCCH skipping. The following answers have been agreed according to RAN1 discussion.

Question 1: RAN2 would like to know whether PDCCH skipping is applied to RNTI(s) monitored during RAR/MsgB window for RAR/MsgB reception.

Answer 1: RAN1 agreed that PDCCH skipping is not applied to RNTI(s) monitored during RAR/MsgB window for RAR/MsgB reception. UE should monitor PDCCH during RAR/MsgB window.

Question 2: RAN2 would like to know if RAN1 prefers to capture the above agreements in RAN1 specification or prefer these to be captured in MAC specification.

Answer 2: RAN1 agreed to capture the agreements in RAN1 specification.

Question 3: RAN2 would like to ask RAN1 whether a) Physical layer of UE reports a value of 1 for Wake-up indication bit to higher layer or b) Physical layer of UE does not report Wake-up indication bit to higher layer, in case UE cannot monitor DCP due to PDCCH skipping; or if Approach 3 can be assumed.

Answer 3: UE needs to monitor DCP if the DCP occasion overlaps with PDCCH skipping duration.

Question 4: RAN2 would like to know whether UE should continue transmitting CSI/SRS during the PDCCH skipping duration or not?

Answer 4: RAN1 agreed that UE should continue transmitting CSI/SRS during the PDCCH skipping duration.

# 2. Actions:

RAN1 respectfully asks RAN2 to take above information into account in their work.

# 3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting #109-e 	 	16 – 27 May 2022				Electronic

TSG-RAN WG1 Meeting #110 		      22 – 26 August 2022			Toulouse, France