3GPP TSG RAN WG1 #108-e	R1-2201174

e-Meeting, February 21st – March 3rd, 2022

Source: 	ZTE

Title: 	Maintenance of Cross-Carrier Scheduling from SCell to PCell

Agenda item:	8.13.1

Document for:	Discussion/Decision

Introduction

During RAN#86 meeting, a new work item on DSS (dynamic spectrum sharing) was approved for Rel-17 [1]. One of the objectives of this WI is to support PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell. From RAN1#103-e, some progress were made for SCell scheduling PCell [2-8].

In this contribution, we provide our analysis on the remaining issues on cross-carrier scheduling from SCell to P(S)Cell.

BD/CCE handling

In the scope of Rel-17 DSS, there is a restriction that the total PDCCH blind decoding budget should not be changed as a result of this work. There are two PDCCH blind decoding budget, one is  or , the other is  or . For each scheduled cell, the UE is not required to monitor on the active DL BWP with SCS configuration  of the scheduling cell more than  PDCCH candidates or more than  non-overlapped CCEs per slot.

In Rel-16 CA, there is only one scheduling cell for a scheduled cell. And the BD/CCE limit on the scheduling cell for the scheduled cell is determined based on the numerology of the scheduling Cell. In Rel-17 DSS with support of SCell scheduling PCell, there are two scheduling cells (PCell and sSCell) for a scheduled cell (PCell), the remaining issues on BD/CCE handling based on Option A are analyzed in this section. Agreement in RAN1#106b-e meeting and RAN1 #107-e meeting are listed below.

[RAN1#106b-e] Agreement: Option A is supported in Rel-17 - At least for Type B UE, when the UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1),[and at least when UE is not provided monitoringCapabilityConfig for any cell, ] - Option A - On P(S)Cell (for self-scheduling) - UE is not required to monitor more than α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than [minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 or MPDCCHmax,slot,μ1] PDCCH BD candidates per sSCell slot - UE is additionally not required to monitor more than (1-α)*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - 0≤α≤1 is based on RRC configuration - α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ is used for P(S)Cell overbooking procedure - When determining MPDCCHtotal,slot,μ and MPDCCHtotal,slot,μ1 - P(S)Cell self-scheduling is counted by applying scaling factor s1 - sSCell to P(S)Cell scheduling is counted additionally (assuming SCS of sSCell) by applying scaling factor s2 - s1=1 and s2=0, FFS other s1 and s2 - 0≤s1≤1 and 0≤s2≤1 are based on RRC configuration - FFS: additional constraints on s1 and s2 e.g., 1 ≤ s1+s2 ≤ 2 or s1 + s2 ≤ 1 - Note: Ncellscap is as in Rel16 - UE capability/incapability indication for below to be discussed as part of UE features discussion - All search space configurations monitored on sSCell for cross-carrier scheduling to P(S)Cell are within a single span of [3] consecutive OFDM symbols within a duration spanning P(S)Cell slot - Same approach as above is used for CCE limits - FFS: Separate vs. same RRC configured scaling factors (corresponding to α) for BD and CCE limits. - When P(S)Cell SCS (μ) is larger than sSCell SCS (μ1), for CCS from sSCell to P(S)Cell and, it is not supported Rel-17 DSS.

[RAN1#107b-e] Agreement: If no additional set of (s1, s2) is introduced, - For Option A BD/CCE limit handling and (s1=1, s2=0) agreed in RAN1#106bis-e, down-select from - Option 2 - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than MPDCCHmax,slot,μ1 PDCCH BD candidates per sSCell slot Agreement - For Option A BD/CCE limit handling agreed in RAN1#106bis-e - Value of parameter α for BD limit handling is configured via RRC from a set of - 8 possible values Agreement - For Option A BD/CCE limit handling agreed in RAN1#106bis-e - For CCE limit handling (i.e., scaling of maximum number of non-overlapping CCEs) - same parameter α agreed for BD limit handling is used

FFS other s1 and s2

If a UE is configured with  downlink cells (Note, assume without M-TRP, that is = and =0, and also assume without span based monitoring) with DL BWPs of the scheduling cells using SCS configuration , where , a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than  PDCCH candidates or more than  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells.

In case that PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell is supported, PDCCH blind decoding will be performed on two cells for this P(S)Cell. In this case, how to calculate the number of P(S)Cell in parameter  is a new issue. For example, assume 5 cells are configured for a UE shown in Figure 1, wherein the SCS = 15kHz for PCell #1 and the SCS = 30kHz for sSCell #2. In Rel-15/Rel-16 case PCell cannot be scheduled by sSCell (the red arrow in Figure 1 is not presented), M_total_15khz = floor(4*44*1/5) = 35; M_total_30khz = floor(4*36*4/5) = 115. In case PCell can be scheduled by sSCell (the red arrow in Figure 1 is presented), PCell is counted as s1=1 cell and s2=0 cell for the two scheduling cells respectively, which is corresponding to the case 1 analyzed below.

Figure 1 CA with SCell-schedule-PCell

- Case 1: s1=1 and s2=0

      - The number of PCell is still counted as 1 only for PCell. This is aligned with the intention of the guidance proposed in RAN1#105-e that the M_total calculation is the same as in Rel-16. That is, M_total_15khz = floor(4*44*1/5) = 35;  M_total_30khz = floor(4*36*4/5) = 115.

      - In this case, the definition of M_total in the specification has been reflected that PCell is only counted into the cell number of M_total_μ0, where μ0 is the numerology for PCell; and is not counted into the cell number of M_total_μ1, where μ1 is the numerology for sSCell. To be more specific, the BD for each scheduling are listed in Table 1. This is aligned with Option A and the agreement in RAN1 #107-e which the total BD per PCell slot for p-p scheduling and s-p scheduling is same as that for PCell in Rel-16.

      - Table 1 BD for Rel-17 DSS and the BD budget is not changed compared with Rel-16

| - Scheduling | - PDCCH BDs |
| --- | --- |
| - P1->P1 | - b1<=α*35 per 1ms |
| - S2->S2 | - b2<=36 per 0.5ms such that b2+b3+b4+b5 <= 115 |
| - S2->S3 | - b3<=36 per 0.5ms such that b2+b3+b4+b5 <= 115 |
| - S2->S4 | - b4<=36 per 0.5ms such that b2+b3+b4+b5 <= 115 |
| - S2->S5 | - b5<=36 per 0.5ms such that b2+b3+b4+b5 <= 115 |
| - S2->P1 | - b6<=36 per 0.5ms; - 2*b6<= (1-α)*35 per 1ms |
|  | - total = (115 @30kHz) +35 @15kHz |

- Case 2: Other s1 and s2 values

      - The number of PCell is counted as s1 and s2 for the two scheduling cells (PCell and sSCell) respectively. For example, s1 = s2 = 0.5, M_total_15khz = floor(4*44*0.5/5) = 17; M_total_30khz = floor(4*36*4.5/5) = 129. For another example, s1 = 1 and s2 = 1, M_total_15khz = floor(4*44*1/6) = 29; M_total_30khz = floor(4*36*5/6) = 120.

      - If other s1 and s2 values are introduced, these cases will result changed BD budget compared with Rel-16. Furthermore, the second BD/CCE handling manner may be introduced for CA scaling and BD/CCE counting. For example, PCell shall be also counted into the cell number of M_total_μ1, where μ1 is the numerology for sSCell.  For another example, UE is not required to monitor more than minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 PDCCH BD candidates per sSCell slot on sSCell for cross-carrier scheduling to PCell.

      - Although we admit the flexible of case 2, it seems difficult to agree other s1 and s2 values at this late stage. So besides s1=1 and s2=0, other values are slightly preferred not to be introduced.

- Proposal 1: Besides s1=1 and s2=0, other values are not introduced.

Scaling factor values

Value of parameter α for BD limit handling is configured via RRC from a set of 8 possible values. Since based on Option A is adopted, and α + β = 1 is agreed that configure only α is enough and β = 1 - α. The candidates value of α can be [0.1, 0.2, 0.3, 0.4, 0.5] due to more PDCCH offloading to sSCell is preferred. Some companies comment that 4/8/16 CCEs should be on PCell for CSS monitoring, then 4/56 = 1/14, 8/56 = 1/7 or 16/56 = 1/4 should be supported. But it is dependent on gNB implementation to arrange the approximate CCE number such as α = 0.1, 0.2 or 0.3 to derived a approximate value to cover above cases.

α = 1 should be also supported to achieve fallback to Rel-16 PCell self-scheduling. It is useful for the case that sSCell is deactivated or dormancy by DCI or MAC CE, α is fallback to 1 without RRC reconfiguration. For the rest two candidates value, we are open to some values between 0.5 to 1, such as 0.6 and 0.8, to reserve more BD/CCE on PCell.

Since type A UE is agreed to reuse the BD/CCE handling of Type B UE, if the total BD/CCE budget is concerned for Type A UE, lower BD/CCE budget for Type A UE compared with Type B UE can be supported by separate configurations of scaling factor for the BD/CCE handling of Type A UE and Type B UE. That is α + β =1 for Type B UE, while α + β ≤ 1 for Type A UE. Otherwise, the complexity of BD/CCE handling of Type A UE and Type B UE are same.

- Proposal 2: 8 possible values for α can be {0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.8, 1}.

- Proposal 3: Separate configurations of scaling factor are applied for the BD/CCE handling of Type A UE and Type B UE. That is α + β =1 for Type B UE, while α + β ≤ 1 for Type A UE.

Search space and DCI formats handling

Type 3 CSS

In RAN1#103-e meeting and RAN1#106b-e meeting, the following conclusion was achieved. It was agreed that the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4, 2_6 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16.

Conclusion - When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16 - FFS: DCI format 2_5 and DCI Format 2_6 handling - Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’ Conclusion - When sSCell to PCell cross-carrier scheduling is configured, DCI format 2_6 (if configured) is monitored only on P(S)Cell

In RAN1#104-e meeting, the following agreement was achieved. It was agreed that UE monitors ‘DCI formats 0_0 and 1_0 in CSS that schedule PDSCH/PUSCH on PCell/PSCell’ only on the PCell/PSCell and not on the sSCell. That means Type 3 CSS with fallback DCI can be reused with no changes compared to Rel-15/Rel-16. In case Type 3 CSS is configured on SCell, fallback DCI in Type 3 CSS cannot be configured which is aligned with the above agreement. In case Type 3 CSS is configured on PCell, whether fallback DCI in Type 3 CSS can be configured or not depends on network implementation. More specifically, fallback DCI in Type 3 CSS can be kept on PCell/PSCell if configured. This can be also confirmed by the agreement for type A and type B UE in RAN1#105-e.

Agreement - When CCS from sSCell to PCell/PSCell is configured, UE monitors ‘DCI formats 0_0 and 1_0 in CSS that schedule PDSCH/PUSCH on PCell/PSCell’ only on the PCell/PSCell and not on the sSCell

Except for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4, 2_6 and fallback DCI (DCI format 1_0/0_0), DCI format 2_5 can also be associated with Type 3 CSS. The handling of remaining DCI format 2_5 in Type 3 CSS is analyzed below.

DCI format 2_5 is used for notifying the availability of soft resources for IAB-DU. Group common DCI format 2_5 in Type 3 CSS can be configured on PCell/PSCell or SCell, which is similar as DCI format 2_0/2_1/2_2/2_3/2_4. So there is no additional issues to decide regarding the location of Type 3 CSS with DCI format 2_5.

Proposal 4: For SCell scheduling PCell, Type 3 CSS with DCI format 2_5 can be reused with no changes compared to Rel-15/Rel-16, i.e., Type 3 CSS with DCI format 2_5 can be configured on PCell/PSCell or SCell.

USS sets for non-fallback DCI

In RAN1#105-e meeting, the following agreement was achieved on two types of UEs for Rel-17 DSS. It was agreed Type B UE with higher capability can monitor PDCCH for scheduling PCell from PCell and sSCell simultaneously. While Type A UE with lower capability can be specified or as Type B UE with restrictions.

Agreement: Two types of UEs (Type A and Type B) can support CCS from sSCell to P(S)Cell - For Type A UE - At least following search space sets on P(S)Cell and search space sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of P(S)Cell and sSCell - search space sets on P(S)Cell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported for Type A UE) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - search space sets on sSCell - USS set(s) for scheduling P(S)Cell - FFS: BD/CCE handling - For Type B UE - Following search space sets on P(S)Cell and search space sets on sSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of P(S)Cell and sSCell - search space sets on P(S)Cell - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - search space sets on sSCell - USS set(s) for scheduling P(S)Cell - For handling ‘USS sets for scheduling P(S)Cell’ on P(S)Cell and/or on sSCell for DCI formats 0_1,1_1,0_2,1_2 - Alt 2-1 is adopted - There is no restriction on Type-0/0A/1/2-CSS sets configurations - FFS: BD/CCE handling - For Type A and/or Type B UE - FFS: switching to ‘normal’ PDCCH monitoring on P(S)Cell when sSCell is deactivated - FFS: Whether Type A is specified or is Type-B with restrictions (as part of UE features discussion) - FFS: Whether the UE can be configured with unaligned CA - FFS: Whether the above applies for multicast PDSCH

For Type B UE, it was agreed that Alt 2-1 is adopted to handle USS sets for non-fallback DCI. That means type B UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously. Type A UE agreed in RAN1#105-e is almost based on Alt 2-4 that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously by gNB configuration. In case USS sets for DCI formats 0_1,1_1,0_2,1_2 on PCell is supported by Type A, it is similar for Type A UE and Type B UE to consider the issue of switching to ‘normal’ PDCCH monitoring on PCell when sSCell is deactivated which is only focus on recover the blind decode capability. In case USS sets with DCI formats 0_1,1_1,0_2,1_2 on PCell is not supported by Type A UE, some additional mechanism may be considered to recover non-fallback DCI formats when sSCell is deactivated. Since TDM monitoring on the two scheduling cells is supported by Type A UE only except PDCCH monitoring for SIB/RAR/Paging, that is not monitoring DCI formats for unicast simultaneously, no matter fallback DCI or non-fallback DCI. Type A UE with support of USS sets for DCI formats 0_1,1_1,0_2,1_2 on PCell is preferred. Note in case of sSCell is deactivated/dormant or sSCell-scheduling-PCell is disabled, how to recover non-fallback DCI or BD/CCE budget is discussed in section 4.2.

Proposal 5: Type A UE supports USS sets for DCI formats 0_1,1_1,0_2,1_2 on PCell.

General issue

Size alignment issue

In case that PDCCH on sSCell can be used to schedule PDSCH or PUSCH on P(S)Cell, P(S)Cell should be additional configured as a scheduled cell of the sSCell. CIF for PCell scheduled by sSCell is explicitly configured according to the agreements reached in RAN1#104b-e meeting as listed below. And in RAN1#107-e meeting, how to use CIF for PCell self-scheduling is determined by the following agreement.

Agreement - When CCS from sSCell to PCell/PSCell is configured - CIF=0 used for sSCell self-scheduling, and CIF for sSCell to PCell cross-carrier scheduling is explicitly configured using RRC signalling

Agreement - Confirm the WA from RAN1#106bis-e with addition of below Note (shown in blue) Working Assumption - When CIF for sSCell to Pcell cross-carrier scheduling is configured, non-fallback DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling - Note: per RAN1#102-e agreement, when sSCell to P(S)Cell scheduling is configured for the UE, cross-carrier scheduling from P(S)Cell to another cell is not allowed. The CIF bits included in non-fallback DCI formats on P(S)Cell are considered reserved.

The remaining issue is how to deal with fields other than CIF. There are three options discussed in RAN1 #107-e meeting and below ‘generic’ approaches for size matching are derived.

  - Option 1

    - If the DCI on the Pcell includes X bits and the DCI on the sSCell includes Y bits, |X-Y| bits are padded to the DCI with the smaller size.

  - Option 2

    - When sSCell to P(S)Cell scheduling is configured for the UE, for a given non-fallback DCI format, the DCI fields and field sizes used for P(S)Cell self-scheduling are used also for sSCell to P(S)Cell cross-carrier scheduling

      - Note: per RAN1#106bis WA – “DCI formats on P(S)Cell include same number of CIF bits as the corresponding non-fallback DCI formats on sSCell that are used for sSCell to P(S)Cell scheduling”

  - Option 3

    - Change the definition of DCI size budget to (scheduling cell, scheduled cell) pair

First, option 3 is not preferred due to the DCI size budget is defined for each scheduled cell and there is still one scheduled cell in Rel-17 DSS. Option 1 and Option 2 can both work, Option 1 is simple but anyway the following special fields should be discussed, such as whether DCI formats with SCell dormancy indication configured on PCell can be used on sSCell for scheduling PCell. Option 2 is already agreed for CIF, it can be also applied for other fields because there are only few fields with the same issue.

  - Scell dormancy Indication

  - Transmission configuration indication

  - “1 bit padding bit in DCI 0_1 and DCI 1_1 according to DCI size alignment procedure step 2 when the size of DCI 0_1 or DCI 1_1 in a USS is equal to the size of DCI 0_0/1_0 in another USS”

For the Scell dormancy Indication in non-fallback DCI, it is only present when this format is carried by PDCCH on the primary cell. In case Rel-17 DSS is configured, the off-loading PDCCH from PCell to sSCell is achieved. Then non-fallback DCI formats on sSCell includes same number of the SCell dormancy indication bits as the corresponding non-fallback DCI formats on PCell that are used for PCell self-scheduling. Note how to deal with the sSCell is indicated as dormancy is another issue because this is a common issue no matter the corresponding DCI is from PCell or sSCell. The intention is ensure the same size for the same DCI format for p-p scheduling and s-p scheduling.

For the Transmission configuration indication, it is definitely dependent on the scheduling cell configuration. TCI size in the same DCI format on PCell and sSCell for both scheduling PCell can be configured same, or using Option 1/2 to derive the size of this field. But anyway it is not an important issue because Rel-17 DSS are within FR1.

For the 1 bit padding in step 2, it is not an issue if the DCI format 0_1 or 1_1 with only one size on PCell and sSCell for scheduling PCell.

Above all, for the same DCI format on sSCell used for schedulng PCell, if SCell dormancy indication can be present and TCI size in the same DCI format can be same with that on PCell for self-scheduling, Option 2 is preferred. Otherwise, Option 1 is applied. We slightly prefer the Option 2 due to that it is a simpler way as CIF which had been agreed.

Proposal 6: When sSCell to P(S)Cell scheduling is configured for the UE, for a given non-fallback DCI format, the DCI fields and field sizes used for P(S)Cell self-scheduling are used also for sSCell to P(S)Cell cross-carrier scheduling (Option 2).

sSCell activation/deactivation and dormancy

In Rel-16, any SCell can be deactivated or switched to dormant BWP. In Rel-17 DSS with support of SCell scheduling PCell, sSCell activation/deactivation operation was agreed in RAN1#104-e meeting listed below.

Agreement - When CCS from sSCell to PCell/PSCell is configured, CA activation/deactivation operation for the sSCell is supported

Dormant BWP operation on sSCell was also agreed in RAN1#106-e meeting listed below.

Agreement Specification supports dormant BWP operation on sSCell for a UE is configured CCS from sSCell to P(S)Cell.

In RAN1#107-e meeting, the following is proposed by FL without result.

- Following is supported - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X1 of P(S)Cell when sSCell deactivation is triggered in slot n of P(S)Cell - Scaling factor α (from RAN1#106bis-e agreement) is not applied for PDCCH monitoring on P(S)Cell from slot n+X2 when sSCell BWP switch to dormant BWP is triggered in slot n of P(S)Cell - X1 = [BWP switching delay specified for P(S)Cell SCS]; X2 = [BWP switching delay specified for P(S)Cell SCS];

For Type A UE without support of non-fallback DCI on PCell when sSCell-schedule-PCell is configured, sSCell can be deactivated or dormant at the sacrifice of system performance because only fallback DCI can be used to schedule PDSCH/PUSCH on PCell. Meanwhile, sSCell cannot be indicated to dormancy by non-fallback DCI because there is no non-fallback DCI on PCell. Because CA activation/deactivation operation and dormant BWP operation for the sSCell are already supported, maybe a supplement mechanism is needed, i.e. USS with non-fallback DCI on PCell for self-scheduling is implicitly enabled/activated when sSCell is deactivated/dormant.

For any Type A UE with/without support of non-fallback DCI on PCell or Type B UE, in case sSCell is deactivated or switched to dormant BWP, BD/CCE budget on PCell should be restored to the Rel-15/Rel-16 budget. Otherwise, using the one of the hard split BD/CCE budget for PCell self-scheduling is inefficient.

Above all, no matter type A UE or type B UE, the proposal is useful. And Scaling factor α (from RAN1#106bis-e agreement) is not applied means α = 1 which is same as Rel-16 PCell self-scheduling.

Proposal 7: For Type A UE not supporting non-fallback DCI on PCell, automatic recovery of USS with non-fallback DCI scheduling on PCell is achieved by not applying α or applying α = 1 once sSCell is deactivated or switched to dormant BWP.

Proposal 8: For Type A UE or Type B UE, automatic recovery of BD/CCE budget on PCell to the Rel-15/Rel-16 budget is achieved by not applying α or applying α = 1 once sSCell is deactivated or switched to dormant BWP.

Conclusion

In this contribution, the remaining issues on cross-carrier scheduling from SCell to P(S)Cell are provided with the following proposals.

Proposal 1: Besides s1=1 and s2=0, other values are not introduced.

- Proposal 2: 8 possible values for α can be {0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.8, 1}.

Proposal 3: Separate configurations of scaling factor are applied for the BD/CCE handling of Type A UE and Type B UE. That is α + β =1 for Type B UE, while α + β ≤ 1 for Type A UE.

Proposal 4: For SCell scheduling PCell, Type 3 CSS with DCI format 2_5 can be reused with no changes compared to Rel-15/Rel-16, i.e., Type 3 CSS with DCI format 2_5 can be configured on PCell/PSCell or SCell.

Proposal 5: Type A UE supports USS sets for DCI formats 0_1,1_1,0_2,1_2 on PCell.

Proposal 6: When sSCell to P(S)Cell scheduling is configured for the UE, for a given non-fallback DCI format, the DCI fields and field sizes used for P(S)Cell self-scheduling are used also for sSCell to P(S)Cell cross-carrier scheduling (Option 2).

Proposal 7: For Type A UE not supporting non-fallback DCI on PCell, automatic recovery of USS with non-fallback DCI scheduling on PCell is achieved by not applying α or applying α = 1 once sSCell is deactivated or switched to dormant BWP.

Proposal 8: For Type A UE or Type B UE, automatic recovery of BD/CCE budget on PCell to the Rel-15/Rel-16 budget is achieved by not applying α or applying α = 1 once sSCell is deactivated or switched to dormant BWP.

Reference

RP-193260, New WID on NR Dynamic spectrum sharing (DSS), RAN #86.

RAN1#103-e meeting, chairman notes.

RAN1#104-e meeting, chairman notes.

RAN1#104b-e meeting, chairman notes.

RAN1#105-e meeting, chairman notes.

RAN1#106-e meeting, chairman notes.

RAN1#106b-e meeting, chairman notes.

RAN1#107-e meeting, chairman notes.