3GPP TSG RAN WG1 #108-e	R1-2201180

e-Meeting, February 21st – March 3rd, 2022

Title: 	Discussion on Rel-17 UE features for LTE based 5G terrestrial broadcast

Source: 	ZTE

Agenda item:	8.16.15

Document for:	Discussion/Decision

Introduction

After RAN1#107-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 LTE [1]. In this contribution, we present our analysis and views on Rel-17 UE features for LTE based 5G terrestrial broadcast.

Discussion

The only remaining issue for FG3-1 is whether to define separate components for different bandwidths.

| 3. LTE_terr_bcast_bands_part1 | 3-1 | Support of new channel bandwidth for PMCH | [TBD: whether separate components are needed for different bandwidths] | Support of dedicated MBMS cells | Yes |  | UE cannot receive MBMS in the corresponding MBSFN area | Per band | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

This issue has been extensively discussed in RAN1#107-e meeting [2]. If separate components are defined for different bandwidths, it may end up with that different UEs support different bandwidths, which may complicate the network deployment. However, if no separate component for different bandwidths, UEs may have to implement some other bandwidths even if one certain bandwidth may not be used in a certain region.

It seems the following two approaches are possible. We propose to further discuss the following two approaches.

Proposal 1: RAN1 further discusses the following two approaches.

Approach 1)

- RAN1 defines separate UE capabilities for 6M, 7M and 8M

- RAN1 indicates to RAN4 that, UE has to indicate support for all the allowed bandwidths among 6MHz, 7MHz and 8MHz for PMCH for a certain band for a certain region

Approach 2’)

- RAN1 doesn't define separate UE capabilities for 6M, 7M and 8M

- RAN1 informs RAN4 of the RAN1's UE capability design. RAN4 can then proceed with their work, including potential refinement of related UE capability design, if needed.

Conclusion

In this contribution, we discuss and analyse Rel-17 UE features for LTE based 5G terrestrial broadcast with the following proposal.

Proposal 1: RAN1 further discusses the following two approaches.

Approach 1)

- RAN1 defines separate UE capabilities for 6M, 7M and 8M

- RAN1 indicates to RAN4 that, UE has to indicate support for all the allowed bandwidths among 6MHz, 7MHz and 8MHz for PMCH for a certain band for a certain region

Approach 2’)

- RAN1 doesn't define separate UE capabilities for 6M, 7M and 8M

- RAN1 informs RAN4 of the RAN1's UE capability design. RAN4 can then proceed with their work, including potential refinement of related UE capability design, if needed.

Reference

- R1-2112900, Updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e, RAN1#107-e,	Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2112146, Summary on UE features for LTE based 5G terrestrial broadcast,	RAN1#107-e, Moderator (NTT DOCOMO, INC.)