3GPP TSG RAN WG1 Meeting #108-e	R1-2201186

e-Meeting, February 21st – March 3rd, 2022

Source:	ZTE

Title:	Remaining issues on multi-TRP enhancements for PDCCH, PUCCH, and PUSCH

Agenda Item:	8.1.2.1

Document for:	Discussion and Decision

Introduction

For NR MIMO in Rel-17, the basic aspects on multi-TRP enhancements for PDCCH, PUCCH and PUSCH have been fully discussed. However, there are still some leftover issues. In this contribution, we provide our views for the remaining issues on these Rel-17 enhancement aspects.

Remaining issues on multi-TRP enhancements for PDCCH, PUCCH and PUSCH in Rel-17

Remaining issues on PDCCH enhancement

In Rel-17, MTRP PDCCH repetition scheme has been supported by using RRC signal linking two PDCCH candidates to form PDCCH repetitions which comprise of the same DCI contents. Through the great progress in the discussion before, most of the ambiguous issues have been addressed. However, some of them are still open and needed to be discussed. In this section, we provide our views on some remaining consideration of PDCCH repetition.

    - SS set group

In RAN1#107-e meeting, the issue on SS set group of the two linked SS sets was discussed by companies. In Rel-16, searchSpaceGroupIdList can be configured per SS set which determines the group or groups that the SS set belongs to. This is used for SS set group switching mechanisms. The network configures at most 2 search space groups per BWP where the group ID is either 0 or 1, and each SS set can belong to one or both of these groups. The two SS set groups are never monitored simultaneously, but it is possible that a SS set is always monitored if it belongs to two groups.

In order to keep the configuration flexibility of gNB, there should be no restriction of SS set group for the two linked SS sets. That is one SS set of the two linked SS sets can be in any one or two of the two SS set group. The two SS sets are linked only when they belong to same SS group.

Proposal 1-1: There is no restriction of SS set group for the two linked SSes. The two SS sets are linked only when they belong to same SS set group.

    - Two linked SSes in two serving cells

To further improve the reliability of PDCCH, it should be supported to link two SS sets in different serving cells.  For example, a first SS set in serving cell 1 and a second SS set in serving cell 2 can be linked. It will improve the reliability of PDCCH when considering frequency selection and interference difference between different serving cells.

Proposal 1-2: It should be supported that two linked SS sets are in two serving cells.

Remaining issues on PUCCH enhancement

Overall, MTRP PUCCH repetition scheme in Rel-17 is to use MAC CE activating two spatial relations (in FR2) or two power control parameter sets (in FR1) for one PUCCH resource which towards to two TRPs. As of now, most of the essential functionalities can be workable and have been captured in the current draft specifications. In this section, we provide our views and TPs on the remaining issues of Rel-17 MTRP PUCCH repetition scheme.

    - Beam mapping/ power control parameter set mapping

In RAN1#104bis-e meeting, the following agreement has been endorsed to specify either cyclic mapping or sequential mapping for beam mapping/ power control parameter set mapping in Rel-17 MTRP PUCCH repetition, plus both inter-slot (scheme 1) and intra-slot (scheme 3) based manners have been supported [1].

Agreement Confirm the following Working Assumption (with small correction of typo and clarification on UE capability in RED): - For beam mapping /power control parameter set mapping for PUCCH repetitions, - For M-TRP PUCCH Scheme 1 in FR1, it is possible to configure either cyclic mapping or sequential mapping of power control parameter sets over PUCCH repetitions (similar to spatial relation info’s over PUCCH repetitions). - For M-TRP PUCCH Scheme 3, reuse the same methods as Scheme 1 (by replacing slots with sub-slots) for beam mapping or power control resource parameter set mapping to sub-slots. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

In RAN1#104-e meeting, the following agreement has been endorsed to support all PUCCH formats for inter-slot (scheme 1) based MTRP PUCCH repetition  [2].

Agreement For M-TRP PUCCH scheme 1, - Support PUCCH formats 0 and 2 (in addition to agreed PUCCH formats 1,3,4)

In RAN1#105-e meeting, the following agreement has been endorsed to support all PUCCH formats for intra-slot (scheme 3) based MTRP PUCCH repetition [3].

Agreement Confirm the working assumption with removing brackets on [consecutive] and adding UE capability. - For PUCCH reliability enhancement, support multi-TRP intra-slot repetition (Scheme 3) for all PUCCH formats. - The same PUCCH resource carrying UCI is repeated for X = 2 [consecutive] sub-slots within a slot. - Refer the design details related to sub-slot configurations (e.g. other values of X) to Rel-17 eIIoT - Note1: The decision of supporting scheme 3 is only applicable for multi-TRP operation. - This feature is optional.

In light of the above, the specified way of beam mapping/ power control parameter set mapping should be used for all PUCCH formats when Rel-17 MTRP PUCCH repetition schemes. Besides, one case is missing that when the number of repetitions is equal to 2 and sequential mapping is configured. Although this case cannot implement PUCCH repetition towards to multiple TRPs in reality, it still can be configured by gNB according to RAN1’s discussion so far.

Consequently, we have the following text proposal. In addition, note that the description of intra-slot (scheme 3) based PUCCH repetition has not been edited in the current specification, the similar modifications should be captured correspondingly.

TP 1: {38.213: 9.2.6 PUCCH repetition procedure}

<Unchanged part omitted> When a PUCCH resource with one of PUCCH formats 0, 1, 2, 3 or 4 used for repetitions of a PUCCH transmission by a UE includes first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clause 7.2.1, the UE - uses the first and second spatial settings, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2 if mappingPattern = ‘cyclicMapping’, - uses the first spatial setting or the first set of power control parameters for first and second repetitions for the PUCCH transmission when NPUCCHrepeat=2 if mappingPattern = ‘sequentialMapping’, - alternates between the first and second spatial settings, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = ‘cyclicMapping’; else, NPUCCHswitch=2. <Unchanged part omitted>

Remaining issues on PUSCH enhancement

Overall, MTRP PUSCH repetition scheme in Rel-17 is to use RRC configuring two SRS resource sets for PUSCH repetitions towards to two TRPs. So far, most of the essential functionalities can be workable and have been captured in the current draft specifications. In this section, we provide our views and TPs on the remaining issues of Rel-17 MTRP PUSCH repetition scheme.

    - SRS resource set configuration

In RAN1#107-e meeting, the following agreement has been endorsed to specify the configured value of NSRS,0_2 in two SRS resource sets should be the same when MTRP PUSCH repetition scheduled by DCI format 0_2, which needs to be captured in the specifications [4]. Based on the current draft CR on 38.212 NR FeMIMO, the “Note 2” in Table 7.3.1.1.2-36 describes the configured value of NSRS,0_2 related rule. Hence this agreement should be captured in this note.

Agreement For mTRP PUSCH repetition scheduled with DCI format 0_2, the value of the NSRS,0_2 in two SRS resource sets configured by higher layer parameter rs-ResourceSetToAddModListDCI-0-2 should be the same.

Based on the above elaborations, we have the following text proposal.

TP 2: {38.212: 7.3.1.1.2 Format 0_1}

<Unchanged part omitted> Table 7.3.1.1.2-36: SRS resource set indication Bit field mapped to index; SRS resource set indication. 0; SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved.. 1; SRS resource indicator field and Precoding information and number of layers field are associated with the second SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved.. 2; SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set.. 3; SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set.. NOTE 1: The first and the second SRS resource sets are respectively the ones with lower and higher srs-ResourceSetId of the two SRS resources sets configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'nonCodeBook' if txConfig=nonCodebook or 'codeBook' if txConfig=codebook. When only one SRS resource set is configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook' respectively, the first SRS resource set is the SRS resource set. The association of the first and second SRS resource sets to PUSCH repetitions for each bit field index value is as defined in Clause 6.1.2.1 of [6, TS 38.214]. NOTE 2: For DCI format 0_2, the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 are composed of the first NSRS, 0_2 SRS resources together with other configurations in the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', respectively, except for the higher layer parameters ‘srs-ResourceSetId’ and ‘srs-ResourceIdList’. Where, the value of NSRS, 0_2 in two SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 is the same.. <Unchanged part omitted>

In RAN1#107-e meeting, the following agreement has been endorsed to specify the association/limitation of SRS resource set configuration of DCI format 0_1 scheduling and SRS resource set configuration of DCI format 0_2 scheduling, which includes both CB and NCB based MTRP PUSCH repetition schemes [4]. Correspondingly, this feature should be captured in the specification.

Conclusion For mTRP PUSCH repetition scheduled with DCI format 0_2, when only one SRS resource set is configured in srs-ResourceSetToAddModListDCI, the UE is not expected to configure with two SRS resource sets in srs-ResourceSetToAddModList-0-2.

Based on the above elaborations, we have the following text proposals.

TP 3: {38.214: 6.1.1.1 Codebook based UL transmission}

<Unchanged part omitted> When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’, one or two SRI(s), and one or two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. The UE applies the indicated SRI(s) and TPMI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. Each TPMI, based on indicated codepoint of SRS Resource Set indicator, is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set, or if a single SRS resource is configured for the applicable SRS resource set TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to the higher layer parameter nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two SRIs are indicated, the UE shall expect the nrofSRS-Ports for the two indicated SRS resources to be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. When two SRS resource sets are configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’, the UE is not expected to be configured with different value of NSRS,0_2 in the two SRS resource sets. For non-codebook based PUSCH transmission(s) scheduled by DCI format 0_2, if only one SRS resource set is configured in srs-ResourceSetToAddModList, the UE is not expect to be configured with two SRS resource sets in srs-ResourceSetToAddModListDCI-0-2. <Unchanged part omitted>

TP 4: {38.214: 6.1.1.2 Non-Codebook based UL transmission}

<Unchanged part omitted> For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder(s) and transmission rank based on the SRI(s) when multiple SRS resources are configured, where the SRI(s) is given by the one or two SRS resource indicator(s) in DCI according to clause 7.3.1.1.2 and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3, or two SRIs given by srs-ResourceIndicator and srs-ResourceIndicator2 according to clause 6.1.2.3.. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. Only one SRS port for each SRS resource is configured. Only one or two SRS resource sets can be configured in srs-ResourceSetToAddModList with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook', and only one or two SRS resource sets can be configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook’, one or two SRIs are given by the DCI fields of two SRS resource indicators in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2. The UE applies the indicated SRI(s) to one or more PUSCH repetitions according to the associated SRS resource set of a PUSCH repetition according to clause 6.1.2.1. The maximum number of SRS resources per SRS resource set that can be configured for non-codebook based uplink transmission is 4. Each of the indicated one or two SRIs in slot n is associated with the most recent transmission of SRS resource(s) of associated SRS resource set identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook’, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. When two SRS resource sets are configured in srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook’, the UE is not expected to be configured with different value of NSRS,0_2 in the two SRS resource sets. For non-codebook based PUSCH transmission(s) scheduled by DCI format 0_2, if only one SRS resource set is configured in srs-ResourceSetToAddModList, the UE is not expect to be configured with two SRS resource sets in srs-ResourceSetToAddModListDCI-0-2. <Unchanged part omitted>

    - Beam mapping

In RAN1#104bis-e meeting, the following agreement has been endorsed to specify both sequential mapping and cyclic mapping for Rel-17 MTRP PUSCH repetition scheme [1].

Agreement Confirm the following working assumption (with removing the last bullet): For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. FFS: Support of half-half mapping.

Similar to MTRP PUCCH enhancement in section 2.2.1, one case is missing that when the number of repetitions is equal to 2 and sequential mapping is configured. Besides, for single-CG based MTRP PUSCH repetition scheme, this case also should be captured, the only differentiation is that all CG based PUSCH repetitions should be associated with the first SRS resource set in such case. Hence we have the following text proposals to capture the above missing case.

TP 5: {38.214: 6.1.2.1 Resource allocation in time domain}

<Unchanged part omitted> When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’ or ‘noncodebook’, for PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, and the association of the first and second SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 to each slot is determined as follows: - if a DCI format 0_1 or DCI format 0_2 indicates codepoint “00” for the SRS resource set indicator, the first SRS resource set is associated with all K consecutive slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint “01” for the SRS resource set indicator, the second SRS resource set is associated with all K consecutive slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint “10” for the SRS resource set indicator, the first and second SRS resource set association to K consecutive slots is determined as follows: - When K = 2 and cyclicMapping in PUSCH-Config is enabled, the first and second SRS resource sets are applied to the first and second slot of 2 consecutive slots, respectively. - When K = 2 and sequentialMapping in PUSCH-Config is enabled, the first SRS resource set is associated with all K consecutive slots. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the first and second SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, first SRS resource set is applied to the first and second slots of K consecutive slots, and the second SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots. - Otherwise, a DCI format 0_1 or DCI format 0_2 indicates codepoint “11” for the SRS resource set indicator, and the first and second SRS resource set association to K consecutive slots is determined as follows, - When K = 2 and cyclicMapping in PUSCH-Config is enabled, the second and first SRS resource set are applied to the first and second slot of 2 consecutive slots, respectively. - When K = 2 and sequentialMapping in PUSCH-Config is enabled, the second SRS resource set is associated with all K consecutive slots. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the second and first SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, the second SRS resource set is applied to the first and second slot of K consecutive slots, and the first SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots. <Unchanged part omitted>

TP 6: {38.214: 6.1.2.3 Resource allocation for uplink transmission with configured grant}

<Unchanged part omitted> For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberOfRepetitions is present in the table; otherwise K is provided by the higher layer configured parameters repK. For PUSCH transmissions with a Type 2 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, the SRS resource set association to (nominal) repetitions follows MappingPattern in ConfiguredGrantConfig as defined in Clause 6.1.2.1 for PUSCH scheduled by DCI format 0_1 and 0_2. For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if two SRS resource indicators and two precoding information are provided, the SRS resource set association to (nominal) repetitions is determined as follows. When K = 2 and cyclicMapping in ConfiguredGrantConfig is enabled, the first SRS resource set is applied to the first and second (nominal) repetition. When K = 2 and sequentialMapping in ConfiguredGrantConfig is enabled, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively. - When K > 2 and cyclicMapping in ConfiguredGrantConfig is enabled, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. - When K > 2 and sequentialMapping in ConfiguredGrantConfig is enabled, first SRS resource set is applied to the first and second (nominal) repetitions, and the second SRS resource set is applied to the third and fourth (nominal) repetitions, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. <Unchanged part omitted>

Conclusions

In this contribution, we analyze the remaining issues on Rel-17 multi-TRP enhancements for PDCCH/ PUCCH/ PUSCH. Overall, two proposals of MTRP PDCCH are provided as follows, and six text proposals of MTRP PUCCH/ PUSCH are given in Sections 2.2 and 2.3.

For PDCCH

Proposal 1-1: There is no restriction of SS set group for the two linked SSes. The two SS sets are linked only when they belong to same SS set group.

Proposal 1-2: It should be supported that two linked SS sets are in two serving cells.

References

Chairman's Notes RAN1#104-e final

Chairman's Notes RAN1#104b-e final

Chairman's Notes RAN1#105-e final

Chairman's Notes RAN1#107-e v18