3GPP TSG RAN WG1 Meeting #108-e                                      R1-2201199

e-Meeting, February 21th – March 3rd, 2022

Source:         ZTE

Title:            UE features for feMIMO

Agenda Item:    8.16.1

Document for:   Discussion and Decision

Introduction

The RAN1 features developed in NR Rel-17 has become more and more stable. After RAN1#107-e, an updated list of UE features is given in [1]. We discuss the remaining open issues in this updated list for feMIMO.

Multi-beam enhancements (23-1)

Four UE features (23-1-1~4) are developed in [1] for Rel-17 beam management, and then we discuss some further details for 23-1 series in this section, especially for the missing new sub-features.

23-1-1	Unified TCI for intra- and inter-cell beam management

Firstly, in the main feature, we need to explicit that the following components should be supported as part of the basic features in 23-1-1:

- Component-1: Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals)

- Component-3: For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS

- Component-4: The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination]

- Component-5-a: The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination]

  - Note: For facilitating unified TCI framework (as an enhancement over legacy Rel-15/16 unified TCI), the UE should support at least 2 activated TCI states per CC mandatorily.

- Component-6: The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS

- Component-8: TCI state indication [mode]: update and activation [in case of updates]
a) MAC CE based TCI state indication [for one active TCI state]
b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment)
c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment)

- Component-11: Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH, and SRS reusing the Rel-15/16 signaling/configuration design(s)

Then, the following functionalities should be supported as part of the basic feature for UE supporting CA in 23-1-1:

- Component-2 Common multi-CC TCI update and activation

- Component-9 Reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC

After that, we have the following comments for other potential UE feature components captured by the moderator.

- Regarding Component-6&14(BAT time), in our views, the Component-6 is sufficient and also aligned with already agreement, but we fail to identify the necessity of Componet-14 and legacy UE feature for PDSCH beam switching can be reused herein.

- Regarding Component 10, we fail to understand the necessity of this component, and it can be well represented by Rel-16 existing UE feature signaling.

- Regarding Component-9, 12, 13 (common TCI state pool), there are several candidate descriptions for Component-13, but in our views, the current description in component-9 and 12 is sufficient and also aligned with already agreement. The further wording-polish for above two components may not be needed. Then, Component 9 can be can made as a prerequisite feature for Component 12.

Proposal 1: For unified TCI for intra- and inter-cell beam management, the following modification in red is proposed.

- Components-{1, 3, 4, 5a, 6, 8, 11} should be supported as a basic feature.

- Components-{2, 9} should be further supported as a basic feature for UE supporting CA.

| 23-1-1 | Unified TCI [with joint DL/UL TCI update] for intra- [and inter-cell] beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) - Common multi-CC TCI update and activation - For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS - The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - The maximum number of MAC-CE activated joint TCI states across all CCs [in a band] [in a band combination] a) The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - Reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC Note: agree component, final wording may change (e.g., when this is merged with other components/FGs) - Maximum number of CCs configured with BFR FFS whether this is a component or just a note in the FG to reuse R16 signaling - Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH, and SRS reusing the Rel-15/16 signaling/configuration design(s) Note: This has no impact on detail signaling design for SRS TCI indication - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band [in a band combination] FFS: Whether to Note: To make component 9 a prerequisite or merge with 9 - [Alt. 1: [The maximum number of PDSCH-Configs containing TCI states that can referred to from a PDSCH-Config without TCI states Alt. 2: Support PDSCH-Config which contains a reference to another CC/BWP, in which the PDSCH-Config contains the TCI state list] [14. The minimum time gap between the beam indication PDCCH and first slot where beam is applied] |
| --- | --- | --- |

23-1-2	Inter-cell measurement and reporting (for inter-cell BM and mTRP)

Firstly, regarding PCI(s) different from serving cell PCI, we prefer to reuse the legacy UE feature of [2-24] SSB/CSI-RS for beam measurement as a starting point.

Then, based on already agreement, the maximum value of supported K is a UE capability, and then the corresponding candidate value should be provided, e.g., {1, 2, 4} as legacy beam reporting.

After that, we have the following comments for other potential UE features captured by the moderator.

- Regarding the basic feature, in our views, component 1~5 can be considered. To be more specific, the component-3 should be considered as the maximum number of RRC-configured PCI(s) different from serving cell PCI for L1-RSRP measurement per CC in a band, and thus this feature can report individual values for FR1 and FR2. Then, this parameter is just relevant to RRC CSI-SSB resource set configuration, and we do not need to consider time domain behavior for measurement/report.

- Regarding component 6 (relationship between L1 and L3 measurement), we think that, based on the gNB configuration for this L1 inter-cell measurement (as intra-cell beam measurement), we do not identify the relationship.

- Regarding component 7 and 8 (configured PCI for transmission), we think that these two parameters should be discussed in FG 23-4.

- Regarding component 9, 10, and 11, we need to wait for the RAN4 reply LS, or if needed, RAN4 can request some related UE feature to RAN2 directly.

- Regarding component 12 and 13, we have the similar requirement for FG 23-5 group based reporting, and in our views, due to the same requirement, the Rel-16 UE FG 16-1g/1g-1 can be refined rather than introducing new one(s).

Proposal 2: For inter-cell measurement and reporting (for inter-cell BM and mTRP), the following modification is proposed in red

- Components-{1, 2, 3, 4, 5} should be supported as a basic feature.

| 23-1-2 | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI 2. Support of up to K[=4] SSBRI-RSRP [pairs/beams] in one report [where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported] Candidate value comprises {1,2,4} 3. The maximum number of [RRC-configured] PCI(s) different from serving cell PCI for L1-RSRP measurement] per CC in a band (FFS: whether to split this for FR1 and FR2) (FFS: whether/how to capture different values/behaviors for periodic/aperiodic/semi-persistent L1-RSRP measurement) [4. The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI [across all CC]] [5. The max number of SSB resources configured to measure L1-RSRP with PCI(s) same as or different from serving cell PCI [across all CC]] [6. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] [7. The maximum number of configured additional PCIs is X1 when time domain positions and periodicity of configured SSBs with additional PCIs are the same as time domain positions and periodicity of the serving cell SSBs] [8. The maximum number of configured additional PCIs is X2 when time domain positions and periodicity of configured SSBs with additional PCIs are different with time domain positions and periodicity of the serving cell SSBs] [9. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC}] [10. Supported mode of measurement over overlapped SSBs: {overlapped, both overlapped and non-overlapped}] [11. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] [12. The maximum total number of SSB/CSI-RS/CSI-IM resources configured to measure within a slot across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g)] [13.[The maximum total number of SSB/CSI-RS/CSI-IM resources configured across all CCs in one frequency range for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, RLM and new beam identification for both intra-cell and inter-cell measurement (Similar to FG 16-1g)] |
| --- | --- | --- |

23-1-3	MPE mitigation

Based on the following agreement, the N P-MPR values and corresponding SSBRI(s)/CRI(s) can be reported. In our views, M should be fixed to 1, and if sharing the same MPE impacts, the UE can provide same P-MPR values for different SSBRI(s)/CRI(s), respectively, in a report instance. Therefore, a new sub-feature of maximum number of P-MPR and SSBRI/CRI pairs can be introduced, and reusing legacy candidate values for normal beam report can be considered as a starting point, i.e., {1, 2, 4}.

Then, based on the following highlighted part, we need to further consider another new UE feature of maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation. As a candidate value, we think that at least the value of ‘64’ should be supported.

Agreement

On Rel.17 enhancements to facilitate MPE mitigation, confirm the following working assumption (in the midst of the previous agreement) as an agreement with the following refinement (highlighted in red):

On Rel.17 enhancements to facilitate MPE mitigation, support the following enhancement on the Rel-16 event-triggered P-MPR-based reporting (included in the PHR report when a threshold is reached, reported via MAC-CE): - In addition to the existing field in the PHR MAC-CE, N≥1 P-MPR values can be reported - The N P-MPR values are reported together with the following: - (Working Assumption) For each P-MPR value, up to M SSBRI(s)/CRI(s), where the SSBRI(s)/CRI(s) is selected by the UE from a candidate SSB/CSI-RS resource pool (FFS: how to perform the selection) - Support M=1 - FFS: The supported value(s) of M - FFS: Additional reporting quantities, e.g. SSBRI/CRI, MPR+DL RSRP, or modified virtual PHR - FFS: additional signaling (e.g. CSI triggering) from the NW

Proposal 3: For MPE mitigation, the following modification is proposed in red

| 23-1-3 | MPE mitigation | 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation Candidate value comprises at least 64. |
| --- | --- | --- |

23-1-4	MPUE support for UL

In RAN1#107bis-e, the following FG has been introduce for MPUE report for UL.

- Based on RAN1 agreement, we have already agreed the terminology of ‘UE capability value set’, so we prefer to use the same terminology to aligned with already agreement and RAN1 spec

- Then, we think that up to 3 value is sufficient, and meanwhile the identical values in the different set should be allowable for facilitating the typical UE design of symmetric panel(s).

- After that, for semi-persistent or periodic reporting, in order to accommodate UE panel switching, UE can further indicate recommended periodicities, as a new component, then gNB configures a periodicity based on the reported periodicities.

- Finally, we do not identify the necessity of introduce ‘0’ in the candidate value set, and if we would like to consider DL-only reporting, we prefer to have a new sub-component rather than a wired signaling

Proposal 4: For MPUE support for UL, the following modification is proposed in red

| 23-1-4 | MPUE support for UL | 1. Supported UE capability value [sets] and corresponding max number of SRS ports for each UE capability value [set] Candidate values: [Up to [4] 3 value [sets] each with one value of {[0,]1,2,4}] Note: the reported list contains only unique value [sets] 2. Supported maximum periodicity for periodic/semi-persistent report Candidate values: {40, 80, 160, 320} slots |
| --- | --- | --- |
| 23-1-4a | DL-only report | - Support of indicating ‘DL-only’ for a reported RS in a beam report instance |

Multi-TRP PDCCH enhancements (23-2)

On MTRP PDCCH repetition enhancement, four FGs (23-2-1, 23-2-1a, 23-2-2 and 23-2-3) are listed in [1]. We have some comments on these FGs as below.

For FG 23-2-1, we are agreeable to it in principle. Regarding the limitation “except FR2” of FDM scheme in component 1, it is NOT in line with any agreement in the previous RAN1 meetings so far, hence we think it should be removed.

For FG 23-2-1a, we think the description of the note is inaccurate based on the following views. Our suggested wording is shown in the proposal as below.

- For the case of BDs number is equal to 2, whether the individual PDCCH candidates can be monitored or not depends on how the UE implements blind detection. More precisely, the individual PDCCH candidates can be monitored when the UE implement individual detection, rather than soft combining.

- For the case of BDs number is equal to 3, the individual PDCCH candidates can always be monitored due to UE has to implement both individual detection and soft combining anyways.

For FG 23-2-2, we think the wording “with non-SFN TDM and/or FDM schem” is redundant and should be remove, because its pre-requisite is FG 23-3-1 which already mentions the same thing.

For FG 23-2-3, we think this extra FG is not needed because there is no much difference between USS and CSS to process PDCCH repetition operation from UE side, and no matter two PDCCH is from USS or CSS, the decoding of two PDCCH repetition is the same, i.e. separate decoding or soft combining. We think it is more suitable to capture this feature in 23-2-1 as the current yellow highlighted part.

In addition, given that inter-span PDCCH repetition was agreed as a UE optional feature in RAN1#106bis-e meeting, we think one new FG should be introduced.

According to the above check points, we have the following proposal.

Proposal 5: Update the yellow parts in 23-2-1 ~ 23-2-3 as follows.

| 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of intra-slot PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) with non-SFN scheme TDM and FDM including PDCCH repetition for Type 3 CSS 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2,] support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET [for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 43. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  |  |  |  |  |  |  |  | Component 1 details FFS Component 2 candidate values: details FFS 2 or 3 Component 3 candidate values: details FFS {monitored, not monitored} Component 4 candidate values: [{0,1,2,3}] details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] Note: for component4, if N PDCCH candidates are overlapped, the number of overlaps is counted as one. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | [23-2-1] |  |  |  |  |  |  |  | [Note: [If 2 is reported in component 2 of FG 23-2-1,] support of whether the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] |
| 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for time-domain overlapping in time CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] | [23-2-1, 23-6-1, 23-6-2] |  |  |  | Per band |  |  | FR2 only |  |
| 23-2-3 | Inter-span PDCCH repetition | Support of inter-span PDCCH repetition |  |  |  |  |  |  |  |  |  |

Multi-TRP PUSCH/PUCCH enhancements (23-3)

On MTRP PUSCH/PUCCH enhancements, several UE features (23-3-1 ~ 23-3-2c) are listed in [1]. We have some further comments on the details as below.

For FG 23-3-1 family, we have the following comments:

- For component 1 in FG 23-3-1, the condition of repetition number when sequential mapping should include “equal to 2”. According to the existing discussion in RAN1, sequential mapping pattern is specified as the first beam is applied to the first and second PUSCH repetitions, and the second beam is applied to the third and fourth PUSCH repetitions, and the same beam mapping pattern continues to the remaining PUSCH repetition. In addition, there is no agreement/ conclusion mentioned that sequential mapping cannot be configured for the case of repetition number equal to 2.

- For component 7 in FG 23-3-1, it is NOT in line with any agreement in the previous RAN1 meetings so far, hence we think it should be removed.

- For component 11 in FG 23-3-1, based on the enhanced aspects in RNA1, we fail to see any discrepancy between CB and NCB based MTRP PUSCH schemes from the perspective of UE capability, plus there is no relationship between the number of SRS resources and MTRP PUSCH related features. Hence we think it should be removed.

- For FG 23-3-1c, it is in line with previous agreement in RAN1,hence we think it can be approved.

- For FG 23-3-1-1, the FFS part should be removed because of the same reason of component 11 in FG 23-3-1.

- For FG 23-3-1a to Fg 23-3-1f, the pre-requisite should be FG 23-3-1. Besides, one editorial issue is that the numeric name of FG 23-3-1d to FG 23-3-1f should be reordered.

According to the above check points, we have the following proposal.

Proposal 6: Update the yellow parts in 23-3-1 ~ 23-3-1-1 as follows.

| 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - sequential mapping for repetitions equal to or larger than 2 - cyclic mapping for 2 repetitions [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) ] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) ] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] 2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23-3-1a | Cyclic mapping | Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A | 23-3-1 |  |  |  |  |  |  |  |  |  |
| 23-3-1b | Second TPC field | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 | 23-3-1 |  |  |  |  |  |  |  |  |  |
| 23-3-1c | Two PHR reporting | Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) | 23-3-1 |  |  |  |  |  |  |  |  |  |
| 23-3-1d | A-CSI report | Support of A-CSI report on two PUSCH repetitions | 23-3-1 |  |  |  |  |  |  |  |  |  |
| 23-3-1e | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions | 23-3-1 |  |  |  |  |  |  |  |  |  |
| 23-3-1f | CG PUSCH transmission | Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) | 23-3-1 |  |  |  |  |  |  |  |  |  |

| 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 23-3-2 family, we have the following comments:

- For component 1 in FG 23-3-2, the condition of “equal to 2” should be added due to the same reason of component 1 in FG 23-3-1.

- For component 2 in FG 23-3-2, we can ve agreeable of it in principle. However, note that up to two spatial relations can be activated per PUCCH resource in FR2 but two power control parameter sets can be activated per PUCCH resource in FR1, the case of MTRP PUCCH in FR1 should be included.

- For FG 23-3-2b, the candidate component values can include 4 and 8 based on the previous agreements in RAN1 as follows.

Agreement (in RAN1#105-e) Confirm the working assumption with removing brackets on [consecutive] and adding UE capability. - For PUCCH reliability enhancement, support multi-TRP intra-slot repetition (Scheme 3) for all PUCCH formats. - The same PUCCH resource carrying UCI is repeated for X = 2 [consecutive] sub-slots within a slot. - Refer the design details related to sub-slot configurations (e.g. other values of X) to Rel-17 eIIoT - Note1: The decision of supporting scheme 3 is only applicable for multi-TRP operation. This feature is optional. Agreement (in RAN1#107-e) For sub-slot based PUCCH repetition, the following agreement from Cov. Enh. WI for slot-based PUCCH repetition is adopted also for sub-slot based PUCCH repetition: Agreement - In Rel-17, reuse the Rel-16 PUCCH repetition factors 2, 4, 8. - Do not support PUCCH repetition factor larger than 8 In Rel-17..

According to the above check points, we have the following proposal.

Proposal 7: Update the yellow parts in 23-3-2 ~ 23-3-2c as follows.

| 23-3-2 | Multi-TRP PUCCH repetition-inter-slot | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions equal to or larger than 2 - cyclic mapping for 2 repetitions [2. Support of up to two PUCCH spatial relation or two power control parameter sets per PUCCH resource] 32. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for both PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 43. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23-3-x | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions equal to or larger than 2 - cyclic mapping for 2 repetitions | FFS |  |  |  |  |  |  |  |  |
| 23-3-2b |  | Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2 | FFS |  |  |  |  |  |  |  | FFS: candidate component values: {4, 8} |
| 23-3-2c |  | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  |

Inter-cell Multi-TRP (23-4)

One UE feature (23-4) has been listed for inter-cell MTRP enhancements in [1]. We have some further comments on the details as below.

For FG 23-3-4, we have the following comments:

- For component 3, the current draft description is NOT in line with the previous agreement in RAN1 as follows. Note that case 1 means the configurations of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI, hence case 2 should capture the complementary set of case 1, which is the configurations of SSB time domain positions and periodicity of the additional PCIs is fully or partially different with SSB time domain positions and periodicity of the serving cell PCI.

- For the candidate values of component 2 and component 3, the value 0 in component 3 should be removed. Given that the description of the note below, when only case 2 is enabled and then the UE reports component 3 candidate value is 0, that is inconsistent with this FG because no additional PCI can be used for MTRP inter-cell operation.

Agreement (in RAN1#106bis-e) Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB. - X1 (Case 1)= The maximum number of configured additional PCIs when each configurations of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI - X2 (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 - Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously - Supported values for X1 and X2 includes at least 0,1,2,3 and 7. FFS on other values - This UE capability has FR1 and FR2 differentiation (FFS : Whether this UE capability is per UE or per band)

According to the above check points, we have the following proposal.

Proposal 8: Update the yellow parts in 23-4 as follows.

| 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. The maximum number of configured additional PCIs is X1 when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI] [3. The maximum number of configured additional PCIs is X2 when the configurations of SSB time domain positions and periodicity of the additional PCIs is fully or partially different with SSB time domain positions and periodicity of the serving cell PCI] [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. default case to be supported, e.g., case2 with X2=1] |  |  |  |  |  |  |  |  | [Component 2 candidate values: {1,2,3,7}] [Component 3 candidate values: {1,2,3,7}] [Note: case1 and case2 cannot be enabled simultaneously] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Multi-TRP beam management (23-5)

In our views, the explicit sub-features of supporting Rel-17 group-based reporting and supporting Rel-17 mTRP BFR should be introduced for [23-5-1] and [23-5-2], respectively.

- Regarding group based report [23-5-1]

  - Regarding #. RS for both CMR sets (component-2/3), we are open to have some further discussion.

  - Regarding FG 23-5-1a, as we mentioned in Section 1, we think that instead of introducing a duplicated FG, the legacy FG 16-1g/1g-1 can be reused with some necessary clarification for that in Rel-17 TS 38.306.

- Regarding MTRP BFR enhancement [23-5-2]

  - For mTRP-BFR, 2 BFD-RS sets should be supported, and so we do not think that we need the component-2 in FG 23-5-2.

  - Regarding Component 3 in FG 23-5-2, as for SCell-BFR, PUCCH-SR should be mandatorily supported for a UE, and due to the same reason, we think that at least one PUCCH-SR should be supported as in a basic feature.

  - Regarding Component 1 in FG 23-5-2a, as we discussed for above Component 3 in FG 23-5-2, the component value of [0, 1] is not needed.

  - Regarding Component 2 in FG 23-5-2a, if we introduce more than one PUCCH-SR, straightforwardly we need to support the association between a BFD-RS resource set on sPCell and a PUCCH SR resource.

Hence, we have the following proposal.

Proposal 9: For beam management for mTRP, the following modification in red is proposed

| 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets [2. Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] [3. Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] |
| --- | --- | --- |
| 23-5-1a | Resources [for beam management , PL measurement, BFD, RLM, and new beam identification] | Note: Strive to align the final implementation of FG 23-5-1a with related R15/16 implementations If 23-5-1a ends up being identical to one or more of Rel.15/ 16 FGs this row will be deleted |
| 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of supported BFD-RS resources per set 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] [3. Support PUCCH-SR resource for MTRP BFRQ] 4. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP |
| 23-5-2a | PUCCH-SR resources for MTRP BFRQ | 1. Max number of PUCCH-SR resources for MTRP BFRQ Component candidate values: {[0,1], 2} [2. Association between BFD-RS resource set on sPCell and a PUCCH SR resource (if component candidate value equals 2)] |

SFN based HST enhancements (23-6)

On SFN based HST enhancements, several FGs (23-6-1 ~ 23-6-4) are listed in [1]. We have some further comments on the details as below.

Firstly, with respect to transmission scheme combinations, the following five options have been supported for Rel-17 SFN based MTRP HST in RAN1, which should be completely captured in FG 23-6-1, FG 23-6-1b, FG 23-6-2 and FG 23-6-2b.

- Scheme A (scheme 1) PDCCH + Rel-17 Scheme A (scheme 1) PDSCH

- Scheme A (scheme 1) PDCCH + Single-TRP PDSCH

- Scheme B (scheme 2) PDCCH + Rel-17 Scheme B (scheme 2) PDCCH

- Single-TRP PDCCH + Rel-17 Scheme A (scheme 1) PDCCH

- Single-TRP PDCCH + Rel-17 Scheme B (scheme 2) PDCCH

Secondly, regarding the part of “default QCL assumption with one or two TCI states for PDCCH” in FG 23-6-1, FG 23-6-1b, FG 23-6-2 and FG 23-6-2b, it is somehow unrelated to these FGs and should be removed for avoiding any ambiguity.

Thirdly, we prefer per band reporting type from the perspective of gNB scheduling.

Fourthly, for FG 23-6-3 on support of simultaneous activation of two TCI states for PDCCH across multiple CCs [2], is not needed from our view. That’s because whether UE supports such feature can completely be inferred by the existing Rel-16 UE FG 16-1b-1 combined with the Rel-17 FG 23-6-1 or 23-6-2.

| 16-1b-1 | TCI state activation across multiple CCs | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH |
| --- | --- | --- |

As we know, for a UE support FG 16-1b-1, some configured CCs in which TCI states can be simultaneously updated for CORESETs with the same ID should share the same RF chain or similar channel conditions for analog beamforming. This is regardless of the number of activated TCI states for each CORESET. So, as long as UE supports FG 16-1b-1, and also supports FG 23-6-1 or 23-6-2, it is natural for this UE to support simultaneous activation of two TCI states across set of configured component carriers.

According to the above check points, we have the following proposal.

Proposal 10: Update the yellow parts in 23-6-1 ~ 23-6-2b as follows.

| 23-6-1 | SFN scheme A (scheme 1) for PDSCH and PDCCH | 1. [Support of SFN scheme A for PDCCH] scheduling single TRP/ SFN Scheme A PDSCH and 2. Support of SFN scheme A for PDSCH [only scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with one or two TCI states for PDSCH] |  |  |  |  | Per band |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH SFN scheme A by TCI state field in DCI formats 1_1, 1_2 | [23-6-1[b]] |  |  |  | Per band |
| 23-6-1b | SFN scheme A (scheme 1) for PDSCH only | 1. Support of SFN scheme A for PDSCH scheduled by single TRP/ Scheme A PDCCH |  |  |  |  | Per band |
| 23-6-2 | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH | 1. [Support of SFN scheme B for PDCCH] scheduling SFN Scheme B PDSCH 2. Support of SFN scheme B for PDSCH only [and default QCL assumption with two TCI states for PDSCH] [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  | Per band |
| 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH SFN scheme B by TCI state field in DCI formats 1_1, 1_2 | [23-6-2[b]] |  |  |  | Per band |
| 23-6-2b | SFN scheme B (TRP based pre-compensation) for PDSCH only | 1. Support of SFN scheme B for PDSCH scheduled by single TRP/ Scheme B PDCCH | [23-6-1] |  |  |  | Per band |

Multi-TRP CSI (23-7)

Several UE features (23-7-1 ~ 23-7-5) are listed for MTRP CSI enhancements in [2]. We generally agree with these UE features, as they are aligned with previous agreements. Some suggestions are given as follows

- In FG 23-7-1,

  - For component-4, we can NOT support the component a/c/e/g that is related to normal sTRP-CSI measurement. Then, if considering a list of combination (although we do not identify clear necessity), the normal UE capability report using the triplet of {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for NCJT. That means that FG 23-7-2 should be considered together.

  - For component-5 and 6, we have already increased the number of occupied CPU(s) compared with s-TRP CSI calculation, and therefore the legacy UE capability report for CPU(s) should be sufficient.

- FG 23-7-3 should be removed as it is similar as component 2 of FG 23-7-1

Proposal 11: For multi-TRP CSI(FG23-7), the following modification is proposed in red

| 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode [selection] of mode 1 with X=0 [and/or] mode 2 - A list of [supported combinations, up to 16, across all CCs simultaneously, where each combination is] - [Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis] - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - [Maximum total number of CMRs for single-TRP measurement] [per CC/across all CCs] - Maximum total number of CMRs for NCJT measurement [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses] [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement] - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] |
| --- | --- | --- |
| 23-7-1a | Additional CSI report mode 1 selection | Maximum value of numberOfSingleTRP-CSI-Mode1 |
| 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |
| 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |
| 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting |
| 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis |

SRS enhancements (23-8)

In RAN1#106bis-e, a new UE optional feature is supported based on the following agreement.

Agreement

For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R.

- The above extension is UE optional

Hence a new FG is needed for such extension of aperiodic antenna switching SRS of 1T4R and 1T2R/2T4R. We have the following proposal.

Proposal 12: Add the following new FG

| 23-8-9 | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R | Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R. | 2-53 |  |  |  | Per FS |
| --- | --- | --- | --- | --- | --- | --- | --- |

Further, one remaining issue in an editor’s note of the Dec. RAN1 specification is whether RPFS is applicable to the case that frequency hopping is not enabled. We think one way to resolve this issue is to introduce a UE capability for the non-FH case. This new UE feature should use 23-8-6 as the prerequisite. If this capability is not reported, the UE cannot support to be configured with partial frequency sounding for non-FH case. Therefore, we have the following proposal.

Proposal 13: Add the following new FG with 23-8-6 as the prerequisite.

| 23-8-6a | Applicability of partial frequency sounding for non-FH case | Support of partial frequency sounding for the case that FH is not enabled. | 23-8-6 |  |  |  | Per band |
| --- | --- | --- | --- | --- | --- | --- | --- |

For other yellow parts in 23-8-1 ~ 23-8-8, we have the following proposal.

Proposal 14: Update the yellow parts in 23-8-1 ~ 23-8-8 as following.

| 23-8-1 | SRS triggering offset enhancement | 1. [Support of/The maximum number of configured available slots offsets for] determining aperiodic SRS location based on available slot [2. Maximum actual slots offset] | 2-52 |  |  |  | Per band |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  | [Per UE or per band] |
| 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the antenna switching impact to downlink receiving in a band 3. Report whether the antenna is switched together with UL Tx in another band | 2-55 |  |  |  | [Per BC] |
| 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  | [Per FS] |
| 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  | Per band |
| 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  | Per band |
| 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodoc SRS | 23-8-6 |  |  |  | Per band |
| 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  | Per band |

FeType II port selection CSI (23-9)

We have the following recommendations on the yellow parts of the 23-9 series.

Proposal 15: Recommend the following on the yellow parts of the 23-9 series

| 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 16-3b, 16-3a-1, 16-3b-1, 2-36, 2-40, 2-41, 2-43 |  |  |  | Per band and per BC |  |  |  | Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}[, {FeType II PS M=2 R=2, NULL}], {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} FFS: The list of {Codebook 2, Codebook 3} may be refined depending on outcome of discussion for FG 23-9-2 Component 1 and FG 23-9-4 Component 2 Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1：if a UE reports one or more codebook combinations in 23-9-5, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1/23-9-2/23-9-4 [Note: If the list of triplets for a codebook combination is empty then the list of triplets is reused from the codebook combination indicated previously] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II 2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support to support Port-selection FeType-II with M=2 and R=2 | 23-9-2 |  |  |  | per band and per BC |  |  |  | Component 2 candidate values: • Maximum 8 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: [{1 to 64}] • Max # total ports: [{4 to 256}] |

Conclusion

We discuss detailed issues on the NR feMIMO UE feature list in this contribution. 15 proposals are given together with detailed update of the FG list are given in Sections 1 - 9.

References

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, Moderators (AT&T, NTT DOCOMO, INC.)