3GPP TSG RAN WG1 #108-e                                                                                 R1-2201272

e-Meeting, February 21st – March 3rd , 2022

Source:	OPPO

Title:	Discussion on enhancement of UL time and frequency synchronization

Agenda Item:	8.4.2

Document for:	Discussion and Decision

Introduction

In this meeting, we discuss the remaining spec CR [2] with respect to the RAN1 agreements [1] to further see how to capture the agreements in the CR.

Discussion

Issue 1: common TA model

In RAN1#107-e meeting, there was an agreements made to define the common TA model.

Agreement

Using indicated Higher-layer Common TA parameters, if configured, the UE can determine the one-way propagation time ( Delaycommon) used for NTA, common  calculation as follows:

Delaycommont= DCommon tepocℎ+ DCommonDrift× t−tepocℎ+DCommonDriftVariation× t−tepocℎ2

Where:

- DCommon = TACommon 2, DCommonDrift=TACommonDrift2 and DCommonDriftVariation=TACommonDriftVariation2

- TACommon, TACommonDrift and TACommonDriftVariation are Common TA parameter defined in RAN1 Meeting #106-bis-e

- Delaycommont is the distance between the satellite and the uplink time synchronization reference point divided by the speed of light. DL and UL are frame aligned at the reference point with an offset given by NTA,offset.

- NTA, common is derived by the UE based on Delaycommont to pre-compensate the two-way transmission delay between the uplink time reference point and the satellite.

This model is important as an assumption for the UE to know and the UE can implement the common TA estimation based on this assumption. However, this information is not captured in the current RAN1 CR. We suggest that this model can be captured in TS 38.211 as an example of TP#1.

------------------------------------ TP#1 TS 38.211 (in bleu)-----------------------------------------------

4.3.1	Frames and subframes

Downlink, uplink, and sidelink transmissions are organized into frames with  duration, each consisting of ten subframes of  duration. The number of consecutive OFDM symbols per subframe is Nsymbsubframe,μ=NsymbslotNslotsubframe,μ. Each frame is divided into two equally-sized half-frames of five subframes each with half-frame 0 consisting of subframes 0 – 4 and half-frame 1 consisting of subframes 5 – 9.

There is one set of frames in the uplink and one set of frames in the downlink on a carrier.

Uplink frame number  for transmission from the UE shall start

TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc

before the start of the corresponding downlink frame at the UE where

- NTA and NTA,offset are given by clause 4.2 of [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used;

-	NTA,adjcommon is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured, otherwise NTA,adjcommon=0; a UE may determine the one-way propagation time Delaycommon, used for NTA,adjcommon calculation as follows:

Delaycommont= DCommon tepocℎ+ DCommonDrift× t−tepocℎ+DCommonDriftVariation× t−tepocℎ2  , where, DCommon , DCommonDrift, and DCommonDriftVariation, are provided by TACommon, TACommonDrift, and TACommonDriftVariation, respectively; and Delaycommont is the distance between the satellite and the uplink time synchronization reference point divided by the speed of light. The reference point is where DL and UL are frame aligned with an offset given by NTA,offset;

-	NTA,adjUE is computed by the UE based on satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

-------------------------------- end of TP#1-------------------------------------------------------------------

Proposal 1: Adopt TP#1.

Issue 2: epoch time definition

In RAN1#107-e meeting, the epoch time for ephemeris data and common TA parameters are agreed. This information should be captured in the spec, otherwise, the UE cannot correctly derive the UE specific TA and common TA.

Agreement

- When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.

- Otherwise, when indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the SI message is transmitted.

- When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number.

Agreement

The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point.

In order to capture the epoch time information in the spec, we suggest the TP#2.

------------------------------------ TP#2 TS 38.211 (in bleu)-----------------------------------------------

4.3.1	Frames and subframes

Downlink, uplink, and sidelink transmissions are organized into frames with  duration, each consisting of ten subframes of  duration. The number of consecutive OFDM symbols per subframe is Nsymbsubframe,μ=NsymbslotNslotsubframe,μ. Each frame is divided into two equally-sized half-frames of five subframes each with half-frame 0 consisting of subframes 0 – 4 and half-frame 1 consisting of subframes 5 – 9.

There is one set of frames in the uplink and one set of frames in the downlink on a carrier.

Uplink frame number  for transmission from the UE shall start

TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc

before the start of the corresponding downlink frame at the UE where

- NTA and NTA,offset are given by clause 4.2 of [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used;

-	NTA,adjcommon is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured, otherwise NTA,adjcommon=0;

-	NTA,adjUE is computed by the UE based on satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

The provided highe layer parameters TACommon, TACommonDrift,TACommonDriftVariation and satellite-ephemeris-related parameters are with reference to an epoch time at a reference point. A UE may assume the epoch time as the start of a subframe n of a SFN m, if m and n are provided; otherwise, the UE may assume the epoch time as the end of a SI window in which the parameters are provided. The reference point is where DL and UL are frame aligned with an offset given by NTA,offset.

-------------------------------- end of TP#2-------------------------------------------------------------------

Proposal 2: Adopt TP#2.

Conclusion

In this contribution, we discuss the remaining issues for the CR.

Proposal 1: Adopt TP#1.

Proposal 2: Adopt TP#2.

Reference

[1]       RAN1#107b-e chairman notes

[2]       R1-2112921, 38.211 CR  version 16.7.0