3GPP TSG RAN WG1 #108-e R1-2201291 e-Meeting, February 21st – March 3rd, 2022 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 16.8.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Correction on eType-3 HARQ-ACK codebook. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_unlic-Core; Date:; 2022-02-07. Category:; D; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; For Type-3 HARQ-ACK codebook, HARQ-ACK bits corresponding to all HARQ processes are transmitted. During the procedure of Type-3 HARQ-ACK codebook construction, UE determines HARQ-ACK information bit, , for each HARQ process. However, in current spec., some HARQ-ACK information bit is descripted for HARQ process number ℎ.. -. Summary of change:; Change “HARQ process number h” to “HARQ process h” in Type-3 HARQ-ACK codebook construction.. Consequences if not approved:; Inconsistent specifications. Clauses affected:; 9.1.4. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR..

#### 9.1.4	Type-3 HARQ-ACK codebook determination

<Unchanged parts are omitted>

Set j=0

while c<NcellsDL

while ℎ<NHARQ,cDL

if NDIHARQ=0

if NHARQ−ACK,cCBG/TB,max>0

while t<NTB,cDL

while g<NHARQ−ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process ℎ of serving cell c, if any; else,

j=j+1

g=g+1

end while

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process ℎ on serving cell c, if any; else,

g=0

j=j+1

t=t+1

end while

else

while t<NTB,cDL

= HARQ-ACK information bit for TB t for HARQ process ℎ of serving cell c, if any; else,

j=j+1

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process ℎ on serving cell c, if any; else,

j=j+1

t=t+1

end while

end if

t=0

else

if NHARQ−ACK,cCBG/TB,max>0

while t<NTB,cDL

if UE has obtained HARQ-ACK information for TB t for HARQ process ℎ on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

while g<NHARQ−ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process ℎ of serving cell c

j=j+1

g=g+1

end while

else

while g<NHARQ−ACK,cCBG/TB,max

j=j+1

g=g+1

end while

end if

g=0

t=t+1

end while

else

while t<NTB,cDL

if UE has obtained HARQ-ACK information for TB t for HARQ process ℎ on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

if harq-ACK-SpatialBundlingPUCCH is not provided

= HARQ-ACK information bit for TB t for HARQ process ℎ of serving cell c

else

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process ℎ of serving cell c. If the UE receives one transport block, the UE assumes ACK for the second transport block

end if

j=j+1

t=t+1

else

= NACK

j=j+1

t=t+1

end if

end while

end if

t=0

end if

ℎ=ℎ+1

end while

ℎ=0

c=c+1

end while