3GPP TSG RAN WG1 #108-e 	                                                         	 R1-2201295

e-Meeting, February 21st – March 3rd, 2022

Source:	OPPO

Title:	HARQ-ACK enhancements for Rel-17 URLLC/IIoT

Agenda Item:	8.3.1

Document for:	Discussion and Decision

Introduction

In RAN1 #104b-e and #106-e, HARQ-ACK feedback enhancements for enhanced IoT and URLLC were discussed with following working assumption and agreement:

Working assumption: To handle the collision for the same HARQ process due to deferred SPS HARQ-ACK the following behaviour is to be specified:

In case the UE receives PDSCH of a certain HARQ Process ID, the deferred SPS HARQ bit(s) for this HARQ Process ID are dropped.

Agreement

For the enhanced Type 3 HARQ-ACK CB of smaller size triggered in a PUCCH slo t, the UE is not expecting HARQ-ACK information in a Type 1 or Type 2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook.

In this contribution, we show our views on the remaining issues on HARQ-ACK enhancements for Rel-17 IoT and URLLC.

SPS HARQ-ACK deferral

Based on the current TS38.213, when the collision for the same HARQ process occurs, UE may have no time to drop the deferred SPS HARQ bit.

#### 9.2.5.4 UE procedure for deferring HARQ-ACK for SPS PDSCH ...... the UE - determines an earliest second slot and, after performing the procedures in clauses 9 and 9.2.5 to resolve overlapping among PUCCHs and PUSCHs, a PUSCH or a PUCCH in the earliest second slot to multiplex HARQ-ACK information bits that include second HARQ-ACK information bits from the first HARQ-ACK information bits …… - the second HARQ-ACK information bits, generated as described in clause 9.1.2, are appended in a HARQ-ACK codebook the UE generates as described in clauses 9.1.2, 9.1.2.1, or 9.1.3.1 - if the UE would receive a PDSCH providing a TB for a same HARQ process as a HARQ-ACK information bit from the second HARQ-ACK information bits prior to transmitting the PUCCH or the PUSCH, the UE does not include the HARQ-ACK information bit in the HARQ-ACK information bits.

One example is shown in Figure 1, a SPS PDSCH with HARQ process number X is transmitted in slot n, and the corresponding SPS HARQ-ACK is deferred to slot n+3. Another PDSCH 1 with the same HARQ process number is transmitted in slot n+2. The gap between the ending of PDSCH 1 and the starting of target PUCCH for SPS HARQ-ACK deferral is less than Rel-15 multiplexing timeline. In this case, UE has no time to reconstruct the HARQ-ACK codebook to be transmitted in slot n+3. To solve this problem, the following solution is proposed to handle the collision for the same HARQ process due to SPS HARQ-ACK deferral:

- If SPS HARQ-ACK corresponding to a SPS PDSCH with a certain HARQ process number is deferred to a target PUCCH/PUSCH, and a later PDSCH with the same HARQ process number is received prior to the target PUCCH/PUSCH,

  - If the later PDSCH and the target PUCCH/PUSCH satisfy Rel-15 multiplexing timeline, the deferred SPS HARQ-ACK is dropped;

  - Otherwise, the deferred SPS HARQ-ACK is transmitted in the target PUCCH/PUSCH.

Figure 1: The collision for the same HARQ process due to SPS HARQ-ACK deferral

Proposal 1: If SPS HARQ-ACK corresponding to a SPS PDSCH with a certain HARQ process number is deferred to a target PUCCH/PUSCH, and a later PDSCH with the same HARQ process number is received prior to the target PUCCH/PUSCH,

  - If the later PDSCH and the target PUCCH/PUSCH satisfy Rel-15 multiplexing timeline, the deferred SPS HARQ-ACK is dropped;

  - Otherwise, the deferred SPS HARQ-ACK is transmitted in the target PUCCH/PUSCH.

Enhanced Type 3 HARQ-ACK CB

Based on the agreement made in RAN1 #106-e, a Type 1/2 CB in one PUCCH slot should be mapped to an eType 3 CB in the same PUCCH slot. Such limitation is applicable when the PUCCH slot lengths of eType 3 CB and Type 1/2 CB are the same or slot-based PUCCH is configured for eType 3 CB. However, when subslot-based PUCCH is configured for eType 3 CB and the PUCCH slot lengths of eType 3 CB and Type 1/2 CB are different, additional clarifications or agreements are needed.

Case 1: A subslot-based HP eType 3 CB and a slot-based LP Type 1/2 CB are allocated in different subslots of one slot, as shown in Figure 2.

Figure 2: A subslot-based HP eType 3 CB and a slot-based LP Type1/2 CB in one slot (case 1)

Case 2: A slot-based PUCCH carrying LP Type 1/2 CB overlaps the subslot configured for HP eType 3 CB, as shown in Figure 3.

Figure 3: A slot-based LP eType 3 CB and a subslot-based HP Type1/2 CB in one slot (case 2)

Case 3: A slot-based PUCCH carrying LP Type 1/2 CB overlaps the PUCCH configured for HP eType 3 CB, as shown in Figure 4.

Figure 4: A slot-based LP eType 3 CB and a subslot-based HP Type1/2 CB in one slot (case 3)

To keep the uniform design for case 1~3, we slight prefer to support that if an eType 3 HARQ-ACK CB is triggered in a given subslot, the HARQ-ACK information in a Type 1/2 HARQ-ACK CB to be transmitted in a slot/subslot overlapping with the given subslot should be mapped to the eType 3 HARQ-ACK CB.

Proposal 2: If an eType 3 HARQ-ACK CB is triggered in a given subslot, the HARQ-ACK information in a Type 1/2 HARQ-ACK CB to be transmitted in a slot/subslot overlapping with the given subslot should be mapped to the eType 3 HARQ-ACK CB.

Conclusions

In this contribution, we show our views on the HARQ-ACK enhancements for Rel-17 IoT and URLLC with following proposals:

Proposal 1: If SPS HARQ-ACK corresponding to a SPS PDSCH with a certain HARQ process number is deferred to a target PUCCH/PUSCH, and a later PDSCH with the same HARQ process number is received prior to the target PUCCH/PUSCH,

  - If the later PDSCH and the target PUCCH/PUSCH satisfy Rel-15 multiplexing timeline, the deferred SPS HARQ-ACK is dropped;

  - Otherwise, the deferred SPS HARQ-ACK is transmitted in the target PUCCH/PUSCH.

Proposal 2: If an eType 3 HARQ-ACK CB is triggered in a given subslot, the HARQ-ACK information in a Type 1/2 HARQ-ACK CB to be transmitted in a slot/subslot overlapping with the given subslot should be mapped to the eType 3 HARQ-ACK CB.

References

- R1-2200775, “Moderator summary #4 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT”, Moderator (Nokia)