3GPP TSG-RAN WG1 Meeting #108-e	R1-2201327

# e-meeting, February 21st – March 3rd, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Type II and enhanced Type II CSI feedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For Type II CSI feedback, it is specified that the elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits, in Clause 5.2.3 [TS 38.214]. There are 2L elements for i1,4,l, i2,1,l and i2,2,l according to the illustration in Caluse 5.2.2.2.3 [TS 38.214]. Hence, their indices i should be 0,1,…, 2L-1. In Clause 6.3.2.1.2 [TS 38.212], the bitwidth of port selection indication i1,1 is calculated as log2PCSI-RS2d for enhanced Type II port selection codebook, where both PCSI-RS and d are configured with the higher layer parameter. When PCSI-RS=2d, the bitwidth of i1,1 is zero according to the computation formula. We can see that port selection indication i1,1 is not always reported. However, in Clause 5.2.3 [TS 38.214], for omission of Part 2 CSI, Group 0 includes indices i1,1, i1,2 (if reported) and i1,8,l (l=1,…,υ), which means that Group 0 always includes i1,1. This makes UE’s behavior be unclear when the bitwidth of i1,1 equals to zero. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the index value of the elements of i1,4,l, i2,1,l and i2,2,l. Group 0 includes indices i1,1 if it is reported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) with their indices i = L,…,2L-1 are not reported. For Enhanced Type II reports, UE’s behaviour of CSI reporting for Group 0 in Part 2 is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===================== Unchanged parts =====================

5.2.3	CSI reporting using PUSCH

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state.

When a DCI format 0_1 schedules two PUSCH allocations, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the aperiodic CSI report is carried on the penultimate scheduled PUSCH.

An aperiodic CSI report carried on the PUSCH supports wideband, and sub-band frequency granularities. An aperiodic CSI report carried on the PUSCH supports Type I, Type II and Enhanced Type II CSI.

A UE shall perform semi-persistent CSI reporting on the PUSCH upon successful decoding of a DCI format 0_1 or DCI format 0_2 which activates a semi-persistent CSI trigger state. DCI format 0_1 and DCI format 0_2 contains a CSI request field which indicates the semi-persistent CSI trigger state to activate or deactivate. Semi-persistent CSI reporting on the PUSCH supports Type I, Type II with wideband, and sub-band frequency granularities and Enhanced Type II CSI. The PUSCH resources and MCS shall be allocated semi-persistently by an uplink DCI.

CSI reporting on PUSCH can be multiplexed with uplink data on PUSCH except that semi-persistent CSI reporting on PUSCH activated by a DCI format is not expected to be multiplexed with uplink data on the PUSCH. CSI reporting on PUSCH can also be performed without any multiplexing with uplink data from the UE.

Type I CSI feedback is supported for CSI Reporting on PUSCH. Type I wideband and sub-band CSI is supported for CSI Reporting on the PUSCH. Type II CSI is supported for CSI Reporting on the PUSCH.

For Type I, Type II and Enhanced Type II CSI feedback on PUSCH, a CSI report comprises of two parts. Part 1 has a fixed payload size and is used to identify the number of information bits in Part 2. Part 1 shall be transmitted in its entirety before Part 2.

-	For Type I CSI feedback, Part 1 contains RI (if reported), CRI (if reported), CQI for the first codeword (if reported). Part 2 contains PMI (if reported), LI (if reported) and contains the CQI for the second codeword (if reported) when RI is larger than 4.

-	For Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the number of non-zero wideband amplitude coefficients per layer for the Type II CSI (see Clause 5.2.2.2.3). The fields of Part 1 – RI (if reported), CQI, and the indication of the number of non-zero wideband amplitude coefficients for each layer – are separately encoded. Part 2 contains the PMI and LI (if reported) of the Type II CSI. The elements of i1,4,l, i2,1,l (if reported) and i2,2,l (if reported) are reported in the increasing order of their indices, i=0,1,…, 2L-1, where the element of the lowest index is mapped to the most significant bits and the element of the highest index is mapped to the least significant bits. Part 1 and 2 are separately encoded.

-	For Enhanced Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the overall number of non-zero amplitude coefficients across layers for the Enhanced Type II CSI (see Clause 5.2.2.2.5). The fields of Part 1 – RI (if reported), CQI, and the indication of the overall number of non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the PMI of the Enhanced Type II CSI. Part 1 and 2 are separately encoded.

A Type II CSI report that is carried on the PUSCH shall be computed independently from any Type II CSI report that is carried on the PUCCH formats 3 or 4 (see Clause 5.2.4 and 5.2.2).

When the higher layer parameter reportQuantity is configured with one of the values 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR', the CSI feedback consists of a single part.

For both Type I and Type II reports configured for PUCCH but transmitted on PUSCH, the determination of the payload for CSI part 1 and CSI part 2 follows that of PUCCH as described in Clause 5.2.4.

When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where  is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority  is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the  CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in increasing order with the lowest subband of csi-ReportingBand as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level.

-	For Enhanced Type II reports, for a given CSI report n, each reported element of indices  i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1 (if reported), i1,2 (if reported) and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the  KNZ/2-υ highest priority elements of i2,4,l and the  KNZ/2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the KNZ/2 lowest priority elements of i2,4,l and the KNZ/2 lowest priority elements of i2,5,l (l=1,…,υ).

===================== Unchanged parts =====================