3GPP TSG RAN WG1 #108-e                                     R1-2201343

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.14.2

Source:	CATT

Title:	Remaining issues on timing relationship enhancement for IoT over NTN

Document for:	Discussion and Decision

Introduction

Based on IoT NTN Chairman's Notes 107-e [1], there are some agreements and working assumption on timing relationship of NTN, and some FFS points are also listed as follows:

Agreement

For IoT NTN, signalling one value for cell-specific K_offset in system information is supported.

Agreement

For IoT NTN, the unit of K_offset is subframe based on a 15kHz subcarrier spacing (i.e. 1 ms).

- Further discuss the case where UL is using 3.75 kHz SCS

Agreement

For IoT NTN, the UE specific K_offset is provided and updated by the network using MAC CE.

Agreement

For IoT NTN, the information of K_mac is carried in system information.

Agreement

For IoT NTN, the unit of K_mac is subframe based on a 15kHz subcarrier spacing (i.e. 1 ms).

- Further discuss the case where UL is using 3.75 kHz SCS

Agreement

Modification of the designation of subframes with NPDCCH monitoring restrictions is needed for at least Cases 1 to 6.

Agreement

Whether/how the “indicated value” of K_offset is translated into number of slots for different numerologies (i.e., 15 kHz and 3.75 kHz) is left to the spec-editor.

- This resolves the bullet from previous agreement: Further discuss the case where UL is using 3.75 kHz SCS

Agreement

For IoT NTN, adopt the NR NTN agreement without modification for FR1: (a) the value range (i.e. 1 ms), (b) the quantity signalled (e.g. a differential UE specific K_offset) for the UE specific K_offset.

Agreement

For IoT NTN, adopt the NR NTN agreement without modification for FR1 for the value range of Kmac.

Agreement

Network can configure UE-specific TA reporting either a TA or UE location for connected mode UE

- In case a TA is configured, NR NTN solutions are a baseline for the following UE-specific TA handling issues,

  - Signaling – quantity (full or delta), range, number of bits

  - Granularity of report

  - Frequency of reporting

  - Means of reporting

  - NOTE: Any changes needed for IoT NTN can be made.

- In case the UE location is configured, RAN2 will design solutions for the UE location information, and it is left to RAN2 to decide whether to support UE location reporting

In this contribution, we further analyze remaining technical issues of timing relationship aspect for IoT NTN and provide our views on these issues.

Preamble retransmission for NB-IoT

The section from 16.3.2 of TS36.213:

a) If a NPDCCH with associated RA-RNTI is detected and the corresponding DL-SCH transport block ending in subframe n contains a response to the transmitted preamble sequence, the UE shall, according to the information in the response, transmit an UL-SCH transport block according to Subclause 16.3.3. b) If a random access response is received and the corresponding DL-SCH transport block ending in subframe n does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n. c) If no NPDCCH scheduling random access response is received in subframe n, where subframe n is the last subframe of the random access response window, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n. d) If an NPDCCH scheduling random access response with associated RA-RNTI is detected and the corresponding DL-SCH transport block reception ending in subframe n cannot be successfully decoded, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

For cases b, c and d, the preamble retransmission is required no later than 12ms of the DL subframe n. There are two interpretations of 12ms among companies.

For the first interpretation, 12ms is the relative time interval, where subframe n is DL subframe and n+12ms is UL subframe. Considering that the UE in the NTN system needs to perform a TA, then timing enhancement is required, as follows in Figure 1. The preamble retransmission subframe shall be delayed by K_offset. In addition, the retransmission preamble detection window of base station, which applies to all UEs in the cell, starts from subframe n+ K_offset to subframe n+ K_offset+12ms with a 12ms detection window.

Figure 1: The preamble retransmission of NB-IoT

For the second interpretation, the 12ms sets the duration of a window within which the UE only has to be ready to retransmit PRACH, and the UE can start to retransmit the preamble at any time. In this way, there is no need for enhancement, but the NodeB needs to wait until the preamble retransmission is received.

Considering the large cell radius in IoT NTN, the second interpretation corresponds to a longer window length and there is a waste of resources. Besides, the NodeB has not obtained the UE_TA related information yet at the preamble retransmission point. Hence, the NodeB does not know the exact UE-NodeB RTT, which will require one larger PRACH detection window. Due to different arrival timing, RACH detection will be problematic when multiple users send the preamble simultaneously.

From the above analysis, it is clear that the correct interpretation should be the first one. Therefore, for the retransmission of preamble, an offset is necessary to be introduced here.

Proposal 1: For NB-IoT in NTN, timing enhancement of preamble retransmission is needed with an additional K-offset.

K_mac Handling

In the last meeting, it has been agreed that the unit of K_mac is subframe based on a 15kHz subcarrier spacing (i.e. 1 ms) for IoT NTN, but for 3.75kHz, it is FFS point. In order to reduce the implementation complexity, for 3.75kHz subcarrier spacing, it is better to define a unified unit as NR NTN regardless of subcarrier spacing for FR1. Similarly, 1ms granularity of K_offset is also applied for 3.75kHz.

Proposal 2:  The unit of K_mac is 1 ms for 3.75 kHz SCS.

Proposal 3:  The unit of K_offset is 1 ms for 3.75 kHz SCS.

TA reporting

Frequently UE-specific TA reporting may cause large signaling overhead and power consumption. Though periodical TA reporting is beneficial for timely TA tracking, however, in most of cases, for example, UE-specific TA is relatively fixed for a long period, which can cover small TA variation. In this case, it’s suitable to set one threshold to trigger the aperiodic TA reporting. For example, when TA’s change exceeds one threshold, UE will report one new TA to NodeB.

Proposal 4: For UE_specific TA reporting, both event triggered and periodic methods should be supported.

Proposal 5: One threshold is used for TA report triggering.

In addition, in order to minimize reporting overhead, it’s desirable that UE only reports TA change between current TA and previous TA with time range. The time range can be pre-configured, which can be mapped to one TA range and expressed by a few bits. For example, ‘00’ denotes [-10ms, -5ms], ‘01’ denotes [-5ms, 0], ‘10’ denotes [0ms, 5ms], ‘11’ denotes [5ms, 10ms].

Proposal 6: Reporting differential TA between current TA and previous TA is preferred.

For the signaling design of TA reporting, RRC signaling or MAC signaling is preferred. Moreover, coarse TA change reporting can be used for reducing signaling overhead, rather than accurate Tc level reporting.

Proposal 7: Using RRC signaling or MAC signaling to report TA can be supported.

Proposal 8: Utilize ms as the unit of reported TA regardless of subcarrier spacing.

Conclusion

In this contribution, we analzyed potential issues of NB-IoT and eMTC over satellite and presented a few enhancements to guarantee satellite IoT performance. In sum, the proposals are listed as follows:

Proposal 1: For NB-IoT in NTN, timing enhancement of preamble retransmission is needed with an additional K-offset.

Proposal 2:  The unit of K_mac is 1 ms for 3.75 kHz SCS.

Proposal 3:  The unit of K_offset is 1 ms for 3.75 kHz SCS.

Proposal 4: For UE_specific TA reporting, both event triggered and periodic methods should be supported.

Proposal 5: One threshold is used for TA report triggering.

Proposal 6: Reporting differential TA between current TA and previous TA is preferred.

Proposal 7: Using RRC signaling or MAC signaling to report TA can be supported.

Proposal 8: Utilize ms as the unit of reported TA regardless of subcarrier spacing.

References

- RAN1 #107e Chairman notes

- 3GPP, TR 38.821, Solutions for NR to support non-terrestrial networks (NTN)

- 3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".