3GPP TSG RAN WG1 #108-e	                                	R1-2201366

e-Meeting, February 21st – March 3rd, 2022

Source:	CATT

Title:	Remaining issues on on-demand DL PRS and positioning for UEs in RRC_INACTIVE state

Agenda item:	8.5.6

Document for:	Discussion and Decision

Introduction

In RAN1#107-e meeting, the following agreements for UEs in RRC_INACTIVE state were agreed [1]:

Agreement For UE in RRC_INACTIVE state can support DL PRS processing outside and inside of the initial DL BWP: - For DL PRS processing outside of the initial DL BWP, the SCS, CP type of DL PRS can be the same or different as for the initial DL BWP - For DL PRS processing inside of the initial DL BWP, the SCS, CP type of DL PRS is the same as for the initial DL BWP. - Potential impact of retuning time and expected RSTD assistance information on DL PRS reception performance is up to RAN4 - UE capability(ies) will be defined for DL PRS processing in RRC_INACTIVE state - details are FFS - Send an LS to RAN4 on agreed by RAN1 UE behavior for reception of DL PRS in RRC INACTIVE state Agreement - For spatial relation of SRS for positioning by RRC_INACTIVE UEs, - Validity criteria for pathloss measurement (OLPC) is reused to determine validity of spatial relation for configured RS - If the UE determines that the UE is not able to meet the above validity criteria for spatial relation then the UE stops transmission of SRS resource for positioning - Note: the RS for spatial relation is a periodic or semi-persistent RS Agreement For OLPC of SRS for positioning by RRC_INACTIVE UEs, the following UE behaviour is used - If the UE determines that it is not able to accurately measure pathloss for pathloss reference RS, UE stops transmission on corresponding SRS resource set for positioning Agreement - The following options are supported for SRS for positioning transmission by RRC_INACTIVE UEs: - Option 1: - Subject to UE capability (which is a prerequisite for option 2), a UE may be configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP. - Option 2: - Subject to UE capability, a UE may be configured with an SRS for Positioning where the following parameters are additionally configured for the transmission of the SRS for Positioning during the RRC_INACTIVE state: frequency location and bandwidth, SCS, CP length. - The UE shall not transmit the SRS for Positioning when it is expected to perform UL transmissions in the initial UL BWP in RRC_INACTIVE state. - RAN1 assumes that - SRS for positioning for UEs in RRC_INACTIVE state is configured using the SRS-PosResourceSet IE - Send LS to RAN2 to define signaling for SRS for positioning configuration for RRC_INACTIVE UEs

In this contribution, we discuss the remaining issues on on-demand transmission and reception of DL PRS positioning solutions for UEs in RRC_INACTIVE state.

Discussion on the on-demand DL PRS request

In RAN1#107-e meeting, a list of higher layer parameters has been agreed for UE-initiated and LMF initiated on-demand DL PRS request [2]. In our opinion, in addition to these parameters, initial UE/gNB measurements are also beneficial for on-demand DL PRS. NR DL PRSs are normally transmitted in a beam sweeping manner. However, this approach results in inefficient usage of the DL resources, since not all beams are detectable by the UEs at all times. To improve the resource usage efficiency, on-demand DL PRS transmitted towards particular directions at a particular time should be supported for both UE-initiated and/or LMF-initiated on-demand DL PRS. The beam information can be included in the on-demand DL PRS request. Take UE-initiated on-demand DL PRS as an example, a UE can provide the DL measurements with the beam information already available in the UE to LMF when it initiates on-demand DL PRS. The measurements may include SSB/CSI-RS Indexes/DL PRS resources IDs, SS-RSRP, CSI-RSRP, etc., measured from the DL reference signals transmitted from the serving gNB and neighboring gNBs. Then, LMF forwards the reported DL measurements to the corresponding serving gNB and neighboring gNBs, where the gNBs can take the information into account and configure the DL PRS beams towards the UE location. The approach will also reduce the time needed for the UE to perform Rx beam sweeping to measure DL PRS, and the measurement latency, and improve the efficiency of DL resources.

Proposal 1: For UE-initiated on-demand DL PRS, the UE may provide the following information to the gNB and/or LMF when the UE sends an on-demand PRS request to the LMF:

- DL measurements available in UE, which may include SS-RSRP, CSI-RSRP, etc., measured from the serving gNB and neighboring gNBs.

Proposal 2: For LMF-initiated on-demand DL PRS, the LMF may request UE to provide the following information to the LMF before LMF sends an on-demand PRS request to the gNBs:

- DL measurements available in UE, which may include SS-RSRP, CSI-RSRP, etc., measured from the serving gNB and neighboring gNBs.

When a serving gNB responds to LMF-initiated on-demand DL PRS request for a UE, the serving gNB provides the information of allocated on-demand DL PRS resources to the LMF. If DL/UL measurements related to the UE are available at the gNB, the gNB may also include the DL/UL measurements in the response, since the DL/UL measurements may be useful for the LMF to determine DL PRS assistance data for the UE.

Proposal 3: When a serving gNB sends the response to LMF-initiated on-demand DL PRS for a UE, the serving gNB may provide the following information to the LMF in addition to the allocated DL PRS resources for supporting the on-demand DL PRS:

- DL measurements reported by the UE if available at the serving gNB, which may include SS-RSRP, CSI-RSRP, etc., measured from the DL RS of serving gNB and neighboring gNBs;

- UL measurements related to the UE if available at the gNB, which may include SRS-RSRP, etc.., measured by the serving gNB.

Positioning for UEs in RRC_INACTIVE state

On SRS-Pos configuration for UEs in RRC_INACTIVE state, we may also consider the following additional methods:

- Method 1: Using a paging message to send SRS-Pos configuration information. The network initiates the UL positioning procedure by transmitting the paging message, which includes SRS-Pos configuration information, at the UE's paging occasion. After receiving the paging message, the UE extracts SRS-Pos configuration information for UL SRS-Pos transmission.

- Method 2: Using SRS-Pos configuration information obtained by the UE in a new RACH procedure. Before sending SRS-Pos, the UE can initiate a new RACH process. In the process of RACH, the SRS-Pos configuration information can be obtained from Msg2 or MsgB.

Proposal 4: Support the following additional SRS-Pos configuration methods for UL positioning in RRC_INACTIVE state:

- UE obtains the SRS-Pos configuration information through the paging message.

- Introducing a new RACH procedure for UE to obtain the SRS-Pos configuration information.

In RAN1#106bis-e meeting, the following agreement was agreed [3]:

Agreement:

- From RAN1 perspective, in RRC_INACTIVE state, reception of DL PRS has lower priority than other DL signals/channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT)

  - FFS how to determine conflicts in DL PRS and other DL signals/channels reception by UE

  - FFS how to handle retuning time for the case when DL PRS and other DL signals/channels are allocated in different BW and/or have the same or different SCS as initial DL BWP

- Send LS to RAN4 (cc RAN2) and ask if there is any feedback

According to the agreement, reception of DL PRS has lower priority than both other DL signals and DL channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT). This is not adequately captured in the spec, where only DL signals are mentioned. We propose the following TP for TS 38.214 [4]:

Proposal 5: Adopt the following TP for the reception of DL PRS:

----------------Start of TP for TS38.214---------------------

- 5.1.6.5	PRS reception procedure

……

The UE in RRC_INACTIVE mode is expected to prioritize the reception of any other DL signals and DL channels than the reception of DL PRS.

……

----------------End of TP for TS38.214---------------------

Conclusion

In this contribution, we discuss on-demand DL PRS and positioning solutions for UEs in RRC_INACTIVE state, and give the following proposals:

Proposal 1: For UE-initiated on-demand DL PRS, the UE may provide the following information to the gNB and/or LMF when the UE sends an on-demand PRS request to the LMF:

- DL measurements available in UE, which may include SS-RSRP, CSI-RSRP, etc., measured from the serving gNB and neighboring gNBs.

Proposal 2: For LMF-initiated on-demand DL PRS, the LMF may request UE to provide the following information to the LMF before LMF sends an on-demand PRS request to the gNBs:

- DL measurements available in UE, which may include SS-RSRP, CSI-RSRP, etc., measured from the serving gNB and neighboring gNBs.

Proposal 3: When a serving gNB sends the response to LMF-initiated on-demand DL PRS for a UE, the serving gNB may provide the following information to the LMF in addition to the allocated DL PRS resources for supporting the on-demand DL PRS:

- DL measurements reported by the UE if available at the serving gNB, which may include SS-RSRP, CSI-RSRP, etc., measured from the DL RS of serving gNB and neighboring gNBs;

- UL measurements related to the UE if available at the gNB, which may include SRS-RSRP, etc., measured by the serving gNB.

Proposal 4: Support the following additional SRS-Pos configuration methods for UL positioning in RRC_INACTIVE state:

- UE obtains the SRS-Pos configuration information through the paging message.

- Introducing a new RACH procedure for UE to obtain the SRS-Pos configuration information.

Proposal 5: Adopt the following TP for the reception of DL PRS:

----------------Start of TP for TS38.214---------------------

- 5.1.6.5	PRS reception procedure

……

The UE in RRC_INACTIVE mode is expected to prioritize the reception of any other DL signals and DL channels than the reception of DL PRS.

……

----------------End of TP for TS38.214---------------------

Reference

3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #107-e

R1-2112976, Consolidated higher layers parameter list for Rel-17 NR, Moderator (Ericsson), RAN1#107-e.

3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #106bis-e

3GPP TS 38.214, “NR; Physical layer procedures for data”, v17.0.0