3GPP TSG RAN WG1 #108e		R1-2201381

e-Meeting, February 21st – March 3rd, 2022

Source:	Panasonic

Title: 	Discussion on TB processing over multi-slot PUSCH

Agenda Item:		8.8.1.2

Document for:	Discussion

Introduction

This document provides our view on the remaining issues related to TB processing over multi-slot PUSCH (TBoMS).

Discussion

Determination of starting coded bit

In RAN#94e, following was endorsed.

- For the determination of the index of the starting coded bit in a transmitted slot for TBoMS

  - Adopt Option C at RAN#94e

  - Task RAN1 to work on the corresponding CR(s) for RAN#95e

Option C is as follows.

- For each transmitted slot for TBoMS, the index of the starting coded bit in the circular buffer is the index continuous from the position of the last bit selected in the previous allocated slot, regardless of whether UCI multiplexing occurred in the previous allocated slot or not.

On how to implement Option C to the specification, feature lead requested to consider following aspects in RAN1#107bis-e [1].

Aspect 1: The definition of G and E for TBoMS

- Alt.1: G is redefined as the total number of coded bits available for transmission of the transport block in a slot

- Alt.2: A new variable H is introduced, only for TBoMS, defined as the total number of coded bit available for transmission of the transport block in a slot

Aspect 2: The value of G and E for TBoMS

- Interpretation 1: The starting index of circular buffer is determined assuming no UCI multiplexing, but the number of bits being selected in bit selection (value E) is determined considering UCI multiplexing.

- Interpretation 2: The starting index of circular buffer is determined assuming no UCI multiplexing and the number of bits being selected in bit selection (value E) is determined assuming no UCI multiplexing.

Aspect 3: Handling of the filler bits in TBoMS

- Direction 1: Filler bits are considered to predetermine the index of the starting bit for each allocated slot for TBoMS, to ensure no overlap exists between bit sequences transmitted over consecutive slots.

- Direction 2: Filler bits are not considered to predetermine the index of the starting bit for each allocated slot for TBoMS and overlap between bit sequences transmitted over consecutive slot is allowed.

On Aspect 1, we are fine with either Alt.1 or Alt.2, while we have slight preference to Alt.2 since the interaction between Aspect 1 and Aspects 2 could be simpler by using different variable. On Aspect 2, our interpretation of the agreed Option C was that only the starting index of circular buffer is determined assuming no UCI multiplexing and no specification change has not been agreed for the rate matching (or bit selection). On Aspect 3, we think Direction 1 is simpler than Direction 2. Therefore, our proposal is the following.

Proposal: Following alternative, interpretation, and direction are taken to implement Option C on the determination of the starting index of circular buffer to the specification.

- Aspect 1: The definition of G and E for TBoMS

  - Alt.2: A new variable H is introduced, only for TBoMS, defined as the total number of coded bit available for transmission of the transport block in a slot

- Aspect 2: The value of G and E for TBoMS

  - Interpretation 1: The starting index of circular buffer is determined assuming no UCI multiplexing, but the number of bits being selected in bit selection (value E) is determined considering UCI multiplexing.

- Aspect 3: Handling of the filler bits in TBoMS

  - Direction 1: Filler bits are considered to predetermine the index of the starting bit for each allocated slot for TBoMS, to ensure no overlap exists between bit sequences transmitted over consecutive slots.

Conclusion

In this contribution, we provide our view on remaining issues related to TB processing over multi-slot PUSCH. We made following proposal.

Proposal: Following alternative, interpretation, and direction are taken to implement Option C on the determination of the starting index of circular buffer to the specification.

- Aspect 1: The definition of G and E for TBoMS

  - Alt.2: A new variable H is introduced, only for TBoMS, defined as the total number of coded bit available for transmission of the transport block in a slot

- Aspect 2: The value of G and E for TBoMS

  - Interpretation 1: The starting index of circular buffer is determined assuming no UCI multiplexing, but the number of bits being selected in bit selection (value E) is determined considering UCI multiplexing.

- Aspect 3: Handling of the filler bits in TBoMS

  - Direction 1: Filler bits are considered to predetermine the index of the starting bit for each allocated slot for TBoMS, to ensure no overlap exists between bit sequences transmitted over consecutive slots.

- Reference

[1] R1-2200752, “Final FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2),” Moderator (Nokia, Nokia Shanghai Bell), RAN1#107bis-e