3GPP TSG RAN WG1 #108e		R1-2201382

e-Meeting, February 21st – March 3rd, 2022

Source:	Panasonic

Title: 	Discussion on the interaction between inter-slot frequency hopping and DMRS bundling

Agenda Item:		8.8.2

Document for:	Discussion

Introduction

This document provides our view on the remaining issues related to the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH / PUSCH repetitions.

Discussion

In RAN1#107e, following agreement was made.

Agreement:

- For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH / PUSCH repetitions, a UE performs the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering, based on the following Option 1.

  - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination”

    - DMRS bundling shall be restarted at the beginning of each frequency hop.

    - DMRS bundling is per actual TDW.

    - FFS: Frequency hopping pattern is determined by physical slot indices.

      - FFS: Different FH pattern determination for PUCCH and PUSCH

      - FFS: Details of FH pattern design

    - Support separate RRC configuration(s) for hopping interval and configured TDW length.

      - If hopping interval is not configured, the default hopping interval is the same as the configured TDW length

        - FFS: If both hopping interval and TDW length are not configured

      - Note: Hopping interval is only determined by the configuration of hopping interval if hopping interval is configured.

The remaining issue is to solve FFS on the above agreement.

One of FFS is whether frequency hopping pattern is based on physical slot or relative slot. In our view, following options can be considered.

- Option 1: Frequency hopping pattern is determined by physical slot indices for both PUSCH and PUCCH.

- Option 2: Frequency hopping pattern is determined by physical slot indices for PUSCH and is determined by relative slot indices for PUCCH.

- Option 3: Frequency hopping pattern is determined by physical slot indices if hopping interval is configured. Frequency hopping pattern is determined by relative slot indices if hopping interval is not configured.

Option 1 can have unified pattern between PUSCH and PUCCH. Option 2 has similar behaviour as in Rel.15/16. Configured TDW is determined by relative slot indices, i.e., first slot of PUSCH repetition is index 0 for configured TDW. Then, even if hopping interval is not configured and default hopping interval is the same as the configured TDW length, the hopping pattern may not be aligned to configured TDW if hopping pattern is determined by physical slot indices as shown in Fig.1. Option 3 can align the frequency hopping interval and configured TDW as shown in Fig.2 and then, it provides better channel estimation gain.

- PUSCH repetition with physical slot counting

- PUSCH repetition with available slot counting

Fig.1: Frequency hopping pattern based on physical slot indices

- PUSCH repetition with physical slot counting

- PUSCH repetition with available slot counting

Fig.2: Frequency hopping pattern based on relative slot indices

On the details of frequency hopping design, frequency hopping pattern based on physical slot indices is realized as following.

- The starting RB during slot ns is given by

  - RBstartns=RBstartns/NFH mod 2=0RBstart+RBoffset mod NBWPsizens/NFH mod 2=1

    - ns is the current slot number within a radio frame

    - RBstart is the starting RB within the UL BWP as calculated from the resource block assignment information

    - RBoffset is the frequency offset in RBs between the two frequency hops

    - NFH is the length of hopping interval

Frequency hopping pattern based on relative slot indices is realized as following.

- The starting RB during slot ns' is given by

  - RBstartns'=RBstartns'NFH mod 2=0RBstart+RBoffset mod NBWPsizens'NFH mod 2=1

    - ns' is the relative slot number. The slot indicated to the UE for the first PUSCH/PUCCH repetition has number 0 and each subsequent slot until the UE transmits the PUSCH/PUCCH in K slots is counted regardless of whether or not the UE transmits the PUSCH/PUCCH in the slot.

On “FFS: If both hopping interval and TDW length are not configured,” if DMRS-bundling is enabled, default hopping interval should be same as the default TDW length. If DMRS-bundling is not enabled, Rel.15/16 hopping pattern should be applied.

In RAN1#107bis-e, following agreement was made.

Agreement:

- For PUCCH repetition and PUSCH repetitions with DMRS bundling, introduce the following two RRC parameters for frequency hopping interval configuration.

  - PUCCH-Frequencyhopping-Interval

  - PUSCH-Frequencyhopping-Interval

- Note: Finalize the details (such as value range, parent IE, etc…) of these two RRC parameters in RAN#107bis-e.

The value range of frequency hopping interval is FFS. The length of frequency hopping interval should consider the length aligned with dl-UL-TransmisisonPeriodicity, which takes the value of {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10}. This is because if hopping interval is the same as the period of TDD {DL+S+UL} slots, hopping pattern can be same regardless of hopping pattern is determined by physical slot or relative slot indices. In addition, the number dividable by 20 ms would be reasonable since the current PUSCH hopping pattern based physical slot index does not include system frame number. Therefore, the potential value range of frequency hopping interval could be {2, 4, 5, 8, 10} consecutive slots. On the necessity of the value “1”, since the inter-slot frequency hopping interval enhancement is targeting for DMRS bundling, value “1” is unnecessary for the purpose of PUSCH repetition Type A and slot-based PUCCH repetition. For PUSCH repetition Type B and sub-slot-based PUCCH repetition, DMRS bundling within a slot can be performed if the value “1” is supported. If the value “1” is used only for PUSCH repetition Type B or sub-slot-based PUCCH with DMRS bundling, we can accept to support it, although our best preference is not to support it.

Based on above discussion, we think at least Option 3 condition should be satisfied. For Option 1 and 2, if the length aligned with dl-UL-TransmisisonPeriodicity is supported, the condition can be satisfied. Therefore, our proposal is as follows.

Proposal 1: Either of following option is taken.

- Option 1: Frequency hopping pattern is determined by physical slot indices for both PUSCH and PUCCH.

- Option 2: Frequency hopping pattern is determined by physical slot indices for PUSCH and is determined by relative slot indices for PUCCH.

- Option 3: Frequency hopping pattern is determined by physical slot indices if hopping interval is configured. Frequency hopping pattern is determined by relative slot indices if hopping interval is not configured.

Proposal 2: Frequency hopping pattern based on physical slot indices is realized as following.

- The starting RB during slot ns is given by

  - RBstartns=RBstartns/NFH mod 2=0RBstart+RBoffset mod NBWPsizens/NFH mod 2=1

    - ns is the current slot number within a radio frame

    - RBstart is the starting RB within the UL BWP as calculated from the resource block assignment information.

    - RBoffset is the frequency offset in RBs between the two frequency hops

    - NFH is the length of hopping interval

Proposal 3: Frequency hopping pattern based on relative slot indices is realized as following.

- The starting RB during slot ns' is given by

  - RBstartns'=RBstartns'NFH mod 2=0RBstart+RBoffset mod NBWPsizens'NFH mod 2=1

    - ns' is the relative slot number. The slot indicated to the UE for the first PUSCH/PUCCH repetition has number 0 and each subsequent slot until the UE transmits the PUSCH/PUCCH in K slots is counted regardless of whether or not the UE transmits the PUSCH/PUCCH in the slot.

Proposal 4: If both hopping interval and TDW length are not configured,

- If DMRS-budling is enabled, default hopping interval should be same as the default TDW length.

- If DMRS-budling is not enabled, Rel.15/16 hopping pattern should be applied.

Proposal 5: The value range of frequency hopping interval is {2, 4, 5, 8, 10} consecutive slots.

Proposal 6: The value “1” of frequency hopping interval can be added with the condition that it is supported only for PUSCH repetition Type B or sub-slot-based PUCCH repetition.

Conclusion

In this contribution, we provide our view related to the interaction between inter-slot frequency hopping and DMRS bundling. We made following proposals.

Proposal 1: Either of following option is taken.

- Option 1: Frequency hopping pattern is determined by physical slot indices for both PUSCH and PUCCH.

- Option 2: Frequency hopping pattern is determined by physical slot indices for PUSCH and is determined by relative slot indices for PUCCH.

- Option 3: Frequency hopping pattern is determined by physical slot indices if hopping interval is configured. Frequency hopping pattern is determined by relative slot indices if hopping interval is not configured.

Proposal 2: Frequency hopping pattern based on physical slot indices is realized as following.

- The starting RB during slot ns is given by

  - RBstartns=RBstartns/NFH mod 2=0RBstart+RBoffset mod NBWPsizens/NFH mod 2=1

    - ns is the current slot number within a radio frame

    - RBstart is the starting RB within the UL BWP as calculated from the resource block assignment information.

    - RBoffset is the frequency offset in RBs between the two frequency hops

    - NFH is the length of hopping interval

Proposal 3: Frequency hopping pattern based on relative slot indices is realized as following.

- The starting RB during slot ns' is given by

  - RBstartns'=RBstartns'NFH mod 2=0RBstart+RBoffset mod NBWPsizens'NFH mod 2=1

    - ns' is the relative slot number. The slot indicated to the UE for the first PUSCH/PUCCH repetition has number 0 and each subsequent slot until the UE transmits the PUSCH/PUCCH in K slots is counted regardless of whether or not the UE transmits the PUSCH/PUCCH in the slot.

Proposal 4: If both hopping interval and TDW length are not configured,

- If DMRS-budling is enabled, default hopping interval should be same as the default TDW length.

- If DMRS-budling is not enabled, Rel.15/16 hopping pattern should be applied.

Proposal 5: The value range of frequency hopping interval is {2, 4, 5, 8, 10} consecutive slots.

Proposal 6: The value “1” of frequency hopping interval can be added with the condition that it is supported only for PUSCH repetition Type B or sub-slot-based PUCCH repetition.