3GPP TSG RAN WG1 #108	R1-2201417

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		8.16.10

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for IAB enhancements

Document for:		Discussion and Decision

# Introduction

In RAN1#107-e the baseline for UE features has been further refined [1, 2], but IAB enhancements has not been part of the scope of RAN1#107-e. In this contribution we present our views on the UE features for IAB enhancements.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#107bis are available in the Annex.

- 31-7:

  - It is fine to confirm the text in brackets in principle. One alternative formulation that is more precise is ”Desired power range adjustment”.

- 31-8:

  - Some further clarification is needed on the intent of this FG. It should not be about enhancement to monitoring DCI 2_5 itself, but on supporting the enhanced resource configurations, which happens to be pointed by DCI 2_5.

  - Consequence if not supported should be: The IAB-node is unable to receive explicit availability indication for FDM Soft resources.

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for IAB enhancements, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#107-e, Nov. 2021

- R1-2112902, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, Moderators (AT&T, NTT DOCOMO, INC.), Nov. 2021

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support Rel-17 DesiredGuardSymbols reporting 2) Support Rel-17 ProvidedGuardSymbols reception | one or more of {31-4, 31-5} | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-2 | IAB-DU beam restriction indication | Support restricted IAB-DU Beam Indication reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-3 | IAB-MT beam recommendation indication | Support recommended IAB-MT Beam Indication transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended IAB-MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-4 | Case 6 timing alignment | Support Case 6 timing alignment indication reception |  | Yes | N/A | Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. When to perform Case 6 timing at the IAB-node cannot be controlled by the parent node. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-5 | Case 7 timing alignment | 1.) Support Case7 timing offset indication reception 2.) Support Case 7 timing at parent-node indication reception |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | 1.) Support Desired DL TX Power Adjustment reporting 2.) Support DL TX Power Adjustment reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting and reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-7 | [Desired] UL TX power adjustment | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | [31-8] | [Dynamic indication of Rel-17 or FDM soft resource availability] | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for Soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-9 | Simultaneous transmission and reception from multiple parent nodes | Support simultaneous transmission and reception from multiple parent nodes |  | Yes | N/A | Simultaneous transmission and reception is not supported in DC scenario | per BC | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |