3GPP TSG RAN WG1 #108-e		R1-2201433

e-Meeting, February 21st – March 3rd, 2022

Source:	Panasonic

Title: 	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz

Agenda Item:		8.2.5

Document for:	Discussion

# Introduction

This document provides our view on the remaining items on multi-PDSCH/PUSCH scheduling for extending NR up to 71 GHz.

# Discussion on multi-PDSCH/PUSCH scheduling

## PUCCH power control

In RAN1#107-bis-e, it has been discussed on how to calculate nHARQ-ACK for PUCCH power control when UCI payload size is equal to or less than 11 bits for both Type-1 and Type-2 codebooks when HARQ-ACK time domain bundling is configured. For Type-1 codebook PUCCH power control, this issue has been concluded. However, for Type-2 codebook PUCCH power control, this issue is still not concluded as shown in FL’s proposal #3.1-2 [1]. From our view, we support FL’s proposal #3.1-2 because the calculation of nHARQ-ACK takes into account the time domain bundling of the serving cell, and whether two sub-codebooks are generated or not. We would like to hear other company's view on FFS points.

FL’s proposal #3.1-2 For type-2 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows. - For a serving cell c configured with numberOfHARQ-BundlingGroups and with NHARQ-ACKTBG,max=1, Nm,creceived in nHARQ-ACK, TB formula can be determined based on the number of DCI formats or the number of transport blocks. - FFS details on Nm,creceived determination - If the UE is provided numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max>1 for a serving cell c, or the UE is not provided numberOfHARQ-BundlingGroups but configured with multi-PDSCH scheduling for a serving cell c, nHARQ-ACK is the summation of nHARQ-ACK,TB and nHARQ-ACK,multi where nHARQ-ACK,multi can be determined based on multi-PDSCH scheduling DCI. - FFS details on nHARQ-ACK,multi determination

Proposal 1: For type-2 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows.

- For a serving cell c configured with numberOfHARQ-BundlingGroups and with NHARQ-ACKTBG,max=1, Nm,creceived in nHARQ-ACK, TB formula can be determined based on the number of DCI formats or the number of transport blocks.

- If the UE is provided numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max>1 for a serving cell c, or the UE is not provided numberOfHARQ-BundlingGroups but configured with multi-PDSCH scheduling for a serving cell c, nHARQ-ACK is the summation of nHARQ-ACK,TB and nHARQ-ACK,multi where nHARQ-ACK,multi can be determined based on multi-PDSCH scheduling DCI.

## Time-domain bundling for Type-2 codebook

Regarding time-domain bundling for Type-2 codebook, it was agreed in RAN1#107-e as follows

Agreement For multi-PDSCH scheduling with a single DCI - Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell. - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell. - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell. - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method. - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured) - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook. - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook, - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

However, it is still not concluded whether PDSCHs corresponding to configured or valid SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups. We support to use “configured SLIVs” because we prefer to have a common rule with HARQ-ACK bit ordering, which is also based on configured SLIV position, for a simplicity of UE implementation.

Proposal 2: Support to update the following RAN1#107-e agreement as follows

Agreement

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell.

  - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell.

  - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell.

  - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method.

    - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured)

    - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case

  - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook.

  - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook,

    - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured.

# Conclusions

In this contribution, we made the following proposals and observations.

Proposals:

Proposal 1: For type-2 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows.

- For a serving cell c configured with numberOfHARQ-BundlingGroups and with NHARQ-ACKTBG,max=1, Nm,creceived in nHARQ-ACK, TB formula can be determined based on the number of DCI formats or the number of transport blocks.

- If the UE is provided numberOfHARQ-BundlingGroups with NHARQ-ACKTBG,max>1 for a serving cell c, or the UE is not provided numberOfHARQ-BundlingGroups but configured with multi-PDSCH scheduling for a serving cell c, nHARQ-ACK is the summation of nHARQ-ACK,TB and nHARQ-ACK,multi where nHARQ-ACK,multi can be determined based on multi-PDSCH scheduling DCI.

Proposal 2: Support to update the following RAN1#107-e agreement as follows

Agreement

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell.

  - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell.

  - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell.

  - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method.

    - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured)

    - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case

  - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook.

  - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook,

    - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured.

# Reference

[1] R1-2200744, " Summary #2 of PDSCH/PUSCH enhancements (Scheduling/HARQ)", Moderator (LG Electronics), RAN WG1 #107bis-e.