3GPP TSG RAN WG1 #108-e			R1- 2201441

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.6.1.1

Source:	China Telecom

Title:	Remaining issues on reduced maximum UE bandwidth for RedCap

Document for:	Discussion

Introduction

According to the Rel-17 work item description on support of reduced capability NR devices [1], the objective on reduced maximum UE bandwidths is as follows:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

In RAN1#107-e, the following agreements and working assumptions on reduced maximum UE bandwidth for RedCap were reached [2]:

Agreement - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable - It is no wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases. Agreement For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation (except for standalone use for RRM measurement) based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB Agreement For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: For SSB and CORESET#0 multiplexing pattern 1, if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

Moreover, the agreements in RAN#94-e were reached as follows [3]:

Agreement: Scheme 1 (i.e. UE in IDLE and INACTIVE monitors paging in an initial BWP associated with CD-SSB) is adopted for further work in Rel-17. Scheme 2 (i.e. UE in IDLE and INACTIVE monitors paging in an initial BWP associated with NCD-SSB) is not considered further in Rel-17. RAN2 should work on the assumption that the cell reselection measurements and cell ranking are performed based on measurements on the CD-SSB. This applies for intra- and inter-frequency measurements, and for IDLE and INACTIVE states.

This contribution provides views on remaining issues related to reduced maximum UE bandwidth for RedCap.

Reduced maximum UE bandwidth

Initial BWP for RedCap

In last RAN#94-e meeting, the agreements on CD-SSB and NCD-SSB were reached [3]. UE in IDLE and INACTIVE monitors paging in an initial BWP associated with NCD-SSB will not be considered further in Rel-17. Furthermore, UE in IDLE and INACTIVE monitors paging in an initial BWP associated with CD-SSB is adopted for further work in Rel-17. However, the case where a separate initial DL BWP does not include the entire CORESET#0 is not precluded. Considering UE needs to rely on CORESET#0 at least for acquiring SIB1, it may need to switch to CORESET #0 when the initial separate DL BWP does not contain the entire CORESET #0.

For a separate initial DL BWP configured for random access while not for paging in idle/inactive mode, it was agreed that RedCap UE does not expect it to contain SSB/CORESET#0/SIB in RAN1#107-e [2]. For random access, if CSS for RACH is not configured in the active BWP, UE can switch to the separate initial BWP. And if the CSS for RACH is not configured in the separate initial BWP, UE can switch to the MIB-configured initial BWP. It is up to UE implementation.

Proposal 1: If CSS for RACH is not configured in the active BWP, RedCap UEs can switch to the separate initial BWP, and if the CSS for RACH is not configured in the separate initial BWP, UE can switch to the MIB-configured initial BWP. It is up to UE implementation.

However, for paging in idle/inactive mode, we prefer to share the legacy paging CSS with non-RedCap UEs in the MIB-configured initial DL BWP during initial access. In the handling process of Rel-15/Rel-16, if the configuration for paging/SIBx is not transmitted in the separate BWP, UE will not monitor the corresponding messages in this BWP. Thus, during initial access, if the configuration for paging/SIBx is not transmitted in the separate initial DL BWP, RedCap UEs can monitor corresponding messages according to the legacy CSS configured in the MIB-configured initial DL BWP. Furthermore, after initial access, if CSS for paging/SIBx is not configured in the active BWP, gNB can deliver paging and SI via UE dedicated RRC signalling.

Proposal 2: During initial access, if the separate initial DL BWP does not include the entire CORESET#0, RedCap UEs can monitor the messages corresponding to the CSS configured for paging/SIBx in the MIB-configured initial DL BWP.

Proposal 3: After initial access, if CSS for paging/SIBx is not configured in the active BWP, gNB can deliver paging and SI via UE dedicated RRC signalling.

Conclusion

In this contribution, we provide the following proposals:

Proposal 1: If CSS for RACH is not configured in the active BWP, RedCap UEs can switch to the separate initial BWP, and if the CSS for RACH is not configured in the separate initial BWP, UE can switch to the MIB-configured initial BWP. It is up to UE implementation.

Proposal 2: During initial access, if the separate initial DL BWP does not include the entire CORESET#0, RedCap UEs can monitor the messages corresponding to the CSS configured for paging/SIBx in the MIB-configured initial DL BWP.

Proposal 3: After initial access, if CSS for paging/SIBx is not configured in the active BWP, gNB can deliver paging and SI via UE dedicated RRC signalling.

- References

RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson, 3GPP TSG RAN #92e, June 2021.

Final_Minutes_report_RAN1#107-e_v100, 3GPP TSG RAN WG1 Meeting #107 e-Meeting, November 11th – 19th, 2021.

Meeting Report RAN#94-e, 3GPP TSG RAN Meeting #94 e-Meeting, December 6th – 17th, 2021.