3GPP TSG RAN WG1 #108-e			R1-2201444

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:			8.8.1.3

Source:			Moderator (China Telecom)

Title:			FL Summary of joint channel estimation for PUSCH

Document for:		Discussion

- Introduction

In RAN #90 e-meeting, a new Rel-17 work item on NR coverage enhancements was approved [1] and was revised in [2]. The objective of this work item is to specify enhancements for PUSCH, PUCCH and Msg3 PUSCH for both FR1 and FR2 as well as TDD and FDD.

The detailed objectives are as follows.

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1, RAN2]

Editors’ CRs on introduction of coverage enhancements have been approved in RAN#94e [3]. This contribution is a summary of the following email discussion.

[108-e-R17-CovEnh-03] Email discussion regarding joint channel estimation for PUSCH – Jianchi (China Telecom)

- 1st check point: February 25

- Final check point: March 3

- Background

2.1 Use cases for joint channel estimation

RAN1 has identified the potential use cases for joint channel estimation for PUSCH.

- Use case 1: back-to-back PUSCH transmissions within one slot.

- Use case 2: non-back-to-back PUSCH transmissions within one slot.

  - Use case 2a: no uplink transmission in the middle of two PUSCH transmissions

  - Use case 2b: other uplink transmissions in the middle of two PUSCH transmissions

- Use case 3: back-to-back PUSCH transmissions across consecutive slots.

- Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

  - Use case 4a: no uplink transmission in the middle of two PUSCH transmissions

  - Use case 4b: other uplink transmissions in the middle of two PUSCH transmissions

- Use case 5: PUSCH transmissions across non-consecutive slots.

  - Use case 5a: no uplink transmission in the middle of two PUSCH transmissions

  - Use case 5b: other uplink transmissions in the middle of two PUSCH transmissions

Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

Note: intervening “other uplink transmissions” can be either on the same component carrier or a different component carrier.

RAN1 has discussed whether joint channel estimation can be applied to the above uses cases. In addition, during online discussion of RAN1 #106-e meeting, according to the guidance from Chair there would be no further discussion for transmissions with different TBs. Based on the discussion and agreements, the situation is summarized in the following table.

| Use cases | Repetition type A for the same TB | Repetition type B for the same TB | Transmissions with different TBs | TBoMS |
| --- | --- | --- | --- | --- |
| 1: B2B PUSCH transmission within one slot | / | Support | Not support | / |
| 2: Non-B2B PUSCH transmission within one slot | / | Not support | Not support | / |
| 3: B2B PUSCH transmissions across consecutive slots | Support | Support | No further discussion | Support |
| 4: Non-B2B PUSCH transmissions across consecutive slots | Support (4a) | Support (4a) | No further discussion | Support (4a) |
|  | Not support (4b) | Not support (4b) |  | Not support (4b) |
| 5: PUSCH transmissions across non-consecutive slots | Not support | Not support | Not support | Not support |

2.2 The maximum duration

In the LS [4][5][7] and agreed way forward [6] in RAN4, RAN4 provided answers to the related questions about the maximum duration.

| RAN1’s questions | RAN4’s answers |
| --- | --- |
| For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it? | Yes, there is a maximum duration but RAN4 has not agreed how many slots it is [4]. Depend on the outcome of “What factors determine the maximum duration”. Note: The number of slots for maximum duration means the consecutive slots. In case of non-scheduled gap and/or other channel transmission, the duration of the non-scheduled gap and/or other channel should be counted [6]. Values RAN4 being considered are 5, 8, 16 or 32 slots [7]. |
| What factors determine the maximum duration? | RAN4 has agreed that TA adjustment should be avoided across the PUSCH/PUCCH transmissions (i.e., from start of first transmission until the end of last transmission) for joint channel estimation. RAN4 is still investigating other factors impact in more detail [4]. Phase and power tolerance within the duration [6]. |
| Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH? | Yes [4]. |
| Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM? | Considering the scenario of coverage extension, RAN4 recommends to only focus on modulation orders not higher than QPSK, i.e., focus on QPSK (PUCCH and PUSCH), Pi/2 BPSK (PUCCH and PUSCH), BPSK (PUCCH). RAN4 is still discussing whether maximum duration depends on modulation order for the above modulation schemes [4]. No. Note: It has been agreed to only focus on the modulation orders not higher than QPSK [6]. |
| Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)? | No [4]. |
| Whether the maximum duration is band specific? | The length of maximum duration is [6]: - Option 1: Band specific - Option 2: FR specific UE reports the single value per band [7]. |
| Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration. | Still under discussion in RAN4 [4]. Down select among the following options once we have the results of the simulations [6]: - Option 1: Subject to a single maximum duration value. - The value is defined for a given set of factors which are depends on the conclusion for the other issues under discussion. - Option 2: Subject to multiple maximum duration value and UE could report the supported value(s). UE reports the single value per band from a set of up to 4 values, and RAN4 does not consider the value more than 32 slots for the capability for maximum duration. Values RAN4 being considered are 5, 8, 16 or 32 slots [7]. |

- Summary of contributions in RAN1#107bis-e

3.1 Time domain window

Issue #1: Events that violate power consistency and phase continuity

Issue #1-1: Events for HD-FDD RedCap UE

Based on the contributions, companies’ views on the events for HD-FDD RedCap UE are summarized as follows:

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where a dropping or cancellation of a PUSCH transmission according dropping rules in [17.2, TS 38.213].

Support: Huawei, HiSilicon, LG, Spreadtrum

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH transmissions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it.

Support: Huawei, HiSilicon, Nokia, NSB, CATT, Spreadtrum, Intel, TCL, LG, Ericsson, CMCC (?)

CMCC: Remove “even if neither of the repetitions overlaps with it”.

Panasonic, vivo: Replace “downlink reception or downlink monitoring” with “an SS/PBCH block provided by ssb-PositionInBurst”.

Huawei proposes to adopt the following 2 TPs:

TP #1

3GPP TS 38.214 ### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS *** Unchanged text is omitted *** - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1, clause 11.2A and clause 17.2 of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6, clause 11.1 and clause 17.2 of [6, TS 38.213]. *** Unchanged text is omitted ***

TP #2

3GPP TS 38.214 ### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS *** Unchanged text is omitted *** Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - For PUSCH transmissions of PUSCH repetition type A or TB processing over multiple slots, the gap between two consecutive PUSCH transmissions, overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it. - For PUCCH transmissions of PUCCH repetition, the gap between two consecutive PUCCH repetitions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it. *** Unchanged text is omitted ***

Spreadtrum proposes to adopt the following TP:

6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged part is omitted > Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: < Unchanged part is omitted > - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1, clause 11.2A and clause 17.2 of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9.2.6, clause 11.1 and clause 17.2 of [6, TS 38.213]. < Unchanged part is omitted >

Nokia proposes to adopt the following TP:

6.1.7 UE procedure for determining time domain windows for bundling DM-RS <<omitted text>> Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - A downlink reception or downlink monitoring for the case of reduced capability half-duplex UE in paired spectrum and SUL band. <<omitted text>>

CATT proposes to adopt the following TP:

6.1.7 UE procedure for determining time domain windows for bundling DM-RS << unchanged text is omitted >> Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots,or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - A downlink reception or downlink monitoring for reduced capability half-duplex UE for paired spectrum. << unchanged text is omitted >>

Intel proposes to adopt the following TP:

------------------------------ TP#2: TS 38.214----------------------------------- ### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged text omitted > Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - For the case of reduced capability half-duplex UE, a downlink reception or downlink monitoring. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols. < Unchanged text omitted >

Issue #1-2: Dynamic & semi-static events

In RAN1 #107bis-e meeting, the following two cases were discussed:

- Case 1: A semi-static event is triggered after one or multiple dynamic events. Whether a new actual TDW is created after the semi-static event?

- Case 2: A semi-static event overlaps with a dynamic event. Whether a new actual TDW is created after the semi-static event?

Fig. Illustration of case 1                          Fig. Illustration of case 2

When UE is capable of restarting DMRS bundling, it’s clear that an actual TDW is created after either semi-static event or dynamic event for the above cases. When UE is not capable of restarting DMRS bundling, it seems the behavior of actual TDW generation is not clear.

Based on the contributions, when UE is not capable of restarting DMRS bundling, majority companies think that a new actual TDW is created after a semi-static event no matter whether there are dynamic events before the semi-static event or dynamic events overlaps with the semi-static event since UE can know semi-static events beforehand, while some companies think that DMRS bundling stops until the end of nominal TDW when dynamic events occur. Detailed companies’ views are summarized as follows.

- Case 1: If a semi-static event is triggered after one or multiple dynamic events

- Option 1-1: a new actual TDW is created after the triggered semi-static event.

Support: vivo, NTT DOCOMO, Nokia, NSB, CATT, CTC, Spreadtrum, CMCC, Samsung, TCL

- Option 1-2: a new actual TDW is not created after the triggered semi-static event.

Support: InterDigital, LG, Ericsson

- Case 2: If a semi-static event overlaps with a dynamic event

- Option 2-1: a new actual TDW is created after the triggered semi-static event.

Support: InterDigital, Nokia, NSB, CATT, CTC, Spreadtrum, CMCC, Samsung, TCL

- Option 2-2: a new actual TDW is not created after the triggered semi-static event.

Support: LG, Ericsson

Nokia: The confirmed working assumption in RAN1#107-e should be read in the context that UE capability of restarting DM-RS bundling is applied for dynamic events only, i.e., “if UE is not capable of restarting DM-RS bundling in response to dynamic events, no new actual TDW is created in response to dynamic events until the end of the configured TDW”.

Samsung: Additional RRC parameter description should be discussed to capture the corresponding agreement (if any) in the UE feature.

Issue #1-3: Invalid symbol pattern for PUSCH repetition type B

Vivo proposes that for PUSCH repetition type B, invalid symbol pattern which creates a gap more than 13 OFDM symbols should be considered as a semi-static event. This issue was discussed in the last meeting and companies’ views are summarized as follows.

Companies (Qualcomm, ZTE, Panasonic, Apple, LG, Xiaomi) think no additional clarification is needed.

Companies (Nokia, Samsung, Sharp, Spreadtrum) are fine with vivo’s proposal.

Issue #1-4: Events for multi-TRP operations

In RAN1 #107bis-e meeting, the following agreement was achieved in AI 8.8.2.

Agreement PUCCH repetitions with different sets of power control parameters in multi-TRP operation should be regarded as a [semi-static] event that causes power consistency and phase continuity not to be maintained across PUCCH repetitions.

Intel proposes that PUSCH repetitions with different sets of power control parameters in multi-TRP operation should also be regarded as a semi-static event and proposes to adopt the following TP for PUSCH repetitions with different sets of power control parameters for multi-TRP operation:

----------------------------------- TP#1: TS 38.214 ----------------------------------- ### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged text omitted > - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213]. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook' or first and second sets of power control parameters are configured as described in [10, TS 38.321] and in clause 7.1.1 of [6, TS 38.213], a different SRS resource set association is or different power control parameters are used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1. - For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations or first and second sets of power control parameters, as described in [10, TS 38.321] and in clause 7.2.1 of [6, TS 38.213], different spatial relations or different power control parameters are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213]. < Unchanged text omitted > The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association or different power control parameters for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, [or in response to the use of different spatial relations for the two PUCCH transmissions of PUCCH repetition,] or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or by MAC-CE, subject to UE capability. < Unchanged text omitted >

3.2 TPC command

Issue #2: Clarification of the Rel-15/16 legacy power control procedure

In RAN1 #107bis-e meeting, the following observations were discussed on the Rel-15/16 legacy power control procedure.

Observations: - Clarification on whether absolute TPC command is supported for group common TPC with DCI format 2_2 for Rel-15/16 is needed. - If supported, whether the timeline of absolute TPC command follows the accumulate TPC command. - Clarification on the interpretation of the definition of KPUSCHi for DG-PUSCH in TS 38.214 for Rel-15/16 is needed. - Interpretation 1: KPUSCHi is defined as the number of OFDM symbols after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission occasion i. With this interpretation, value of KPUSCHi for a PUSCH transmission occasion is different from the one for another PUSCH transmission occasion among the same set of PUSCH repetitions for a TB. - Interpretation 2: KPUSCHi is defined as the number of OFDM symbols after a last symbol of a corresponding PDCCH reception and before a first symbol of the first PUSCH repetition for a TB. With this interpretation, value of KPUSCHi for all PUSCH transmission occasions are the same for the TB.

For the 1st observation, the majority companies think absolute TPC command is supported for group common TPC with DCI format 2_2 and the timeline of absolute TPC command follow the accumulate TPC command, while some company doesn’t think so. For the 2nd observation, companies have different understandings on the interpretation of the definition of K_PUSCH (i) for DG-PUSCH.

In RAN1 #107bis-e meeting, companies think it is necessary to clarify the UE behaviour in Rel-15 CR session. In RAN1 #108-e meeting, relevant contributions are submitted to Rel-15 CR session in AI 7.1. As per Chair’s guidance, this is handled in [108-e-NR-CRs-03].

Issue #3: TPC command

Issue #3-1: About the working assumption for group common TPC commands with format 2_2

In RAN1 #107e, the following working assumption was achieved for TPC commands.

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

The majority companies (Huawei, HiSilicon, Nokia, NSB, vivo, ZTE, OPPO, Sharp, CTC, Intel, Xiaomi, Samsung, TCL) support to confirm the WA. Some companies (Panasonic, Spreadtrum, Interdigital, LG, Ericsson) propose only to confirm the main bullet and 1st sub-bullet, while Apple proposes only to confirm the main bullet. LG proposes to remove the 2nd sub-bullet.

Companies’ further views on the above WA are summarized as follows:

- Remove the FFS bullet.

  - Support: Huawei, HiSilicon, vivo, ZTE, Panasonic, CTC, Xiaomi, Samsung, TCL, CMCC

  - Not Support: Intel

- Replace all the “configured TDW” to “actual TDW”.

  - Support: Huawei, HiSilicon, CMCC

CMCC proposes the following modification:

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured actual TDW, UE accumulates TPC commands without taking effect during the current configured actual TDW. TPC commands take effect after the current configured actual TDW. - If UE is not configured to accumulate TPC commands - Tthe last TPC command that would take effect within a configured actual TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured actual TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

Issue #3-2: How to capture the working assumption in Issue #3-1

During the discussion in RAN1 #107bis-e meeting, three options are discussed on how to capture the working assumption in Issue #3-1 into the spec. Based on companies’ contributions, there can be two ways to go in RAN1#108-e.

- Alt.1: Down select one of Option 0 ~ 3.

  - Option 0: For DG PUSCH or PUCCH associated with a DCI, Keep Rel-15/16 legacy power control procedure.

Support: ZTE, NTT DOCOMO

  - Option 1: Legacy definition of KPUSCHi is preserved for PUSCH transmissions without DM-RS bundling. Redefine KPUSCHi for PUSCH transmissions within a nominal TDW in case of DM-RS bundling. e.g., KPUSCHi is a number of symbols from K symbols before the start of the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i.

FFS: the value of K, e.g., K is “a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c”.

Support: Nokia, NSB

  - Option 2: Modify the TPC command value set Di, e.g. if transmission occasion i is not the first transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window.

Support: Qualcomm

  - Option 3: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands,

    - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window.

    - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself).

Support at least the spirit: CATT, Samsung, ZTE, Ericsson, Huawei, HiSilicon, vivo, Sharp, Intel, LG (?)

- Alt.2: It’s up to Editor how to capture TPC enhancement to support DMRS bundling into the specification.

Apple: For the absolute power control, in working assumption the last TPC command is applied by the UE after the current configured TDW, the received last TPC command could be smaller than the UE processing time, which could not be applied to the current configured TDW.

CTC has the following observation.

- All the three options on the TPC enhancement to support DMRS bundling can work properly for CG-PUSCH and DG-PUSCH. - For Option 1, - The timeline is same for CG-PUSCH and DG-PUSCH. - UE behavior for DG-PUSCH is different from legacy UE behavior. - For Option 2 and Option 3, - The timeline is different for CG-PUSCH and DG-PUSCH. - Legacy UE behavior for DG-PUSCH is kept. - Option 2 and Option 3 have the same effect on TPC commands.

CATT has the following proposal.

For group common TPC commands with format 2_2 for CG-PUSCH, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occuring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect from KPUSCH(i1) symbols before the transmission occasions i1 to KPUSCH(k) symbols before the transmission occasion k.

Samsung and ZTE have the following proposal.

If the UE is provided PUSCH-DMRS-bundling = ‘enable’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occuring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself)

Qualcomm proposes to capture the agreement on deferring TPC updates to the end of nominal TDWs using the following TP in 38.213, Section 7.1.1:

“If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, and transmission occasion i is not the last transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window.”

Ericsson proposes that the Rel-15/16 (slot based) transmission occasion definitions are used to specify power control operation for DCI format 2_2 and DMRS bundling and adopt the following TP for accumulated TPC operation.

If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, if a transmission occasion i containing a PUSCH transmission configured by ConfiguredGrantConfig occurs within a nominal time domain window determined as described in [6, TS 38.214], then fb,f,ci,l=fb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window. For a first transmission occasion i2 after the transmission nominal time domain window, fb,f,ci2,l=fb,f,ci1,l+i=i1+1i2m=0CDi-1δPUSCH,b,f,c(m,l), with Di as defined above.

Huawei proposes to adopt the following 2 TPs:

TP#1 for case with accumulation of TPC commands (TS 38.213).

## 7 Uplink Power control ## 7.1 Physical uplink shared channel ## 7.1.1 UE behaviour < Unchanged parts are omitted > - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is not provided tpc-Accumulation, where - The values are given in Table 7.1.1-1 - is a sum of TPC command values in a set of TPC command values with cardinality that the UE receives between symbols before PUSCH transmission occasion and symbols before PUSCH transmission occasion on active UL BWP of carrier of serving cell for PUSCH power control adjustment state , where is the smallest integer for which symbols before PUSCH transmission occasion is earlier than symbols before PUSCH transmission occasion - If the UE is provided PUSCH-DMRS-Bundling =‘enabled’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, - fb,f,c'i,l=fb,f,c'i-i0,l+m=0CDi-1δPUSCH,b,f,cm,l, where fb,f,c'0,l=fb,f,c0,l - If the transmission occasion i is the first transmission occasion within an actual time domain window determined as described in [6, TS 38.214], or if the transmission occasion i is a transmission occasion that is not within an actual time domain window, then fb,f,ci,l=fb,f,c'i,l, otherwise fb,f,ci,l=fb,f,c'i1,l where the transmission occasion i1 is the first transmission occasion within the same actual time domain window as the transmission occasion i. < Unchanged parts are omitted >

TP#2 for case without accumulation of TPC commands configured (TS 38.213).

## 7 Uplink Power control ## 7.1 Physical uplink shared channel ## 7.1.1 UE behaviour < Unchanged parts are omitted > - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - absolute values are given in Table 7.1.1-1 - If the UE is provided PUSCH-DMRS-Bundling =‘enabled’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, - fb,f,c'i,l=δPUSCH,b,f,ci,l, where fb,f,c'0,l = fb,f,c0,l - If the transmission occasion i is the first transmission occasion within an actual time domain window determined as described in [6, TS 38.214], or if the transmission occasion i is a transmission occasion that is not within an actual time domain window, then fb,f,ci,l=fb,f,c'i,l, otherwise fb,f,ci,l=fb,f,c'i1,l where the transmission occasion i1 is the first transmission occasion within the same actual time domain window as the transmission occasion i. < Unchanged parts are omitted >

Nokia proposes to adopt the following TP (TS 38.213):

***Unchanged text is omitted *** 7.1.1 UE behaviour ***Unchanged text is omitted *** - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c from K symbols before the first symbol of the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i, where the nominal time domain window is determined as described in [6, TS 38.214] and K is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c. - If the UE is not provided PUSCH-DMRS-Bundling = ‘enabled’, - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=0 - If j=1, l is provided by the value of powerControlLoopToUse - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’ and tpc-Accumulation, fb,f,c(i,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i, which is identical for all transmission occasions belonging to a nominal TDW, and equal to the absolute value δPUSCH,b,f,c carried by the last TPC command before the first symbol of the nominal TDW including transmission occasion i, and provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI - If the UE is not provided PUSCH-DMRS-Bundling = ‘enabled’, fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 ***Unchanged text is omitted *** 7.2.1 UE behaviour ***Unchanged text is omitted *** - For the PUCCH power control adjustment state for active UL BWP of carrier of primary cell and PUCCH transmission occasion - is a TPC command value included in a DCI format scheduling a PDSCH reception for active UL BWP of carrier of the primary cell that the UE detects for PUCCH transmission occasion , or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 38.212], as described in clause 11.3 - if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo - If the UE obtains a TPC command value from a DCI format scheduling a PDSCH reception and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of through the link to a corresponding p0-PUCCH-Id index - If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI, the value is provided by the closed loop indicator field in DCI format 2_2 - is the current PUCCH power control adjustment state for active UL BWP of carrier of primary cell and PUCCH transmission occasion , where - The values are given in Table 7.1.2-1 - is a sum of TPC command values in a set of TPC command values with cardinality that the UE receives between symbols before PUCCH transmission occasion and symbols before PUCCH transmission occasion on active UL BWP of carrier of primary cell for PUCCH power control adjustment state, where is the smallest integer for which symbols before PUCCH transmission occasion is earlier than symbols before PUCCH transmission occasion - If the UE is provided PUCCH-DMRS-Bundling = ‘enabled’, KPUCCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c from K symbols before the first symbol of the time domain window including the transmission occasion i to the first symbol of the transmission occasion i, where the time domain window is determined as described in [6, TS 38.214] and K is a number of symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP of carrier of primary cell . - If the UE is not provided PUCCH-DMRS-Bundling = ‘enabled’, - If the PUCCH transmission is in response to a detection by the UE of a DCI format, is a number of symbols for active UL BWP of carrier of primary cell after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission - If the PUCCH transmission is not in response to a detection by the UE of a DCI format, is a number of symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP of carrier of primary cell - If the UE has reached maximum power for active UL BWP of carrier of primary cell at PUCCH transmission occasion and , then - If UE has reached minimum power for active UL BWP of carrier of primary cell at PUCCH transmission occasion and , then ***Unchanged text is omitted ***

Vivo proposes to adopt the following TP:

------------------------------------------------Start of TP#1 for section 7.1.1 of 38.213 V17.0.0---------------------------------- 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as [dBm] where, - PCMAX,f,c(i)is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of serving cell c in PUSCH transmission occasion i. <<unchanged text omitted>> - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’ and not provided tpc-Accumulation,, - for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], fb,f,ci,l=fb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window; - for the first transmission occasion j occurs after the previous nominal time domain window, fb,f,cj,l=fb,f,ci1,l+δaccuTDW, where δaccuTDW is all the TPC command values that would take effect between the first symbol of the previous nominal time domain window and the first symbol of the current nominal time domain window. - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=0 - If j=1, l is provided by the value of powerControlLoopToUse - fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’ and provided tpc-Accumulation, - for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], then fb,f,ci,l=fb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window, - for the first transmission occasion j occurs after the previous nominal time domain window, fb,f,cj,l=δPUSCH,b,f,c(i2,l), where δPUSCH,b,f,ci2,l is the last TPC command value that would take effect before the first symbol of the current nominal time domain window. If the UE transmits a PUSCH associated with the first RS resource index qd, the UE applies the first PO_UE_PUSCH,b,f,cj value, the first αb,f,cj value, and fb,f,ci,l for determining PPUSCH,b,f,c(i,j,qd,l). If the UE transmits a PUSCH associated with the second RS resource index qd, the UE applies the second PO_UE_PUSCH,b,f,cj value, the second αb,f,cj value, and fb,f,ci,l or fb,f,ci,0 if twoPUSCH-PC-AdjustmentStates is provided or not provided, respectively, for determining PPUSCH,b,f,c(i,j,qd,l). <<unchanged text omitted>> ------------------------------------------------End of TP#1 for section 7.1.1 of 38.213 V17.0.0---------------------------------- ------------------------------------------------Start of TP#2 for section 7.2.1 of 38.213 V17.0.0---------------------------------- 7.2.1 UE behaviour If a UE transmits a PUCCH on active UL BWP b of carrier f in the primary cell c using PUCCH power control adjustment state with index l, the UE determines the PUCCH transmission power PPUCCH,b,f,c(i,qu,qd,l) in PUCCH transmission occasion i as [dBm] where - PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of primary cell c in PUCCH transmission occasion i <<unchanged text omitted>> - gb,f,ci,l=gb,f,ci-i0,l+m=0CDi-1δPUCCH,b,f,c(m,l) is the current PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i, where - The δPUCCH,b,f,c values are given in Table 7.1.2-1 - m=0CDi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i - If the UE is provided PUCCH-DMRS-Bundling = ‘enabled’, - for a transmission occasion i occurs within a nominal time domain window determined as described in [6, TS 38.214], gb,f,ci,l=gb,f,c(i1,l), where transmission occasion i1 is a first transmission occasion within the nominal time domain window; - for the first transmission occasion j occurs after the previous nominal time domain window, gb,f,cj,l=gb,f,ci1,l+δaccuTDW, where δaccuTDW is all the TPC command values that would take effect between the first symbol of the previous nominal time domain window and the first symbol of the current nominal time domain window. - If the PUCCH transmission is in response to a detection by the UE of a DCI format, KPUCCHi is a number of symbols for active UL BWP b of carrier f of primary cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission <<unchanged text omitted>> ------------------------------------------------End of TP#2 for section 7.2.1 of 38.213 V17.0.0----------------------------------

OPPO proposes to adopt the following TP (TS 38213-h00):

**************************************Start of the TP************************************** ## 7.1 Physical uplink shared channel <text omitted> fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, if TPC command is received in DCI format 2_2 and the TPC command would take into effect during a nominal TDW, as defined in clause 6.1.7 of [6, TS 38.214], the TPC commands is not applied during the nominal TDW and would be applied after the nominal TDW. - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=0 - If j=1, l is provided by the value of powerControlLoopToUse - fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 - For PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, when PUSCH-DMRS-Bundling is enabled, if TPC command(s) is received in DCI format 2_2 , and if one or more TPC command(s) that would take effect in a nominal TDW, as defined in clause 6.1.7 of [6, TS 38.214], are received, only the last TPC command is applied after the nominal TDW while the others are omitted. <text omitted> **************************************End of the TP**************************************

Intel proposes to adopt the following TP:

-----------------------------------TP#3: TS 38.213----------------------------------- 7.1.1 UE behaviour < Unchanged text omitted > - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If the UE is provided PUSCH-DMRS-bundling = ‘enable’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, fb,f,ci,l=fb,f,ci1,l, where i1 is the first transmission occasion within a nominal time domain window determined as described in [6, TS 38.214] and i is a transmission occasion within the nominal time domain window after i1. - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=0 - If j=1, l is provided by the value of powerControlLoopToUse - fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 - If the UE is provided PUSCH-DMRS-bundling = ‘enable’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, fb,f,ci,l=fb,f,ci1,l, where i1 is the first transmission occasion within a nominal time domain window determined as described in [6, TS 38.214] and i is a transmission occasion within the nominal time domain window after i1. < Unchanged text omitted >

Sharp proposes to adopt the following TP:

7.1.1 UE behaviour <Unchanged parts are omitted> - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,cm,l is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCHi symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCHi-i0 symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCHi symbols before PUSCH transmission occasion i - If the UE is provided PUSCH-DMRS-Bundling = ‘enabled’, and for processing TPC command values provided by DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, fb,f,ci,l=fb,f,c(i-i0',l), where i0'≥0 is determined such that PUSCH transmission occasion i-i0' is the first PUSCH transmission occasion within the nominal TDW if PUSCH transmission occasion i is within the nominal TDW and not the first PUSCH transmission occasion within the nominal TDW. - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission <Unchanged parts are omitted>

LG proposes that Only TPC commands expected to be applied within nominal TDW when joint channel estimation is not enabled should be accumulated and are updated in units of nominal TDW and proposes to adopt the following TP:

================== Start of Text Proposal for TS38.213 ================== 7.1.1 UE behaviour <--------------------------------------Other parts are omitted--------------------------------------> - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - When PUSCH-DMRS-Bundling is enabled and if transmission occasion i is within the first nominal TDW, m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i1-i0-1 symbols before PUSCH transmission occasion i-i1-i0 and KPUSCH(i-i1) symbols before PUSCH transmission occasion i-i1 on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i1≥0 is the largest integer that transmission occasion i1 and transmission occasion i are within same nominal TDW and i0>0 is the smallest integer for which KPUSCH(i-i1-i0) symbols before PUSCH transmission occasion i-i1-i0 is earlier than KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 - When PUSCH-DMRS-Bundling is enabled and if transmission occasion i is not within the first nominal TDW, m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i1-i0-1 symbols before PUSCH transmission occasion i-i1-i0 and KPUSCH(i-i1) symbols before PUSCH transmission occasion i-i1 on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i1≥0 is the largest integer that transmission occasion i1 and transmission occasion i are within same nominal TDW and i0>0 is the largest integer that transmission occasion i-i1-i0 is within the previous nominal TDW, for which KPUSCH(i-i1-i0) symbols before PUSCH transmission occasion i-i1-i0 is earlier than KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 - When PUSCH-DMRS-Bundling is not enabled, m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission <--------------------------------------Other parts are omitted--------------------------------------> - gb,f,ci,l=gb,f,ci-i0,l+m=0CDi-1δPUCCH,b,f,c(m,l) is the current PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i, where - The δPUCCH,b,f,c values are given in Table 7.1.2-1 - When PUCCH-DMRS-Bundling is enabled, m=0CDi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and KPUCCHi symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i - m=0CDi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and KPUCCHi symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i - m=0CDi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i <--------------------------------------Other parts are omitted--------------------------------------> ================== End of Text Proposal for TS38.213 ==================

3.3 RRC parameters

Nokia, Panasonic: The value range for both PUSCH-TimeDomainWindowLength and PUCCH-TimeDomainWindow Length is INTEGER (2, ..., 32).

ZTE:

- The value range of PUSCH-TimeDomainWindowLength is INTEGER (2..32).

- The value range of PUCCH-TimeDomainWindowLength is INTEGER (2..8).

Ericsson: Inform RAN4 that the minimum value that can be reported as a maximum duration for DMRS bundling at least for TDD should be no less than 8, and is preferably larger from a RAN1 perspective.

3.4 Others

The start of configured TDW for CG PUSCH

The issue was discussed during RAN1 #107bis-e meeting. According to the discussion, it seems the majority think the current spec is clear, which is aligned with the agreements and the Interpretation 2 illustrated in the following figure. Samsung proposes for CG-PUSCH, the start of first nominal TDW should be the first allocated slot in a configured grant periodicity.

High power UE

Vivo: For high power UE, if DMRS bundling is enabled, the power class for Tx power setting should be determined prior to the first PUSCH transmission within one configured TDW.If the percentage of uplink symbols transmitted in a certain evaluation period would exceed the duty cycle in the upcoming configured TDW, UE should reduce transmission power from the first repetition and adopt the same transmission power for all repetitions in the configured TDW.

Figure 1. Example of power change at UE due to high duty cycle

Precoding cycling

Panasonic: Specify that a UE expects to perform the same precoder of precoding cycling within an actual TDW.

PT-RS

InterDigital: Support enabling PT-RS during DM-RS bundling

Qualcomm: Support different criteria for activation of PTRS or its density for the case of joint channel estimation.

CA/DC:

MediaTek: JCE is not supported for CA/DC cases in R-17.

Applicability of DMRS Bundling

Qualcomm: Restrict DMRS bundling for PUSCH to only MCS values that correspond to QPSK or lower modulation orders.

Autonomous UE Tx power changes

MediaTek: JCE may be affected by UE autonomous Tx power changes due to PL changes and P-MPR changes satisfying SAR requirements. Our preference would be for RAN4 to not restrict the UE’s ability to perform those functions during JCE. Proposal: No restriction on UE autonomous power adjustments (e.g., due to PL changes or P-MPR changes) for JCE.

OPPO proposes to adopt the following TP for clarification on describing first PUSCH transmission, available slot and downlink monitoring (TS 38.214-h00):

**************************************Start of the TP************************************** ### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS <text omitted> - For PUSCH transmission of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when AvailableSlotCounting is enabled, and for TB processing over multiple slots: - The start of the first nominal TDW is the first slot determined for the first PUSCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot determined for PUSCH transmission after the last slot determined for PUSCH transmission of a previous nominal TDW. - For PUSCH transmissions of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 and PUSCH repetition Type A with a configured grant, when the UE is not configured with AvailableSlotCounting or when AvailableSlotCounting is disabled, and for PUSCH repetition type B: - The start of the first nominal TDW is the first slot scheduled for the first PUSCH transmission. - The end of the last nominal TDW is the last slot scheduled for the last PUSCH transmission. - The start of any other nominal TDWs is the first slot after the last slot of a previous nominal TDW. - For PUCCH transmissions of a PUCCH repetition: - The start of the first nominal TDW is the first slot determined for the first PUCCH transmission. - The end of the last nominal TDW is the last slot determined for the last PUCCH transmission. - The start of any other nominal TDWs is the first slot determined for PUCCH transmission after the last slot determined for PUCCH transmission of a previous nominal TDW. For PUSCH transmissions of a PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, PUSCH repetition Type A with a configured grant, PUSCH repetition Type B and TB processing over multiple slots, a nominal TDW consists of one or multiple actual TDWs. The UE determines the actual TDWs as follows: - The start of the first actual TDW is the first symbol of the first PUSCH transmission in a slot determined for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW. - The end of an actual TDW is - The last symbol of the last PUSCH transmission in a slot determined for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, if the actual TDW reaches the end of the last PUSCH transmission within the nominal TDW. - The last symbol of a PUSCH transmission before the event, if an event occurs which causes power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, and the PUSCH transmission is in a slot determined for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A wth with a configured grant, or PUSCH repetition type B or TB processing over multiple slots. -<text omitted> Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. Or, downlink RS or PDCCH monitoring as descried in clause 5, clause 6 and clause 11 of [6, TS 38.213]. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols. <text omitted> **************************************End of the TP**************************************

- Email discussion (1st round)

4.1 Time domain window

Issue #1: Events that violate power consistency and phase continuity

Issue #1-1: Events for HD-FDD RedCap UE

Proposal 1a:

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where a dropping or cancellation of a PUSCH transmission according dropping rules in [17.2, TS 38.213].

Companies are encouraged to provide comments on the above proposal.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support. |
| ZTE | Fine with the proposal. |
| QC | Support |
| vivo | Support. |
| Panasonic | For proposals 1a and 1b, it is not clear whether "an event is constituted for a case " is a dynamic event or semi-static event as it is not described in the texts. We would like to clarify the intention of FL for the proposals 1a and 1b shown as follows? - Firstly, just to agree it as an event in either proposal 1a or proposal 1b - Secondly, depending on whether it is triggered by dynamic or semi-static manner, the event is further categorized as a dynamic event or semi-static event @FL, please share your intention/view if we missed anything. Thanks. |
| InterDigital | Support |
| NTT DOCOMO | Support the proposal. |
| Intel | We are fine with the proposal. |
| LG | Fine with the proposal. |
| Ericsson | OK |
| CATT | Support. |
| Samsung | We don’t think there is a need for an agreement or a conclusion. Whether it is a HD-FDD RedCap UE or non-RedCap UE, if the UE can’t transmit in a slot, DMRS bundling would be interrupted. |
| Xiaomi | Support |
| Sharp | Support |
| Spreadtrum | Support |
| Apple | OK |
| CMCC | Support |

Proposal 1b:

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH transmissions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it.

Companies are encouraged to provide comments on the above proposal.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Support the proposal |
| Huawei, HiSilicon | Support. |
| ZTE | The event could be simplified as downlink reception or downlink for HD-FDD. |
| QC | Support |
| vivo | Support the proposal in principle. |
| Panasonic | Same comment as the above for the proposal 1a |
| InterDigital | Support |
| NTT DOCOMO | Support the proposal. |
| Intel | We are fine with the proposal 1b in principle. We suggest to update this as follows. Proposal 1b: - For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH transmissions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it. It is up to editor to capture this in the spec. |
| LG | Fine with the proposal. |
| Ericsson | Support |
| CATT | Support. |
| Samsung | Support. |
| Xiaomi | Support |
| Sharp | Support |
| Spreadtrum | Support |
| Apple | OK |
| CMCC | Support |
| CMCC | Support |

Issue #1-2: Dynamic & semi-static events

FL comments: This issue was raised in RAN1#107b-e and many contributions discuss this issue in RAN1#108-e. Companies have different understandings on the current spec and agreements. Clarification is needed.

Proposed observation:

Clarification on the following two cases is needed.

- Case 1: A semi-static event is triggered after one or multiple dynamic events. Whether a new actual TDW is created after the semi-static event?

- Case 2: A semi-static event overlaps with a dynamic event. Whether a new actual TDW is created after the semi-static event?

| Company | Comments |
| --- | --- |
| Nokia/NSB | Yes, in our view and based on current specification wording, a new actual TDW is always created after a semi-static event even if the semi-static event is triggered after one or multiple dynamic events or overlaps with a dynamic event. While discussing the UE capability of restarting DM-RS bundling in response to dynamic event, it was argued by the proponents that the main reason for introducing UE capability of restarting DM-RS bundling in response to dynamic events is that some UEs may not have capability of quickly reacting to dynamic events and hence cannot be prepared beforehand for maintaining power consistency and phase continuity. This difficulty does not exist for semi-static events, which are known beforehand by RRC configuration, even if they end up overlapping or being after dynamic events. Therefore, regardless of whether dynamic events happen before a semi-static event or not, the UE should always be able to prepare a new actual TDW after the semi-static event. |
| ZTE | We agree more discussion is needed. In the following figure, the semi-static event is after the dynamic grant in slot 2. The UE should be able to re-start the TDW from slot 3 as it has enough preparation time for the semi-static time. For the dynamic event at the beginning of slot 4, it may or may not trigger a new TDW based on the UE capability. Figure 1 Similarly, if the semi-static event is before or overlaps with the dynamic event, The UE should also be able to trigger a new TDW at the beginning of slot 3. - For slot 2 in Figure 1, if the dynamic event is at the end of the slot (or saying at the beginning of slot 3), it is the case that semi-static event is before the dynamic event. - For slot 2 in Figure 1, if the semi-static event is also at the beginning of event, e.g., the first symbol is a semi-static DL symbol, it is the case that two events overlap with each other. Overall, our understanding is: 1) A new TDW shall be always created if it is triggered by a semi-static event, no matter it is before or after or overlapping with a dynamic event. 2) A new TDW is not created if it is only triggered by a dynamic event and the UE has no capability of re-starting DMRS bundling. |
| QC | We assume this is in the context of a UE that has not indicated a capability to resume bundling after a dynamic event. For such a UE, we should not assume any further capability after a dynamic event has occurred. This is already quite clear from previous agreements. No further discussion is needed. |
| vivo | Support a new actual TDW is created for Case 1 and Case 2. |
| Panasonic | Yes for both cases 1 and 2 because UE is mandatory to support restarting DM-RS bundling due to semi-static event as shown in the following agreement. Agreement: - If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events. - An event is regarded as a dynamic event if it is triggered by a DCI or MAC-CE, otherwise it is regarded as a semi-static event. - Note: At least frequency hopping event is considered as semi-static event.. |
| InterDigital | In our understanding, a UE not capable of restarting bundling after a dynamic event, does not restart bundling before the end of the configured TDW. This seemed to be the intention of the confirmed WA of RAN1#107-e (“If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW.”). This seems simpler from UE (and possibly NW) implementation perspective and testing. We are fine to apply this principle to both Case 1 and Case 2, i.e. UE does not restart in that Case 2 either. |
| NTT DOCOMO | Agree with the proposed observations. It is better to clarify the two scenarios. |
| Intel | Case 1: based on the existing agreement, it is clear that if a semi-static event is triggered after one or multiple dynamic events, UE will not restart the DMRS bundling if UE does not support restarting of DMRS bundling after dynamic events. Case 2: our view is that this could be considered as a corner case. It is not clear to us why we need to consider simultaneous semi-static and dynamic events for DMRS bundling. |
| LG | We would like to point out an agreement made in the previous meeting before discussion. Agreement: Update the description of the RRC parameters PUSCH-Window-Restart and PUCCH-Window-Restart as follows. - UE bundles PUSCH DM-RS remaining in a nominal time domain window after dynamic event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements - UE bundles PUCCH DM-RS remaining in a nominal time domain window after dynamic event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements Note: Events should be excluded, which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded. It is our understanding that only for the UE with capability, actual TDW is created after the dynamic event according to the description of UE with capability. Since the description of capability depends on the dynamic event only, whether it is overlapped with semi-static event or not does not matter. Therefore for both of cases, a new actual TDW is created for UE with capability only following previous agreement. |
| Ericsson | Similar understanding as Qualcomm regarding if a UE does not support restarting DMRS bundling: as soon as a dynamic event occurs, bundling stops until the end of the current nominal TDW, regardless of the presence of a semi-static event. If a UE does support restarting DMRS bundling, the symbols excluded from bundling are the union of the symbols of all events, regardless of the types of the events. Therefore, the current definition of events in 38.214 seems clear with respect to the interaction of dynamic and semi-static events. |
| CATT | We share similar understanding with Nokia and ZTE. For both cases, we think a new actual TDW should be restarted. |
| Samsung | We don’t think there is a need to agree to these observations. Proposal 2 seems to be sufficient. |
| Xiaomi | Support a new actual TDW is created for Case 1 and Case 2. |
| Sharp | We are fine with the observation. |
| Spreadtrum | Share similar view as Nokia/NSB. Yes to both cases. |
| CMCC | Support a new TDW is created for both Case 1 and Case 2. |

FL comments: Based on companies’ contributions, the majority support a new actual TDW is created after a semi-static event no matter whether there are dynamic events before the semi-static event or dynamic events overlaps with the semi-static event since UE can know semi-static events beforehand.

Proposal 2:

For UEs not capable of restarting DM-RS bundling,

- If a semi-static event is triggered after one or multiple dynamic events, a new actual TDW is created after the triggered semi-static event.

- If a semi-static event overlaps with a dynamic event, a new actual TDW is created after the triggered semi-static event.

- Note: No specification impact is expected.

Companies are encouraged to provide comments on the above proposal.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Support the proposal |
| ZTE | As discussed above, we think adopting the following conclusion seems more accurate. - A new TDW shall be always created if it is triggered by a semi-static event, no matter it is before or after or overlapping with a dynamic event. - A new TDW is not created if it is only triggered by a dynamic event and the UE has no capability of re-starting DMRS bundling. |
| QC | Do not support. This is not in line with previous agreements. Please also see response to the observation above. |
| vivo | Support the proposal. Current spec is clear and confirms to the majority understanding. |
| Panasonic | Support the proposal 2. |
| InterDigital | Do not support, based on explanation above. |
| NTT DOCOMO | Support the proposal. |
| Intel | We do not see the need for the first sub-bullet. As mentioned above, our understanding is that if a semi-static event is triggered after one or multiple dynamic events, UE will not restart the DMRS bundling based on existing agreement if UE does not support restarting of DMRS bundling after dynamic events. For the second bullet, we are fine to make the procedure clear, although we think this is the corner case and no need to optimize it in maintenance phase. |
| LG | As commented in previous observation, we think the previous agreement regarding RRC parameter PUSCH-Window-Restart and PUCCH-Window-Restart should be revised to support this proposal. |
| Ericsson | Do not support, given the reasons above. |
| CATT | Support. |
| Samsung | Fine with the proposal. A description needs to be added in the corresponding UE feature - we suggest to add Note2 as follow: Proposal 2-v1: For UEs not capable of restarting DM-RS bundling, - If a semi-static event is triggered after one or multiple dynamic events, a new actual TDW is created after the triggered semi-static event. - If a semi-static event overlaps with a dynamic event, a new actual TDW is created after the triggered semi-static event. - Note1: No specification impact is expected. - Note2: Additional RRC parameter description should be discussed to capture this agreement in the UE feature. |
| Xiaomi | Support |
| Sharp | Support |
| Spreadtrum | Support the proposal |
| CMCC | Fine with the 1st two bullets. But considering though based on current TP companies still have different understanding, we are not sure if the there won’t be any specification impact. |

FL comments: Companies not supporting the above Proposal 2 are encouraged to provide comments on how to understand the following agreement if there are dynamic events in the first hop within the nominal TDW.

Agreement:

- DMRS bundling shall be restarted at the beginning of each frequency hop.

| Company | Comments |
| --- | --- |
| QC | No, such a UE will not be capable of restarting bundling in subsequent hops. That’s precisely what the UE capability is trying to convey. Not sure what the confusion here is. This UE is designed to be incapable to reprogramming its RF after the dynamic event to continue supporting bundling in the remaining duration. We don’t see why we should impose any further restrictions on such a UE. |
| InterDigital | The UE does not restart DMRS bundling in this case. The quoted agreement is actually a portion (sub-bullet) of a larger agreement made to define interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, within PUCCH AI in RAN1#107e. It does not concern the issue of whether to restart or not after dynamic event. |
| Intel | Our understanding is that frequency hopping is neither considered as semi-static nor dynamic event. UE would restart the DMRS bundling if frequency hopping occurs. This may not be related to the above proposals. |
| LG | Since it was agreed that frequency hopping is “considered as semi-static event” and captured in TS 38.214 as following, it is our understanding that actual TDW is created for every hop with and without capability. We do not see any controversial point here. <TS38.214> The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or by MAC-CE, subject to UE capability. |
| Ericsson | From RAN1#107, we have the following agreed. We think the behavior should be clear for a UE not capable of restarting DM-RS bundling (for dynamic events). - If the power consistency and phase continuity are violated due to an event, whether a new actual TDW is created is subject to UE capability of supporting restarting DMRS bundling. - If UE is capable of restarting DM-RS bundling, one new actual TDW is created after the event, - FFS: The start of the new actual TDW is the first available slot/symbol for PUSCH transmission after the event. - If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW. - FFS: UE capability of restarting DMRS bundling is applied only to dynamic event or not |

Issue #1-3: Invalid symbol pattern for PUSCH repetition type B

Vivo proposes that for PUSCH repetition type B, invalid symbol pattern which creates a gap more than 13 OFDM symbols should be considered as a semi-static event.

Companies are encouraged to continue the discussion on the above proposal by vivo.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Whether the invalid symbol pattern is applied or not still depends on the indicated value of a DCI field. Suggest proponents to clarify why it is a semi-static event. TS 38.212 Invalid symbol pattern indicator – 0 bit if higher layer parameter invalidSymbolPatternIndicatorDCI-0-1 is not configured; otherwise 1 bit as defined in Clause 6.1.2.1 in [6, TS 38.214]. TS 38.214, S6.1.2.1 - if the PUSCH is scheduled by DCI format 0_1, or corresponds to a Type 2 configured grant activated by DCI format 0_1, and if invalidSymbolPatternIndicatorDCI-0-1 is configured, - - if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern; - - otherwise, the UE does not apply the invalid symbol pattern; |
| ZTE | There are two kinds of invalid symbols. The first one is configured by numberOfInvalidSymbolsForDL-UL-Switching, it should be regarded as a semi-static event. Another is configured by invalidSymbolPattern, which may or may not be enabled by DCI depending on whether invalidSymbolPatternIndicatorDCI-0-1 or invalidSymbolPatternIndicatorDCI-0-2 is configured. So, it can be regarded as a semi-static event if not enabled by DCI or a dynamic event if can be enabled by DCI. Anyway, an event causing by a gap with more than 13 symbols is already captured in the specification, and whether it is a semi-static or dynamic event is also clear according to current specification. We don’t see any spec impact here. |
| vivo | The motivation is to clarify this is a semi-static event. If invalid symbol pattern for PUSCH repetition type B is configured via invalidSymbolPatternIndicatorDCI-0-1 or invalidSymbolPatternIndicatorDCI-0-2, invalid symbol pattern can be enabled by setting ‘invalid symbol pattern indicator’ field to 1 in DCI. Frequency hopping, which is configured by RRC but triggered by the hopping flag field in DCI, is considered as the semi-static event as well. The reason to consider the frequency hopping as semi static event is the ‘frequency hopping flag’ is indicated together with the UL grant, which is known prior to the first PUSCH repetition. Hence, the event indicated by the same scheduling DCI can be considered as semi-static events. Following the same logic, the invalid symbols pattern indicated by scheduling DCI for type B PUSCH repetition should also be considered as a semi-static event. |
| InterDigital | Prefer to still consider this a dynamic event. |
| NTT DOCOMO | We are fine with the proposal. However, it is against the agreement that the DMRS bundling mechanism is not optimized for PUSCH repetition type B. Hence, we prefer to deprioritize this issue if the consensus is not reached quickly. |
| Intel | We do not support the proposal. |
| LG | Just quick clarification. Isn’t it included in the following event? (captured from the list of events in TS 38.214) - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols. |
| Ericsson | The agreement to support repetition Type B was that it reuse only those mechanisms defined for repetition Type A. We do not think the proposal is in line with that agreement. |
| CATT | Taking invalid symbol pattern as an event is already captured in current spec, as pointed out by LG. Whether it is dynamic or semi-static even is clearly defined by current spec, as point out by HW and ZTE. The reason we agree to consider inter-slot hopping as semi-static is to guarantee the JCE performance, since FH is widely use. Invalid symbol pattern seems not comparable to FH in this regard. |
| Xiaomi | Taking invalid symbol pattern as an event is already captured in current spec, as pointed out by LG. |
| Sharp | It is already specified as one of the events that “the gap between any two consecutive PUSCH transmissions exceeds 13 symbols”. |
| Spreadtrum | Prefer to regard this as a dynamic event. |
| Apple | We also consider this was covered by existing event, i.e., larger than 13 symbols, as pointed out by LG. |
| CMCC | Share similar view with LG, it should be covered by the gap in-between two transmissions. |

Issue #1-4: Events for multi-TRP operations

Intel proposes that PUSCH repetitions with different sets of power control parameters in multi-TRP operation should also be regarded as a semi-static event

Companies are encouraged to provide comments on the above proposal by Intel.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Support |
| vivo | In our understanding, for PUSCH repetition for multiple TRP operation, the power control parameter for different TRP is automatically changed since associated SRS resource are changed. Hence, current spec already covers this case. From the TPs provided by Intel, we do not recognize the intention to regard that as a semi-static event. |
| Panasonic | Support |
| Intel | Support |
| LG | Fine with the proposal. |
| Ericsson | Support |
| CATT | Support. |
| Samsung | Support |
| Xiaomi | Support |
| Sharp | Support |
| Spreadtrum | Support |
| Apple | Just want to clarify the relationship between the proposal and the below agreements. To us, the proposal seems already covered by the agreements. Configuring different set of power control parameter itself doesn’t change the UE behaviour, only UE switches the transmission to another TRP with another set of power control parameter could break the power consistency, thus, it’s an event. Agreement If DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously, UL beam switching for multi-TRP operation is regarded as a semi-static event. |
| CMCC | support |

4.2 TPC command

Issue #3: TPC command

Issue #3-1: About the working assumption for group common TPC commands with format 2_2

FL comments: Since whether absolute TPC command is supported for group common TPC with DCI format 2_2 is under discussion in [108-e-NR-CRs-03]. Let’s confirm accumulate TPC command first.

@Huawei, CMCC, regarding the comment “replace configured TDW with actual TDW”, from FL perspective, we have devoted great efforts and worked out this working assumption as a compromise in RAN1#107-e, I don’t think companies are willing to re-open the discussion.

@CMCC, Regarding the comment on absolute TPC command, I would like to check if there are other companies supporting the revision.

Proposal 3: For the following working assumption,

- Confirm the main bullet

- Confirm the 1st sub-bullet for accumulate TPC commands

- Keep the 2nd sub-bullet for absolute TPC commands as working assumption and remove FFS sub-bullet.

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

Companies are encouraged to provide comments on the above proposal.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Support FL proposal |
| Huawei, HiSilicon | Thanks for FL’s reply. We understand that the current Working Assumption was a compromise. However, how the WA can be in line with the existing agreement is more important. Agreement - For non-back-to-back PUSCH/PUCCH transmissions across consecutive slots, the other uplink transmission in the middle of two PUSCH/PUCCH transmissions constitutes an event that violates power consistency and phase continuity.. According to the agreement above, the DG PUSCH transmission or URLLC transmission in Case 2 and Case 3 and Case 4 constitutes an event that violates power consistency and phase continuity for DMRS bundling for CG PUSCHs or eMBB PUSCHs, where Case 2 - 4 are, - Case 2: CG PUSCH transmissions and DG PUSCH transmission(s) are interlaced. - Case 3: CG PUSCH transmissions are cancelled by DG PUSCH transmission(s). - Case 4: eMBB PUSCH transmissions are cancelled by URLLC transmission(s) via UL CI. Replacing the “configured TDW” to “actual TDW” can align the Working Assumption with the agreement. Otherwise, since the closed-loop PC state has been changed by the agreed event for “the other uplink transmission”, during the second actual TDW (after the event) of the configured TDW the UE cannot restore the previous closed-loop PC state that has been applied in the first actual TDW because of the limited number of closed-loop PC state. For example, slot#0 and slot#1 are a configured TDW where a first PUSCH with two repetitions is scheduled in the first 7 symbols of each slot, with +1 dB PC state. A second PUSCH is scheduled at the gap between two repetitions of the first PUSCH, so that the configured TDW is split into two actual TDW. The closed-loop PC state for the second PUSCH is changed to +3 dB by its own TPC command. The UE cannot transmit the second half of the first PUSCH with +1 dB PC state anymore because its memory of PC has been refreshed into +3 dB by the event. Therefore, we have the following proposal. Proposal 3-1: For the following working assumption, - Confirm the main bullet - Confirm the 1st sub-bullet for accumulate TPC commands with “configured TDW” replaced by “actual TDW” - Keep the 2nd sub-bullet for absolute TPC commands as working assumption and remove FFS sub-bullet. |
| ZTE | Fine with the proposal. |
| QC | It might be best to NOT confirm even the main bullet. In the absence of a consensus on how TPC commands are to be handled across PUSCH repetitions, it might be best to treat TPC commands as events and move on. Lets wait for more progress from 108-e-NR-CRs-03. |
| vivo | Support the proposal. |
| Panasonic | We support the proposal 3. |
| InterDigital | Support FL proposal |
| NTT DOCOMO | Support the proposal. |
| Intel | We are fine with to confirm the working assumption. But we think the last FFS should be confirmed: - no more than 1 TPC command is expected to take effect during a configured TDW For the last FFS, our view is that for coverage enhancement, it is expected that UE would typically already apply maximum transmit power for PUSCH/PUCCH repetition We do not think this is typical scenario that gNB sends more than one TPC command during configured TDW window. |
| LG | Support the proposal for the progress. |
| Ericsson | OK to confirm the first main bullet and first subbullet. Prefer to not make any change to the status or content of the second main bullet, pending further discussion in 108-e-NR-CRs-03 and in this agenda item. Also OK to wait a little to see if 108-e-NR-CRs-03 can shed light on both main bullets. |
| CATT | We feel hesitant to support. As point out by many companies (also in Issue#-2), Interpretation 1 is correct on current DG-PUSCH power control. Therefore, for DG-PUSCH, the power among DG-PUSCH repetitions does not change by nature (Option 0), regardless JCE is configured or not, and regardless group common TPC is configured or not. Furthermore, legacy behaviour of DG-PUSCH power control (Option 0) seems even more stringent than the WA, since all repetitions must keep the same power. It seems unnecessary to confirm this WA for DG-PUSCH case. Is it aim to cover CG-PUSCH case only? |
| Samsung | Support FL proposal. |
| Xiaomi | Support FL’s proposal |
| Sharp | Support the proposal in principle, but we are not sure if we can make the confirmation on “TPC commands take effect after the current configured TDW” in the 1st sub-bullet before concluding the Rel-15 CR discussion between interpretations 1 and 2. This is related to our comment for Issue#3-2. |
| Spreadtrum | Support FL proposals. |
| Apple | We are ok to confirm the main bullet. For the first bullet, the majoritiy agreed with interpretation 1 on Kpusch, so the transmission power will not adjust during the repetition. In this case, the timeline to apply the TPC command is not clear if TPC commands take effect after the current configured TDW for DG PUSCH repetition. |
| CMCC | Thanks for FL’s reply and sorry for the late response. Our proposal is as below. The sub-bullet of the non-accumulated TPC command could be updated as below, - The last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. |

Issue #3-2: How to capture the working assumption in Issue #3-1

CTC observes that for DG-PUSCH with interpretation 1, the effect of Option 2 and Option 3 is equivalent to Option 0 and legacy UE behavior is kept.

Companies are encouraged to provide comments on the above observation.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Keeping legacy UE behavior for DG-PUSCH would not be compliant with the WA. Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. The red part of the WA would indeed not be satisfied with legacy behavior of DG-PUSCH, since all repetitions would be characterized by the same power even if multiple nominal TDW are created within the set of repetitions. |
| Huawei, HiSilicon | The observation seems based on incorrect assumption that interpretation 1 would be correct. According to interpretation 1 for DG-PUSCH, a UE is required to keep unchanged transmission power for all repetitions in all cases, which the UE is incapable of. |
| ZTE | For DG-PUSCH with interpretation 1, Option 2 is equivalent to Option 0/legacy behavior if the TPC command values received via DCI format 2_2 contained in the set Di is not modified, i.e., based on legacy rules with interpretation 1. For DG-PUSCH with interpretation 1, Option 3 is equivalent to Option 0/legacy behavior, assuming ‘would take effect’ in Option 3 follows the same rules as legacy with interpretation 1. |
| QC | I think we will need resolution in 108-e-NR-CRs-03 to be able to make any progress. If interpretation 1 prevails, then yes Option 0 might suffice for DG-PUSCH. |
| vivo | In our understanding, the observation is reasonable and we support the Interpretation 1. |
| InterDigital | Option 0 does not seem equivalent to Option 2 or Option 3 if the configured TDW does not span the whole set of repetitions. @Huawei: Interpretation 1 does not imply this in our understanding? It may imply that the same TPC adjustment is applied to all repetitions. |
| Intel | It may be good to wait for the progress in the email discussion for Rel-15/16 behaviour for TPC command |
| LG | We are not sure how legacy UE behaviour can support any options since the configured TDW is a new concept. |
| Ericsson | Agree with CTC’s observation and support Interpretation 1. |
| CATT | Fine with the observation. And, if Interpretation 1 is valid (which is likely to be), Option 0 is sufficient to DG-PUSCH. Option 0 seems even more stringent than the Option 1~3 since it mandates the same power for all repetitions. |
| Samsung | Proposal 3 (if agreed) is sufficient. Then next step is to discuss the options that FL summarizes below. |
| Xiaomi | Support interpretation 1 |
| Sharp | Even if Interpretation 1 is correct, the effect of Option 2 and Option 3 is not equivalent to Option 0. This is because all the accumulated TPC commands take effect after whole duration of all the repetitions in the legacy procedure while the accumulated TPC commands take effect after the configured TDW (which may be shorter than the whole duration of all the repetitions) in Option 2 and 3. |
| Spreadtrum | Share similar view as Intel. We can wait the progress of Rel-15/16 behaviour for TPC command. |
| Apple | Agree with CTC’s observation. |

FL comments: It seems the current situation is a bit complicated. On one hand, clarification on absolute TPC command and the definition of KPUSCHi for DG-PUSCH is under discussion in [108-e-NR-CRs-03]. On the other hand, companies have different understandings on how to capture the working assumption into the specification while it somehow depends on the outcome of discussion in [108-e-NR-CRs-03]. From FL understanding, there can be two ways to go in RAN1#108-e as listed in section 3.2.

Proposal 4:

- If interpretation 1 is the correct understanding on the definition of KPUSCHi for DG-PUSCH in TS 38.214 for Rel-15/16,

  - Down select one of Option 1 ~ 3 for CG-PUSCH in RAN1#108-e.

  - Down select one of Option 0 ~ 3 for DG-PUSCH in RAN1#108-e.

- If interpretation 2 is the correct understanding on the definition of KPUSCHi for DG-PUSCH in TS 38.214 for Rel-15/16, down select one of Option 1 ~ 3 for both CG-PUSCH and DG-PUSCH in RAN1#108-e.

- If no consensus on the above down selection can be reached in RAN1#108-e, it’s up to Editor how to capture TPC enhancement to support DMRS bundling into the specification.

- Option 0: For DG PUSCH or PUCCH associated with a DCI, Keep Rel-15/16 legacy power control procedure. - Option 1: Legacy definition of KPUSCHi is preserved for PUSCH transmissions without DM-RS bundling. Redefine KPUSCHi for PUSCH transmissions within a nominal TDW in case of DM-RS bundling. e.g., KPUSCHi is a number of symbols from K symbols before the start of the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i. - FFS: the value of K, e.g., K is “a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c”. - Option 2: Modify the TPC command value set Di, e.g. if transmission occasion i is not the first transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window. - Option 3: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself).

Companies are encouraged to provide comments on the above proposal.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Before down-selecting between the listed options, a clarification on the role of TPC commands received through DCI format 0_1 scheduling a different PUSCH than the active one with DMRS bundling is necessary. Let us take the following example: where PUSCH1~PUSCH4 are PUSCH repetitions belonging to a same nominal TDW (red dashed box) and δc is the TPC command carried by a DCI format 0_1 scheduling a DG-PUSCH (PUSCH5) outside the current nominal TDW. Let us also note that this could be a very relevant case during network operation, for a gNB to avoid breaking of phase continuity within the nominal TDW. In addition, reception of a TPC command in format 0_1 within a nominal TDW is not listed in the list of (dynamic) events causing phase and power consistency not to be maintained, meaning that neither of PUSCH2, PUSCH3 and PUSCH4 will account for δc in setting their transmission power. Assuming this is common understanding, defining K_PUSCH(i) to start at the start of the nominal TDW (Option 1) does not create any disadvantage and allows a UE to keep phase continuity and power consistency even if a TPC command in DCI 0_1 is received within the nominal TDW. On the other hand, in such a scenario it would become redundant to define two sets of Di (one for format 2_2 and one for format 0_1) since RAN1 has not agreed that reception of a TPC command in DCI 0_1 constitutes an event that breaks power consistency, and hence its application would anyway have to be postponed at least until the start of the actual PUSCH5 transmission. However, if this is not common understanding, we would appreciate if feature lead could trigger a corresponding discussion. |
| ZTE | It’s better to explicitly clarify what is interpretation 1 and 2 in the proposal. If no consensus can be made, it could be simply treat TPC as an event and there would be no spec impacts on TS 38213. |
| QC | For sure, we should not leave to Editor’s judgement. Don’t support the third bullet. Seems okay otherwise, although, we think both interpretations may not work if we are to support CG-DG interlacing. Okay to take TPC as an event as an alternative. We are okay to wait until the second week assuming some additional guidance emerges from the CR thread. @Nokia, the action time of a TPC command received via DCI format 0_1 needs additional clarification. Its not clear whether it will take effect before PUSCH2/PUSCH3/PUSCH4 --- the current description doesn’t seem to preclude this. |
| vivo | Support the proposal in principle. We can continue to discuss this issue until the outcome of discussion in [108-e-NR-CRs-03]. |
| InterDigital | Regardless of which of Interpretation 1 or 2 is correct, we would prefer down-selecting to same Option for both CG and DG. |
| NTT DOCOMO | We are generally fine with the proposal's motivation. However, Option 1 is different from Option 0, 2 and 3 in a sense that the definition of K is the symbol from the start of nominal time domain window instead of the first transmission occasion. This difference could change the time duration monitoring TPC commands, as the unit of nominal time domain windows is a slot while that of transmission occasions is a symbol. Therefore, we suggest modifying Option 1 into the following approach. - Option 1: Legacy definition of KPUSCHi is preserved for PUSCH transmissions without DM-RS bundling. Redefine KPUSCHi for PUSCH transmissions within a nominal TDW in case of DM-RS bundling. e.g., KPUSCHi is a number of symbols from K symbols before the start of the first repetition within the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i. FFS: the value of K, e.g., K is “a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c”. |
| Intel | It may be good to wait for the progress in the email discussion for Rel-15/16 behaviour for TPC command |
| Ericsson | Ok with the first two bullets for discussion now, but prefer not to have the third at least at this stage. Agree with ZTE it would be more clear to elaborate on what Interpretations 1 and 2 are (maybe reference the FL summary) if we go for a formal agreement. |
| CATT | We have the same understanding with Nokia that the δc in the TPC command in DCI 0_1 would not be accumulated in neither of PUSCH2, PUSCH3 and PUSCH4. According to the following highlighted description in TS38.213, only the δc in the scheduling DCI for the PUSCH transmissions is included in the set of accumulated TPC command as well as other group common TPC commands. Since there is no scheduling DCI for CG-PUSCH repetitions, only TPC commands in DCI format 2_2 should be considered. - For the PUSCH power control adjustment state fb,f,c(i,l) for active UL BWP b of carrier f of serving cell c in PUSCH transmission occasion i - δPUSCH,b,f,c(i,l) is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in clause 11.3. Hence we think it is necessary to introduce the advance for the start of the configured TDW (details in R1-2201375). Also we echo ZTE and QC’s view that if no consensus can be made, it could be simply list TPC as an event, so no spec changes on power control in 38.213. |
| Samsung | We don’t think there is a need for any agreement on the interpretation of the current specifications, and/or consider different options for CG-PUSCH and DG-PUSCH. Option 3 seems to capture the WA. |
| Sharp | If interpretation 1 is correct, Option 0 should be supported. This is because it is unnecessary restriction that accumulated TPC commands are applied at every configured TDW while the accumulated TPC commands are applied after end of repetition in the legacy procedure and the configured TDW is equal to or smaller than duration of all the repetitions. |
| Apple | Ok with first two proposal. if no consensus could be reached, considering the TPC as event seems still problematic, when to apply the TPC command is still debatable, i.e., timeline. |

Proposal 5: Agree in principle to the following in order to capture the working assumption on group common TPC commands with format 2_2.

- For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands,

    - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window.

    - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself).

Companies are encouraged to provide comments on the above proposal.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Do not support for the reasons stated above |
| Huawei, HiSilicon | As commented above, replacing the “configured TDW” to “actual TDW” can align the Working Assumption with the existing agreement. Therefore, we suggest Proposal 5-rev: Agree in principle to the following in order to capture the working assumption on group common TPC commands with format 2_2. - For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a [nominal] time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the [nominal] time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself). - FFS: how to accommodate the case where a sequence of CG PUSCH repetitions is interrupted by a DG-PUSCH, e.g. whether or not “nominal” TDW above should be replaced with actual TDW |
| ZTE | Fine with the proposal. |
| QC | The second sub-bullet may or may not line up with legacy behavior. Lets wait for more clarity. |
| vivo | Support the proposal. |
| Intel | We are fine with the proposal 5. |
| Ericsson | OK with the proposal |
| CATT | The proposal needs update. As commented above, we think the start and the end time period of TPC accumulation for δj should be slightly modified, i.e. advanced. In addition, we hope to figure out whether this is only applied to CG-PUSCH. |
| Samsung | Support Proposal 5. |
| Xiaomi | Fine with the proposal |
| Sharp | Support the proposal |
| Spreadtrum | Support |
| Apple | Sharing the similar view as CATT, the timeline to determine δj is still not clear for DG PUSCH. |

4.3 RRC parameters

FL comments: As per Chair’s guidance, RAN1 will send the updated RRC parameters to RAN2 at the end of Week 1, which means that we have to finalize all relevant issues in Week 1. There are remaining issues: value range for PUSCH-TimeDomainWindowLength and PUCCH-TimeDomainWindowLength, and default value for PUCCH-TimeDomainWindowLength. For the value range, since it depends on RAN4, the discussion will be open after the progress in RAN4. As per Chair’s guidance, another LS will be sent to RAN2 at the end of Week 2 including this issue.

For PUSCH-TimeDomainWindowLength, the default value is as follows.

- For PUSCH repetition type A/B, if PUSCH-TimeDomainWindowLength is not configured, the default value of PUSCH-TimeDomainWindowLength is the minimum value in the unit of consecutive slots of the time duration for the transmission of K repetition and the maximum duration defined in TS38.101-1/2.

- For TBoMS, if PUSCH-TimeDomainWindowLength is not configured, the default value of PUSCH-TimeDomainWindowLength is the minimum value in the unit of consecutive slots of the duration of TBoMS transmission (including repetition of TBoMS) and the maximum duration defined in TS38.101-1/2.

It seems the default value for PUCCH-TimeDomainWindowLength is missing.

Proposal 6:

- For PUCCH repetition, if PUCCH-TimeDomainWindowLength is not configured, the default value of PUCCH-TimeDomainWindowLength is the minimum value in the unit of consecutive slots of the time duration for the transmission of K repetition and the maximum duration defined in TS38.101-1/2.

Companies are encouraged to provide comments on the above proposal.

| Company | Comments |
| --- | --- |
| Nokia/NSB | Our understanding is that this is already captured in TS 38.214. We would appreciate if FL could elaborate more on the need of this proposal. |
| vivo | Support. |
| Panasonic | We do not understand motivation for a need of the proposal 6. This is because it repeats the same thing that we agreed in the 2nd bullet of the below agreement. Agreement: Support Option 1’-a Option 1’-a: - If L is configured, the maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements. - If L is not configured, the default value of L = min (maximum duration, duration of all PUSCH repetitions). |
| NTT DOCOMO | Support the proposal. |
| Intel | Share similar view as Nokia. |
| LG | Same view with Nokia/NSB and Intel. |
| Ericsson | Same view/question as Nokia. |
| CATT | Understand the intension but seems Nokia is correct. In latest 214: - For PUCCH transmissions of PUCCH repetition, the duration of each nominal TDW except the last nominal TDW, in number of consecutive slots, is: - Given by PUCCH-TimeDomainWindowLength, if configured. - Computed as min ([maxDMRS-BundlingDuration], M), if PUCCH-TimeDomainWindowLength is not configured, where M is the time duration in consecutive slots from the first slot determined for PUCCH transmissions of PUCCH repetition to the last slot determined for PUCCH transmissions of PUCCH repetition according to clause 9.2.6 of [6, TS 38.213].. |
| Samsung | OK |
| Xiaomi | support |
| Spreadtrum | Support |

4.4 Others

FL comments: Regarding the issues summarized in section 3.4. These issues seem not so critical or they have already been discussed in previous meetings.

Any comments on the issues summarized in section 3.4?

| Companies | Comments |
| --- | --- |
| Samsung | Regarding “Panasonic: Specify that a UE expects to perform the same precoder of precoding cycling within an actual TDW” – the use of same precoder within the actual TDW seems reasonable. Given we are in maintenance phase, it is worth further considering this proposal only if consensus can be reached quickly. |

- Proposals for GTW

Proposal 1a:

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where a dropping or cancellation of a PUSCH transmission according dropping rules in [17.2, TS 38.213].

Support: Huawei, HiSilicon, ZTE, Qualcomm, vivo, InterDigital, NTT DOCOMO, Intel, LG, Ericsson, CATT, Xiaomi, Sharp, Spreadtrum, Apple, CMCC

Not needed: Samsung

Proposal 1b:

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH transmissions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it.

Support: Nokia, NSB, Huawei, HiSilicon, ZTE, Qualcomm, vivo, InterDigital, NTT DOCOMO, LG, Ericsson, CATT, Samsung, Xiaomi, Sharp, Spreadtrum, Apple, CMCC

Proposed observation:

Clarification on the following two cases is needed.

- Case 1: A semi-static event is triggered after one or multiple dynamic events. Whether a new actual TDW is created after the semi-static event?

- Case 2: A semi-static event overlaps with a dynamic event. Whether a new actual TDW is created after the semi-static event?

- Agreements at RAN1#107b-e

Conclusion:

- It is not expected to redefine transmission occasion for PUSCH/PUCCH for DMRS bundling in Rel-17.

Agreement:

- The value range of PUSCH-TimeDomainWindowLength is INTEGER (2..[32]).

- The value range of PUCCH-TimeDomainWindowLength is INTEGER (2..[8]).

- Note: the value shall not exceed the maximum duration.

Agreement:

Adopt the following TP to TS 38.214

6.1.7 UE procedure for determining time domain windows for bundling DM-RS < unchanged text omitted> - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213]. < unchanged text omitted>

Agreement:

Send an LS to RAN4 asking the following question

- For extended CP, is 11-symbol the maximum length for the non-zero un-scheduled gap in-between the PUSCH transmission or PUCCH repetition, when UE is required to maintain power consistency and phase continuity?

Agreement

Final LS R1-2200773 on DMRS bundling for PUSCH and PUCCH is endorsed.

Agreement:

- If DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously, UL beam switching for multi-TRP operation is regarded as a semi-static event.

Agreement:

Update the description of the RRC parameters PUSCH-Window-Restart and PUCCH-Window-Restart as follows.

- UE bundles PUSCH DM-RS remaining in a nominal time domain window after dynamic event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements

- UE bundles PUCCH DM-RS remaining in a nominal time domain window after dynamic event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements

Note: Events should be excluded, which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.

- Agreements at RAN1#107-e

Agreement:

Support Option 1’-a

Option 1’-a:

- If L is configured, the maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- If L is not configured, the default value of L = min (maximum duration, duration of all PUSCH repetitions)

Agreement:

- For non-back-to-back PUSCH/PUCCH transmissions across consecutive slots, the other uplink transmission in the middle of two PUSCH/PUCCH transmissions constitutes an event that violates power consistency and phase continuity.

Conclusion:

- Dynamic indication of the window length L of the configured TDW by DCI or indicated by TDRA table with one additional entry is not supported.

Agreement:

This working Assumption is confirmed.

Working assumption: - The start of the first actual TDW is the first available symbol (at least determined by TDRA table) in available slot for the first PUSCH transmission in an available slot within the configured TDW. - The end of the actual TDW is - the last available symbol (at least determined by TDRA table) in available slot for the last PUSCH transmission in an available slot within the configured TDW if the actual TDW reaches the end of the last PUSCH transmission within the configured TDW. - the last available symbol (at least determined by TDRA table) in available slot of the PUSCH transmission right before the event if an event occurs that violates power consistency and phase continuity, and the PUSCH transmission is in an available slot. - For UE capable of restarting DM-RS bundling, the start of the new actual TDW is the first available symbol (at least determined by TDRA table) in available slot for PUSCH transmission after the event violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

Agreement:

- The action of gNB indicated TA commands constitutes an event that violates power consistency and phase continuity.

Agreement:

- If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

  - An event is regarded as a dynamic event if it is triggered by a DCI or MAC-CE, otherwise it is regarded as a semi-static event.

  - Note: At least frequency hopping event is considered as semi-static event.

Working assumption:

- The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity.

  - If UE is configured to accumulate TPC commands,

    - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW.

  - If UE is not configured to accumulate TPC commands

    - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW.

      - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

Agreement:

The following working Assumption is confirmed.

Working assumption: For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs. - Each configured TDW consists of one or multiple consecutive physical slots. - The window length L of the configured TDW(s) can be explicitly configured with a single value and L is no longer than the maximum duration. - FFS: The maximum value of L is the duration of all repetitions - FFS: Solutions to error propagation issue if for L is longer than the maximum duration is to be discussed further. - FFS: The window length L is configured per UL BWP - The start of the first configured TDW is the first PUSCH transmission - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission. - The start of other configured TDWs can be implicitly determined prior to first repetition. - FFS: The configured TDWs are consecutive for paired spectrum/SUL band - FFS: The start of the configured TDWs for unpaired spectrum is implicitly determined based on semi-static DL/UL configuration. - The end of the last configured TDW is the end of the last PUSCH transmission. - FFS: The end of the configured TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission. - Within one configured TDW, one or multiple actual TDWs can be implicitly determined: - The start of the first actual TDW is the first PUSCH transmission within the configured TDW. - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission. - After one actual TDW starts, UE is expected to maintain the power consistency and phase continuity until one of the following conditions is met, then the actual TDW is ended. - The actual TDW reaches the end of the last PUSCH transmission within the configured TDW. - FFS: The end of the actual TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission. - An event occurs that violates power consistency and phase continuity - FFS: The events may include e.g., a DL slot based on DL/UL configuration for unpaired spectrum, the actual TDW reaches the maximum duration, DL reception/monitoring occasion for unpaired spectrum, high priority transmission, frequency hopping, precoder cycling. - FFS: The end of the actual TDW is the last available slot/symbol of the PUSCH transmission right before an event such that the power consistency and phase continuity are violated. - If the power consistency and phase continuity are violated due to an event, whether a new actual TDW is created is subject to UE capability of supporting restarting DMRS bundling. - If UE is capable of restarting DM-RS bundling, one new actual TDW is created after the event, - FFS: The start of the new actual TDW is the first available slot/symbol for PUSCH transmission after the event. - If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW. - FFS: UE capability of restarting DMRS bundling is applied only to dynamic event or not Note 1: A ‘configured TDW’ refers to a time domain window whose length can be configured to ‘L’ and whose start and end is determined as described above. Note 2: An ‘actual TDW’ refers to a time domain window during whose entire duration the DM-RS bundling is actually applied. An ‘actual TDW’ duration is always less than or equal to the ‘configure TDW’ duration. Note 3: Whether the terms ‘configured TDW’ and ‘actual TDW’ are revised to other terms and if such terminology is used in specifications is to be further discussed.

Agreement:

- The candidate values of the window length L of the configured TDW can be any integer value that is larger than 1 and no larger than the maximum duration.

Agreement: The following agreement is clarified as follows.

- For PUSCH repetition type A counting based on available slots,

  - “The configured TDWs are determined based on available slots” in the agreement means “The start of the configured TDWs is determined based on available slots”

Agreement - For PUSCH repetition type A counting based on physical slots - The configured TDWs are consecutive, where the start of other configured TDWs is the first physical slot right after the last physical slot of a previous configured TDW. - For PUSCH repetition type A counting based on available slots - The configured TDWs are determined based on available slots, where start of a configured TDWs is the first available slot after the last available slot of a previous configured TDW. - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Agreement:

- UE should not perform UE autonomous TA adjustment during the actual time domain window.

Agreement:

- The TDW determination procedure agreed for PUSCH repetition type A is reused, when applicable, for PUSCH repetition type B and TBoMS with or without repetition.

- No additional specification enhancements for PUSCH repetition type B and TBoMS.

Agreement:

- If DMRS bundling and UL beam switching for multi-TRP operation are configured simultaneously, UL beam switching for multi-TRP operation constitutes an event that violates power consistency and phase continuity.

  - FFS: UL beam switching for multi-TRP operation is regarded as a semi-static event.

- Agreements at RAN1#106b-e

Agreement:

- For PUSCH repetition type A counting based on physical slots

  - The start of the first configured TDW is the first physical slot for the first PUSCH transmission.

  - The end of the last configured TDW is the last physical slot for the last PUSCH transmission.

- For PUSCH repetition type A counting based on available slots

  - The start of the first configured TDW is the first available slot for the first PUSCH transmission.

  - The end of the last configured TDW is the last available slot for the last PUSCH transmission.

  - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Conclusion:

- Joint channel estimation over PUSCH transmissions across non-consecutive slots is not supported in Rel-17.

Agreement:

Down-select one of the following options in this meeting:

Option 1:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

Option 1’:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

    - If L is not configured, the configured TDW length is equal to all repetitions

    - If L is not configured, default behavior should be defined, e.g., the configured TDW length is equal to all repetitions

Option 3’:

- Whether the window length L of the configured TDW can be longer than maximum duration is subject to UE capability.

  - If UE is capable of L being longer than maximum duration,

    - The maximum value of the window length L of the configured TDW is the duration of all repetitions.

      - FFS: whether L cannot be other values other than the duration of all repetitions, if it is longer than the maximum duration.

    - If L is longer than the maximum duration, UE does not expect dynamic events.

      - FFS: details of dynamic events

Agreement

- For DG-PUSCH, Type1 CG-PUSCH and Type2 CG-PUSCH, the window length L of the configured TDW is at least configured by RRC.

- FFS: For DG-PUSCH and Type2 CG-PUSCH, whether the window length L of the configured TDW can be indicated by DCI or indicated by TDRA table with one additional entry.

Agreement

- The window length L of the RRC configured TDW is configured separately for PUSCH and PUCCH.

  - For PUSCH, L is configured per BWP.

- FFS whether the window length L can be configured with each row in the TDRA table

Agreement

- For PUSCH repetition type A counting based on physical slots

  - The configured TDWs are consecutive, where the start of other configured TDWs is the first physical slot right after the last physical slot of a previous configured TDW.

- For PUSCH repetition type A counting based on available slots

  - The configured TDWs are determined based on available slots, where start of a configured TDWs is the next first available slot after the conclusion last available slot of a previous configured TDW.

  - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Working assumption:

- The start of the first actual TDW is the first available symbol (at least determined by TDRA table) in available slot for the first PUSCH transmission in an available slot within the configured TDW.

- The end of the actual TDW is

  - the last available symbol (at least determined by TDRA table) in available slot for the last PUSCH transmission in an available slot within the configured TDW if the actual TDW reaches the end of the last PUSCH transmission within the configured TDW.

  - the last available symbol (at least determined by TDRA table) in available slot of the PUSCH transmission right before the event if an event occurs that violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

- For UE capable of restarting DM-RS bundling, the start of the new actual TDW is the first available symbol (at least determined by TDRA table) in available slot for PUSCH transmission after the event violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

Agreement

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for one TB processed over multiple slots

    - It’s subject to UE capability

    - if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A

Agreement

- For non-back-to-back PUSCH transmissions across consecutive slots (no uplink transmission in the middle of two PUSCH transmissions), support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over non-back-to-back PUSCH transmissions for one TB processed over multiple slots

    - It’s subject to UE capability

    - if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A

Agreement

Down-select one of the following options:

- Option 1: If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

- Option 2: UE capability of restarting DMRS bundling is applied to both semi-static events and dynamic events.

Agreement

- Support at least the following events that violate power consistency and phase continuity.

  - Dropping/cancellation based on Rel-15/16 collision rules.

  - FFS: Rel-17 collision rules.

  - DL slot or DL reception/monitoring based on semi-static DL/UL configuration for unpaired spectrum.

  - FFS: Other UL transmission in between PUSCH/PUCCH transmissions.

  - Gap between two PUSCH/PUCCH transmissions exceeds 13 symbols.

  - FFS: Transmission parameters need to be changed due to network-indicated operations, including: Tx power, UL beam/TPMI, and RB allocation.

  - FFS: TPC command.

  - FFS: TA adjustment.

  - FFS: The actual TDW reaches the maximum duration.

  - FFS: Frequency hopping.

  - FFS: Precoder cycling.

  - FFS: other events.

  - FFS: whether events are semi-static events or dynamic events.

  - FFS: the time duration of an event.

Agreement

- Introduce two RRC parameters to indicate enabling of DM-RS bundling and the window length of the configured TDW respectively.

Agreement

- Introduce a new RRC parameter for when UE restarts a PUSCH bundling window

- Agreements at RAN1#106-e

Agreement: Confirm the following working assumption.

Working assumption:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over non-back-to-back PUSCH transmissions with different TBs

  - FFS: Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

Conclusion

- Optimization of DMRS location in time domain for PUSCH is not considered for joint channel estimation in Rel-17.

Agreement

- Joint channel estimation for PUSCH transmissions and the time domain window are jointly enabled or disabled via RRC configuration for a UE.

  - Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity.

Agreement

Make down-selection between the following two alternatives:

- Alt 1: UE is not expected to receive TPC commands during the current time domain window.

- Alt 2: UE receives and accumulates TPC commands without taking effect during the current time domain window.

Agreement

- UE should not perform TA adjustment during the time domain window.

  - FFS: UE does not expect to receive TA command to indicate TA adjustment during the TDW.

  - FFS: UE ignores any TA command which indicates TA adjustment during the TDW.

  - FFS: UE performs TA adjustment after the TDW if it receives any TA command indicating TA adjustment during the TDW.

Working assumption:

For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs.

- Each configured TDW consists of one or multiple consecutive physical slots.

- The window length L of the configured TDW(s) can be explicitly configured with a single value and L is no longer than the maximum duration.

  - FFS: The maximum value of L is the duration of all repetitions

  - FFS: Solutions to error propagation issue if for L is longer than the maximum duration is to be discussed further.

  - FFS: The window length L is configured per UL BWP

- The start of the first configured TDW is the first PUSCH transmission

  - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission.

- The start of other configured TDWs can be implicitly determined prior to first repetition.

  - FFS: The configured TDWs are consecutive for paired spectrum/SUL band

  - FFS: The start of the configured TDWs for unpaired spectrum is implicitly determined based on semi-static DL/UL configuration.

- The end of the last configured TDW is the end of the last PUSCH transmission.

  - FFS: The end of the configured TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission.

- Within one configured TDW, one or multiple actual TDWs can be implicitly determined:

  - The start of the first actual TDW is the first PUSCH transmission within the configured TDW.

    - FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission.

  - After one actual TDW starts, UE is expected to maintain the power consistency and phase continuity until one of the following conditions is met, then the actual TDW is ended.

    - The actual TDW reaches the end of the last PUSCH transmission within the configured TDW.

      - FFS: The end of the actual TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission.

    - An event occurs that violates power consistency and phase continuity

      - FFS: The events may include e.g., a DL slot based on DL/UL configuration for unpaired spectrum, the actual TDW reaches the maximum duration, DL reception/monitoring occasion for unpaired spectrum, high priority transmission, frequency hopping, precoder cycling.

      - FFS: The end of the actual TDW is the last available slot/symbol of the PUSCH transmission right before an event such that the power consistency and phase continuity are violated.

  - If the power consistency and phase continuity are violated due to an event, whether a new actual TDW is created is subject to UE capability of supporting restarting DMRS bundling.

    - If UE is capable of restarting DM-RS bundling, one new actual TDW is created after the event,

      - FFS: The start of the new actual TDW is the first available slot/symbol for PUSCH transmission after the event.

    - If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW.

    - FFS: UE capability of restarting DMRS bundling is applied only to dynamic event or not

Note 1: A ‘configured TDW’ refers to a time domain window whose length can be configured to ‘L’ and whose start and end is determined as described above.

Note 2: An ‘actual TDW’ refers to a time domain window during whose entire duration the DM-RS bundling is actually applied. An ‘actual TDW’ duration is always less than or equal to the ‘configure TDW’ duration.

Note 3: Whether the terms ‘configured TDW’ and ‘actual TDW’ are revised to other terms and if such terminology is used in specifications is to be further discussed.

- Agreements at RAN1#105-e

Agreement:

- Joint channel estimation over non-back-to-back PUSCH transmissions within one slot is not supported.

Agreement:

- Definition of the maximum duration: a maximum time duration during which UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- FFS whether or not such a definition is necessary for RAN1 specifications.

  - Note: whether such a definition is to be specified in RAN4 specifications is up to RAN4.

- FFS the maximum duration may be reported by UE.

- Note: it is understood that for a UE, the maximum duration is no less than the time domain window duration

Agreement: Send LS to RAN4 asking the following questions

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

  - What factors determine the maximum duration?

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

  - Whether the maximum duration is band specific?

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

Agreement:

- Optimization of DMRS granularity in time domain for PUSCH is not considered for joint channel estimation in Rel-17.

Agreement:

- For back-to-back PUSCH transmissions within one slot, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A with consecutive slots

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

- Joint channel estimation over back-to-back PUSCH transmissions with different TBs within one slot is not supported.

Working assumption:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over non-back-to-back PUSCH transmissions with different TBs

  - FFS: Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

Agreement:

- Joint channel estimation for PUSCH transmissions is enabled or disabled via RRC configuration for a UE

  - FFS: whether additional dynamic signaling is needed to enable/disable joint channel estimation for PUSCH transmissions

  - Note: the enabling of such a feature is subject to certain prerequisites

  - FFS RRC parameter details (including explicit vs. implicit configuration)

- FFS For joint channel estimation for PUSCH, the time domain window is not explicitly enabled or disabled separately from joint channel estimation.

Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity.

Agreement:

For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, down select one of the following alternatives for the time domain window.

- Alt 1: All the repetitions are covered by one single time domain window

  - The start of the window is the first PUSCH transmission

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum

  - FFS: frequency hopping and precoder cycling

- Alt 2: All the repetitions are covered by one or multiple time domain windows

  - For the start of each window,

    - The start of the first window is the first PUSCH transmission.

    - FFS: how to determine the start of other windows, e.g., whether multiple windows are consecutive or non-consecutive, whether the start of the window depends on DL/UL configuration for unpaired spectrum

  - For the length of each window,

    - FFS Each window consists of at least two adjacent physical slots for UL transmission.

    - The length of each window is no longer than the maximum duration.

    - FFS: how to determine the length of each window

    - FFS: whether the length of each window depends on DL/UL configuration for unpaired spectrum

  - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum.

  - FFS: frequency hopping and precoder cycling

- Other alternatives are not precluded.

- Agreements at RAN1#104b-e

Agreements:

- For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window

  - FFS the units the time domain window (e.g. repetitions, slots, and/or symbols)

    - FFS : association between the potential use case(s) and units of the time window

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS whether or not to further consider impacting of timing advance

Agreements:

- A new DMRS pattern equally spaced among PUSCH transmissions is not considered for joint channel estimation in Rel-17.

Agreements:

- For inter-slot frequency hopping with inter-slot bundling, down select on the following two options:

  - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size.

  - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size.

    - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined.

    - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD.

    - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

Conclusion:

- For optimization of DMRS granularity in time domain with joint channel estimation, the proponents are encouraged to provide more simulation results in next meeting

Agreements:

- For the time domain window for joint channel estimation, down select on the following two options:

  - Option 1: The unit of the time domain window is defined separately for the following PUSCH transmissions:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

  - Option 2: The unit of the time domain window is the same for the following PUSCH transmission:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

Agreement:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over back-to-back PUSCH transmissions with different TBs

- Agreements at RAN1#104e

Agreements:

- Following potential use cases are considered for joint channel estimation for PUSCH:

  - Use case 1: back-to-back PUSCH transmissions within one slot.

  - Use case 2: non-back-to-back PUSCH transmissions within one slot.

  - Use case 3: back-to-back PUSCH transmissions across consecutive slots.

  - Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

  - Use case 5: PUSCH transmissions across non-consecutive slots.

Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

Agreements:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation at least for the following case:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant

  - FFS details (including possible other cases)

Agreements:

- For joint channel estimation, define a time domain window is introduced to facilitate further discussion, during which UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS: whether the window should be specified

  - FFS: the length of the time domain window is defined by a set of repetitions/slots/symbols

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: the time domain window may or may not be configured or specified.

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS: Whether the window is determined by the power consistency and phase continuity requirements and/or by other factors is to be decided.

Agreements:

- Companies are encouraged to study optimization of DMRS granularity in time domain with joint channel estimation, including:

  - Use cases

  - Simulations results

  - Enhanced schemes, e.g.,

    - Different DMRS density for different PUSCH transmissions

    - No DMRS for some PUSCH transmissions

  - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method.

- Companies are encouraged to study optimization of DMRS location in time domain with joint channel estimation, including:

  - Use cases

  - Simulations results

  - Enhanced schemes, e.g.,

    - DMRS equally spaced among PUSCH transmissions

    - DMRS located in special slots

    - Orphan symbol上 used for DMRS

  - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method.

- Note: the simulation assumptions for DM-RS in TR 38.830 are used as baseline for performance evaluation on optimization of DMRS location/granularity in time domain.

  - Take into account impairments such as frequency offset, and report corresponding parametrization together with the results. Further discuss impairment details.

Working assumption:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for TB processing one TB processed over multiple slots

    - It’s subject to UE capability

Agreements:

- For joint channel estimation.

  - Take into account the residual frequency error, e.g., +/- 0.1 ppm as upper bound.

  - Companies can report other values and frequency error model.

- Reference

- 3GPP RP-202928, “New WID on NR coverage enhancements”, China Telecom, RAN#90e, December 7th – 11th, 2020.

- 3GPP RP-211566, “Revised WID on NR coverage enhancements”, China Telecom, RAN#92e, June 14th – 18th, 2021.

- 3GPP RP-212973, “Introduction of coverage enhancements in NR”, RAN1, RAN#94e, December 6th – 17th, 2021.

- 3GPP R4-2114991, “LS on joint channel estimation for PUSCH and PUCCH (R1-2106212, R4-2111706)”, Qualcomm, RAN4#100-e, Aug. 2021.

- 3GPP R4-2120002, Reply LS on PUCCH and PUSCH repetition, RAN4, Qualcomm, RAN4#101-e, November 1th – 12th, 2021.

- 3GPP R4-2120003, WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei, HiSilicon, RAN4#101-e, November 1th – 12th, 2021.

- 3GPP R4-2202368, Reply LS on Maximum duration for DMRS bundling, RAN4, Qualcomm, RAN4#101b-e, January 17th – 25th, 2022.

- R1-2200968	Discussion on joint channel estimation for PUSCH	Huawei, HiSilicon

- R1-2201014	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2201106	Remaining issues on joint channel estimation for PUSCH	vivo

- R1-2201166	Discussion on remaining issues for joint channel estimation for PUSCH	ZTE

- R1-2201285	Consideration on Joint channel estimation for PUSCH	OPPO

- R1-2201375	Remaining issues on joint channel estimation for PUSCH	CATT

- R1-2201434	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

- R1-2201443	Remaining issues on joint channel estimation	China Telecom

- R1-2201444	FL Summary of joint channel estimation for PUSCH	Moderator (China Telecom)

- R1-2201489	Remaining issues on joint channel estimation for PUSCH	NTT DOCOMO, INC.

- R1-2201555	Discussion on joint channel estimation for PUSCH	Spreadtrum Communications

- R1-2201659	Discussion on joint channel estimation	InterDigital, Inc.

- R1-2201710	Remaining details on joint channel estimation for PUSCH	Intel Corporation

- R1-2201782	Remaining issues on cross-slot channel estimation for PUSCH	Apple

- R1-2201870	Remaining issues on joint channel estimation for PUSCH	CMCC

- R1-2201912	Remaining issues on joint channel estimation for PUSCH	Xiaomi

- R1-2201963	Remaining Issues for Joint Channel Estimation for PUSCH	Ericsson

- R1-2202028	Joint channel estimation for PUSCH	Samsung

- R1-2202085	Discussion on Joint channel estimation over multi-slot	MediaTek Inc.

- R1-2202153	Joint channel estimation for PUSCH	Qualcomm Incorporated

- R1-2202198	Joint channel estimation for PUSCH	Sharp

- R1-2202237	Discussion on joint channel estimation for PUSCH	TCL Communication Ltd.

- R1-2202301	Discussions on joint channel estimation for PUSCH	LG Electronics

- R1-2201169	Discussion on RRC parameters for coverage enhancement	ZTE