3GPP TSG RAN WG1 #108-e		R1-2201454

e-Meeting, February 21st – March 3rd, 2022

Title: 	Discussion on RAN2 LS on Enhanced TCI state indication for UE-specific PDCCH MAC CE

Source: 	Lenovo, Motorola Mobility

Agenda item:	5

Document for:	Discussion and decision

# Introduction

RAN2 agreed to introduce the new “Enhanced TCI state indication for UE-specific PDCCH MAC CE” for activating two TCI states for SFN-based PDCCH transmission. But RAN2 think whether the Enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET zero is up to RAN1 decision. Thus, an LS [1] was sent to RAN1 to check the view whether “Enhanced TCI state indication for UE specific PDCCH MAC CE” can be applied to CORESET zero or not.

The following related agreement made in RAN1#106b-e [2] are shown as following:

Agreement 1

When SFN PDSCH is not configured by RRC, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL,

- For DCI format 1_1/1_2, support both configurations with and without TCI state field.

- [If enableTwoDefaultTCIStates is not configured,] for both cases with and without TCI state field,

  - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception

    - FFS: Whether above applies for TRP-based pre-compensation if TRP-based pre-compensation is agreed to be support in FR2

  - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH

Agreement 2

For CSS associated with SFN CORESET, study the following alternatives and down-select in RAN1#107e [3]:

- Alt 2: UE doesn’t expect PDCCH candidates in CSS to be associated with CORESET activated with two TCI states, except for CSS type 3 associated with CORESET configured with scheme 1

- Alt 3: If PDCCH candidates in CSS 0/0A/1/2/3 are associated with CORESET that activated with two TCI states, the first TCI state is applied for the CSS reception, except for CSS type 3 associated with CORESET configured with scheme 1.

  - For CSS type 3 associated with CORESET configured with scheme 1,  both TCI states can be applied for the CSS reception.

The following related agreement made during RAN1#107-e are shown as following.

Agreement 3

When SFN PDSCH is not configured by RRC and there is no TCI codepoint which indicates two TCI states activated for the PDSCH (i.e. Rel-16 MTRP PDSCH is not configured) and SFN transmission scheme 1 is configured for PDCCH, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2 without TCI field, if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL if applicable and the CORESET which schedules the PDSCH is indicated with two TCI states, the default TCI state is defined as the first TCI state of the CORESET

Agreement 4

If PDCCH candidates in CSS 3 are associated with CORESET that is activated with two TCI states and configured with enhanced SFN scheme 1 or TRP based pre-compensation, both TCI states can be applied for the CSS reception.

- FFS: Whether/How specification change is needed is up to the editor

We show our understanding in this contribution on whether “Enhanced TCI state indication for UE specific PDCCH MAC CE” can be applied to CORESET zero or not based on available agreement in RAN1.

# Discussions

We will discuss whether “Enhanced TCI state indication for UE specific PDCCH MAC CE” can be applied to CORESET zero or not” after initial access stage case by case based on PDCCH carried in CORESET 0.

Case 1: UE specific search space set associated with DCI format 1_0

Based on RAN1 agreement 1 and 3 shown in section 1, TCI state determination scheme is agreed when SFN PDSCH is not configured by RRC and PDSCH is scheduled by DCI format 1_0 with enhanced SFN PDCCH. If CORESET 0 carries DCI format 1_0 monitored by USS, enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET 0.

Case 2: Type3 common search space set associated with DCI format 1_0

Based on RAN1 agreement 4 shown in section 1, both TCI states can be applied for the CSS reception if PDCCH candidates in CSS 3 are associated with CORESET that is activated with two TCI states and configured with enhanced SFN scheme 1 or TRP based pre-compensation. If CORESET 0 carries DCI format 1_0 monitored by Type3 CSS, enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET 0.

Case 3: Other common search space set, except Type 3 common search space set, associated with DCI format 1_0

Based on RAN1 agreement 2, down-selection for Alt.2 and Alt.3 can be made in RAN1#107e. However, there are different views on CSS except for CSS type 3 associated with SFN CORESET. Some companies prefer scheme 2 that UE doesn’t expect PDCCH candidates in CSS to be associated with CORESET activated with two TCI states. Some companies prefer scheme 3 that the first TCI state is applied for the CSS reception, except for CSS type 3 associated with CORESET configured with scheme 1. Some companies think there is no need for further discussion since DCI format 1_0 with enhanced SFN PDCCH is already supported in agreement 1 and 3 shown in section 1. Thus, there is no conclusion for this case and we need further discussion. In our view, we prefer Alt.3 on account that it provides flexibility to carry multiple DCIs monitor by USS and CSS in CORESET 0. This can save CORESET number and provide more freedom for gNB’s realization. Also, it provides good compatibility with legacy UE monitoring behavior by only one selected TCI state.

Based on above discussion, we have the following observation and proposal:

Observation 1: If CORESET 0 carries DCI format 1_0 monitored by USS or by Type 3 CSS, enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET 0.

- Proposal 1: When CORESET 0 carries DCI format 1_0 monitored by CSS except Type 3 CSS, enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET zero and the first TCI state is applied for the CSS reception except Type 3 CSS.

- Proposal 2: From RAN1 perspective, enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET zero.

# Conclusion

Based on above discussion, we have the following observation and proposal:

Observation 1: If CORESET 0 carries DCI format 1_0 monitored by USS or by Type 3 CSS, enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET 0.

- Proposal 1: When CORESET 0 carries DCI format 1_0 monitored by CSS except Type 3 CSS, enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET zero and the first TCI state is applied for the CSS reception except Type 3 CSS.

- Proposal 2: From RAN1 perspective, enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET zero.

# References

R1-2200886, LS on Enhanced TCI state indication for UE-specific PDCCH MAC CE, Samsung, RAN1#108-e

3GPP RAN1 #106b-e Meeting Chairman Notes

3GPP RAN1 #107-e Meeting Chairman Notes