3GPP TSG RAN WG1 #108-e			R1-2201416

e-Meeting, February 21st – March 3rd, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SL type 1 HARQ-ACK codebook for boundary of resource pool period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2022-02-xx |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-16, PSFCH occasion is provided in a periodic manner. The periodicity is 1, 2, or 4. For example when 4 is (pre-)configured, basically one PSFCH occasion is associated with 4 PSSCH occasions. One key point here is that around boundary of two resource pool periods, there is a special PSFCH occasion not associated with 4 PSSCH occasions due to a situation where the number of slots available for SL is not multiple of 4. This is illustrated as below. In addition in Rel-16, SL HARQ-ACK can be reported to gNB as type-1 codebook. Section 16.5.1.1 of TS38.213 specifies how to construct the codebook. However, it seems that the spec text has abmiguity on the abovementioned special PSFCH occasion. The section has the following text: Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH MA=MA∪j; j=j+1; nF=nF+1; end while. where N_PSFCH is the value of the period of PSFCH transmission occasion resources for the sidelink resource pool. Question 1: which number is set to N_PSFCH for the special PSFCH occasion? Question 2: If N_PSFCH value is common among any PSFCH occasions, which n_F value is corresponding to the PSSCH occasions associated with the special PSFCH occasion? Regarding Q1, it seems that N_PSFCH is common among any PSFCH occasions i.e. N_PSFCH = 4 in the above example, since the value is set before ‘while’ loop of codebook construction. Then Q2 comes. This Q2 issue is illustrated as below. To solve this issue, the following two solutions are considered. Solution 1: N_PSFCH value is determined per PSFCH occasion. Solution 2: Clarify which is correct interpretation: interpretation 1 or 2 in the above illustration. In our view, solution 1 is easier, and slightly beneficial from codebook size perspective. Based on this, the spec update is proposed. |
| Summary of change: |  | N_PSFCH value is determined in ‘while’ loop so that N_PSFCH value for the special PSFCH occasion is the number of PSSCH occasions corresponding to the special PSFCH occasion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB and UE might have different understanding on which HARQ-ACK bit in a type 1 HARQ-ACK codebook is corresponding to which PSSCH occasion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

#### 16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<Unchanged parts omitted>

Set j=0 - index of occasion for candidate PSSCH transmissions with corresponding PSFCH reception occasions

Set MA=∅

Set CK1 to the cardinality of set K1

Set k=0 – index of slot timing values K1,k, in descending order of the slot timing values, in set K1

Set NPSFCH to the value of the period of PSFCH transmission occasion resources for the sidelink resource pool

while k< CK1

if mod nU-K1,k+1,max2μUL-μSL,1=0

Set nS=0 – index of a SL slot within an UL slot

while nS<max2μSL-μUL,1

if slot nU starts at a same time as or after a slot for an active UL BWP change on the PCell and slot nU-K1,k⋅2μSL-μUL+nS is before the slot for the active UL BWP change on the PCell

nS=nS+1;

else

if slot nU-K1,k⋅2μSL-μUL+nS belongs to the sidelink resource pool and includes PSFCH resources as indicated by a sidelink resource pool bitmap and sl-PSFCH-Period, where K1,k is the k-th slot timing value in set K1

Set nF=0 – index of a SL slot within an PSFCH period

Set NPSFCH to the number of PSSCH transmission occasions corresponding to the PSFCH occasion

while nF<NPSFCH

MA=MA∪j;

j=j+1;

nF=nF+1;

end while

end if

nS=nS+1;

end if

end while

end if

k=k+1;

end while

<Unchanged parts omitted>