3GPP TSG RAN WG1 #108-e	R1-2201464

e-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC

Title:	Remaining issues on MTRP for reliability

Agenda Item:	8.1.2.1

Document for: 	Discussion and Decision

Introduction

In this contribution, we discuss remaining issues on repetition over multiple TRPs for PDCCH/PUSCH to improve reliability and robustness.

PDCCH repetition over multiple TRPs

Following agreement was made in RAN1#107e [1]. How to resolve ambiguity for PUCCH resource determination for Case c2 (i.e., the case where SS sets 1 and 2 are linked, AL8 candidate in SS set 1 is linked with AL8 candidate in SS set 2, AL16 candidate in SS set 1 is linked with AL16 candidate in SS set 2, AL8 candidate and AL16 candidate in at least one of the SS sets have the same start CCE) was not decided.

Agreement To address the ambiguity issue between AL8 and AL16 candidates in the presence of PDCCH repetition: - For two linked AL8 PDCCH candidates in a first and second SS sets and two linked AL16 candidates in a third and fourth SS sets, UE expects different starting CCEs in a CORESET for any of the linked AL8 candidates and any of the linked AL16 candidates if the CORESET spans one OFDM symbol (i.e., Case c1 is not expected by UE) - If two PDCCH candidates with AL8 and AL16 have the same start CCE in a CORESET with one OFDM symbol: - When at least one of the AL8 or AL16 candidates is linked with other PDCCH candidate, and UE receives a DCI on any of the AL8 or AL16 candidates, a scheduled PDSCH by the DCI is rate matched around the AL16 candidate and any PDCCH candidate linked with any of the AL8 or AL16 candidates (i.e., rate matching in Cases a, b and c2 is around the union of candidates) - When one of the AL8 candidate or the AL16 candidate is linked to another PDCCH candidate for PDCCH repetition (i.e., Cases a or b), interpretation of a detected DCI via any of the first or second PDCCH candidates is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate). - FFS (to be resolved in this meeting): Whether/how to resolve potentially ambiguity for PUCCH resource determination for Case c2

For PUCCH resource determination in case of PDCCH repetition, it was agreed that starting CCE of the CORESET associated with the SS set with lowest SS set ID among the linked PDCCH candidates is applied. For the above Case c2, when the two PDCCH candidates with AL8 and AL16 having same start CCE are in a first SS set that is linked to a second SS set, and the second SS set has lower ID compared to the first SS set, following two alternatives for PUCCH resource determination were identified in RAN1#107e meeting.

- Alt1: UE expects the linked AL8 candidate and the linked AL16 candidate in the second SS set to also have the same start CCE

- Alt2: If the linked AL8 candidate and the linked AL16 candidate in the second SS set do not have the same start CCE, the one with lower starting CCE is assumed for PUCCH resource determination

From our perspective, Alt.1 leads to unnecessary restriction on flexibility, and thus, Alt.2 is preferred to solve this issue.

Proposal 2-1:

- If two PDCCH candidates with AL8 and AL16 have the same start CCE in a CORESET with one OFDM symbol,

  - when the two PDCCH candidates are in a first SS set that is linked to a second SS set, and the second SS set has lower ID compared to the first SS set,

    - if the linked AL8 candidate and the linked AL16 candidate in the second SS set do not have the same start CCE, the one with lower starting CCE is assumed for PUCCH resource determination

PUSCH repetition over multiple TRPs

In Rel-17 FeMIMO WI, M-TRP for PUSCH repetition type A is supported. Meanwhile, in Rel-17 Coverage Enhancement WI, available slot counting is supported for PUSCH repetition type A. In our view, available slot counting is also beneficial for PUSCH repetition type A with M-TRP and it can be easily extended to M-TRP case. Thus, it is proposed to support available slot counting for M-TRP PUSCH repetition Type A.

Proposal 3-1:

- Support available slot counting for M-TRP PUSCH repetition type A.

In current specification in TS 38.214 [2], M-TRP PUSCH repetition type A is supported with PUSCH repetitions transmitted in K consecutive slots. To support available slot counting for M-TRP PUSCH repetition Type A, following CR is proposed.

Proposal 3-2:

- Adopt the following CR for section 6.1.2 in TS 38.214.

< Unchanged text is omitted> When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', for PUSCH repetition Type A, in case K>1, - if AvailableSlotCounting is enabled, the same symbol allocation is applied across the K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. - Otherwise, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, and the association of the first and second SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 to each slot is determined as follows: - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "00" for the SRS resource set indicator, the first SRS resource set is associated with all K consecutive slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "01" for the SRS resource set indicator, the second SRS resource set is associated with all K consecutive slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "10" for the SRS resource set indicator, the first and second SRS resource set association to K consecutive slots is determined as follows: - When K = 2, the first and second SRS resource sets are applied to the first and second slot of 2 consecutive slots, respectively. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the first and second SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, first SRS resource set is applied to the first and second slots of K consecutive slots, and the second SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots. - Otherwise, a DCI format 0_1 or DCI format 0_2 indicates codepoint "11" for the SRS resource set indicator, and the first and second SRS resource set association to K consecutive slots is determined as follows, - When K = 2, the second and first SRS resource set are applied to the first and second slot of 2 consecutive slots, respectively. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the second and first SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, the second SRS resource set is applied to the first and second slot of K consecutive slots, and the first SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots. < Unchanged text is omitted>

Furthermore, TBoMS is supported in Rel-17 Coverage Enhancement WI. In our view, similar as M-TRP for PUSCH repetition, it is also beneficial to support M-TRP for TBoMS. And with similar rule as M-TRP PUSCH repetition, two beams/power control parameters can be mapped to multiple repetitions of TBoMS with one beam applied to each repetition of TBoMS.

Proposal 3-3:

- Support M-TRP TBoMS.

- Two beams/power control parameters are mapped to multiple repetitions of TBoMS. One beam is applied to each repetition of TBoMS.

And to support M-TRP TBoMS, following CR is proposed.

Proposal 3-4:

- Adopt the following CR for section 6.1.2 in TS 38.214.

< Unchanged text is omitted> When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', for TBoMS, in case K>1, the same symbol allocation is applied across the N*K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N*K slots applying the same symbol allocation in each slot, and the association of the first and second SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 to each slot is determined as follows: - if a DCI format 0_1 or DCI format 0_2 indicates codepoint “00” for the SRS resource set indicator, the first SRS resource set is associated with all N*K slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "01" for the SRS resource set indicator, the second SRS resource set is associated with all N*K slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "10" for the SRS resource set indicator, the first and second SRS resource set association to the N*K slots is determined as follows: - When K = 2, the first and second SRS resource sets are applied to the first and second N slots, respectively. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the first and second SRS resource sets are applied to the first and second N slots of the N*K slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of the N*K slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, first SRS resource set is applied to the first and second N slots of the N*K slots, and the second SRS resource set is applied to the third and fourth N slots of the N*K slots, and the same SRS resource set mapping pattern continues to the remaining slots of the N*K slots. - Otherwise, a DCI format 0_1 or DCI format 0_2 indicates codepoint "11" for the SRS resource set indicator, and the first and second SRS resource set association to the N*K slots is determined as follows, - When K = 2, the second and first SRS resource set are applied to the first and second N slots, respectively. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the second and first SRS resource sets are applied to the first and second N slots of the N*K slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of the N*K slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, the second SRS resource set is applied to the first and second N slots of the N*K slots, and the first SRS resource set is applied to the third and fourth N slots of the N*K slots, and the same SRS resource set mapping pattern continues to the remaining slots of the N*K slots. < Unchanged text is omitted>

Conclusion

In this contribution, we discussed the reliability and robustness enhancements for simultaneous multi-TRP transmission for PDCCH and PUSCH. We have following proposals.

[PDCCH]

Proposal 2-1:

- If two PDCCH candidates with AL8 and AL16 have the same start CCE in a CORESET with one OFDM symbol,

  - when the two PDCCH candidates are in a first SS set that is linked to a second SS set, and the second SS set has lower ID compared to the first SS set,

    - if the linked AL8 candidate and the linked AL16 candidate in the second SS set do not have the same start CCE, the one with lower starting CCE is assumed for PUCCH resource determination

[PUSCH]

Proposal 3-1:

- Support available slot counting for M-TRP PUSCH repetition type A.

Proposal 3-2:

- Adopt the following CR for section 6.1.2 in TS 38.214.

< Unchanged text is omitted> When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', for PUSCH repetition Type A, in case K>1, - if AvailableSlotCounting is enabled, the same symbol allocation is applied across the K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. - Otherwise, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, and the association of the first and second SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 to each slot is determined as follows: - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "00" for the SRS resource set indicator, the first SRS resource set is associated with all K consecutive slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "01" for the SRS resource set indicator, the second SRS resource set is associated with all K consecutive slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "10" for the SRS resource set indicator, the first and second SRS resource set association to K consecutive slots is determined as follows: - When K = 2, the first and second SRS resource sets are applied to the first and second slot of 2 consecutive slots, respectively. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the first and second SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, first SRS resource set is applied to the first and second slots of K consecutive slots, and the second SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of K consecutive slots. - Otherwise, a DCI format 0_1 or DCI format 0_2 indicates codepoint "11" for the SRS resource set indicator, and the first and second SRS resource set association to K consecutive slots is determined as follows, - When K = 2, the second and first SRS resource set are applied to the first and second slot of 2 consecutive slots, respectively. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the second and first SRS resource sets are applied to the first and second slot of K consecutive slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, the second SRS resource set is applied to the first and second slot of K consecutive slots, and the first SRS resource set is applied to the third and fourth slot of K consecutive slots, and the same SRS resource set mapping pattern continues to the remaining slots of the K consecutive slots. < Unchanged text is omitted>

Proposal 3-3:

- Support M-TRP TBoMS.

Proposal 3-4:

- Adopt the following CR for section 6.1.2 in TS 38.214.

< Unchanged text is omitted> When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', for TBoMS, in case K>1, the same symbol allocation is applied across the N*K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N*K slots applying the same symbol allocation in each slot, and the association of the first and second SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 to each slot is determined as follows: - if a DCI format 0_1 or DCI format 0_2 indicates codepoint “00” for the SRS resource set indicator, the first SRS resource set is associated with all N*K slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "01" for the SRS resource set indicator, the second SRS resource set is associated with all N*K slots, - if a DCI format 0_1 or DCI format 0_2 indicates codepoint "10" for the SRS resource set indicator, the first and second SRS resource set association to the N*K slots is determined as follows: - When K = 2, the first and second SRS resource sets are applied to the first and second N slots, respectively. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the first and second SRS resource sets are applied to the first and second N slots of the N*K slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of the N*K slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, first SRS resource set is applied to the first and second N slots of the N*K slots, and the second SRS resource set is applied to the third and fourth N slots of the N*K slots, and the same SRS resource set mapping pattern continues to the remaining slots of the N*K slots. - Otherwise, a DCI format 0_1 or DCI format 0_2 indicates codepoint "11" for the SRS resource set indicator, and the first and second SRS resource set association to the N*K slots is determined as follows, - When K = 2, the second and first SRS resource set are applied to the first and second N slots, respectively. - When K > 2 and cyclicMapping in PUSCH-Config is enabled, the second and first SRS resource sets are applied to the first and second N slots of the N*K slots, respectively, and the same SRS resource set mapping pattern continues to the remaining slots of the N*K slots. - When K > 2 and sequentialMapping in PUSCH-Config is enabled, the second SRS resource set is applied to the first and second N slots of the N*K slots, and the first SRS resource set is applied to the third and fourth N slots of the N*K slots, and the same SRS resource set mapping pattern continues to the remaining slots of the N*K slots. < Unchanged text is omitted>

- References

[1] Final Report of 3GPP TSG RAN WG1 #107-e v1.0.0, Nov. 2021

[2] 3GPP TS 38.214 V17.0.0 (2021-12)