3GPP TSG RAN WG1 #108-e			     R1-2201478

e-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on HARQ enhancements for NR NTN

Agenda Item:	8.4.3

Document for: 	Discussion and Decision

- Introduction

At the RAN1#107-e meeting [1], there were discussions on HARQ enhancements for NR NTN and several agreements were reached. In this contribution, we share our further views for maintenance of HARQ enhancements for NR NTN.

- Discussions

  - SPS with feedback-disabled process

Agreement HARQ feedback for SPS activation may be additionally enabled by the network by RRC configuration. - If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of whether HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation - Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation, - FFS between Alt1 and Alt2 - [Alt-1: UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process - Alt-2: UE follows the feedback-enabled/disabled configuration of the SPS PDSCH]

One key issue on SPS is how to handle HARQ feedback corresponding to the initial SPS PDSCH after receiving activation DCI in case where feedback-disabled process is used for the PDSCH, and the above agreement was reached at the last meeting. Here still one FFS point is there, but we think the FFS includes two issues mixedly. Actually they are independent each other, they are discussed separately below.

Observation 1:

- An FFS in the agreement reached for HARQ feedback for SPS activation includes two issues mixedly. They should be discussed separately.

- HARQ feedback corresponding to the initial SPS PDSCH after receiving activation DCI

The above captured agreement at the last meeting intends this issue mainly. RRC configuration can be provided for this issue, and if enabled, UE reports HARQ-ACK for the PDSCH regardless of feedback-enabling/disabling configuration. Then, if disabled/not provided, the straightforward option is only that UE follows the feedback-enabling/disabling configuration corresponding to the initial SPS PDSCH. Other option would not be aligned with intention of introducing the RRC parameter.

Proposal 1:

- When RRC configuration of HARQ feedback for SPS activation is not enabled, UE follows feedback-enabling/disabling configuration corresponding to the initial SPS PDSCH after SPS activation.

- HARQ feedback-enabling/disabling configuration for SPS

Alt 2 in the above agreement is a feature for feedback of entire SPS PDSCHs rather than feedback of the initial SPS PDSCH after activation. Related to this issue, there were discussions on whether the agreement of feedback-enabling/disabling configuration per HARQ process covers SPS or not. In addition, whether the configuration is shared among dynamically scheduled PDSCH and SPS PDSCH or not is also an unclear point currently. In our view, at least feedback-enabling/disabling should be separately configured between dynamically scheduled PDSCH and SPS PDSCH.

In SPS, process number determination is dependent on the reception timing and a parameter nrofHARQ-Processes. In this sense, if common configuration with dynamic case is applied and it is intended that a SPS is configured with feedback-enabling, all HARQ processes allocated for the SPS shall be configured with feedback-enabling so that any SPS PDSCH receptions follow the same configuration of enabled feedback. However, it will degrade dynamic scheduling flexibility since HARQ processes allocated for the SPS can be used for dynamic scheduling as well. Any dynamic PDSCH scheduling with disabled feedback shall be transmitted via one of the other HARQ processes in this case. One note is that RTT in NTN is quite large, i.e. more HARQ processes will be needed for each SPS with enabled feedback. Another note is that the following is the process number determination of SPS PDSCH.

- HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes, or

- HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes + harq-ProcID-Offset

- where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame

Fig. 1: Common/Separate configuration per HARQ process between dynamic PDSCH and SPS.

Observation 2:

- If feedback-enabling/disabling configuration per HARQ process is common between dynamic PDSCH and SPS PDSCH, many HARQ processes would be configured with enabled feedback so that enabled feedback is applied to all of the SPS receptions.

  - This aspect degrades flexibility of dynamic PDSCH scheduling with disabled feedback.

Proposal 2:

- Feedback-enabling/disabling for SPS PDSCH is separately configured from configuration for dynamically scheduled PDSCH.

  - Processing time constraints for SPS

Another sub-issue on SPS is processing time constraints. At the previous meetings, processing time constraints for PDSCH receptions with disabled feedback were agreed as captured below.

Agreement: For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. - Working assumption: X = T_proc,1 - FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0 - Note: The TB of the two PDSCHs can be either same or different Agreement: Confirm the previous working assumption for X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH for the given HARQ process.

38.214 V17.0.0 5.1 UE procedure for receiving the physical downlink shared channel ... ... When HARQ feedback for the HARQ process ID is not disabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213]. For HARQ-ACK subject to HARQ-ACK deferral described in Clause 9.2.5.4 of [6 TS 38.213], the expected transmission of HARQ-ACK corresponds to the expected transmission HARQ-ACK in a first slot. When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. ... ...

After the end of a PDSCH reception at time t, UE does not expect to receive a PDCCH carrying a DCI scheduling another PDSCH that starts within [t, t+T_proc,1). The corresponding text is added as above with green color. However, this does not cover SPS case except the initial one with activation since there is no corresponding PDCCH. The agreement at the last meeting should be updated as the following proposal with red color so that any SPS PDSCH reception is included.

Fig. 2: Processing time constraints for SPS PDSCH receptions

Proposal 3:

- Update the RAN1#105-e agreement and apply the following TP.

  - Confirm the previous working assumption for X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH or the PDSCH without corresponding PDCCH for the given HARQ process.

38.214 V17.0.0 5.1 UE procedure for receiving the physical downlink shared channel ... ... When HARQ feedback for the HARQ process ID is not disabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213]. For HARQ-ACK subject to HARQ-ACK deferral described in Clause 9.2.5.4 of [6 TS 38.213], the expected transmission of HARQ-ACK corresponds to the expected transmission HARQ-ACK in a first slot. When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH scheduled for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. ... ...

  - Disabled HARQ feedback in Type 2 HARQ-ACK CB

Agreement The bit-fields related to the HARQ-ACK feedback (i.e., PRI, PUSCH-to-HARQ_feedback timing, DAI) are unchanged for the DCI of PDSCH with feedback-disabled HARQ process in Rel-17 with the same interpretation from UE as for feedback-enabled HARQ process - Note: The interpretation regarding the DAI for Type-2 codebook is up to the progress of codebook design.

38.213 V17.0.0 9.1.3 Type-2 HARQ-ACK codebook determination ... If a UE is provided HARQ-feedbackEnabling-disablingperHARQprocess indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in PDSCH reception occasion m on serving cell c, the UE does not multiplex a HARQ-ACK information bit corresponding to the transport block in a Type-2 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived or of NSPS,c in clause 9.1.3.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE does not multiplex HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-2 HARQ-ACK codebook and does not consider the CBGs as received in the determination of Nm,creceived,CBG in clause 9.1.3.1. If the UE is also provided HARQ-feedbackEnablingforSPSactive, the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception. ...

At the last meeting, for feedback-disabled HARQ process, interpretations of DCI fields related to HARQ feedback are agreed except for DAI field. Regarding DAI field, we prefer to use the DAI field to detect the last DCI misdetection. However, companies’ views are not converged so far, then it is questionable that RAN1 can have agreement of such a mechanism in maintenance phase. If the situation is not changed, the possible outcome would be only that DAI in DCI with feedback-disabled HARQ process is ignored.

Then even in this case, we believe that an appropriate spec update is necessary. It seems that the latest specification has a text corresponding to a direction of ignoring TB scheduled by DCI with feedback-disabled HARQ process, for purpose of type 2 HARQ-ACK CB construction. We guess that the text is intended to ignore the corresponding DAI as well as the TB. However, the current spec text still has an ambiguity on whether DCI, i.e. DAI, corresponding to the transport block is considered or not. Therefore, we propose the following text update.

Proposal 4:

- Confirm that DAI in a DCI with disabled feedback is ignored, and apply the following TP.

38.213 V17.0.0 9.1.3 Type-2 HARQ-ACK codebook determination ... If a UE is provided HARQ-feedbackEnabling-disablingperHARQprocess indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in PDSCH reception occasion m on serving cell c, the UE does not multiplex a HARQ-ACK information bit corresponding to the transport block in a Type-2 HARQ-ACK codebook and does not consider the transport block and the corresponding PDCCH, if any, as received in the determination of Nm,creceived or of NSPS,c in clause 9.1.3.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE does not multiplex HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-2 HARQ-ACK codebook and does not consider the CBGs and the corresponding PDCCH, if any, as received in the determination of Nm,creceived,CBG in clause 9.1.3.1. If the UE is also provided HARQ-feedbackEnablingforSPSactive, the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception. ...

  - 32 HARQ processes

38.214 V17.0.0 5.1 UE procedure for receiving the physical downlink shared channel For downlink, a maximum of 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of μ = 5 and μ = 6. The number of processes the UE may assume will at most be used for the downlink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPDSCH, and when no configuration is provided the UE may assume a default number of 8 processes. ... 6.1 UE procedure for transmitting the physical uplink shared channel ... For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of μ = 5 or μ = 6.

In Rel-17 NTN, maximum of 32 HARQ processes was agreed to handle the quite large propagation delay. The same mechanism was introduced for FR2-2 as well. However, the current specification mentions only the FR2-2 perspective. The following text proposal is submitted to cover NTN.

Proposal 5:

- Apply the following TP.

38.214 V17.0.0 5.1 UE procedure for receiving the physical downlink shared channel For downlink, a maximum of 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of μ = 5 and μ = 6 or for the cases of communications via satellite/HAPS or with ATG gNB as specified in [8, TS 38.101-X]. The number of processes the UE may assume will at most be used for the downlink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPDSCH, and when no configuration is provided the UE may assume a default number of 8 processes. ... 6.1 UE procedure for transmitting the physical uplink shared channel ... For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of μ = 5 or μ = 6 or for the cases of communications via satellite/HAPS or with ATG gNB as specified in [8, TS 38.101-X].

- Conclusion

In this contribution, we discussed HARQ enhancements on NR NTN. Observations/Proposals are summarized as following:

Observation 1:

- An FFS in the agreement reached for HARQ feedback for SPS activation includes two issues mixedly. They should be discussed separately.

Proposal 1:

- When RRC configuration of HARQ feedback for SPS activation is not enabled, UE follows feedback-enabling/disabling configuration corresponding to the initial SPS PDSCH after SPS activation.

Observation 2:

- If feedback-enabling/disabling configuration per HARQ process is common between dynamic PDSCH and SPS PDSCH, many HARQ processes would be configured with enabled feedback so that enabled feedback is applied to all of the SPS receptions.

  - This aspect degrades flexibility of dynamic PDSCH scheduling with disabled feedback.

Proposal 2:

- Feedback-enabling/disabling for SPS PDSCH is separately configured from configuration for dynamically scheduled PDSCH.

Proposal 3:

- Update the RAN1#105-e agreement and apply the following TP.

  - Confirm the previous working assumption for X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH or the PDSCH without corresponding PDCCH for the given HARQ process.

38.214 V17.0.0 5.1 UE procedure for receiving the physical downlink shared channel ... ... When HARQ feedback for the HARQ process ID is not disabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213]. For HARQ-ACK subject to HARQ-ACK deferral described in Clause 9.2.5.4 of [6 TS 38.213], the expected transmission of HARQ-ACK corresponds to the expected transmission HARQ-ACK in a first slot. When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH scheduled for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. ... ...

Proposal 4:

- Confirm that DAI in a DCI with disabled feedback is ignored, and apply the following TP.

38.213 V17.0.0 9.1.3 Type-2 HARQ-ACK codebook determination ... If a UE is provided HARQ-feedbackEnabling-disablingperHARQprocess indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in PDSCH reception occasion m on serving cell c, the UE does not multiplex a HARQ-ACK information bit corresponding to the transport block in a Type-2 HARQ-ACK codebook and does not consider the transport block and the corresponding PDCCH, if any, as received in the determination of Nm,creceived or of NSPS,c in clause 9.1.3.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE does not multiplex HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-2 HARQ-ACK codebook and does not consider the CBGs and the corresponding PDCCH, if any, as received in the determination of Nm,creceived,CBG in clause 9.1.3.1. If the UE is also provided HARQ-feedbackEnablingforSPSactive, the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception. ...

Proposal 5:

- Apply the following TP.

38.214 V17.0.0 5.1 UE procedure for receiving the physical downlink shared channel For downlink, a maximum of 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of μ = 5 and μ = 6 or for the cases of communications via satellite/HAPS or with ATG gNB as specified in [8, TS 38.101-X]. The number of processes the UE may assume will at most be used for the downlink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPDSCH, and when no configuration is provided the UE may assume a default number of 8 processes. ... 6.1 UE procedure for transmitting the physical uplink shared channel ... For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of μ = 5 or μ = 6 or for the cases of communications via satellite/HAPS or with ATG gNB as specified in [8, TS 38.101-X].

References

- 3GPP RAN1#107-e meeting, chairman’s notes