3GPP TSG RAN WG1 #108-e			     R1-2201495

e-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on sidelink resource allocation for reliability and latency

Agenda Item:	8.11.1.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#107bis-e meeting [1], there were discussions on resource allocation for reliability/latency improvements and several agreements were reached. In this contribution, we share our further views to conclude resource allocation enhancement for better reliability and latency.

- Discussions

  - Inter-UE coordination - scheme 1

    - Cast-type finalization

Agreement For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request Working Assumption For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported

In this section, cast type for IUC scheme 1 is discussed. In request-based scheme 1, only unicast is assumed. Meanwhile, in condition-based scheme 1, there are several FFSs.

For condition-based scheme 1, if preferred resources are transmitted from UE-A to multiple UEs, as the results, multiple RX UEs might use the same resource, which is not intended situation. Thus we believe that groupcast/broadcast for preferred resources should not be supported to avoid collision increase due to IUC scheme 1.

Regarding further condition to determine cast type, there seems no motivation to specify it. When IUC message can be transmitted only when the UE-A has data to a destination UE, cast type of the IUC message is determined automatically. When IUC message can be transmitted anytime by up to UE implementation, then it would be straightforward that the cast type is also determined by UE implementation.

Proposal 1:

- For transmission of preferred resource set triggered by a condition other than explicit request, groupcast/broadcast is not used.

- For transmission of non-preferred resource set triggered by a condition other than explicit request, which cast type is used is determined by UE-A implementation.

    - Details of SCI 2-C

Agreement The following working assumption is confirmed with modification in RED. - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3] Agreement For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - (Working Assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A - A single format SCI 2-C is used for inter-UE coordination information and request - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information - SCI 2-C is UE RX optional - It is up to UE implementation to additionally use 2nd SCI (for UE-B). - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A Working Assumption First resource location of each TRIV is a slot offset with respect to a reference slot - The slot offset is the number of logical slots from the reference slot - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256] - FFS: The detailed value range including granularity - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information - For the reference slot, - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

For transmissions of IUC message or explicit request, a new SCI format – SCI format 2-C was introduced at the previous meeting. There are several agreements on this format, while further discussions are necessary for details of SCI 2-C. As high-level view, our preference is not to increase payload size for SCI 2-C to keep good decoding performance.

On required fields in SCI 2-C, our view is that fields in SCI 2-A should be copied since SCI 2-C is used for transmissions with data. It would be natural that fields used for data transmissions are included in SCI 2-C. One big issue is whether 2-B-specific fields are also introduced or not. Currently we do not see strong motivation to use groupcast for IUC message while available, thereby we think that groupcast Option 1 with IUC message is not an essential feature. Unicast would be the most valuable cast type for IUC scheme 1. Moreover, if IUC message is intended for data transmission in groupcast Option 1, then they can choose the option where UE can send IUC message without multiplexing with data. Then IUC message can be transmitted via groupcast without feedback.

Regarding N value for preferred/non-preferred resources, it seems that the size for IUC message could become too large as over 140 bits and be much larger than that for request when N=3. For example,

- SCS = 120 kHz, 16 available periodicities, 26 subchannels with 10 PRBs subchannel (i.e. 400 MHz bandwidth): max 156 bits,

- SCS = 30 kHz, 16 available periodicities, 27 subchannels with 10 PRBs subchannel (i.e. 100 MHz bandwidth): max 156 bits,

- SCS = 30 kHz, 16 available periodicities, 6 subchannels with 12 PRBs subchannel (i.e. 30 MHz bandwidth): max 136 bits.

The following tables are format size analysis for each of IUC message and explicit request, in the first example. Therefore, we believe that smaller N value should be considered. ‘2’ rather than ‘1’ or ‘3’ seems to be the best approach as the bar so that the payload size is reduced and a reservation chain based on preferred resources is possible. If the bar to use SCI 2-C is ‘1’, the reservation chain is not available, which is a bit strange for reliability enhancement purpose.

The last issue is value range of slot offset used for each TRIV. Now preferred/non-preferred resources are indicated by a combination of reference slot / slot offset up to [256] / TRIV. The combination would have sufficient flexibility to indicate any slots. Further value range is not preferable for further optimization.

Table 1: Format size (SCS = 120 kHz, 16 available periodicities, 26 subchannels with 10 PRBs subchannel)

Proposal 2:

- SCI format 2-C includes the same fields as in SCI format 2-A. Format 2-B specific fields are not included.

- SCI format 2-C can be used for N <= 2.

- The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to 256.

    - Prioritization of IUC message and explicit request

Agreement For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data Agreement For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B. - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data Agreement For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. - FFS: Otherwise, the priority value is determined by UE-A’s implementation. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

Most prioritization procedures for IUC scheme 1 were already agreed, while priority value of condition-based IUC message without data multiplexing when (pre-)configuration is not provided is not concluded yet. In condition-based scheme 1, basically UE-A does not know whether UE-B has data to be transmitted and which priority UE-B will use for its own transmission if any. From this perspective, we believe that IUC-related message based on condition should be associated with the lowest priority. There is no motivation to set high priority at UE-A side. High priority should not be used to avoid undesirable collision between the IUC message and other UE’s important data transmissions. Alternatively, no definition for such a case would be fine. (Pre-)configuration shall be provided for condition-based scheme 1 in this alternative.

Proposal 3:

- If priority value is not (pre-)configured for condition-based IUC message, priority value of the IUC message is the largest value, i.e. 8.

    - UE-B’s behavior after receiving IUC message

Agreement In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A: - For preferred resource set, the following two options are supported: - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met - FFS: Details of condition(s) - This option is supported when UE-B performs sensing/resource exclusion - FFS: Other details (if any) - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set - This option is supported at least when UE-B does not support sensing/resource exclusion - FFS: Whether the support is conditional or UE capability - FFS: Other details (if any) - FFS: Other option(s), and other details (if any)

38.214 8.1.4C UE procedure for using a received resource set A UE configured with the higher layer parameter interUECoordinationScheme1 enabling reception of preferred or non-preferred inter-UE co-ordination information uses a received resource set as follows when performing resource (re-)selection: - if the received resource set is a preferred resource set, the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set when the UE has no own sensing result, otherwise the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set in combination with its own sensing result. - if the received resource set is a non-preferred resource set, the UE excludes resource(s) obtained after Step 6) of clause 8.1.4 overlapping with the non-preferred resource set.

UE-B’s behavior after receiving coordination message was discussed above. Regarding Option B, the agreement is saying that this option is supported at least when UE-B does not support sensing/resource exclusion. However, the current specification is not aligned with the previous agreement, which is the text with green color.

One important observation would be that Option B is not good for other UE’s transmission performance since UE-B does not consider the surrounding environment, including any other UEs ‘reservations, which is a kind of random selection from perspective of the surrounding UEs. Even if UE-B’s performance is good in Option B, Option A should be prioritized when applicable.

Fig. 1: Issue of Option B.

Proposal 4:

- Option B of preferred resource set is available only for either of the following UE:

  - UE that does not support sensing/resource exclusion.

  - UE that supports sensing/resource exclusion but performs random selection for corresponding transmission.

- Apply the following TP.

38.214 8.1.4C UE procedure for using a received resource set A UE configured with the higher layer parameter interUECoordinationScheme1 enabling reception of preferred or non-preferred inter-UE co-ordination information uses a received resource set as follows when performing resource (re-)selection: - if the received resource set is a preferred resource set, the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set when the UE has no own sensing result when the UE does not support full sensing/partial sensing or the UE performs random selection, otherwise the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set in combination with its own sensing result. - if the received resource set is a non-preferred resource set, the UE excludes resource(s) obtained after Step 6) of clause 8.1.4 overlapping with the non-preferred resource set.

    - UE-B’s behavior when UE-B receives multiple IUC messages from a single UE-A

38.214 8.1.4C UE procedure for using a received resource set A UE configured with the higher layer parameter interUECoordinationScheme1 enabling reception of preferred or non-preferred inter-UE co-ordination information uses a received resource set as follows when performing resource (re-)selection: - if the received resource set is a preferred resource set, the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set when the UE has no own sensing result, otherwise the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set in combination with its own sensing result. - if the received resource set is a non-preferred resource set, the UE excludes resource(s) obtained after Step 6) of clause 8.1.4 overlapping with the non-preferred resource set.

At section 8.1.4C of TS 38.214, UE-B’s behavior after receiving preferred/non-preferred resources are described. However, it seems that UE behavior when UE-B receives multiple IUC messages from a single UE-A is unclear yet. In our view, if preferred resources are received multiple times, only the latest set should be used since channel condition at UE-A might have been updated, e.g. preferred resource might become non-preferred. The old information is not used at UE-B if the new information is available.

Meanwhile, if two non-preferred sets are received, it would be reasonable to use both sets at UE-B since non-preferred resource will continue to be non-preferred. Similarly, if one preferred set and one non-preferred set are received, both information would be beneficial for the UE-B.

Fig. 2: Using only the latest preferred set.

Proposal 5:

- For scheme 1, when UE-B receives multiple preferred/non-preferred sets from a single UE-A,

  - UE-B uses the preferred set received later.

  - UE-B uses any of the non-preferred sets.

    - UE-B’s behavior when UE-B receives multiple IUC messages from multiple UE-As

Similarly to the issue discussed at the last section, UE behavior when UE-B receives multiple IUC messages from multiple UE-As is unclear yet. In other words, whether does UE-B use all received sets for transmissions to any UE?

Fig. 3: Issue on whether received set is used for transmissions to other UE.

In our view, “preferred resources” are preferred ones for transmissions to the UE-A; e.g. when UE-B receives a set from UE-A1 and another set from UE-A2, and the UE-B would transmit to UE-A1, then the set from UE-A1 should be used preferentially. There is no motivation to use the set received from UE-A2 since whether UE-A2 can receive the set or not does not matter.

Meanwhile, “non-preferred resources” are non-preferred ones for transmissions to any UE; e.g. in the above example, UE-B should not use the set received from UE-A2, even for transmissions to UE-A1. Otherwise, collision at UE-A2 side would occur in the resource where UE-A2 receive TB from other UE. The reason of “non-preferred” is unknown at UE-B side, thus not to use the resource is the best choice for safer side.

Proposal 6:

- For scheme 1, when UE-B receives multiple preferred/non-preferred sets from multiple UE-As,

  - UE-B uses the preferred set from UE-A that is a destination UE of UE-B’s transmission.

  - UE-B uses any of the non-preferred sets.

  - Inter-UE coordination - scheme 2

    - UE-A’s behavior by using the added indicator at SCI 1-A

Working Assumption For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not. - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication

1-bit indicator to inform UE-A of whether the source UE can be UE-B or not was introduced for the purpose of avoiding the case where UE-A transmits a collision indication to UE not supporting IUC scheme 2. Meanwhile, how to use the 1-bit indicator is not concluded yet.

For UE-B perspective, we believe that UE supporting scheme 2 should basically send ‘1’ as the indicator to become UE-B. When the UE reserves some future resources, we do not see any motivation not to send ‘1’ as the indicator. For the best way of UE-B determination at UE-A side, the UE should not send ‘0’ if supported. Meanwhile, when this UE does not reserve any future resource, any indication is unnecessary and UE-A will never use the information in the collision indication field. In other words, either information can be sent from UE not reserving any resource and the information is ignored at UE-A side.

At UE-A side, there is an agreement on UE-B determination based on priorities attached to TBs. Then regarding relationship with the agreed priority-based UE-B determination, two directions are considerable for UE-B determination.

- Alt 1: Firstly capability-based selection, then priority-based selection. After this rule, certainly one UE is selected as UE-B if at least one UE supports scheme 2.

- Alt 2: Firstly priority-based selection, then capability-based selection. After this rule, none of UE might be selected as UE-B.

In our view, Alt 2 is not preferable since collision indication is not sent and collision is not avoided in some cases. There was an argument that collision indication might be sent to UE transmitting a TB with lower priority, which is not a good way. However, collision indication to UE transmitting a TB with lower priority was already agreed as a case where “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and destination UE of either of the conflicting TBs is not UE-A. Thus the argument is not valid comment. Appropriate (pre-)configuration of collision detection, e.g. SIR-based detection, can handle such a situation.

Proposal 7:

- UE supporting scheme 2 and indicating future resource for reservation shall send ‘1’ at Second UE flag field in SCI 1-A.

- When at least one of UEs scheduling conflicting TBs is incapable of being UE-B, UE-A determines UE-B as the following procedure:

  - Firstly, candidate UE(s) is(are) determined among UEs scheduling the conflicting TBs with ‘1’ at Second UE flag field in SCI 1-A.

  - Then, UE-B is determined based on the working assumption of priority-based UE-B determination, from the candidate UE(s).

    - Details of the “next” reserved resource

Agreement For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

For IUC scheme 2, UE-B’s behavior after receiving collision indication was agreed above. When collision indication is received, the “next” reserved resource is interpretated as collided and the corresponding resources are excluded at higher layer. However, we believe that definition of the “next” reserved resource is quite unclear. In both options of PSFCH occasion determination, it is possible that there is another reserved resource between PSFCH resource and the reserved resource with collision, which is illustrated below. The resource with collision is the 2nd reserved resource, but the 1st resource is not collided with other UE’s reservation.

Fig. 4: Ambiguity of the “next” reserved resource.

If no further clarification is agreed, it might mean that the “next” reserved resource is the 1st reserved resource without any collision in the above examples. In this case, UE-B reports resources overlapping with the 1st reserved resource to higher layer, but this is not the intention of the above agreement in our understanding. Rather, the “next” reserved resource should be the earliest resource after the decoding since UE-B requires processing time to decode the PSFCH. The processing time was already agreed as T_3, thereby it would be valid that the “next” reserved resource is determined based on T_3. With the following, the “next” reserved resource is always the resource with collision at least in Option 2 of PSFCH occasion determination.

Proposal 8:

- For UE-B receiving collision indication, the “next” reserved resource is the earliest resource T_3 or more later from the PSFCH resource.

Regarding the “next” reserved resource, even if the above clarification is introduced, the “next” reserved resource might not be resource with collision in Option 1 of PSFCH occasion determination. This case is illustrated as below. RAN1 should have common understanding of whether this situation is valid or not and whether corresponding solution is necessary or not.

Fig. 5: The “next” reserved resource without collision.

Observation 1:

- For UE-B receiving collision indication, the “next” reserved resource might not be resource with collision when PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted.

    - m_0/m_CS

Conclusion For Scheme 2, there is no consensus to support the indication of the following - Condition type of a resource conflict - Time location of a resource conflict

At the previous meetings, detailed parameters of PSFCH for scheme 2 were agreed. How to set m_CS and m_0 is the remaining issue.

Regarding m_0, in Rel-16, each PSFCH resource index is associated with one of available m_0 values. Which m_0 is used is determined based on which PSSCH resource was used and which UE transmitted on the PSSCH resource. We do not see strong motivation to change this mechanism, so no enhancement should be agreed. One argument to change the Rel-16 mechanism might be for the purpose of CDM between PSFCH for scheme 2 and PSFCH for HARQ feedback. However, FDM (with larger NCSPSFCH if necessary) is possible by using sl-PSFCH-RB-Set and rbSetPSFCHScheme2. There would be no motivation to use CDM rather than FDM.

Regarding m_CS, based on the above conclusion at the last meeting, there is no motivation to use m_CS value other than 0.

Proposal 9:

- For PSFCH in scheme 2,

  - m_0 is determined as in Rel-16.

  - m_CS is 0.

- Conclusion

In this contribution, we discussed resource allocation for reliability and latency enhancements. Observations/Proposals are summarized as following:

Proposal 1:

- For transmission of preferred resource set triggered by a condition other than explicit request, groupcast/broadcast is not used.

- For transmission of non-preferred resource set triggered by a condition other than explicit request, which cast type is used is determined by UE-A implementation.

Proposal 2:

- SCI format 2-C includes the same fields as in SCI format 2-A. Format 2-B specific fields are not included.

- SCI format 2-C can be used for N <= 2.

- The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to 256.

Proposal 3:

- If priority value is not (pre-)configured for condition-based IUC message, priority value of the IUC message is the largest value, i.e. 8.

Proposal 4:

- Option B of preferred resource set is available only for either of the following UE:

  - UE that does not support sensing/resource exclusion.

  - UE that supports sensing/resource exclusion but performs random selection for corresponding transmission.

- Apply the following TP.

38.214 8.1.4C UE procedure for using a received resource set A UE configured with the higher layer parameter interUECoordinationScheme1 enabling reception of preferred or non-preferred inter-UE co-ordination information uses a received resource set as follows when performing resource (re-)selection: - if the received resource set is a preferred resource set, the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set when the UE has no own sensing result when the UE does not support full sensing/partial sensing or the UE performs random selection, otherwise the UE uses in its resource (re-)selection resource(s) belonging to the preferred resource set in combination with its own sensing result. - if the received resource set is a non-preferred resource set, the UE excludes resource(s) obtained after Step 6) of clause 8.1.4 overlapping with the non-preferred resource set.

Proposal 5:

- For scheme 1, when UE-B receives multiple preferred/non-preferred sets from a single UE-A,

  - UE-B uses the preferred set received later.

  - UE-B uses any of the non-preferred sets.

Proposal 6:

- For scheme 1, when UE-B receives multiple preferred/non-preferred sets from multiple UE-As,

  - UE-B uses the preferred set from UE-A that is a destination UE of UE-B’s transmission.

  - UE-B uses any of the non-preferred sets.

Proposal 7:

- UE supporting scheme 2 and indicating future resource for reservation shall send ‘1’ at Second UE flag field in SCI 1-A.

- When at least one of UEs scheduling conflicting TBs is incapable of being UE-B, UE-A determines UE-B as the following procedure:

  - Firstly, candidate UE(s) is(are) determined among UEs scheduling the conflicting TBs with ‘1’ at Second UE flag field in SCI 1-A.

  - Then, UE-B is determined based on the working assumption of priority-based UE-B determination, from the candidate UE(s).

Proposal 8:

- For UE-B receiving collision indication, the “next” reserved resource is the earliest resource T_3 or more later from the PSFCH resource.

Observation 1:

- For UE-B receiving collision indication, the “next” reserved resource might not be resource with collision when PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted.

Proposal 9:

- For PSFCH in scheme 2,

  - m_0 is determined as in Rel-16.

  - m_CS is 0.

References

- 3GPP RAN1#107bis-e meeting,	chair’s notes