3GPP TSG RAN WG1 #108-e   	           R1-2201501

e-Meeting, February 21st – March 3rd, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel-17 FeMIMO UE features

Agenda Item:	8.16.1

Document for: 	Discussion and Decision

Introduction

Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e is captured in R1-2200780. In this contribution, we provide our views on Rel.17 FeMIMO UE features.

Discussions

  - Multi-beam Operation

We believe FG23-1-1 and FG23-1-2 should be reported per band.

For FG23-1-1, we have the following suggestion:

- Component 4: In Rel.15, the number of configured TCI state for PDSCH per CC is reported, however, the number of configured TCI states across all BWPs/CCs is not reported. Since TCI state is configured per BWP, component 4 should be reported per BWP per CC.

  - For the candidate values of component 4, in Rel.15, UE is mandated to report the number of RRC configured TCI states as “64” in FR2 and “the maximum number of allowed SSBs” in FR1. Since Rel.17 unified TCI framework replaces all of Rel.15/16 beam indication mechanism, the mandatory value of Rel.15/16 TCI state should be supported in Rel.17 TCI state. That is, UE is mandated to report the number of RRC configured TCI states per BWP in a CC as “64” in FR2 and “the maximum number of allowed SSBs” in FR1.

- Component 5/5a: In Rel.15, the number of activated TCI state for PDSCH per BWP in a CC is reported, however, the number of activated TCI states across all BWPs/CCs is not reported. Hence, component 5/5a should be reported per BWP in a CC.

- Component 6/14: Component 6 should be confirmed because RAN1 agreed to introduce the FG. Component 14 is not necessary, because it is redundant.

- Component 10: We think this component 10 is not needed. We can reuse Rel.15/16 UE capabilities.

- Component 11: For CHs/RSs which cannot share the indicated Rel.17 TCI state, it is inevitable to be indicated Rel.17 TCI state by reusing the Rel-15/16 signaling/configuration design(s), because Rel. 15/16 TCI state and Rel.17 TCI state cannot be configured in the same band. Hence, this should be basic FG. On the other hand, for CHs/RSs which can share the indicated Rel.17 TCI state, whether to support Rel.17 TCI state by reusing the Rel-15/16 signaling/configuration design(s) can be optional FG. To clarify this, we suggest to add “for CHs/RSs which can share the indicated Rel.17 TCI state”.

- The basic FGs:

  - The basic FGs should be component 3/4/5a/6/8.

  - The basic FGs for UEs supporting CA: We believe either CC-common TCI pool configuration or CC-specific TCI pool configuration should be a basic FG for CA. During the RAN1#107bis-e meeting, more number of companies suggest CC-common TCI pool configuration as basic FG, and we are fine with it. Hence, we suggest to make component 2/9 are basic FGs for CA. In this case, we need new component to indicate CC-specific TCI pool configuration. Hence, we suggest to update component 13 to indicate “support of CC-specific TCI pool configuration”. In case UE supports CC-specific TCI pool configuration, UE should support it on all CCs, otherwise it does not work when the number of CCs supported for CA is larger than the number of CCs supported for CC-specific TCI pool configuration.

For FG23-1-2, we have the following suggestion:

- FG23-1-2 should cover both L1/L2 inter cell mobility and M-TRP inter cell.

- Component 2: We suggest to update as “where at least one [pair/beam] can be associated with a PCI different from serving cell PCI”. The discussion point was that the selected pair/beam is not always associated with a PCI different from serving cell PCI. However, it is true that the selected pair/beam “can be” associated with a PCI different from serving cell PCI.

- Component 3: The difference between component 3 and component 7/8 should be clarified, otherwise, either component(s) should be removed. Also, since FG23-1-2 would be reported per band, we don’t need to differentiate the component 3 for FR1 and FR2.

- Component 7/8: We are fine to have the component 7/8 per agreement in M-TRP inter cell. We believe the scenario of component 7 is more typical in commercial use-case.

- Component 10/11: whether SSB of serving cell PCI and SSB of non-serving cell are overlapped is up to gNB implementation. gNB already transmits SSB for Rel.15/16 UEs, it is difficult to change SSB location for Rel.17 UEs. Since we already have component 7/8, UE can report the number of PCIs for both two modes. Hence, we don’t need component 10/11.

- Component 4/5/6/12/13: We don’t think these components are needed.

For FG23-1-3, we have the following suggestion

- Component 1: Regarding the text in bracket, we think it can be kept. From our perspective, it is fine to clarity that the report is in PHR report.

- Component 2: Considering multiple entry PHR, PHRs and PMPRs of multiple CCs are included in one MAC CE, it is better to clarify the maximum number of reported P-MPR and SSBRI/CRI pairs is per CC or across all CCs. From companies’ input in email discussion during RAN1#107bis-e, it can be observed that companies have different understanding on this. Some company mentioned that it is max number per CC, while some company mentioned that it is max number in a single report which include multiple CCs in case of multi-entry PHR. In our view, the maximum number of reported P-MPR and SSBRI/CRI pairs should be per CC.

- Component 3: Regarding candidate value, {4,8,16,32,64} can be considered.

For FG23-1-4, we support to keep the text “sets”, which is aligned with RAN1 agreement. We support the candidate value of up to 4 value sets. We support to have one value of 0, which represent “DL only” panel, since we have agreed that UL panels can be a subset of DL panels.

Proposal 1: Adopt the following for Rel-17 Multi-beam Operation.

| 23. NR_FeMIMO | 23-1-1 | Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) Basic FGs are component 3/4/5a/6/8. Basic FGs for UEs supporting CA are component 2/9. FFS: separate FGs for inter/intra/joint/separate - Common multi-CC TCI update and activation - For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS - The maximum number of configured joint TCI states per BWP per CC in a band - a) The maximum number of MAC-CE activated joint TCI states per BWP per CC in a band - The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS - Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS - TCI state indication update and activation a) MAC CE based TCI state indication for one active TCI state b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) for more than one active TCI states c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) for more than one active TCI states - Support of CC-common TCI pool configuration and reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC - Maximum number of CCs configured with BFR FFS whether this is a component or just a note in the FG to reuse R16 signaling - Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH, and SRS reusing the Rel-15/16 signaling/configuration design(s), for CHs/RSs which can share the indicated Rel.17 TCI state Note: This has no impact on detail signaling design for SRS TCI indication - The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band FFS: Whether to make component 9 a prerequisite or merge with 9 - Support of CC-specific TCI pool configuration. |  |  |  |  | Per band |  |  |  | For component 4, UE supports FG23-1-1 shall report at least “64” in FR2 and “the maximum number of allowed SSBs” in FR1. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting (for inter-cell BM and mTRP) | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI FFS: whether to include the following components 2-13 into this FG or one or more separate FGs FFS: Whether basic FGs are defined, and if so, which components are basic FGs, i.e., a UE that supports FG 23-1-2 must also support said basic FGs 2. Support of up to K=4 SSBRI-RSRP pairs/beams in one report where at least one pair/beam can be associated with a PCI different from serving cell PCI can be reported (FFS: if K is a component candidate value) 3. The maximum number of RRC-configured PCI(s) different from serving cell PCI for L1-RSRP measurement] 7. The maximum number of configured additional PCIs is X1 when time domain positions and periodicity of configured SSBs with additional PCIs are the same as time domain positions and periodicity of the serving cell SSBs 8. The maximum number of configured additional PCIs is X2 when time domain positions and periodicity of configured SSBs with additional PCIs are different with time domain positions and periodicity of the serving cell SSBs [9. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC}] | [2-24, 2-29] |  |  |  | per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced PHR reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs per CC 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  |  |  |  |  |  |  |  | 2. Candidate value of {1,2,3, 4} 3. Candidate value {4,8,16,32,64} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | 1. Supported UE capability value sets and corresponding max number of SRS ports for each UE capability value set |  |  |  |  | per band |  |  |  | Candidate values: Up to 4 value sets each with one value of {0,1,2,4} Note: the reported list contains only unique value sets This FG is a working assumption | Optional with capability signalling |

  - Multi-TRP for PDCCH

For FG23-2-1, we have following suggestion.

- Component 1: regarding the text in bracket, we think the text except “(except FR2)” should be kept. In our view, if a UE supports PDCCH repetition, it supports both TDM and FDM repetition and also supports repetition for Type3 CSS. We don’t see the intention to add “(except FR2)”.

- Component 3: since it was agreed that UE capability for such overlap is introduced, we support to have component 3. Meanwhile, we think it is better to keep the Note for component 3 to clarify how the number of overlaps is counted when N PDCCH candidates are overlapped, i.e., counted as one overlap or counted as N overlaps. Thus, we suggest to keep the Note for component 3.

For FG23-2-1a, we think the Prerequisite feature groups should be kept. Regarding the note, in our understanding, when 3 is reported in FG23-2-1 component 2, the individual candidate is always monitored in case of overlapping because there is always one individual decoding when the number of BDs is reported as 3. On the other hand, when 2 is reported in FG23-2-1 component 2, whether individual candidate is monitored in case of overlapping needs to be reported, because when 2 BDs is reported, either individual decoding or soft combining may be performed by UE.

For FG23-2-2, we support to remove the bracket and keep the text.

For FG23-2-3, we think this FG should be removed. As analyzed above, if a UE supports PDCCH repetition, it supports both TDM and FDM repetition and also supports repetition for Type3 CSS, thus, there is no need to have separate capability for Type3 CSS.

Proposal 2: Adopt the following for Rel-17 M-TRP PDCCH

| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs with non-SFN scheme TDM and FDM including PDCCH repetition for Type 3 CSS 2. Support of reporting one number as required number of BDs for the two PDCCH candidates 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  |  |  |  |  |  |  |  | Component 2 candidate values: 2 or 3 Component 3 candidate values: [{0,1,2,3}] Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot Note: for component 3, if N PDCCH candidates are overlapped, the number of overlaps is counted as one. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | 23-2-1 |  |  |  |  |  |  |  | Note: If 2 is reported in component 2 of FG 23-2-1, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET is reported. If 3 is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition with non-SFN TDM and/or FDM scheme | 23-2-1 |  |  |  | Per band |  |  | FR2 only |  | Optional with capability signalling |

  - Multi-TRP for PUSCH, PUCCH

For M-TRP PUCCH, for FG23-3-1, we have following suggestion.

- Component1: the text “CB” can be removed, because this component should be common for CB and NCB. The text “cyclic mapping for 2 repetitions” should be kept, because if 2 repetitions is configured, it is cyclic mapping by default, and it should be supported by UE.

- Component2 (maximum number of PHR): this component should be moved to FG23-3-1c

- Component3 (dynamic switching): we support this component.

- FFS: We support the FFS as well as the candidate component values.

For FG23-3-1a, we have following suggestion.

- Remove “for single DCI based M-TRP PUSCH repetition Type A”, because this FG should be common for repetition Type A and Type B.

For FG23-3-1c, we support the text in yellow.

For FG23-3-1-1, we have following suggestion.

- Component1: similar as FG23-3-1, the text “CB” should be removed. And similar as FG23-3-1, support of “sequential mapping for repetitions larger than 2” and “cyclic mapping for 2 repetitions” should be added.

- A new component of “support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission” should be added similar as FG23-3-1.

- FFS: we support the FFS as well as the candidate component values.

Proposal 3: Adopt the following for Rel-17 M-TRP PUSCH

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 3. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources |  |  |  |  |  |  |  |  | Candidate component values: {CB, non-CB, both} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1a | Cyclic mapping | Support of cyclic mapping when the number of repetitions is larger than 2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1b | Second TPC field | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | 1.Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) 2. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1e | A-CSI report | Support of A-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1f | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1g | CG PUSCH transmission | Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 [-CB] | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 3. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources |  |  |  |  |  |  |  |  | Candidate component values: {CB, non-CB, both} | Optional with capability signalling |

For M-TRP PUCCH, for FG23-3-2, we have following suggestion.

- Component1: the text in bracket should be kept. If 2 repetitions is configured, it is cyclic mapping by default, and it should be supported by UE.

- Component2: this component should be removed. If UE supports M-TRP PUCCH repetition, it means UE supports two PUCCH spatial relations/power control parameters per PUCCH resource. There is no need to have a separate component.

For FG23-3-3, similar as FG23-3-2, the text in bracket should be kept.

Proposal 4: Adopt the following for Rel-17 M-TRP PUCCH

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-2b |  | Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2 | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2c |  | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-3 | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |

  - Inter-cell multi-TRP

Component 2 is well aligned with the agreement. But component 3 needs to be updated to be aligned with the agreement, which should be ‘not according to the case in component 2’. The deployment scenario in component 2 is important for commercial NW, hence, it could be defined as a default case for inter-cell MTRP operation, i.e., candidate value of 0 is not needed for component 2.

Regarding the note, it is true that case1 and case2 cannot be enabled simultaneously. However, it is related to NW configuration instead of UE feature, thus, the note can be removed.

Proposal 5: Adopt the following for Rel-17 beam management for M-TRP.

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs is X1 when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 3. The maximum number of configured additional PCIs is X2 when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to the case in component 2 |  |  |  |  |  |  |  |  | Component 2 candidate values: {1,2,3,7} Component 3 candidate values: {0,1,2,3,7} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Beam management for multi-TRP

For 23-5-1, the component 2 and 3 could be removed. In Rel-15, max number of SSB/CSI-RS for beam measurement for L1-RSRP is defined, regardless of non-group based or group-based beam reporting, which could be reused for this Rel-17 feature. If a new UE feature has to be introduced for Rel-17 group based L1-RSRP beam reporting, component 2 is sufficient.

For 23-5-1a, it is also not needed as Rel-16 has introduced max number of resources for beam management, pathloss measurement, BFD, RLM and new beam identification, which could be reused for this Rel-17 feature.

For 23-5-2, component 2 is naturally supported for MTRP BFR and separate component is not needed. To make the description more complete, ‘two BFD-RS sets’ can be added into component 1. For PUCCH-SR for BFR, Rel-16 does not introduce related UE feature, hence, it is assumed that one PUCCH-SR for BFR could be baseline so that component 3 can be kept.

For 23-5-2a, candidate value of 2 is needed for component 1. As one PUCCH-SR for BFR is baseline, there is no need to support candidate values of 0 and 1. For component 2, we have related agreement so it should be kept.

Proposal 6: Adopt the following for Rel-17 beam management for M-TRP.

| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets |  |  |  |  |  |  |  |  | Component 1 candidate values: {1,2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 23. NR_FeMIMO | 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of BFD-RS resources per set, for two BFD-RS sets 3. Support of one PUCCH-SR resource for MTRP BFRQ 4. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP |  |  |  |  |  |  |  |  | Candidate values: {1, 2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-5-2a | PUCCH-SR resources for MTRP BFRQ | 1. Max number of PUCCH-SR resources for MTRP BFRQ 2. Association between BFD-RS resource set on sPCell and a PUCCH SR resource (if component candidate value equals 2) |  |  |  |  |  |  |  |  | Component candidate values: {2} | Optional with capability signalling |

  - HST-SFN deployment

We have the following comments:

- For FG23-6-1/23-6-1a and FG23-6-2/23-6-2a,

  - During the RAN1 discussion for Rel.17 HST, the assumption of the basic operation is a combination of “1) SFN-PDCCH + SFN-PDSCH”, and some UE vendors claim difficulty of supporting other combinations. Hence, the basic capability of FG23-6-1 and FG23-6-2 should be the combination of “1) SFN-PDCCH + SFN-PDSCH”, and other combinations should be reported as optional FGs. The other combinations are “2) S-TRP PDCCH + SFN-PDSCH” and “3) SFN-PDCCH + S-TRP PDSCH”. Note that 3) is only supported in SFN scheme A (FG23-6-1/23-6-1a) based on the previous agreements.

- For FG 23-6-4/23-6-4a,

  - Pre-requisite feature should not be “23-6-1, 23-6-2”. If the pre-requisite feature is “ 23-6-1, 23-6-2”, it means UE supporting FG 23-6-4/23-6-4a should support both “ 23-6-1, 23-6-2”. However, FG 23-6-1 and FG 23-6-2 are independent features, and it is possible that UE supports either FG 23-6-1 or FG 23-6-2. One possible solution is to make FG23-6-1 as basic FG for HST, and make FG23-6-1 as pre-requisite feature of FG 23-6-2 and FG 23-6-4 and FG23-6-4a. We believe the most of functions of FG23-6-1 are included in FG23-6-2.

  - “FR2 only” for FG23-6-4 is incorrect. For FG23-6-4, the threshold of timeDurationForQCL is only reported in FR2. In RAN1 agreements for HST and Rel.15/16/17 spec., the UE behavior of no TCI state field is captured as “scheduling offset equal or larger than the threshold, if applicable”. This is because the UE behavior of no TCI state field in FR1is specified. However, in the current UE feature list, there is no FG to report the support of no TCI state field in FR1. Hence, we suggest to add “(if applicable)” in component 2 and update as “FR2 only for component 1/3”.

- Report granularity:

  - We believe FG23-6-1/23-6-1a/23-6-1b/23-6-2/23-6-2a/23-6-2b can be reported per band.

Proposal: Adopt the following for HST-SFN deployment

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) for PDSCH and PDCCH | 1. Support of SFN scheme A for PDCCH scheduling [single TRP/] SFN Scheme A PDSCH [and default QCL assumption with one or two TCI states for PDCCH] |  |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1, 1_2 | [23-6-1[b]] |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1b | SFN scheme A (scheme 1) for PDSCH only | 1. Support of SFN scheme A for PDSCH [only scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with one or two TCI states for PDSCH] |  |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH | 1. Support of SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH [and default QCL assumption with one or two TCI states for PDCCH] | 23-6-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and PDSCH SFN scheme B by TCI state field in DCI formats 1_1, 1_2 | [23-6-2[b]] |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2b | SFN scheme B (TRP based pre-compensation) for PDSCH only | 1. Support of SFN scheme B for PDSCH [only and default QCL assumption with two TCI states for PDSCH] [scheduled by [single TRP/Scheme B] PDCCH] | 23-6-1 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC) | Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE | 23-6-1 or 23-6-2 |  |  |  | Per UE |  | Yes |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4 | Default DL beam setup for SFN | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold (if applicable) 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold | 23-6-1 |  |  |  | Per band |  | FR2 only for component 1/3 |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4a | Default UL beam setup for SFN | 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | 23-6-1 |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signalling |

  - SRS

For FG23-8-1, we have the following two comments:

- For component 1, we are fine with either “Support of” or “The maximum number of configured available slots offsets for” at the beginning of the text, while we think the latter one could be clearer. Also, we are fine with removing 3 from candidate 1 component values.

- For component 2, we are not sure which agreement in WI implies to define this component. We think this issue needs to be discussed in WI at first.

For FG23-8-2, we are fine with either per UE or per band.

For FG23-8-3, our view is as follows:

- As we described multiple times during the last e-meeting, we believe downgraded configurations for each combination of xTyR should be configurable. Component 1 candidate values should capture them explicitly. We observe the following alternatives on this issue:

  - Alt-0: Define FG23-8-3 for reporting only the mandatory configurations

    - Only a mandatory configuration can be reported. Any downgrading configuration is not supported for antenna architecture with Rx more than 4.

    - Component 1 can indicate either of {t1r6, t2r6, t1r8, t2r8, t4r8}

  - Alt-1: Define FG23-8-3 for reporting the mandatory configuration, and all the downgrading configurations corresponding to the reported mandatory configuration

    - Either “only mandatory configuration” or “mandatory configuration + all the corresponding downgrading configurations” can be reported. Support of only a part of downgrading configurations is not considered.

    - Component 1 can indicate either of {t1r6, t1r1-t1r2-t1r4-t1r6, t2r6, t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6, t1r8, t1r1-t1r2-t1r4-t1r6-t1r8, t2r8, t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8, t4r8, t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6t1r8-t2r8-t4r8}

  - Alt-2: Define FG23-8-3 for reporting the mandatory configuration and all the downgrading configurations with the exception for 6Rx support for UE with 8Rx

    - On top of the candidate report available for Alt-1, for UE that implements 8 Rx antenna architecture, support of downgrading configurations other than the ones with 6 Rx can be reported. This restriction is added because of concern that UE supports 8Rx may not always supports 6Rx.

    - Component 1 can indication either of {t1r6, t1r1-t1r2-t1r4-t1r6, t2r6, t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6, t1r8, t1r1-t1r2-t1r4-t1r6-t1r8, t1r1-t1r2-t1r4-t1r8, t2r8, t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8, t1r1-t1r2-t2r2-t1r4-t2r4-t1r8-t2r8, t4r8, t1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8-t4r8, t1r1-t1r2-t2r2-t1r4-t2r4-t1r8-t2r8-t4r8}

  - Alt-3: Define FG23-8-3 for reporting one or multiple configurations for antenna switching, regardless of mandatory or downgrading ones

    - Any combination of configurations can be reported, while a note can also be added to avoid considering non-practical combinations (e.g., t2r6 and t1r8) for smaller reporting overhead. For example, in case of reporting of multiple configurations, one should be mandatory configuration and the others should be downgrading configuration(s) of the mandatory configuration.

    - Component 1 can indicate one or more of {t1r1, t1r2, t2r2, t1r4, t2r4, t4r4, t1r6, t2r6, t1r8, t2r8, t4r8}

  - Alt-4: Define separate FGs (e.g., FG23-8-3 and a new FG) for reporting mandatory configuration and downgrading configurations, respectively

    - A mandatory configuration can be reported per FG23-8-3. A supported combination of downgrading configurations can be reported per the new FG.

    - Component 1 in FG23-8-3 can indicate either of {t1r6, t2r6, t1r8, t2r8, t4r8}

    - A component in the new FG can indicate one or more of {t1r1, t1r2, t2r2, t1r4, t2r4, t4r4, t1r6, t1r8, t2r8}. No report of the new FG implies no support of any downgrading configuration for antenna architecture with the number of Rx more than 4

Since we would like to unlock downgrading configurations, we prefer to go with the alternatives other than Alt-0 above. Among the alternatives with downgrading configuration, our best preference is Alt-1 since the least signaling overhead is assumed. On other hand, we can understand that just to define the choice like “support all, or not support at all” may result in the difficulty for UE vendors to support downgrading configurations, which is not our preference either. If the issue is only about the support of 6 Rx when the UE supports the antenna architecture with 8 Rx, we would suggest to go with Alt-2. Otherwise, we think it would be good to take either Alt-3 or Alt-4. Between Alt.3-4, we think Alt-4 could be slightly better since “no support of downgrading configuration” can be achieved without signaling overhead. Although we reflect our best preference (i.e. Alt-1) in the following table, we are open to discuss further on this issue from options between Alt.1-4.

- For Component 2 and 3, we are ok with supporting each integer from 1 to 32 as candidate values, which is the same candidate value set as what FG2-55 defines for the corresponding components. Meanwhile, it may be a bit strange that the value for “Report whether …” is an integer. The component descriptions could be updated to improve the readability by reusing the existing text in 38.306.

- Since the prerequisite FG is per-BC, we think FG23-8-3 should also be per-BC capability.

- For a FFS “FFS for component 2&3: impact when UE reports component 1 as xTyR with x=y”, we are not sure what needs to be considered further for component 2 and 3 when UE reports x=y.

For FG23-8-4, we support to define this per BC as the prerequisite FG is defined per BC.

Proposal: Adopt the following for Rel-17 SRS.

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | 1. The maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot | 2-52 |  |  |  | Per band |  |  |  | Candidate 1 component values: {1, 2, 4} [Candidate 2 component values: FFS] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 |  |  |  | [Per UE or per band] |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-55 |  |  |  | Per BC |  |  |  | Component 1 candidate values: - 't1r6’ for 1T6R - 't2r6’ for 2T6R - 't1r8’ for 1T8R - 't2r8’ for 2T8R - 't4r8’ for 4T8R - 't1r1-t1r2-t1r4-t1r6’ for 1T=1R/1T2R/1T4R/1T6R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R - 't1r1-t1r2-t1r4-t1r6-t1r8’ for 1T=1R/1T2R/1T4R/1T6R/1T8R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6-t1r8-t2r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R/1T8R/2T8R - 't1r1-t1r2-t2r2-t1r4-t2r4-t1r6-t2r6t1r8-t2r8-t4r8' for 1T=1R/1T2R/2T=2R/1T4R/2T4R/1T6R/2T6R1T8R/2T8R/4T8R Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} NOTE: The first-listed band with UL includes a band associated with FeatureSetUplinkId set to 0 corresponding to the support of SRS-SwitchiingTimeNR | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 |  |  |  | Per FS |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodoc SRS | 23-8-6 |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  | Per band |  |  |  |  | Optional with capability signalling |

  - M-TRP CSI

For 23-7-1, the candidate value of component 2 could start from 4. For component 3, it is better to define default CSI report mode for MTRP CSI instead of CSI report mode selection. On the other hand, the UE supporting mode 2 could easily support mode 1 with X=0, hence, we suggest supporting both mode 1 with X=0 and mode 2 in component 3. For component 4, 5, 6, we’re okay to keep them.

Proposal: Adopt the following for Rel-17 M-TRP CSI.

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of Nmax=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode of mode 1 with X=0 and mode 2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for single-TRP measurement [per CC/across all CCs] - Maximum total number of CMRs for NCJT measurement [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement - A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC - A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs | FFS |  |  |  | [Per band and/ per BC] |  |  |  | Component 2 candidate value set: { 4, 5, 6, 7, 8} Component 4 candidate values: - [{2, 4, 8, 12, 16, 24, 32}] - {2, 4, 8, 12, 16[, 24, 32]} - [{1,2,3,4 … 64}] - {2,3,4 … 164} - [{4,5,6, …, 256}] - {2,3,4, …, 256} [Component 5: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 6: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 selection | Maximum value of numberOfSingleTRP-CSI-Mode1 |  |  |  |  |  |  |  |  | Component 1 candidate value set: {X=1, X=2} | Optional with capability signalling |

  - FDD CSI

The UE feature on port selection CSI is close to stable now.

For 23-9-4, there is duplicated ‘to support’ in component 2, which should be removed. We’re okay with the candidate values of max number of resource and max number of total ports in component 2.

For 23-9-5, to align with 23-9-2 and 23-9-4, the FFS part of (codebook 2, codebook 3) can be revised as {FeType II PS R=2, NULL}.

Proposal: Adopt the following for Rel-17 port selection CSI.

| 23. NR_FeMIMO | 23-9-2 | Support of M=2 and R=1 for FeType-II | 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1 2. Support parameter combinations with M=2 | 23-9-1 |  |  |  | per band and per BC |  |  |  | Component 1 candidate values - Maximum 16 8 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II 2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=2 | 23-9-2 |  |  |  | per band and per BC |  |  |  | Component 2 candidate values: • Maximum 8 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: {1 to 64} • Max # total ports: {4 to 256} | Optional with capability signalling |

| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 16-3b, 16-3a-1, 16-3b-1, 2-36, 2-40, 2-41, 2-43 |  |  |  | Per band and per BC |  |  |  | Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL}, {FeType II PS M=2 R=1, NULL}, {FeType II PS R=2, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256}] Note 1：if a UE reports one or more codebook combinations in [23-9-5], then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1/23-9-2/23-9-4 [Note: If the list of triplets for a codebook combination is empty then the list of triplets is reused from the codebook combination indicated previously] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Summay

Based on the discussions on Rel.17 FeMIMO UE features in Session 2, we propose to adopt the updated UE feature tables for each UE feature.