3GPP TSG RAN WG1 #108-e	                	                                                                                              R1-2201507

e-Meeting, February 21th – March 3th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel.17 UE features for UE power saving enhancements

Agenda Item:	8.16.7

Document for: 	Discussion and Decision

Introduction

Updated Rel.17 UE feature list after RAN1#107b-e including UE power saving enhancements has been agreed [1]. In this contribution, we present our views and updates regarding the UE features for UE power saving enhancements based on the list.

Rel-17 UE features for UE power saving

We itemized our suggested corrections to the existing UE features as follows.

- FG 29-1:

  - Type should be per UE

- FG 29-2:

  - Regarding whether to support FG 29-2 as optional with capability signaling or optional without capability signaling, we think it can be left to RAN2 discussion.

  - Regarding prerequisite feature groups, given UEs that support this feature may not support FG 29-1, we don’t think FG 29-1 should be pre-requisite.

  - Type should be per UE

  - Regarding the note, it should be confirmed since we had the following agreement in the last meeting

Agreement Confirm the following working assumption Working Assumption If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration. Agreement If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI Note: Huawei, MTK and ZTE have concern on the agreement.

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  | FFS | N |  | UE cannot receive TRS resources for idle/inactive mode | Per UE | N | N | N | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1 | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG 29-3a to 29-3d:

  - Type should be per UE

According to the above comments, the following revision is proposed for UE power saving.

Proposal 1: Adopt the following changes on UE power saving feature groups:

- FG 29-2:

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  | FFS | N |  | UE cannot receive TRS resources for idle/inactive mode | Per UE | N | N | N | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1 | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Summary of proposals

The proposals in this paper are summarized as follows.

Proposal 1: Adopt the following changes on UE power saving feature groups:

  - FG 29-1: Type should be per UE

  - FG 29-2:

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  | FFS | N |  | UE cannot receive TRS resources for idle/inactive mode | Per UE | N | N | N | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1 | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - FG 29-3: Type should be per UE

Reference

[1]	Moderators (AT&T, NTT DOCOMO, INC.), R1-2200678, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107b-e, Jan. 2022.