3GPP TSG RAN WG1 #108-e			R1-2201519

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.11

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for NR sidelink enhancement

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.11 regarding UE features for NR sidelink enhancement and captures the following email discussion.

[108-e-R17-UE-features-Sidelink-01] Email discussion on UE features for NR sidelink enhancements – Shinya (DOCOMO) - 1st check point: February 25 - Final check point: March 3

In the updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e [1], there are following feature groups for NR sidelink enhancement.

- 32-2	Receiving NR sidelink of PSFCH/S-SSB

- 32-4	Transmitting NR sidelink mode 2 with partial sensing

- 32-4a	Transmitting NR sidelink mode 2 with random resource selection

- 32-5a	Inter-UE coordination scheme 1 in NR sidelink mode 2

- 32-5b-1	Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2

- 32-5b-2	Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2

Also, in the updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e [2], there are following feature groups for NR sidelink enhancement.

- 4-1		[Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB]

- 4-2		[Receiving NR sidelink of PSFCH/S-SSB only]

- 4-3		Transmitting NR sidelink mode 2 with full sensing

- 4-4		Transmitting NR sidelink mode 2 with partial sensing

- 4-5		Inter-UE coordination in NR sidelink mode 2

The issues to be discussed are tagged and colour coded with High priority, Medium priority, or Low priority, considering RAN2 impact especially for capability signaling design.

- General matters

This section discusses general matters applicable to all Rel-17 SL FGs.

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [1] | Huawei, HiSilicon | For the all newly defined/introduced UE features, the support the configuration by network should not be a mandatory since there are PC5 only band at least for ITS band 47. This issue has been treated by added a note in Rel-16 like: “Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1”. We think this principle should also been used for Rel-17. Proposal 10: Add a note to the newly defined Rel-17 UE features to treat the supporting of network configuration for PC5 only band cases, e.g. - “Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1”. |
| --- | --- | --- |
| [13] | MediaTek Inc. | We agree with the above working assumption and we prefer to confirm it. Proposal 1: Confirm the following working assumption from RAN1-107-e. - FGs 32-x are not basic FGs for sidelink enhancement. We agree with the above working assumption and we prefer to confirm it. Proposal 2: Confirm the following working assumption from RAN1-107bis-e, - Necessary Rel-16 FGs are handled, via pre-requisites of Rel-17 FGs for sidelink enhancements - Necessary components in Rel-16 FGs are added to Rel-17 FG components for sidelink enhancements if an entire Rel-16 FG cannot be included as pre-requisites. |

Discussion

Low priority proposal 2-1:

- Add a note to the newly defined Rel-17 SL UE features to treat the supporting of network configuration for PC5 only band cases as follows:

  - “Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1”

    - Support: HW/HiSi

| Company | Comment |
| --- | --- |
| NTT DOCOMO | Although we are fine with the intention of the note itself, it is unclear for us whether all FGs shall have this note. In Rel-16 FGs, it seems that there are some FGs that do not have this note. What is difference between Rel-16 FGs and Rel-17 FGs for this note? |
| ZTE,Sanechips | Ok with such note @DCM, perhaps at least some M1 related FGs shall not need this note for Rel-16 UE features, while all the Rel-17 FGs need such a note. |
| Qualcomm | Support |
| Huawei, HiSilicon | Support |
| Panasonic | Support |
| Futurewei | We are ok with this note |

- 32-2 for NR: Receiving NR sidelink

In [1], FG 32-2 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-2 | Receiving NR sidelink of PSFCH/S-SSB | 1) UE can receive NR PSFCH/S-SSB. FFS whether other components will be included | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [2] | FUTUREWEI | For the Type B UE feature, i.e., 32-2 receiving NR sidelink of PSFCH/S-SSB only, an FG 32-2 was agreed in RAN1#107-e with FFS on whether further split the reception of PSFCH and S-SSB and FFS on the components, as shown in Table 2, i.e., the updated table in [6]. However, in RAN1#107bis-e due to diverse view on whether further split the FG, no consensus can be made. It was concluded that the whole FG is highlighted as unstable with the FFS on the further split removed. As discussed in our previous contributions [7][8], since the UE reception types are used as the reference for evaluation and designing of SL power saving features in R17, there were no agreements to support these UE features. Therefore, we may not need to specify UE features for all these SL reception types. Also based on the WID in [3], the objective for power saving is to introduce sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. Further, it is included in the WID that enhancements introduced in Rel-17 should be based on the functionalities specified in Rel-16. The WID indicates that Rel-16 NR sidelink resource allocation mode 2 is the prerequisite of the power saving features for Rel-17 sidelink. Therefore, it is not necessary to define FGs for the three UE types. If there is no consensus, we prefer to drop FG 32-2. If FG 32-2 for Type B UE reception is kept as most companies want it, we do not support further split of the capability of PSFCH and S-SSB as we did not have any discussions, agreement, or evaluation in RAN1 SL group on such new type of UEs. This is also aligned with the conclusion from last meeting for no consensus in RAN1. Proposal 3: Prefer not to include the FG 32-2. - If FG32-2 is agreed, do not support further split the capabilities for PSFCH and S-SSB receptions as different FGs If FG 32-2 is kept, here are the discussions on the related Rel-16 UE features as the pre-requisites or the components of this new feature. - 15-1: For UE SL reception FG 15-1, all are not supported except component 8 as SCS and CP still needs to be reported for PSFCH reception. - 15-3: All transmission features in FG 15-3 are not needed as Type B UE feature is for receiving capability only. - 15-4: Synchronization is needed for SL transmission. S-SSB reception is supported in this feature. Therefore, FG 15-4 is supported. - 15-5: Since type B UE does not support PSCCH reception, it cannot measure or process CBR measurements. As agreed in RAN1#107-e, when no SL CBR measurement result is available, a (pre-)configured SL CBR value is used. Congestion control based on CBR and CR is still supported. However, since there is no CBR measurement, there is no need to consider the congestion control feature support here. Instead, it can be considered for FGs of Tx capabilities. - 15-11: This FG is needed as PSFCH reception is supported in this new feature - 15-23: This FG cannot be supported as UE is unable to receive the RSRP report from Rx UE for open loop power control. Based on above discussions, in summary, component 8 of 15-1 is included as a component of FG 32-2 and FGs 15-4 and 15-11 are included as prerequisites of FGs 32-2. Since FG 32-2 is defined as a receive capability, as companies preferred in RAN1#107bis-e, we are fine to include only the Rx related components of 15-4 and 15-11 as components of FG 32-2. Proposal 4: The components of FG 32-2 (if it is specified) include component 8 of 15-1 and Rx related components of FGs 15-4 and 15-11. |
| --- | --- | --- |
| [7] | NTT DOCOMO, INC. | This FG’s issue was not converged at RAN1. Further discussion at RAN plenary is assumed, thereby our view on this FG is omitted from this RAN1 contribution. |
| [8] | Intel Corporation | For FG 32-2 it needs to be clarified whether reception of PSFCH is supported only for HARQ or both HARQ and IUC operation. In our view, RAN1 can proceed with either of the alternatives: - Alt.1: Clarify that “NR PSFCH/SSB reception” means “NR PSFCH HARQ/SSB” reception. In this case UE does not support resource reselection based on Scheme-2 IUC feedback. - Alt 2: Introduce two FGs in terms of PSFCH reception for HARQ and PSFCH reception for IUC as PSFCH reception for IUC require changes in UE resource allocation behavior. In our view, pre-requisites need to be defined for FG 32-2 as UE supporting PSFCH/SSB reception is not a useful feature and thus at least FG 32-4a (TX-M2 w/random resource selection) should be pre-requisite for FG 32-2. - Select one of the following alternatives for FG 32-2 (Receiving NR sidelink of PSFCH/S-SSB): - Alt.1: Clarify that FG 32-2 is applicable for PSFCH HARQ reception - Alt.2: Split FG 32-2 into two FGs: - FG 32-2a: Receiving NR sidelink of S-SSB and PSFCH for HARQ operation - FG 32-2b: Receiving NR sidelink of S-SSB and PSFCH for IUC operation - FG32-2a is a pre-requisite of FG 32-2b - Define FG 32-4a (TX-M2 w/random resource selection) as a pre-requisite for FG 32-2/a/b |
| [13] | MediaTek Inc. | We have a preference to split as FG 32-2 into two FGs. These PHY channels essentially have different functionalities in possibly different user scenarios. A UE may prefer to have RX capability for PSFCH in a performance-sensitive SL use case wherein reliable packet transmissions are essential while reception of S-SSB is not necessary (e.g., in sidelink use cases for in-coverage scenario). Also, support for S-SSB reception may be an important necessity for UE if the coverage conditions are varying, although high system reliability may not be required. For these reasons, in our view, it is preferable to split the reception of PSFCH and S-SSB. We propose the following. Proposal 3: FG 32-2 should be split into two FGs as “reception of PSFCH” and “reception of S-SSB”. |
| [16] | Ericsson | The feature group 32-2 indicates that a UE is able to receive the PSFCH and S-SSB. This FG is useful for indicating limited SL capabilities. We are supportive of including this FG since it is a new capability agreed in Rel-17 and it is aligned with the agreements and conclusions taken in RAN1. Moreover, in our view, no other components should be included in this FG. No other components shall be included in the FG 32-2. In our view, there is no need to split the capability further since both the S-SSB and the PSFCH are needed for a UE that has some basic SL functionalities. The rationale is the following: - The S-SSB reception is needed for a proper functionality of the SL feature due to the synchronization requirements. If the UE is not able to receive S-SSB it cannot operate out-of-coverage. - The PSFCH reception is needed in order to monitor the success or failure of the UE previous transmission, and therefore, increase the reliability of the transmissions. The FG 32-2 does not need to be split further. The S-SSB and PSFCH reception is not signalled in different FG. |

Discussion

| Company | Comment |
| --- | --- |
| Moderator | As concluded in the last RAN1 meeting as follows, RAN1 couldn’t achieve any consensus on whether to split the capabilities for PSFCH and S-SSB receptions as different FGs. RAN1 can come back FG 32-2 after some decision/guidance is made in RAN plenary. Conclusion - There is no consensus in RAN1 on “FFS whether to split the capabilities for PSFCH and S-SSB receptions as different FGs” - Remove the FFS in FG 32-2, highlight FG 32-2 |

- 32-4 to 32-4a for NR: Transmitting NR sidelink mode 2

In [1], FGs 32-4 to 32-4a are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-4a | Transmitting NR sidelink mode 2 with random resource selection | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added | [TBD] | [Yes] | [No] | UE does not support transmission according to the random resource selection and resource allocation | [Per band] | N.A. | N.A. | N.A. | Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: Component 4 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-4 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-4 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-4 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 6 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [1] | Huawei, HiSilicon | For FG 32-4a Transmitting NR sidelink mode 2 with random resource selection, it can leave the space to achieve extreme low cost device without any receiving capability. So it doesn’t need any prerequisite feature groups of NR sidelink reception. Which means if UE only report 32-4a, it doesn’t need to have any receiving capabilities. Since FG 32-4a can imply the case of UE without NR SL reception, FG 15-4 defined in Rel-16 which is synchronization sources for NR sidelink includes sync source of GNSS, SyncRef UE and gNB can’t be directly used for synchronization capability of such UE as SyncRef UE is not able to be synchronization source. We need to include related components of GNSS and gNB sync source in FG 15-4 as components of FG32-4a to solve the synchronization issue for UE without NR SL reception However, random resource selection can also be supported by a UE which has full or partial sensing capability, and in that case the full list of sync sources is assumed to be supported. These components are in common with FG 15-4 for a UE with sidelink reception capability, and thus do not imply extra signaling or duplicate reporting for such a UE. Proposal 1: Add synchronization source as components of FG32-4a to indicate UE without NR SL reception - 8) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. - 9) UE can transmit NR sidelink based on the synchronization to an gNB - 10) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. - 11) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. - After adding the above, no other components are needed, hence the related FFS can be removed. Proposal 2: The ‘Note’ regarding Rel-16 random selection in the exceptional pool is not necessary, since that feature is always a part of the MAC procedures for any UE. Unlike the other ‘Notes’, which have spec impact, this note has no such impact, and hence can be removed. For synchronization, we can reuse existing FG 15-4 and include it as pre-requisite for FG 32-4. And a note is needed in FG 32-4 as follows: “If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B”. For the note regarding Rel-16 exceptional pools, support of this feature is always a part of the MAC procedures, whereas the Rel-17 feature is new and already agreed as a separate capability. In a later release of a feature, there is a general principle not to couple features together to allow faster independent implementation. Proposal 3: For FG 32-4 Transmitting NR sidelink mode 2 with partial sensing: - Prerequisite feature group is 15-4. - Add a note: If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B. - Remove the FFS “whether any other components should be added”. - Confirm the note regarding Rel-16 random selection in an exceptional pool. For FG 32-4 Transmitting NR sidelink mode 2 with partial sensing, there is no difference from receiving aspect compared with mode 2 full sensing scheme. It also need to receive and decode SCIs to perform resource exclusion. It is a UE with SL reception, and we notice that the FGs 15-1, 15-4, 15-5, 15-11 and 15-23 are basic FGs in Rel-16. This principle can be applicable in all Rel-17 UE with SL reception including UE supports inter-UE coordination. Besides, FG 15-14 was defined as mandatory with capability signalling for UEs supporting NR sidelink, in order to ensure the performance of NR sidleink, it can also be pre-requisite of Rel-17 UE with sidelink reception. Proposal 4: A Rel-17 UE with NR SL reception needs to indicate FG 15-1, 15-4, 15-5, 15-11, 15-14, 15-23. This can be captured by pre-requisite listing in Rel-17, or by a separate agreement in TR 38.822. We assume RAN2 will handle the definition of capabilities related to SL DRX without input from RAN1. Based on RAN1 agreements that resource allocation schemes (full sensing, partial sensing, and random resource selection) are configured per resource pool, it is essential for gNB to be aware of the UE features on what resource allocation schemes it supports. For inter-UE coordination, it is also agreed that features for both scheme 1 and scheme 2 can be enabled or disabled or controlled by (pre-)configuration, and therefore gNB should be informed with UE capability. Proposal 11: UE features regarding resource allocation schemes and inter-UE coordination schemes need to be informed to gNB. In Rel-16, it was decided that a UE would need to support mode 1 in licensed spectrum where gNB is defined. This was to ensure network operator control could be exercised over sidelink when in licensed spectrum. The Rel-16 principle and agreement extend to Rel-17, where otherwise a UE might report only Rel-17 FGs (and associated pre-requisites), all of which apply to mode 2 only, and hence be always beyond network control in licensed spectrum. The simplest way to capture this in the Rel-17 FG list is, as with Rel-16, adding to the notes column of the rows for partial sensing, random selection, inter-UE scheme 1, and inter-UE scheme 2. Proposal 12: Add a note for Rel-17 resource allocation FGs 32-4, 32-4a, 32-5a-a, 32-5a-b, 32-5b-a, 32-5b-b: “For UE supports this FG, and NR sidelink in licensed spectrum where gNB is defined, UE must indicate FG 15-2 is supported.” |
| --- | --- | --- |
| [2] | FUTUREWEI | As aforementioned, in RAN1#107bis-e, it was concluded that the UE without NR SL reception is supported in Rel-17. However, it is meaningless if such UE does not support any other Rel-17 UE features. The purpose of the conclusion is to support Type A UE which is only for Rel-17 SL transmissions. Therefore, a Type A UE should support a transmit feature which is clearly FG 32-4a RRS. In order to support transmissions in SL, such a UE has to support certain synchronization in order to transmit on the SL frame structure. In RAN1#107bis-e, a new synchronization FG 32-4b was proposed and discussed but no consensus was achieved. In the FL proposal, the components introduced in FG 32-4b are the components for the Rel-16 synchronization FG 15-4 except that synchronization to a synchRef UE is removed as the UE-A does not have the capability of receiving S-SSB for synchronization. However, there were some questions on some components. First, the gNB is included as one synchronization source. Since UE-A does not have capability of reception of S-SSB, it is not clear that it has the capability of reception of the synchronization signal from gNB. For this issue, we think that during the discussions we only agreed that Type A UE is not capable of performing reception of any SL signals and channels. We did not discuss whether UE-A is capable of reception of gNB signals. Although the Rel-17 SL WI focus on mode 2, it does not mean that it is only for out-of-coverage case. We prefer that Type-A UE has the capability of sync with gNB. Therefore, we prefer to include the sync with gNB case in FG 32-4b. The other question is whether to include the transmission feature component in the synchronization FG 32-4b. Since this is the synchronization FG which is not viewed as either a reception capability or transmit capability, following the Rel-16 synchronization FG 15-4, we should include the components of the transmit features. In particular, the capability of transmitting S-SSB should be included as GNSS sync is supported so that the UE can be a syncRef UE. Proposal 1: Support adding an FG for synchronization sources for NR sidelink transmission FG 32-4b - Include components of Rel-16 FG 15-4 as components of FG 32-4b (except the sync to the syncRef UE) including the component of transmitting S-SSB. 32. NR_SL_enh; 32-4b; Synchronization sources for NR sidelink transmission; 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit NR sidelink based on the synchronization to an eNB. 6) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 7) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 8) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a FFS whether other components will be included; At least one of 15-1,15-2, 15-3, 32-4, 32-4a; Yes; No; Per band; N.A.; N.A.; N.A.; This FG can only be reported as a pre-requisite to FG 32-4a Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. Regarding the prerequisite, we can follow FG 15-4 and include at least one of Rel-17 FGs, i.e., FG 32-4a RRS and FG 32-4 partial sensing, and related Rel-16 FGs, i.e., FG 15-1, FG 15-2, and FG 15-3. Proposal 2: The prerequisite of Rel-17 sync FG 32-4b is at least one of 15-1, 15-2, 15-3, 32-4, 32-4a. For FG 32-4a RRS feature, since it is a transmitter capability, all UE receiving FGs are not applicable. Only transmitting FG 15-3 and FG 15-5 for congestion control are applied. - 15-3: Since Type A/B UEs can support random resource selection but not full sensing, to avoid the conflict with features of type A/B UEs, component 4 for full sensing capability in FG 15-3 should be excluded. - 15-5: Since a type A/B UE does not support PSCCH reception, it cannot measure or process CBR measurements. Therefore, component 1 is not supported. As agreed in RAN1#107-e, for random resource selection, when no SL CBR measurement result is available, a (pre-)configured SL CBR value is used. Congestion control based on CBR and CR is still supported. Component 2 can be updated as UE can adjust its radio parameters based on a pre-configured CBR value and CRlimit. For component 3, type A/B UE can only processes CR. As shown in APP-1, in the updated FG 32-4a agreed in RAN1#107bis-4, the components of FG 15-3 were included in FG 32-4 except component 4 for full sensing. However, the components for congestion control were not included, which we propose to include as the components of FG 32-4a. Proposal 5: Include congestion control components as in FG 15-5 to FG 32-4a RRS with necessary updated based on Rel-17 agreements. For FG 32-4a RRS, no prerequisite is necessary. Therefore, the prerequisite of FG 32-4a is none. In addition, if FG 32-4a is not supported, a UE does not support random resource selection in a shared resource pool. However, a UE may still support random resource selection in the exception pool if UE supports Rel-16 basic feature. Correspondingly, the column “Consequence if the feature is not supported by the UE” for FG 32-4a is updated in Table 5. Proposal 6: For FG 32-4a RRS, in the column of “Consequence if the feature is not supported by the UE”, the description should be updated as - UE does not support transmission according to the Rel-17 random resource selection and resource allocation In the agreed FG 32-4a RRS from RAN1#107bis-e, the note on the Random selection according to Rel-16 in the exceptional pool is to be confirmed. One issue of the note is that the meaning of “according to Rel-16” is not clear. Since there is no RRS FG specified and UE may only support Rel-17 RRS and basic sync FG, but not support any Rel-16 features, how to access exceptional pool according to Rel-16 procedures is not well defined. However, from PHY point of view, there is no problem for a UE capable of Rel-17 RRS to access the exceptional pool with RRS. The issue may be an upper layer problem. Therefore, if there is no issue from upper layer, UE can support RRS according to Rel-16 in the exceptional pool. Another way is that we can specify that the exceptional pool in Rel-16 is supported in Rel-17, we can simply remove the wording “according to Rel-16”. Proposal 7: For the note on Random selection in FG 32-4a RRS, - Alt. 1 confirm the note on random selection according to Rel-16 in the exceptional pool if there no issue in the upper layer procedure. - Alt. 2 remove “according to Rel-16” if the exception pool in Rel-16 is supported in Rel-17. For the agreed FG 32-4 partial sensing feature, as discussed in RAN1#107bis-e, full sensing is not prerequisite of the partial sensing. Then in the updated FG 32-4 agreed in RAN1#107bis-e, the components of Rel-16 FG 15-3 except full sensing component 4 were included. First, as agreed, RRS is supported in partial sensing as one of the options when minimum M slots for CPS cannot be guaranteed. Therefore, FG 32-4a RRS should be a prerequisite of FG 32-4 partial sensing. In general, the reception capability FG 15-1 should be the prerequisite of FG 32-4. Also, since congestion control and CBR measurement is supported for partial sensing, FG 15-5 is a prerequisite FG for 32-4. Proposal 8: Add FG 32-4a, FG 15-1, and FG 15-5 as the prerequisite of FG 32-4 partial sensing. Regarding the note on the Random selection in FG 32-4, similar to that in FG 32-4a, Proposal 7 is applicable. In FGs 15-3, 32-4 and 32-4a for SL transmissions, as in the components, up to B sidelink processes are supported. One issue arises that if UE reports more than one FGs of these three, how many processes should UE support it. We prefer that in this case, the total number of processes is B instead of sum of each value B in all reported FGs. A note can be added in both FG 32-4 and 32-4a. Proposal 9: Add the following note to FGs 32-4 and 32-4a - If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B. |
| [3] | vivo | In the case that a UE reports both FG 32-4 (i.e., partial sensing) and 32-4a (i.e., random selection), and when FG 32-4 indicates “up to 8 SL processes are supported”, while 32-4a indicates “up to 16 SL processes are supported”, it is then not clear how many sidelink processes are supported by the UE. In our view, it should not be understood as the UE supports up to 8+16=24 sidelink processes. One possible solution is to report the same number of B for each of FG 32-4, 32-4a and 15-3. However, it precludes the possibility that UE supports a smaller number of full/partial sensing sidelink processes, which is desirable for the UE because a larger number of sensing processes requires larger buffer size and higher power consumption. Therefore, it is proposed that the supported sidelink processes for a given UE is the largest number of B among the reported FGs of 32-4, 32-4a and 15-3. Proposal 1: The total sidelink processes supported by the UE is the largest number of value B among the reported FGs of 32-4, 32-4a and 15-3. |
| [4] | OPPO | In Rel-16, FG 15-4 is defined as basic FG and each UE who support NR SL should support this FG. FG 15-4 includes components related to SL/S-SSB transmission and SL/S-SSB reception. In Rel-17, Rel-16 basic FGs are not basic FGs any more. UE needs to report which FG it supports. Some new FGs which is related to SL transmission, such as FG 32-4/32-4a/32-5b-1, or SL reception, such as FG32-2/32-5b-2, are introduced respectively. Type-A UE (without SL reception capability) is also supported in Rel-17, which can only perform SL transmission. For type-A UE, S-SSB transmission can be supported while not for S-SSB reception. Therefore, it is preferred to introduce new FG for synchronization, and furthermore, this FG is split into two FGs for S-SSB transmission and reception respectively. Proposal 1: - New FGs for synchronization are supported in Rel-17. - FGs for synchronization are split into two FGs for S-SSB transmission and reception respectively. In FG 15-4, supporting SyncRef UE as the synchronization reference is one of component which means synchronizing to SyncRef UE is supported for each UE. In Rel-17, we also think it shall be supported by UEs who have SL reception capability, such as Type-B and Type-D UE. Considering supporting SyncRef is related to S-SSB reception, it is preferred to include SyncRef UE related components to FG for S-SSB reception. Proposal 2: - SyncRef UE as the synchronization reference shall be supported in Rel-17. - SyncRef UE related components is added into FG for S-SSB reception. Proposal 3: The new added FG for S-SSB transmission and reception is as follows: 32. NR_SL_enh; 32-4b; Synchronization sources for NR sidelink transmission; 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 4) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 5) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 6) UE can transmit NR sidelink based on the synchronization to GNSS 7) UE can transmit NR sidelink based on the synchronization to an gNB 8) UE can transmit NR sidelink based on the synchronization to an eNB 9) UE can transmit S-SSB in NR sidelink; None; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 2/3/6/7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. 32. NR_SL_enh; 32-4c; Synchronization sources for NR sidelink reception; 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 4) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 5) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 7) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 8) UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 9) UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 10) UE can receive NR sidelink based on the synchronization to GNSS 11) UE can receive NR sidelink based on the synchronization to an gNB 12) UE can receive NR sidelink based on the synchronization to an eNB 13) UE can receive NR sidelink based on the synchronization to a SyncRef UE 14) UE can receive S-SSB in NR sidelink; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 2/3/6/7/10/11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. According to the proposal in section 2.1, there are two FG for synchronization are introduced. If UE supports FG 32-4, which is based on partial sensing, UE has the reception capability. Both FG 32-4b (synchronization for SL transmission) and FG 32-4c(synchronization for SL reception) can be pre-requisite. If UE supports FG 32-4a, which is based on random selection, it is possible that UE does not have reception capability. In this case, only FG 32-4c((synchronization for SL reception) can be pre-requisite. Proposal 4: - FG 32-4b and FG 32-4c are pre-requisites of FG 32-4. - FG 32-4c is pre-requisite of FG 32-4a. The total number SL processes UE can support is independent of how many FG it supports. We propose to add a note to clarify that. Proposal 5: - Add a note in FG 32-4: “The reported value B is the total number of SL processes that UE can support. UE is not required to support the sum of reported value B in each FG” - Add a note in FG 32-4a: “The reported value B is the total number of SL processes that UE can support. UE is not required to support the sum of reported value B in each FG” Another open issue is about the note: “Note: Random selection according to Rel-16 in the exceptional pool is supported”. Considering a new FG 32-4a for random resource selection, which is different with random selection in Rel-16, we think it is necessary to clarify in the note that the random selection is based on Rel-16, not Rel-17. Proposal 6: It is necessary to clarify in the note that it is based on Rel-16: “Note: Random selection according to Rel-16 in the exceptional pool is supported” |
| [5] | CATT, GOHIGH | When a UE performs sidelink transmission, it is required to be synchronized with sidelink reference timing regardless of its capability on sidelink reception or not. Since UE without NR SL reception is supported in Rel-17, a new FG on synchronization capability without NR SL reception shall be introduced. In RAN1#107b-e meeting, a new FG on synchronization capability is discussed but no consensus was achieved. From our understanding, the proposed FG is generally fine in the direction. But we have some comments as follows. - On component 3) and 4) for “if the UE supports Components 1 and 2”, the red sentence should be removed, there is no dependency between component 1/2 and component 3/4. - On component 5)-7), it is preferred to be a separate FG which is similar as that of Rel-16 FGs, since some UE may only operate in only NR Uu mode. - On the note part “This FG can only be reported as a pre-requisite to FG 32-4a”, it is not necessary to restrict this FG is only be pre-requisite of FG32-4a, it can be also the pre-requisite of FG32-4(i.e. UE with partial sensing). Based on the above comments, the updated proposal is provided as follows. Proposal 1: Two FGs on synchronization capability for power saving UEs should be supported, one is derived from 15-4(i.e. 32-4b), the other is derived from 15-15(i.e. 32-4c), 32-4b should be the pre-requisite of 32-4c. 32. NR_SL_enh; 32-4b; Synchronization sources for NR sidelink transmission; 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit NR sidelink based on the synchronization to an eNB. 6) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 7) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 5) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a FFS whether other components will be included; Yes; No; Per band; N.A.; N.A.; N.A.; This FG can only be reported as a pre-requisite to FG 32-4a Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. 32. NR_SL_enh; 32-4c; eNB type Synchronization sources for NR sidelink transmission; 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true.; 32-4b; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. There are two remaining issues on FG32-4 and FG32-4a, one is for synchronization capability, the other is on how to define total SL processes of UE reports more than one FGs of 15-3, 32-4 and 32-4a. Regarding the synchronization capability, in order to obtain sidelink reference timing for NR sidelink transmission, at least one of 15-4 and 32-4b should be the pre-requisite of FG32-4/FG32-4a. Regarding total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. Proposal 2: For FG32-4/FG32-4a, in order to obtain sidelink reference timing for NR sidelink transmission, at least one of 15-4 and 32-4b should be the pre-requisite of FG32-4/FG32-4a. Proposal 3: Add a note in FG32-4/FG32-4a, “If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B” |
| [7] | NTT DOCOMO, INC. | FGs 15-3/32-4/32-4a have the same parameter B. Even when UE reports support of more than one from these FGs, it should not mean that UE can support 2*B or 3*B. In other words, the reported B is total SL processes supported by the UE regardless of resource allocation mechanism. With this understanding, the FL proposal shared before end of the last meeting would be OK and agreeable. Proposal 1: - Add a note in FG 32-4: - If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B. In our understanding, Rel-17 random selection is assumed to be applied in a normal resource pool. Random selection mechanism at the exceptional pool, which was defined in Rel-16, is not enhanced in Rel-17. Proposal 2: - Keep “according to Rel-16” from the note in FG 32-4. Pre-requisite for this FG is still not fixed. Basically required feature for transmission with partial sensing is only synchronization perspective. However, the existing FG for synchronization, i.e. FG 15-4, includes a lot of components. In Rel-17 SL, some UE would support limited features, thereby it is not valid to set FG 15-4 as pre-requisite of FG 32-4. At the last meeting, a new FG with minimum components for synchronization completion was proposed, we believe that the FG should be introduced and set as pre-requisite of FG 32-4. Regarding eNB-related components, we feel that forcing power saving UE to support eNB part is a bit excessive requirement. Proposal 3: - Introduce a FG containing minimum components for synchronization purpose: FG 32-4b. - FG 32-4b is added as pre-requisite of FG 32-4. 32-4b; Synchronization sources for NR sidelink transmission; 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a.; Yes; No; [Per band]; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. These aspects for FG 32-4, i.e. partial sensing, can be the same as in FG 15-3 for full sensing. Proposal 4: - FG 32-4 is reported to gNB but not reported to UE. As discussed above, the note should be added to FG 32-4a as well as FG 32-4. Proposal 5: - Add a note in FG 32-4a: - If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B. After checking the latest RAN2 spec based on Rel-17 agreements [2][3], we think random selection at the exceptional pool is used in some situations regardless of resource allocation mechanism. Therefore, the note should be kept as it is. Proposal 6: - Keep the note of random selection at the exceptional pool in FG 32-4a. As discussed above, the newly introduced FG with minimum components for synchronization completion should be set as pre-requisite of FG 32-4a. Proposal 7: - FG 32-4b is added as pre-requisite of FG 32-4a. These aspects for FG 32-4a, i.e. random selection, can be the same as in FG 15-3 for full sensing and as in FG 32-4 for partial sensing. Proposal 8: - FG 32-4a is reported to gNB but not reported to UE. At the last meeting, there was a discussion on whether a FG of UE behavior to consider RX-UE’s DRX active time should be newly introduced at RAN1 or not. We are fine with either handled at RAN1 or at RAN2, but at least a corresponding capability is necessary at RAN. Whether RAN2 introduces a FG for this behavior rather than for the UE performing DRX is unclear, so an LS to request the new FG with the PHY behavior should be sent to RAN2. Proposal 9: - Send an LS to RAN2 to ask to cover the following PHY behavior in FG defined by RAN2 side. - When PHY layer is indicated with an active time of RX UE from MAC layer for candidate resource selection, PHY layer selects and reports candidate resources in which at least a subset of the candidate resources is within the indicated active time of the RX UE. |
| [8] | Intel Corporation | Components of FG 15-1 and FG 15-4 are consistent with components of the FG 32-4. Given that FG 15-1 is a pre-requisite of FG 15-4, only FG 15-4 can be considered and added as a prerequisite for FG 32-4. As an optimization it can be considered to add only part of the components from FG 15-4 (e.g., corresponding to subset of synchronization sources). Besides FG for synchronization, the FG 15-11 PSFCH Format 0 for sidelink HARQ transmission/reception can be added as FG or separate components. On the other hand, PSFCH support for HARQ can be indicated separately. - FG 15-4 (Synchronization sources for NR sidelink) is a pre-requisite of the FG 32-4 - Consider defining FG 15-11 (PSFCH Format 0) as a pre-requisite of the FG 32-4 Components and pre-requisites of the FG 32-4a are still under discussion. RAN1 concluded that UE without NR SL reception is supported in Rel-17. Sidelink transmission with random resource selection can be performed by UEs that do not support sidelink reception. UEs w/o sidelink reception capability still require support of sidelink synchronization w/o SSB reception. Therefore FG 15-4 cannot be fully reused as a pre-requisite for FG 32-4a. Therefore, the following alternatives can be considered - Alt.1: Define new FG for sidelink synchronization w/o SSB reception support - Alt.2: Add components of the FG 15-4 except SSB reception support Support of the FG 32-4a does not necessary imply UE without sidelink reception capabilities. For UEs with sidelink reception capabilities, the FG 15-4 can be reused. Considering that FG 32-4a can be applied for UEs with and w/o sidelink reception capabilities, we have following proposal: - Define new R17 FG for sidelink synchronization w/o SSB reception support: - UE supports GNSS as the sidelink synchronization reference - UE supports gNB as the sidelink synchronization reference - UE can transmit S-SSB in NR sidelink when it uses GNSS or gNB as the sidelink synchronization reference - The following FGs are supported as a pre-requisite for FG 32-4a: - FG 15-4 (Sync sources for sidelink) - R17 FG for sidelink synchronization w/o SSB reception component |
| [9] | Apple | It was concluded that a UE without NR sidelink reception is supported in Rel-17. To support NR sidelink transmission, a UE without NR sidelink reception should rely on the synchronization source of GNSS, gNB or eNB. This motivates the introduction of a new feature of “synchronization sources for NR sidelink transmission”. The components of this new feature 32-4b were discussed in last RAN1 meeting. In our view, these components properly capture UE’s possible synchronization references and could be kept. We are open for any discussions of the restrictions of these components. This feature does not have pre-requisite feature group. Proposal 1: Introduce a new feature 32-4b “synchronization sources for NR sidelink transmission” without any pre-requisite feature group and with components: - UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. - UE can transmit NR sidelink based on the synchronization to an gNB. - UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. - UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. - UE can transmit NR sidelink based on the synchronization to an eNB. - UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. - UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. - UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a - FFS whether other components will be included The new feature 32-4b provides the synchronization sources for a Rel-17 UE’s sidelink transmission, based on either partial sensing or random resource selection. Hence, feature 32-4b can serve as the pre-requisite feature group for both feature 32-4 and feature 32-4a. Proposal 2: In both feature 32-4 and feature 32-4a, pre-requisite feature group is 32-4b. Both feature 32-4 and feature 32-4a contain a component mentioning “up to B sidelink processes are supported”. It is open how to define total sidelink processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a. In our view, the reported value B is the total number of sidelink processes for features 15-3, 32-4 and 32-4a. A straightforward way is to add a note to clarify that the reported value B is the total number of sidelink processes for features 15-3, 32-4 and 32-4a if supported. Proposal 3: In feature 32-4 and feature 32-4a, add a note that the reported value B is the total number of SL processes for features 15-3, 32-4 and 32-4a if supported. In features 32-4, 32-4a and 32-4b, a UE may report these capabilities to gNB so that gNB know how many UEs using a resource pool performs partial sensing, performs random resource selection, or has synchronization sources for sidelink transmission. This facilitates gNB’s configuration of resource pools, e.g., whether a resource pool supports mixed resource allocation schemes. On the other hand, we do not think UE’s capability of partial sensing, random resource selection or having synchronization sources needs to be exchanged between UEs. Proposal 4: In features 32-4, 32-4a, 32-4b, - need for the gNB to know if the feature is supported. - no need for other UEs to know if the feature is supported. |
| [10] | CMCC | For FG 32-4, when UE is configured to perform partial sensing, the target is to select one or more available resources for UE’s SL transmission, thus, FG 15-3 should certainly be included as a pre-requisite for this feature group. In addition, for a UE performs partial sensing operation, it needs to have the PSCCH reception capability to achieve the operation of sensing, hence, FG 15-1 should also be a pre-requisite of FG 32-4. Moreover, in last meeting, the following two working assumption/agreement has been reached in RAN1 [1][2]. Working assumption For UE performs partial sensing or random resource selection, Rel-16 SL CR evaluation is directly reused. Agreement When UE is configured to perform partial sensing by a UE higher layer (including when SL DRX is configured), SL RSSI is measured in slots where the UE performs partial sensing and PSCCH/PSSCH reception over the SL CBR measurement window defined in Rel-16. The calculation of SL CBR is limited within the slots for which the SL RSSI is measured. - If the number of SL RSSI measurement slots is below a (pre-)configured threshold, a (pre-)configured SL CBR value is used.. Congestion control is a basic feature for Rel-16 sidelink, and it has been agreed that congestion control should also be supported by a UE which is configured to perform partial sensing, meanwhile, according to the aforementioned working assumption/agreement, the design of congestion control mechanism for a UE which performs partial sensing has almost been finished. Thus, now FG 15-5 should be added as a pre-requisite of FG 32-4 as well. Proposal 1: At least FG 15-1, 15-3 and 15-5 should be included as the pre-requisites of FG32-4. For FG 32-4a, when UE is configured to perform random selection, the target is also to select one or more available resources for UE’s SL transmission, then, FG 15-3 should be included as a pre-requisite for this feature group as well. Regarding congestion control for a UE which is configured to perform random selection, following working assumption/agreement has been achieved during the previous meetings [1]. Working assumption For UE performs partial sensing or random resource selection, Rel-16 SL CR evaluation is directly reused. Agreement When UE performs random resource selection, LTE principle is reused: - The UE is not required to measure CBR. - When no SL CBR measurement result is available, a (pre-)configured SL CBR value is used.. Even though there is no need for such kind of UE to measure CBR, as mentioned in the agreement above, a (pre-)configured SL CBR value is used for proceeding the congestion control procedure, and UE should also perform the transmission under the constrain of the corresponding CBR/CR value. From this point of view, FG 15-5 should also be added as a pre-requisite of FG 32-4a. Proposal 2: At least FG 15-3 and 15-5 should be included as the pre-requisites of FG32-4a. There were some discussions on whether re-evaluation and pre-emption operation should be a separate FG or should be included as components in FG 32-4/ FG 32-4a, since in Rel-16 there are no such FGs on re-evaluation and pre-emption operation, there is no need to include separate FGs for these procedures, the motivation is to inherit the Rel-16 rationale. Proposal 3: Re-evaluation and pre-emption operation should not be introduced as new and separate FGs. However, from our point of view, re-evaluation/pre-emption should be added as components for FG 32-4. The reason is, for a UE which is configured to perform partial sensing, re-evaluation/pre-emption may need additional sensing slots compared to initial resource selection, i.e., UE may need to continue to do SCI monitoring in the slots after initial resource selection, we think this operation is different from full sensing case in which UE will always perform sensing. For a UE which is configured to random selection, it has not been achieved in RAN1 that whether a UE can do re-evaluation/pre-emption after performing random selection, if the answer is “yes”, re-evaluation/pre-emption should also be added as components for FG 32-4a. Proposal 4: Re-evaluation and pre-emption operation should be included as a component of FG32-4. Proposal 5: If RAN1 agrees that UE can do re-evaluation/pre-emption after performing random selection, re-evaluation and pre-emption operation should be included as a component of FG32-4a. |
| [11] | Xiaomi | In RAN1#107bis-e, it has been concluded that UE without sidelink reception capability is supported in Rel-17 [1]. However, even though UE does not have sidelink capability, it needs to be able to achieve sidelink synchronization from GNSS/gNB for potential sidelink transmissions. Therefore, it was proposed to define a new FG on synchronization capability, to make sure that a Rel-17 UE with sidelink Tx capability but without any sidelink RX capability can still achieve synchronization for sidelink Tx. On the other hand, as only Rel-17 UE with Tx capability of random resource selection can has no sidelink reception capability, the same synchronization capabilities can also be defined as pre-requisite or components of FG 32-4a. From our point of view, all the options can work well. Defining the synchronization capabilities as components of FG 32-4a is slightly preferred as less new FGs would be introduced in Rel-17. Proposal 1: Defining the synchronization capability as the components of FG 32-4a instead of defining new FG for synchronization capability |
| [12] | Samsung | For the first FFS on FG 32-4a above, our preference is to add the synchronization capability via another FG. This is because copying all necessary components from FG 15-4 into FG 32-4a will add unnecessary redundancy when such components are needed by other FGs. In addition, requiring FG 15-4 as pre-requisite will necessitate that the UE supports SyncRef UE as a synchronization source which might not be the case for UEs that do not support SL reception according to the conclusion below from RAN1 #107bis-e [1]: Conclusion - UE without NR SL reception is supported in Rel-17 For the 2nd FFS, if a UE reports more than one FG of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes for 15-3, 32-4 and 32-4a. This is essential to reduce the processing burden on the UE. Note that the above proposals also apply to the FFSs of FG 32-4 that was agreed in RAN1 #107bis-e. Proposal 1: The followings are proposed for FGs 32-4 and 32-4a: - The capability for synchronization is added via another FG. - If UE reports more than one FG of 15-3, 32-4 and 32-4a, the value B (i.e. either 8 or 16) is the total number of SL processes for 15-3, 32-4, and 32-4a. |
| [13] | MediaTek Inc. | In the previous RAN1-106bis-e meeting, there were discussions whether multiple sensing schemes (i.e., any combination of full sensing, partial sensing, and/or random resource selection) should be bundled as a separate UE capability. Hence an FFS point was noted on such TX capability. Each of these sensing schemes are intended to provide UE with a level of power saving gain at the expense of decreased reliability Each sensing scheme can essentially be applied in a different use case. If UE intends to support a different use case tailored for a different power saving or reliability target, the UE can indicate capability for more than one of these sensing schemes. However, the benefit of supporting a single capability that indicates a set of different sensing schemes is not clear. Each sensing scheme in Rel-17 is designed essentially to be self-sufficient without any additional pre-requisite of supporting another sensing scheme. With these views, we propose the following. Proposal 4: Do not support any TX capabilities as bundled with more than one sensing schemes (e.g., {full sensing, partial sensing, random selection}, {partial sensing, random selection}) as FGs for Rel-17 SL. |
| [14] | Qualcomm Incorporated | In general, many of the new features are impacted by the currently supported featureset. For example, the ability to process the additional information required for inter-UE coordination could be impacted by support of operations in other bands that would increase the baseband processing effort. Similarly transmitting or receiving such information would be impacted by other reception and transmission. Another example is sensing and partial sensing, which both require baseband processing taking away processing resources from other operations. Therefore, we propose that the FGs be defined per featureset. Proposal 1: UE features for sidelink enhancements are defined per featureset. In Release-16, all sidelink UEs were required to support synchronization to GNSS. However, the same should not apply to Release-17 since it expands the use cases and synchronization to GNSS might not be required, or even feasible, for some of them such as indoor commercial uses. Reception of SL-SSB was also required in Release-16 of all UEs; however, RAN1 already agreed to support Release-17 SL UEs without any SL reception capability and those UEs would not be able to receive SL-SSB. Based on these two points, we propose the following for synchronization. Proposal 2: Support of GNSS as a synchronization reference is only required in a band indicated with only the PC5 interface. Proposal 3: Support of gNB as a synchronization reference is only required in a band not indicated with only the PC5 interface. Proposal 4: Support of SyncRef UEs as a synchronization reference is required of UEs that support SL reception. The UE reports a maximum number of sideslink processes as part of Feature Groups 15-3, 32-4, and 32-4a. This number should be a maximum, and not the sum, across all features. Proposal 5: If the UE reports support for more than one of FG 15-3, 32-4, and 32-4a, it supports up to the reported B value, not the total number of processes across those FGs. |
| [15] | LG Electronics | One of remaining issues needed to be resolved is how to define the number of supported SL processes when a UE reports more than one FGs of 15-3 (i.e., full sensing), 32-4 (i.e., partial sensing) and 32-4a (i.e., random selection). Our understanding of Rel-15 LTE V2X is that the number of resource (re)selection schemes supported by the UE does not affect the total number of its supported SL processes. We think that the same principle can be kept for Rel-17 NR SL enhancement. Proposal 1: If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value of B in each FG is the total number of SL processes (i.e., UE is not required to support the sum of each reported value B). In the last meeting, there was a discussion on what kind of random selection should be assumed to be applied in the exceptional pool. Since the RAN1 has only discussed an enhancement of random selection in the normal pool so far, we think that it would be reasonable to assume that the random selection defined in Rel-16 NR V2X is supported in the exceptional pool. Proposal 2: Add a note in FG 32-4 and FG 32-4a: “Random selection according to Rel-16 is supported in the exceptional pool”. |
| [16] | Ericsson | For the remaining unstable fields of FG 32-4, i.e., pre-requisite FGs, whether the gNB needs to know about the feature, signalling to other UEs about the feature and the granularity of the feature, we propose the following: - Include as required pre-requisite the FG 32-2 if agreed - Need for the gNB to know if the feature is supported: Yes - Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No - Granularity of the feature: Per band For the FG 32-4, the remaining unstable fields are completed as follows: - Include as required pre-requisite the FG 32-2 if agreed - Need for the gNB to know if the feature is supported: Yes - Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No - Granularity of the feature: Per band Moreover, in our view, for the note related to random resource selection, we think it is fine to keep the note as it is, without highlighting the wording “according to Rel-16”. Given the lack of agreements stating anything else, Rel-16 random resource selection is applied in the exceptional pool. Remove the highlighted part for the wording “according to Rel-16” in the note related to random resource selection. For the remaining unstable fields of 32-4a, i.e., pre-requisite FGs, whether the gNB needs to know about the feature, signalling to other UEs about the feature and the granularity of the feature, we propose the following: - Include as required pre-requisite the FG 32-2 if agreed - Need for the gNB to know if the feature is supported: Yes - Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No - Granularity of the feature: Per band For the FG 32-4a, the remaining unstable fields are completed as follows: - Include as required pre-requisite the FG 32-2 if agreed - Need for the gNB to know if the feature is supported: Yes - Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: No - Granularity of the feature: Per band Regarding the note for random resource selection, in our view, the note regarding the random resource selection should be kept since a Rel-17 UE should be able to perform random resource selection in an exceptional pool as it was already supported in Rel-16. Keep the note regarding the random resource selection in the exception resource pool. |
| [17] | ZTE, Sanechips | For 32-4 transmitting NR sidelink mode 2 with partial sensing, given the necessary components for mode 2 Tx have already been captured, the prerequisites should include the basic components related to the functionality of NR sidelink such as 15-1 receiving NR sidelink and 15-4 Synchronization for NR sidelink. To reflect the above changes, the following proposal is made, 32-4; Transmitting NR sidelink mode 2 with partial sensing; 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 24) UE can perform periodic-based partial sensing and resource allocation operation. 35) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added; 15-1,15-4; [Yes]; [No]; UE does not support transmission according to the partial sensing and resource allocation; [Per band]; N.A.; N.A.; N.A.; Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. During previous RAN1 meeting, there was discussion as to whether to handle the non-SL sync sources as dedicated feature group or as components to the random selection feature group. It was clarified by the proponent that these components are for random selection purpose only, thus it's slightly preferred to capture them in the random selection FG for reduced capability indication. In case the SL sync sources are additionally supported, 15-14 can be used as prerequisite. Moreover, if there is a need to support UE not relying on any external sync sources, a note can be added Adopt the sync related components in the FG 32-4a for random selection. 32-4a; Transmitting NR sidelink mode 2 with random resource selection; 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. 2) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 3) UE can transmit PSSCH according to the normal 64QAM MCS table. 4) UE supports PT-RS transmission in FR2. 5) UE can perform periodic-based partial sensing and resource allocation operation. 6) UE can perform contiguous partial sensing and resource allocation operation. 7) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 910) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 1011) DL pathloss based open loop power control when mode 2 is configured by NR Uu 11) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 12) UE can transmit NR sidelink based on the synchronization to an gNB. 13) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 14) UE additionally supports gNB, GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true.; [TBD]; [Yes]; [No]; UE does not support transmission according to the random resource selection and resource allocation; [Per band]; N.A.; N.A.; N.A.; The support of components 11-14; Optional with capability signalling.. To resolve the FFS on the upper bound of the number for SL process supported if more than one of 15-3, 32-4 and 32-4a are supported, a note reflecting the number refers to the sum instead of for each individual capability can be captured in 32-4 and 32-4a . The following proposal is made to reflect that. Adopt the following note to reflect the number of B SL processes covers all the Tx capability if more than one of 15-3, 32-4, 32-4a are supported. 32-4; Transmitting NR sidelink mode 2 with partial sensing; 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform periodic-based partial sensing and resource allocation operation. 5) UE can perform contiguous partial sensing and resource allocation operation. 6) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 8) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 10) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 11) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added; [TBD]; [Yes]; [No]; UE does not support transmission according to the partial sensing and resource allocation; [Per band]; N.A.; N.A.; N.A.; Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: When more than one of 15-3, 32-4 and 32-4a are supported, the total number SL processes for all the supported capability are up to B Note: Component 6 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-6 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-6 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-6 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 10 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 11 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. 32-4a; Transmitting NR sidelink mode 2 with random resource selection; 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. Up to B sidelink processes are supported. 2) UE can transmit PSSCH according to the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can transmit using the subcarrier spacing and CP length defined for a given band in RAN4 5) Supports 14-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {12, 9} for slots w/wo PSFCH. If UE signals support of ECP, support 12-symbol SL slot with all DMRS patterns corresponding to {#PSSCH symbols} = {10,7} for slots w/wo PSFCH. 6) UE can transmit using 30 kHz and normal CP subcarrier spacing in FR1, 120 kHz subcarrier spacing with normal CP FR2 7) DL pathloss based open loop power control when mode 2 is configured by NR Uu FFS whether any other components should be added; [TBD]; [Yes]; [No]; UE does not support transmission according to the random resource selection and resource allocation; [Per band]; N.A.; N.A.; N.A.; Note: Random selection according to Rel-16 in the exceptional pool is supported. Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for B are {8,16} FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a Note: When more than one of 15-3, 32-4 and 32-4a are supported, the total number SL processes for all the supported capability are up to B Note: Component 4 is not required to be signalled in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Component-4 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-4 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} Component-4 candidate value set for CP length: {NCP,NCP and ECP} (ECP only applies to SCS of 60 kHz) Note: Component 6 is only required in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 7 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. For the note on performing SL reception according to Rel-16 random selection in this FG, without the note, it's assumed the UE can support random selection according to Rel-17 configuration or triggering in all the RPs, thus the additional implication shall be UE can support random selection in the exceptional resource pool and thus perform SL communication with Rel-16 SL UE therein. It's suggested to keep the note. The note in 32-4a 'Note: Random selection according to Rel-16 in the exceptional pool is supported.' is kept. |

Discussion

[FL1] High priority proposal 4-1:

- For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, down select from following options

  - Option 1:

    - Add a new FG 32-4b for synchronization sources for NR sidelink transmission as follows

| 32. NR_SL_enh | 32-4b | Synchronization sources for NR sidelink transmission | 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit NR sidelink based on the synchronization to an eNB. 6) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 7) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 8) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | This FG can only be reported as a pre-requisite to FG 32-4a Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4/5/6/7 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - FG 32-4b is a prerequisite FG for FGs 32-4/4a

  - Option 2: Add following components for synchronization sources for NR sidelink transmission in FGs 32-4/4a

    - UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false.

    - UE can transmit NR sidelink based on the synchronization to an gNB

    - UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. if the UE supports Components 1 and 2

    - UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2.

    - [UE can transmit NR sidelink based on the synchronization to an eNB.]

    - [UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb.]

    - [UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true.]

    - UE can transmit S-SSB in NR sidelink

  - Option 3: Add FG 15-4 as a prerequisite FG for FGs 32-4/4a

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - FG 32-4 - Option 1: OPPO, DCM, Apple, [CATT] - Option 2: - Option 3: HW, Intel, ZTE, [CATT] - FG 32-4a - Option 1: OPPO, DCM, Apple, [CATT] - Option 2: HW, Xiaomi, ZTE - Option 3: Intel, [CATT] |
| NTT DOCOMO | We are fine with either Option 1 or Option 2. Our feeling was that Option 1 is the easiest solution, but it seems that Option 2 can achieve the same thing. Regarding Option 3, at least this option would not be reasonable for FG 32-4a since some UEs supporting 32-4a do not have S-SSB RX capability, which is included in FG 15-4. |
| vivo | We prefer option 1 (i.e., a new FG). However, for this new FG, UE should be able to indicate {support, not support} to each of the components, instead of being required to support all the listed components. For example, a UE can indicate supporting gNB as Synchronization reference, but not eNB, or supporting GNSS, but not gNB or eNB, etc. This is essential for NR only commercial UE supporting, or for UE supporting mode-2 only, etc. |
| ZTE,Sanechips | For 32-4, option 3 shall work. Given there is no other FG other than 32-4a requring the new sync components, option 2 shall be ok for 32-4a. |
| Qualcomm | Synchronization to eNB was a separately optional FG 15-15 in Rel-16 and shouldn’t be included here. The UE can declare support for FG 15-15 if it supports eNB as a synchronization reference. Synchronization to a SyncRef UE should be required if the UE supports SL reception. The components in Option 2 should have notes similar to those of the corresponding components in Option 1. With the above modifications applied, our first preference is Option 2. We’d be ok with Option 2 if the “This FG can only be reported as a pre-requisite to FG 32-4a” is also removed. The main goal of introducing the synchronization components or FG here is to avoid some of the components in FG 15-4, hence we don’t think that Option 3 is viable. |
| Apple | We are fine with Option 1 or Option 2, slightly prefer Option 1 for its clean design. Regarding Option 3, the prerequisite of FG 15-4 is at least one of FG 15-1, 15-2, 15-3. However, this is not applicable at least to a random resource selection UE, since a random resource selection UE does not have to support SL reception or SL transmission based on mode 1 or full sensing. |
| OPPO | We prefer option 1. we have concern on option 3 because the pre-requisite of FG 15-4 is at least FG 15-1, FG 15-2 or FG 15-3. While for 15-4a, it is possible that the UE does not support any one of these 3 FGs if the UE does not have SL reception capability and work in mode 2. |
| Samsung | We prefer option 1 because copying all necessary components from FG 15-4 into FG 32-4a will add unnecessary redundancy when such components are needed by other FGs. In addition, requiring FG 15-4 as pre-requisite will necessitate that the UE supports SyncRef UE as a synchronization source which might not be the case for UEs that do not support SL reception. |
| xiaomi | We prefer option 2 for FG 32-4a, but we can accept option 1 for FG 32-4a. We support option 3 for FG 32-4, but do not think option 3 is feasible for FG 32-4a. |
| Huawei, HiSilicon | For the synchronization sources for NR sidelink transmission for FGs 32-4, we support option 3. As UE supporting partial sensing UE can receive NR sidelink, we can simply use FG 15-4 as prerequisite. For the synchronization sources for NR sidelink transmission for FGs 32-4a, we support option 2 with modification. Our intention is to realize UEs without any SL reception capabilities by only reporting FG 32-4a. In Rel-16, eNB type synchronization source for NR sidelink is optional, we suggest to add a note “Note: the following components are not required to be supported if eNB type synchronization source for NR sidelink is not supported” - [UE can transmit NR sidelink based on the synchronization to an eNB.] - [UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb.] [UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true.] |
| CATT, GOHIGH | We prefer the direction of option 1, but we have following comments on option 1: - eNB-based synchronization (yellow highlight part) should be separated, since some UE may only operate in only NR Uu mode. We think the same principle has been used in Rel-16 UE feature., which is slight different from that of 15-115(UE can transmit or receive NR sidelink based on the synchronization to an eNB.) - For component 3/4, we don’t see the need of “if the UE supports components 1 and 2”. Regarding to synchronization capability pre-requisites for 32-4/4a, from our understanding, at least one of 15-4 and 32-4b should be the pre-requisite of FG32-4. For UE is only capable of transmitting by 32-4a, the 32-4b is the pre-requisite of FG32-4a. |
| Ericsson | We are supportive of Option 1. |
| Spreadtrum | We are fine with option 1 or option 2. |
| Futurewei | We prefer option 1. For FG32-4 partial sensing, we are also ok to add FG 15-4 instead of FG 32-4b as one of its prerequisite. |
| Moderator | Summary of companies view - FG 32-4 - Option 1: OPPO, DCM, Apple, CATT (w/o eNB part), vivo (support per component), QC(?), SS, E///, SPRD, FW - Option 2: QC (w/o eNB part, w/ note), SPRD - Option 3: HW, Intel, ZTE, [CATT], Xiaomi, FW - As UE supporting partial sensing UE can receive NR sidelink - FG 32-4a - Option 1: OPPO, DCM, Apple, CATT (w/o eNB part), E///, SPRD, FW - Option 2: HW (add note), Xiaomi, ZTE, xiaomi, SPRD - Not support: SS - copying all necessary components from FG 15-4 into FG 32-4a will add unnecessary redundancy when such components are needed by other FGs - Option 3: Intel, [CATT], - Not support: DCM, QC, Apple, OPPO, SS, Xiaomi - 32-4a do not have S-SSB RX capability, which is included in FG 15-4 Given majority companies support Option 1 for FG 32-4/4a, following proposal is made [GTW1] High priority proposal 4-1: - For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, - Add a new FG 32-4b for synchronization sources for NR sidelink transmission as follows 32. NR_SL_enh; 32-4b; Synchronization sources for NR sidelink transmission; 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit NR sidelink based on the synchronization to an eNB. 6) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 7) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 8) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4/5/6/7 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 5/6/7 are not required to be supported if eNB type synchronization source for NR sidelink is not supported; Optional with capability signalling.. |
| FL2 | Following was agreed in the GTW session on Feb 22. Agreement - For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, - Add a new FG 32-4b for synchronization sources for NR sidelink transmission as follows - eNB parts are included as another FG, FFS detail 32. NR_SL_enh; 32-4b; Synchronization sources for NR sidelink transmission; 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a 6) UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1.; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. 1) Regarding the FFS for the eNB parts, following proposal is made as starting point for further discussion, which is based on FG 15-15 [FL2] High priority proposal 4-1: - For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, add another FG 32-4c for eNB type synchronization sources for NR sidelink transmission as follows 32. NR_SL_enh; 32-4c; eNB type synchronization sources for NR sidelink transmission; 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true.; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. 2) Regarding the component 6 in FG 32-4b, based on comment in the GTW session, confirmation of component 6 may need to wait until FG 32-2 makes some progress. It is moderator’s understanding that no RAN2 signaling impact is expected on component 6 regardless of the outcome of FG 32-2, but if you see any impact, please provide the view. 3) Regarding the type for FG 32-4b, companies are invited to provide view whether it can be per band as FG 15-4. [FL2] Medium priority proposal 4-1a: - Type of FG 32-4b is per band |
| NTT DOCOMO | For proposal 4-1, we are fine with the new FG for eNB perspective, which would be aligned with relationship between FG 15-4 and FG 15-15. For 2), we have the same understanding. For proposal 4-1a, we support the proposal. |
| Apple | For 1), we are fine with the new FG for eNB. We are not sure if components (5) and (6) in FG 32-4b apply to this new FG as well. For 2), we share the same view as FL. For 3), we agree it is per band as in FG 15-4. |
| CATT, GOHIGH | We support both proposal 4-1 and 4-1a. |
| OPPO | 1）Support 2）same view 3) support |
| ZTE,Sanechips | 1) Prefer to put the components in 32-4b, the reason why eNB sync ref is split is to make 15-4 basic - this issue no longer exists in Rel-17. 2) Can be directly confirmed with the new wording from Rel-16 15-4, also ok this way for progress. 3) Support |
| vivo | - Support - Same view - Support |
| Qualcomm | - We agree with the proposal - We share the same view as the moderator. - We are ok with per band signaling for this case. |
| Futurewei | 1) Ok with new FG for adding eNB type sync. 2) same view. 3) support. |
| Xiaomi | 1) support; 2) same view; 3) support |
| Huawei, HiSilicon | - We are fine to define the new FG for eNB type synchronization sources. - We share the same understanding - We support the proposal with per band signaling |
| Nokia, NSB | Support |
| Ericsson | - We are in general supportive of the FG, but we do not understand why we have the following sentence in components 2) and 3) “, UE additionally supports eNB ” - We have the same view - Support per band |
| Moderator | For 1), most companies support High priority proposal 4-1. One company suggest to discuss whether to add components 5/6 in FG 32-b into this FG. The proposal is updated for further discussion [GTW2] High priority proposal 4-1: - For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, add another FG 32-4c for eNB type synchronization sources for NR sidelink transmission as follows 32. NR_SL_enh; 32-4c; eNB type synchronization sources for NR sidelink transmission; 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 4) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a 5) UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1.; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. For 2), all companies share the same view. This highlighted part can be discussed after FG 32-2 makes some progress For 3) all companies support the proposal. [GTW2] Medium priority proposal 4-1a: - Type of FG 32-4b is per band |
| FL3 | proposal 4-1 was discussed in the GTW session on Feb 24 but no consensus was achieved. Based on the discussion, let’s further discuss for down selection among 3 options directly over the reflector. High priority proposal 4-1: - For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, down select from one of the following options - Option 1: add another FG 32-4c for eNB type synchronization sources for NR sidelink transmission with FG 32-4b as prerequisite 32. NR_SL_enh; 32-4c; eNB type synchronization sources for NR sidelink transmission; 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true.; 32-4b; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. - Option 2: add another FG 32-4c for eNB type synchronization sources for NR sidelink transmission without FG 32-4b as prerequisite 32. NR_SL_enh; 32-4c; eNB type synchronization sources for NR sidelink transmission; 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 4) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a 5) UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1.; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. - Option 3: add following capabilities for eNB type synchronization sources for NR sidelink transmission into FG 32-4b - UE can transmit NR sidelink based on the synchronization to an eNB. - UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. - UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. Following proposal could not be discussed in the GTW session but it seems stable. No further input is necessary unless you have concern on the following proposal. [FL3] Medium priority proposal 4-1a: - Type of FG 32-4b is per band |
| Apple | For proposal 4-1a, we support FG 32-4b is per band, considering that FG 15-4 and 15-15 are per band. |
| Moderator | proposal 4-1 was discussed over the RAN1 reflector. Seems Option 1 is acceptable to all. We can quickly agree on this proposal via email endorsement Summary of companies view - Option 1: ZTE (2), SS(2), DCM, QC(2), vivo, CATT, HW(1), Apple(2), FW - Option 2: DCM, Apple(1), QC(1), vivo - Option 3: ZTE (1), SS(1), HW(2) “If UE supports component 1 in FG 32-4b,” is added at the beginning of components 2/3 based on the comment from QC. [email1] High priority proposal 4-1: - For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, add another FG 32-4c for eNB type synchronization sources for NR sidelink transmission with FG 32-4b as prerequisite 32. NR_SL_enh; 32-4c; eNB type synchronization sources for NR sidelink transmission; 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true.; 32-4b; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. Following proposal is stable for more than 24 hours. We can quickly agree on this proposal via email endorsement [email1] Medium priority proposal 4-1a: - Type of FG 32-4b is per band |
| Moderator | Followings were agreed via email endorsement on Mar 1. Agreement - For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, add another FG 32-4c for eNB type synchronization sources for NR sidelink transmission with FG 32-4b as prerequisite 32. NR_SL_enh; 32-4c; eNB type synchronization sources for NR sidelink transmission; 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true.; 32-4b; Yes; No; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling.. Agreement - Type of FG 32-4b is per band Let’s further discuss the reporting type of FG 32-4c directly over the reflector. |

[FL1] High priority question 4-2:

- Companies are encouraged to provide views on whether to send an LS to RAN2 to ask to cover PHY behavior for RX-UE’s DRX in FG defined by RAN2 side

  - Support: DCM

| Company | Comment |
| --- | --- |
| NTT DOCOMO | OK. The agreed PHY behavior is one performed at any UE including UE not configured with SL-DRX. Whether RAN2 considers this or not would be unclear. |
| vivo | We think an LS is not needed – RAN2 can handle this. Alternatively, we can have a RAN1 conclusion, then RAN2 can start the work reference the conclusion, without waiting for RAN1’s LS. |
| ZTE,Sanechips | Similar view as vivo. |
| Qualcomm | We do not think an LS is needed. |
| Apple | We support the proposal. At least the resource selection feature in considering RX UE’s DRX is a RAN1 feature, and RAN2 may need to check if a separate FG needs to be defined in RAN1. |
| OPPO | OK. |
| Samsung | Similar view as vivo. |
| Huawei, HiSilicon | No, RAN2 are expert to handle it without input from RAN1 |
| Panasonic | Support |
| CATT, GOHIGH | Agree with vivo’s comment. |
| Ericsson | There is no need to send an LS to RAN2. They will define the UE features based on the RAN1 list. |
| Spreadtrum | We think there is no need to send LS to RAN 2. |
| Futurewei | We do not think this is necessary. |
| Moderator | Summary of companies view - Support: DCM, Apple, OPPO, Pana - Not support: vivo, ZTE, QC, SS, HW, CATT, E///, SPRD, FW [GTW1] Given more companies do not think this LS is necessary, this issue will be discussed in the GTW session whether we can conclude on it. |
| FL2 | This issue was discussed in the GTW session on Feb 22. As discussed, we will wait for the progress in AI8.11.1.1 |

[FL1] Medium priority proposal 4-3:

- For the FFS how to define total SL processes if UE reports more than one FGs of 15-3, 32-4 and 32-4a, down select from the following options

  - Option 1: The reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B

  - Option 2: The total number of SL processes supported by the UE is the largest number of value B among the reported FGs of 32-4, 32-4a and 15-3

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Option 1: HW, FW, OPPO, CATT, DCM, Apple, SS, QC, LGE, ZTE - Option 2: vivo - vivo: One possible solution is to report the same number of B for each of FG 32-4, 32-4a and 15-3. However, it precludes the possibility that UE supports a smaller number of full/partial sensing sidelink processes, which is desirable for the UE because a larger number of sensing processes requires larger buffer size and higher power consumption. |
| NTT DOCOMO | Support Option 1. Reporting different values would not be valid case. We do not think that different number of processes are applied for different RA mechanisms. |
| vivo | We support option 2 as it is much flexible than option 1. One motivation is to support low-end UE that supports a smaller number of full/partial sensing sidelink processes and a larger number of random selection processes, as a larger number of sensing processes requires larger buffer size and higher power consumption. |
| ZTE,Sanechips | Option 1. Option 2 may cause confusiong when the number of values supported are the same and the sum are fewer than the largest defined. |
| Qualcomm | We prefer Option 1. |
| Apple | Support Option 1. Option 2 does not provide sufficient information on each FG of 32-4, 32-4a and 15-3. gNB does not know how many SL processes supported in one FG if it knows how many SL processes are supported for the other two FGs. |
| OPPO | Support option 1. No necessary to optimize/complex this feature |
| Samsung | Support Option 1. |
| Xiaomi | We prefer option 1. |
| Huawei, HiSilicon | Option 1. Reporting the same value B of these FGs is reasonable for UE to process different RA mechanisms and the total SL processes should not exceed the maximum capability of the UE. |
| Panasonic | We support option 1. |
| CATT, GOHIGH | Option 1 |
| Ericsson | Option 1. |
| Spreadtrum | Option 1. |
| Futurewei | We prefer option 1. |
| FL2 | Summary of companies view - Option 1: HW, FW, OPPO, CATT, DCM, Apple, SS, QC, LGE, ZTE, Xiaomi, Pana, E///, SPRD - Option 2: vivo - to support low-end UE that supports a smaller number of full/partial sensing sidelink processes and a larger number of random selection processes, as a larger number of sensing processes requires larger buffer size and higher power consumption Given most companies prefer Option 1, following proposal is set [FL2] Medium priority proposal 4-3: - If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes. UE is not required to support the sum of each reported value B |
| NTT DOCOMO | Support. |
| Apple | Support. |
| CATT, GOHIGH | Support |
| OPPO | Support |
| ZTE | The rational of the second sentence may be confusing in a way that still there will be multiple B reported. Can we just delete that? |
| vivo | We can accept this proposal. |
| Qualcomm | Support |
| Futurewei | Support |
| Xiaomi | Support |
| Huawei, HiSilicon | We support this proposal. But we want to confirm it is common understanding that if UE reports more than one of FGs 15-3, 32-4 and 32-4a, the reported values of B in 15-3, 32-4 and 32-4a is the same. |
| Ericsson | Support |
| Moderator | Most companies are fine with the proposal in general. The proposal is updated to clarify that the reported values of B in 15-3, 32-4 and 32-4a is the same [GTW2] Medium priority proposal 4-3: - If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs. UE is not required to support the sum of each reported value B |
| FL3 | This proposal could not be discussed in the GTW session on Feb 24. No further input is necessary unless you have concern on this proposal [FL3] Medium priority proposal 4-3: - If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs. UE is not required to support the sum of each reported value B |
| Apple | Support. |
| Moderator | Following proposal is stable for more than 24 hours. We can quickly agree on this proposal via email endorsement [email1] Medium priority proposal 4-3: - If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs. UE is not required to support the sum of each reported value B |
| Moderator | Followings was agreed via email endorsement on Mar 1. Agreement - If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs. |

[FL1] Medium priority question 4-4:

- Companies are encouraged to provide views on whether the type of FGs 32-4 and 32-4a should be per band or per FS

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Per band: E/// - Per FS: QC |
| NTT DOCOMO | Slightly prefer per band. |
| vivo | We think per band would be enough, but we are open to either one. |
| ZTE,Sanechips | Per band |
| Nokia, NSB | Per band |
| Qualcomm | Per FS. Partial sensing requires baseband processing and the UE might not be able to support partial sensing in the reported band(s) depending on what other features are also supported. |
| Apple | In FG 32-4 and 32-4a, the total number of SL processes may be defined in a per band per band combination. Hence, we prefer Per FS. |
| OPPO | Support per band |
| Samsung | Prefer per band |
| Huawei, HiSilicon | Support per band, but also open to per FS. Rel-16 15-3 full sensing is defined as per-band, we don’t see the difference from resource allocation aspect, 32-4 and 32-4a are more related to baseband less related to RF, only per-band is needed. |
| Panasonic | Per band |
| Ericsson | Per band |
| Spreadtrum | Per band |
| Futurewei | We think per band would be sufficient. |
| FL2 | Summary of companies view - Per band: E///, DCM, vivo, ZTE, Nokia, OPPO, SS, HW, Pana, SPRD, FW - Rel-16 15-3 full sensing is defined as per-band - Per FS: QC, Apple, [vivo], [HW] - Partial sensing requires baseband processing and the UE might not be able to support partial sensing in the reported band(s) depending on what other features are also supported Given majority companies prefer to support as per band, following proposal is made [FL2] Medium priority proposal 4-4: - Type of FGs 32-4 and 32-4a are per band |
| NTT DOCOMO | Support |
| Apple | We can accept the proposal, considering FG15-3 is defined per band. |
| OPPO | Support |
| ZTE,Sanechips | Support |
| vivo | Support |
| Qualcomm | We’re ok to have 32-4a (random selection) be per band. However, in our view, 32-4 (partial sensing) has baseband impact which could be significant for power saving UEs and should be per FS. |
| Futurewei | Support. |
| Huawei, HiSilicon | Support |
| Nokia, NSB | Support |
| Ericsson | Support |
| Moderator | All companies are fine with per band for FG 32-4. Most companies also support per band for FG 32-4a. The same proposal is set for further discussion. [GTW2] Medium priority proposal 4-4: - Type of FGs 32-4 and 32-4a are per band |
| FL3 | This proposal could not be discussed in the GTW session on Feb 24. No further input is necessary unless you have concern on this proposal [FL3] Medium priority proposal 4-4: - Type of FGs 32-4 and 32-4a are per band |
| Apple | We are fine with the proposal, considering FG 15-3 is defined per band. |
| Moderator | Following proposal is stable for more than 24 hours. We can quickly agree on this proposal via email endorsement [email1] Medium priority proposal 4-4: - Type of FGs 32-4 and 32-4a are per band |
| Moderator | This proposal could not be agreed via email endorsement on Mar 1. Let’s further discuss directly over the reflector. |

[FL1] Medium priority question 4-5:

- Companies are encouraged to provide views on whether the column of “Need for the gNB to know if the feature is supported” for FGs 32-4 and 32-4a should be “Yes” or “No”

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Yes: HW, DCM, Apple, E/// - No: |
| NTT DOCOMO | Yes |
| vivo | Yes |
| ZTE,Sanechips | Yes |
| Nokia, NSB | Yes |
| Qualcomm | We prefer to not report. |
| Apple | Yes |
| OPPO | Yes |
| Samsung | Yes |
| Xiaomi | Yes |
| Huawei, HiSilicon | Yes |
| Panasonic | Yes |
| Ericsson | Yes |
| Spreadtrum | Yes |
| Futurewei | Yes. |
| FL2 | Summary of companies view - Yes: HW, DCM, Apple, E///, vivo, Nokia, OPPO, SS, Xiaomi, Pana, SPRD, FW - No: QC Given majority companies prefer “Yes”, following proposal is made [FL2] Medium priority proposal 4-5: - The column of “Need for the gNB to know if the feature is supported” for FGs 32-4 and 32-4a are “Yes” |
| NTT DOCOMO | Support |
| Apple | Support |
| OPPO | Support |
| ZTE, Sanechips | Support |
| vivo | Support |
| Qualcomm | We can accept the proposal for progress even though we don’t think this information would be utilized by the gNB. |
| Futurewei | Support |
| Xiaomi | Support |
| Huawei, HiSilicon | Support |
| Nokia, NSB | Support |
| Ericsson | Support |
| Moderator | QC showed their flexibility to accept the proposal. The same proposal is set. [GTW2] Medium priority proposal 4-5: - The column of “Need for the gNB to know if the feature is supported” for FGs 32-4 and 32-4a are “Yes” |
| FL3 | This proposal could not be discussed in the GTW session on Feb 24. No further input is necessary unless you have concern on this proposal [FL3] Medium priority proposal 4-5: - The column of “Need for the gNB to know if the feature is supported” for FGs 32-4 and 32-4a are “Yes” |
| Apple | We are fine with the proposal 4-5. (We assume we are talking about proposal 4-5, rather than 4-4.) [Moderator] Thank you very much for pointing out the typo. It is fixed now. |
| Moderator | Following proposal is stable for more than 24 hours. We can quickly agree on this proposal via email endorsement [email1] Medium priority proposal 4-5: - The column of “Need for the gNB to know if the feature is supported” for FGs 32-4 and 32-4a are “Yes” |
| Moderator | Followings was agreed via email endorsement on Mar 1. Agreement - The column of “Need for the gNB to know if the feature is supported” for FGs 32-4 and 32-4a are “Yes” |

[FL1] Medium priority question 4-6:

- Companies are encouraged to provide views on whether the column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-4 and 32-4a should be “Yes” or “No”

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Yes: - No: DCM, Apple, E/// |
| NTT DOCOMO | No |
| vivo | Yes |
| ZTE,Sanechips | Yes |
| Qualcomm | We prefer to report. |
| Apple | No, it is unclear to us how a UE could make use of this information when receiving it from another UE. |
| OPPO | No |
| Samsung | No |
| Xiaomi | No |
| Huawei, HiSilicon | No, we agree there is no use a UE could make use of receiving such information. |
| Panasonic | No |
| CATT, GOHIGH | No |
| Ericsson | No |
| Futurewei | No. This may not be necessary. |
| FL2 | Summary of companies view - Yes: vivo, ZTE, QC, - No: DCM, Apple, E///, OPPO, SS, Xiaomi, HW, Pana, CATT, FW - unclear how a UE could make use of this information when receiving it from another UE Given majority companies prefer “No”, following proposal is made. Companies still prefer “Yes” are encouraged to provide view how UE can use the information [FL2] Medium priority proposal 4-6: - The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-4 and 32-4a are “No” |
| NTT DOCOMO | Support |
| Apple | Support |
| CATT, GOHIGH | Support |
| OPPO | Support |
| ZTE,Sanechips | Considering in Rel-16, UE exchange of mode 1 or mode 2 Tx is not supported, we can accept this. But we think this information can still be useful in case of RP sharing between UEs with or without sensing operations. |
| vivo | OK |
| Qualcomm | We are ok to not report 32-4a but would still prefer to report 32-4 to other UEs. |
| Futurewei | Support |
| Xiaomi | Support |
| Huawei, HiSilicon | Support |
| Ericsson | Support |
| Moderator | ZTE, vivo, and QC showed their flexibility to accept the proposal. The same proposal is set [GTW2] Medium priority proposal 4-6: - The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-4 and 32-4a are “No” |
| FL3 | This proposal could not be discussed in the GTW session on Feb 24. No further input is necessary unless you have concern on this proposal [FL3] Medium priority proposal 4-6: - The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-4 and 32-4a are “No” |
| Apple | Support. |
| Moderator | Following proposal is stable for more than 24 hours. We can quickly agree on this proposal via email endorsement [email1] Medium priority proposal 4-6: - The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-4 and 32-4a are “No” |
| Moderator | Followings was agreed via email endorsement on Mar 1. Agreement - The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-4 and 32-4a are “No” |

Low priority question 4-7:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 32-4 and 32-4a which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 32-5a to 32-5b-2 for NR: Inter-UE coordination in NR sidelink mode 2

In [1], FGs 32-5a to 32-5b-2 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a | Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-1 | Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. | 32-5b-2, TBD | [Yes] | [Yes] | UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| 32. NR_SL_enh | 32-5b-2 | Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. | [TBD] | [Yes] | [Yes] | UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [1] | Huawei, HiSilicon | For inter-UE coordination, we suggest to split the capabilities based on transmission and reception of inter-UE coordination. First, a UE may only receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection, meanwhile it may not need to have the capabilities to transmit inter-UE coordination information for some power efficient UEs. Second, the pre-requisites for transmission and reception are different. For example, a UE need to perform full or partial sensing to provide inter-UE coordination information, but a UE may not need to have the capabilities of full sensing to support receiving coordination information. Besides, we don’t see further splitting on inter-UE coordination is needed, e.g. for scheme 1 prefer preferred resource set and non-preferred resource set, UE-A performs sensing to find inter-UE coordination information and send to UE-B, UE-B uses the coordination information for its resource exclusion. The procedure for UE-A and UE-B are the same for both prefer preferred resource set and non-preferred resource set. Proposal 5: Split FGs 32-5a into transmission and reception capabilities for their respective inter-UE coordination scheme. - 32-5a-1: Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2 - 32-5a-2: Receiving Inter-UE coordination scheme 1 in NR sidelink mode 2 No further splitting on inter-UE coordination is needed We think a single design should be applied for both preferred resource set and non-preferred resource set, and explicit request for inter-UE coordination information can be applied for both of them. From explicit request transmission and reception aspect, there’s no difference of preferred resource set and non-preferred resource set. Proposal 6: The component of explicit request for inter-UE coordination information is applicable to both preferred resource set and non-preferred resource set Since MAC CE is always used as the container of inter-UE coordination information and an explicit request between UE A and UE B, it is not needed to define MAC CE in FG 32-5a-1 and FG 32-5a-2. But since SCI 2-C is UE RX optional, we need to define the capability of receiving SCI 2-C. Since it is (pre-)configured at a resource pool level, if MAC CE and 2nd SCI are (pre-)configured to use as containers, the transmitter needs to know the capability of receiver to decide to use SCI 2-C or SCI 2-A/2-B. The capability signalling of receiving SCI 2-C needs to be exchanged between UE-A and UE-B. Proposal 7: Define FG on SCI-2C as the container of inter-UE coordination information transmission and an explicit request as following: Index; Feature group; Components; Prerequisite feature groups; Applicable to the capability signalling exchange between UEs (V2X WI only).. 32-5a-3; Receiving SCI 2-C conveying an explicit request; 1) UE can additionally receive SCI format 2-C which conveys an explicit request for inter-UE coordination information.; 32-5a-1; Yes. 32-5a-4; Receiving SCI 2-C conveying inter-UE coordination information; 1) UE can additionally receive SCI format 2-C which conveys inter-UE coordination information of preferred resource set/non-preferred resource set, and use the received information in its own resource (re-)selection in NR sidelink mode 2.; 32-5a-2; Yes. We think similar feature group of PSFCH format 0 used to convey the presence of HARQ-ACK and expected/potential resource conflict on reserved resource(s) in inter-UE coordination scheme 2 in NR sidelink mode 2 should be defined to treat the supported transmission and reception capability of PSFCH format 0. And we also noted that there would be cases where the transmission/reception between PSFCH format 0 for inter-UE coordination in NR sidelink and PSFCH format 0 for sidelink HARQ feedback may happen in the same PSFCH slots. Considering that, jointly reporting is sufficient, including HARQ-ACK and coordination indication. For the value (N, M) of reception/transmission PSFCH(s) resources in FG 32-5b-1/5b-2, one simple way is to reuse the candidate values set for M and N in Rel-16, since the same PSFCH format 0 is used. For a Rel-17 UE, if the UE reports more than one of FGs 15-11 and 32-5b-X, if the reported value M or N in different FGs is different, the meaning is unclear. One simple approach is to expect the UE to report the same value of FG 15-11 and 32-5b-X, and it depends on prioritization procedures to allocate different resources to inter-UE coordination and HARQ-ACK reporting. In this case, the candidate values for Rel-16 and Rel-17 can be the same, since the network can already infer which releases of UEs are present, if it wishes to use that information in PSFCH resource allocation. Proposal 8: Capability of PSFCH format 0 transmission/reception for scheme 2 is a component of the scheme 2 FGs: - PSFCH Tx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting in FG 32-5b-1 - The candidate value range for M is {4, 8, 16} - PSFCH Rx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting in FG 32-5b-2 - The candidate value range for N is {5, 15, 25, 32, 35, 45, 50, 64} - If UE reports more than one of FGs 15-11 and 32-5b-1/5b-2, the reported values of M and N in FG 15-11 and FG 32-5b-1/5b-2 is shall be the same. Proposal 9: The following FGs are defined for inter-UE coordination with pre-requisites and components as shown: Index; Feature group; Components; R17 prerequisite feature groups. 32-5a-1; Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set. 2) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set; 15-1, 15-3, 15-4, 15-5, 15-11, 15-23. 32-5a-2; Receiving Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set.; 15-1, 15-4, 15-5, 15-11, 15-23 And at least one of 15-3, 32-4. 32-5a-3; Receiving SCI 2-C conveying an explicit request; 1) UE can additionally receive SCI 2-C which conveys an explicit request transmission from UE-B to UE-A.; 32-5a-1. 32-5a-4; Receiving SCI 2-C conveying inter-UE coordination information; 1) UE can additionally receive SCI 2-C which conveys inter-UE coordination information of preferred resource set/non-preferred resource set, and use the received information in its own resource (re-)selection in NR sidelink mode 2.; 32-5a-2. 32-5b-1; Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2; 1) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. FFS whether/how to split FG 32-5b into multiple FGs 2) UE can transmit up to M PSFCH(s) resources in a slot The candidate value ranges for M is {4, 8, 16}; 32-5b-2 15-1, and either 15-4 or 32-2.. 32-5b-2; Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 2) UE can receive up to N PSFCH(s) resources in a slot. The candidate value ranges for N is {5, 15, 25, 32, 35, 45, 50, 64}; At least one of 15-3, 32-4 15-4. Based on RAN1 agreements that resource allocation schemes (full sensing, partial sensing, and random resource selection) are configured per resource pool, it is essential for gNB to be aware of the UE features on what resource allocation schemes it supports. For inter-UE coordination, it is also agreed that features for both scheme 1 and scheme 2 can be enabled or disabled or controlled by (pre-)configuration, and therefore gNB should be informed with UE capability. Proposal 11: UE features regarding resource allocation schemes and inter-UE coordination schemes need to be informed to gNB. In Rel-16, it was decided that a UE would need to support mode 1 in licensed spectrum where gNB is defined. This was to ensure network operator control could be exercised over sidelink when in licensed spectrum. The Rel-16 principle and agreement extend to Rel-17, where otherwise a UE might report only Rel-17 FGs (and associated pre-requisites), all of which apply to mode 2 only, and hence be always beyond network control in licensed spectrum. The simplest way to capture this in the Rel-17 FG list is, as with Rel-16, adding to the notes column of the rows for partial sensing, random selection, inter-UE scheme 1, and inter-UE scheme 2. Proposal 12: Add a note for Rel-17 resource allocation FGs 32-4, 32-4a, 32-5a-a, 32-5a-b, 32-5b-a, 32-5b-b: “For UE supports this FG, and NR sidelink in licensed spectrum where gNB is defined, UE must indicate FG 15-2 is supported.” |
| --- | --- | --- |
| [2] | FUTUREWEI | For FG 32-5a inter UE coordination scheme 1, we do not need to separate the Tx vs Rx for inter UE coordination features. We can see that it may be possible a Scheme 1 Rx UE only uses RX coordination information without performing sensing and does not try to help others by TX. However, a Scheme 1 Rx UE needs to detect the PSCCH/PSSCH to receive coordination information as MAC-CE is always used for coordination transmission (only coordination information in SCI is Rx optional). And even for the RX only UE, it also may need to send the coordination request or some info via RRC signaling to UE-A which is related to Tx functionality. Similarly for Scheme 1 Tx UE, i.e., UE-A, UE-A needs the reception capability of PSCCH for sensing to generate coordination information, i.e., again Tx and Rx mixed. Therefore, there is no need to split the Tx and Rx features of Scheme 1. For preferred vs non-preferred, many unified designs, e.g., container, content format, were agreed. We do not think it is necessary to split them. Similarly for request vs condition- based coordination, further splitting the FG is unnecessary. We have agreed RRC parameters. These combinations can be controlled or enabled/disabled by RRC parameters. Therefore, we do not support further splitting of FG 32-5a unless there is an agreement on some UE feature which is subject to UE capability. Proposal 10: Do not support further split of FG 32-5a unless there is an agreement on some UE feature which is subject to UE capability. For both inter-UE coordination features, it is not necessary that a UE supporting inter-UE coordination must support the partial sensing feature or vice-versa. For inter-UE coordination, the mechanism of partial sensing is not suitable for Scheme 1 UE to generate coordination information and Scheme 2 Tx UE to detect the potential/expected conflict. Therefore, full-sensing FG 15-3 should be the pre-requisite of FG 32-5a and FG 32-5b-1. For inter-UE coordination, synchronization from a syncRef UE is beneficial. Therefore, inter UE coordination should support Rel-16 FG 15-4 for synchronization. Supporting both Tx and Rx PSFCH features is necessary for Scheme 1 and Scheme 2 Tx UEs. Then FG 15-11 should also be supported for both FG 32-5a and FG 32-5b-1. Other Rel-16 mode 2 basic features such as FG 15-5 congestion control and 15-23 FG power control should also be supported for Scheme 1 UEs as they transmit and receive PSSCH for coordination information and data. For scheme 2 Tx UE, it is not necessary to support these two features. Since some Rel-16 FGs are prerequisites of another Rel-16 FG, e.g., the receive capability FG 15-1 is the prerequisite of FG 15-3, an issue arises whether such prerequisite structure should be inherited or not. If not, FG 15-1 should then be listed as the prerequisite for FG 32-5a and FG 32-5b-1. Although not inheriting Rel-16 prerequisite provides larger flexibility in defining the prerequisite of Rel-17 FGs, we do not see the need based on the current agreement. Inheriting Rel-16 prerequisite may also facilitate the implementation of Rel-17 SL if Rel-16 SL is implemented. Proposal 11: Support inheriting the prerequisite FGs of a Rel-16 FG as the requisite of a Rel-17 FG if the Rel-16 FG is listed as the prerequisite of the Rel-17 FG. Proposal 12: Add FGs 15-3, 15-4, 15-5, 15-11, 15-23 as the prerequisite of FGs 32-5a. Proposal 13: Add FGs 15-3, 15-4, and 15-11 as the prerequisite of FG 32-5b-1. Regarding FG 32-5b-2 Scheme 2 Rx feature, if FG 32-2 is defined, it is possible that Scheme 2 Rx UE may only be capable of S-SSB and PSFCH, but not capable of any type of sensing. Then FG 32-2 should be the requisite of FG 32-5b-2 instead of Rel-16 FG 15-3. Although FG 32-2 includes synchronization, it only includes the Rx related component. For inter UE coordination Scheme 2, it is also important that Scheme 2 Rx UE can be syncRef UE so that Scheme 2 Tx UE can synchronize with it. Therefore, full synchronization FG 15-4 should be the prerequisite of FG 32-5b-2. If FG 32-2 is not agreed, the Rel-16 FG 15-11 should be included as the prerequisite of FG 32-5b-2. Proposal 14: Add at least one of FG 32-2 (if specified) or FG 15-11 as the prerequisite of FG 32-5b-2. For inter-UE coordination scheme 2, PSFCH format 0 transmission and reception are supported for reporting and receiving the conflict report as coordination information, respectively. Since a non-destination of a TB from UE-B can be UE-A, UE-A needs to transmit more PSFCHs than a Rel-16 UE for HARQ-ACK purpose only. Also, since UE-B needs to receive a PSFCH for the conflict report without transmitting a PSSCH, UE-B needs to receive more PSFCHs than a Rel-16 UE for HARQ-ACK only. Therefore, the number of simultaneous PSFCH receptions and/or transmissions shall be increased. For simplicity on specification support, it would be preferred to report them separately for inter-UE coordination and HARQ-ACK. However, in RAN1#107bis-e, companies preferred to jointly report of the PSFCH transmission and reception. In this case, we propose to set a minimum number of PSFCHs for Scheme 2 conflict indication, i.e., the joint report in FG 32-5b-1 for PSFCH Tx and in FG 32-5b-3 for PSFCH Rx should be greater than corresponding PSFCH HARQ-ACK Tx in FG 15-11 and PSFCH HARQ-ACK Rx in FG 32-2 (if specified) or FG 15-11. The minimum is then dependent on the granularity of the number of simultaneous PSFCH transmissions and receptions. Proposal 15: Support additional PSFCH simultaneous transmission and reception for inter-UE coordination scheme 2 and report them separately from that for HARQ-ACK. Proposal 16: If joint report of PSFCH simultaneous transmission and reception for coordination Scheme 2 and for HARQ-ACK is supported, the number of simultaneous PSFCH transmission and reception supported in Scheme 2 FGs should be greater than the one reported in FGs for HARQ-ACK. - If UE reports more than one FGs of 15-11, and 32-5b-1, the reported value M in FG 32-5b-2, is the total number, which should be greater than the one in FG 15-11. - If UE reports more than one FGs of 15-11, 32-2 (if specified), and 32-5b-2, the reported value N for simultaneous PSCFH receptions in FG 32-5b-2, is the total number, which should be greater than the one in FG 15-11 or FG 32-2. Since 2nd stage SCI is supported as the container for inter-UE coordination in Scheme 1, and 2nd stage SCI is UE Rx optional, a new FG should be specified to support the reception of 2nd stage SCI for Scheme 1 inter-UE coordination information. FG 32-6 was introduced in RAN1#107bis-e for this capability, which is then supported. In addition, UE who support this FG should support inter-UE coordination Scheme 1. Therefore, FG 32-5a should be the prerequisite of this new FG. Proposal 17: Support a new FG, i.e., FG 32-6, for reception of Scheme 1 inter-UE coordination information over 2nd stage SCI. - FG 32-5a for inter-UE coordination Scheme 1 should be prerequisite of FG 32-6. Similar to the case for coordination information transmissions, the 2nd stage SCI is supported as the container for transmission of the explicit request, which is UE Rx optional. A Rel-17 FG should be specified to include such capability of reception of 2nd stage SCI for the explicit request. As shown in above agreement, a single format SCI 2-C is used for inter-UE coordination information and request. Therefore, the capabilities of reception of the coordination information and the explicit request should be supported in one FG. Proposal 18: Support for reception of an explicit request and coordination information in Scheme 1 inter-UE coordination over 2nd SCI should be included in the same FG, i.e., FG 32-6. If this working assumption is confirmed, option 4, the SIR based criteria is supported. However, the support of option 4 is subject to UE capability as UE-A needs to perform two RSRP measurements and find the difference between these two measurements, which is new behavior on UE SL sensing. A new FG is needed to specify this feature. Proposal 19: Support a new FG, i.e., FG 32-7, for determination of expected conflict in Scheme 2 based on RSRP difference. |
| [3] | vivo | Similarly, inter-UE coordination scheme 1 FG (i.e., 32-5a) should be further split into two FGs for Tx and Rx capabilities. A UE may only support receiving the inter-UE coordination information for resource selection, but is not capable to perform resource recommendations for the other UEs, especially for the UE not performing full/partial sensing. Furthermore, there are two separate sub-schemes defined for scheme 1, namely scheme 1 with preferred resource set, scheme 1 with non-preferred resource set. These sub-schemes are targeting different scenarios and device types. A power-limited or low-end SL UE can only support one sub-scheme (e.g., scheme 1 with preferred resource set), while a normal VUE can support both. Therefore, it is natural and reasonable to further split the FG 32-5a (i.e., Inter-UE coordination scheme 1) into two FGs for two sub-schemes respectively. Proposal 2: Inter-UE coordination scheme 1 FG (i.e., 32-5a) should be further split, based on Tx and RX capabilities, and based on different sub-schemes, i.e., scheme 1 with preferred resource set and scheme 1 with non-preferred resource set, respectively. Besides, in the previous meeting it has been agreed to introduce the Rx capability of SCI 2-C for inter-UE coordination scheme 1, where SCI 2-C is used for either explicit request or inter-UE coordination information transmissions. Thus, the following capability should be added. 32. NR_SL_enh; 32-6; Reception of SCI 2-C; 1) UE can receiver SCI format 2-C for inter-UE coordination Scheme 1.; No; Yes. Proposal 3: Introduce a new FG (e.g., 32-6) for indicating support of SCI format 2-C. Moreover, according to the following agreement in RAN1#107, the support of option 4 (i.e., conflict determination based on relative RSRP measurement) is subject to UE capability. Therefore, the following UE FG 32-7 should be supported. 32. NR_SL_enh; 32-7; Determination of expected conflict in Scheme 2 based on RSRP difference; 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations; No; No. Proposal 4: Introduce a new FG (e.g., 32-7) for indicating support of conflict determination based on RSRP difference of the two overlapping reservations. On the other hand, regarding the inter-UE coordination scheme 2, one remaining issue is how to report the Tx and Rx capabilities of PSFCH. Firstly, from implementation perspective, the hardware capability of PSFCH is same regardless of the usage. Secondly, it is desirable that the maximum number of simultaneous PSFCH receptions and/or transmissions is shared for both inter-UE coordination and HARQ-ACK reporting, so that in the case that inter-UE coordination feature is not enabled, the capability of PSFCH receptions is not wasted. Therefore, the PSFCH capability is jointly reported for both inter-UE coordination and HARQ-ACK reporting. Proposal 5: The max number of simultaneous PSFCH receptions and/or transmissions is jointly reported for inter-UE coordination (scheme 2) and HARQ-ACK reporting. |
| [4] | OPPO | Similar as FG 32-5b, we suggest to further split FG 32-5a per transmission/reception of inter-UE coordination information as below, as the prerequisites of them are different. There is no need to further split FG32-5a-1 per preferred resource set or non-preferred resource set, as based on the RAN1 agreement achieved in the last meeting, UE-A is expected to provided either preferred resource set or non-preferred resource set based on UE-B’s request or its implementation, there is no specific use case or scenario where only one of them is provided. For 32-5a-2, we are open to further split it per preferred or non-preferred resource set, as for a UE in power saving mode, it may only use preferred resource set. 32. NR_SL_enh; 32-5a-1; Determine and transmit preferred resource set/non-preferred resource set in NR sidelink mode 2; 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set].. 32. NR_SL_enh; 32-5a-2; Receive and use of preferred resource set/non-preferred resource set in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set].. FG 15-1(Receiving NR sidelink) and FG 15-3 (Transmitting NR sidelink mode 2) should be prerequisites of FG 32-5a-1 and 32-5-b-1, as only a UE having full sensing capability can receive the explicit request and determine inter-UE coordination information in Scheme 1 and determine/transmit resource conflict in Scheme 2. The prerequisites of FG 32-5a-2 should be 15-1(Receiving NR sidelink) and at least one of 15-3 (Transmitting NR sidelink mode 2) or 32-4(Transmitting NR sidelink mode 2 with partial sensing). The prerequisites of FG 32-5b-2 should be 15-1(Receiving NR sidelink) and at least one of 15-3 (Transmitting NR sidelink mode 2). Proposal 7: - FG 32-5a should be split into 2 FGs per transmission/reception of inter-UE coordination information. - Prerequisites of FG 32-5a-1 and 32-5-b-1 should be FG 15-1(Receiving NR sidelink) and FG 15-3 (Transmitting NR sidelink mode 2). - Prerequisites of 32-5a-2 should be 15-1(Receiving NR sidelink) and at least one of 15-3 (Transmitting NR sidelink mode 2) or 32-4(Transmitting NR sidelink mode 2 with partial sensing). - prerequisites of FG 32-5b-2 should be 15-1(Receiving NR sidelink) and at least one of 15-3 (Transmitting NR sidelink mode 2) In the last meeting, the necessity of FG32-6 was discussed, as it was agreed that 2nd SCI is used for the inter-UE coordination transmission, and the reception of 2nd SCI is RX optional, this FG should be introduced, and it should be applicable to the capability signaling exchange between UEs. 32. NR_SL_enh; 32-6; Reception of Scheme 1 inter-UE coordination information over 2nd SCI; 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission.; No; YES. Proposal 8: FG 32-6 (Reception of Scheme 1 inter-UE coordination information over 2nd SCI) should be introduced and it is applicable to the capability signaling exchange between UEs. |
| [5] | CATT, GOHIGH | For inter-UE coordination scheme 1, since the resource set type can be carried by inter-UE coordination information, it means the UE should support to perform resource selection based on both preferred and non-preferred resource set, therefore the preferred/non-preferred resource set should not be separated as different FGs. Considering that a power efficient UE may not be capable of transmitting inter-UE coordination information, but it is capable of performing resource selection based on the received inter-UE coordination information. it is preferred to separate the FG32-5a into two FGs, one is for transmitting capability of scheme 1 inter-UE coordination information, the other is for receiving capability of scheme 1 inter-UE coordination information. Proposal 4: FG32-5a should be separated into two FGs, one is for transmitting capability of scheme 1 inter-UE coordination information, the other is for receiving capability of scheme 1 inter-UE coordination information. Regarding whether the PSFCH Tx/Rx capability is jointly reported for both inter-UE coordination scheme 2 and SL HARQ-ACK, a joint manner is preferred since there is no fundamental difference between them. And when a UE report more than one FGs of 15-11 and 32-5b-1/32-5b-1, the reported number of PSFCH TX/RX capability in each FG should be the total number. Proposal 5: PSFCH Tx capability is jointly reported for inter-UE coordination and SL HARQ-ACK as a component of 35-5b-1. - Add a note: “If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number, UE is not required to support the sum of each reported value M.” Proposal 6: PSFCH Rx capability is jointly reported for inter-UE coordination and SL HARQ-ACK as a component of 35-5b-2. - Add a note: “If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number, UE is not required to support the sum of each reported value N.” Based on the working assumption, one FG should be introduced accordingly. Proposal 7: A new FG should be introduced to support RSRP difference based resource conflict determination. |
| [6] | Nokia, Nokia Shanghai Bell | - 32-5a: - A similar split can be considered as for 32-5b, i.e. based on transmission and reception of inter-UE coordination. - 32-5x/y: - Add new FGs to reflect the RAN1#107-bis-e agreement on support of inter-UE coordination based on 2nd SCI transmission/reception. Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list. |
| [7] | NTT DOCOMO, INC. | At the last meeting, it was confirmed that any combinations of preferred/non-preferred and request-based/condition-based are supported in Rel-17. Then possible split-directions are the following: Option 1: TX vs RX Option 2: Preferred vs Non-preferred Option 3: Request-based vs Condition-based Regarding Option 1, we do not see the necessity to split between TX side and RX side. Although FGs for scheme 2 were split, the motivation is to cover UE supporting PSFCH RX but incapable of sensing. IUC message is conveyed on PSCCH/PSSCH, thus UE supporting RX perspective of scheme 1 will support sensing. The UE should be capable of TX perspective of scheme 1 as well. For Option 2, we slightly prefer not to split by this direction. Corresponding UE-A’s behavior is specified in section 8.1.4A of TS 38.214 and UE-B’s behavior is described in section 8.1.4C of TS 38.214. We feel that they are not much different. To split based on this criterion, a clear motivation should be presented. For Option 3, the key differences would be 1) UE-B’s request transmission, 2) UE-A’s request reception. In this sense, condition-based operation is easier than request-based scheme 1. With this understanding, we think either ‘not split’ or ‘separate with a pre-requisite’ is the feasible approach. Proposal 10: - FG 32-5a is not split between TX and RX. - FG 32-5a is not split between preferred resources and non-preferred resources. - FG 32-5a is not split between request-based and condition-based, or is split between request-based (FG 32-5a-1) and condition-based (FG 32-5a-2) with setting FG 32-5a-2 as pre-requisite of FG 32-5a-1. Pre-requisite of FG 32-5a is dependent on whether to split the FG. After concluding the issue, pre-requisite can be discussed. It would be beneficial that NW knows whether the UE does IUC scheme 1 for SL and/or UL scheduling. Support/Not support should be shared between a pair of two UEs since a part of scheme 1 targets unicast communications. Proposal 11: - FG 32-5a is reported to gNB and UE. At the last meeting, RAN1 had long discussions on this issue but still companies’ views are divergent. In our understanding, this issue is the same as the number of HARQ processes, which is discussed above. From RF perspective, the purpose of PSFCH TX/RX cannot be distinguished. Basically different value between FG 15-11 and Rel-17 FG would be invalid. Meanwhile, there is a claim that UE supporting IUC might not support HARQ feedback. We are not sure whether this is really typical, but at least ‘possible’. With this thinking, similar note to in FGs 32-4/32-4a would be the best approach for converging. Proposal 12: - Add a component in FG 32-5b-1 to indicate supporting number of PSFCH TX with a note as follows: - 2) UE can transmit up to M PSFCH(s) resources in a slot - Note: If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number of PSFCH transmissions. UE is not required to support the sum of each reported value M. - Add a component in FG 32-5b-2 to indicate supporting number of PSFCH RX with a note as follows: - 2) UE can receive up to N PSFCH(s) resources in a slot - Note: If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number of PSFCH transmissions. UE is not required to support the sum of each reported value N. For 32-5b-1, UE-A’s behavior can be written as 1) sensing, 2) collision detection, 3) collision indication TX. For 3), this UE shall support PSFCH TX. The corresponding FG is FG 15-11. In addition, 1) and 2) can be performed by UE supporting either FG 15-3 or FG 32-4. They should be added as pre-requisite in “one of {x, y}” manner. Proposal 13: - FG 15-11 is added as pre-requisite of FG 32-5b-1. - Either FG 15-3 or FG 32-4 is added as pre-requisite of FG 32-5b-1, as “one of {15-3, 32-4} For 32-5b-2, UE-B’s behavior can be written as 1) reservation transmission, 2) collision indication RX, 3) reselection. For 1) and 3), this UE will support SL TX, thereby at least support of FG 32-4b is necessary. In addition, as UE behavior of 2) is PSFCH RX, at least support of FG 32-2 is necessary. Proposal 14: - FGs 32-2 and 32-4b are added as pre-requisite of FG 32-5b-2. Similarly to FG 32-5a, these FGs should be reported to both gNB and UE. Proposal 15: - FGs 32-5b-1 and 32-5b-2 are reported to gNB and UE. At the previous meeting, there was the above working assumption including UE capability perspective. Corresponding FG, which indicates whether Option 4 is supported or not under scheme 2, needs to be added. Proposal 16: - Add a new FG as follows. 32. NR_SL_enh; 32-X; Determination of expected conflict in Scheme 2 based on RSRP difference; 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations; 32-5b-1; [No]; [Yes]; UE does not support SIR-based collision detection in inter-UE coordination scheme 2.; [Per band]; N.A.; N.A.; N.A.; Optional with capability signalling.. The above agreement also includes UE capability discussion. We think that request/IUC-message via SCI 2-C can be covered in a single FG. Proposal 17: - Add a new FG as follows. 32. NR_SL_enh; 32-X; Reception of Scheme 1 inter-UE coordination information over 2nd SCI; 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. 2) UE can receive Scheme 1 explicit request transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same explicit request in the same transmission.; 32-5a; [No]; [Yes]; UE supports only MAC CE to receive inter-UE coordination information and explicit request in inter-UE coordination scheme 1.; [Per band]; N.A.; N.A.; N.A.; Optional with capability signalling.. |
| [8] | Intel Corporation | For inter-UE coordination scheme-1 we prefer to further split functionality required for transmission and reception of request and condition based inter-UE coordination feedback. In addition, the following container options were agreed for carrying scheme-1 IUC feedback: 1) MAC-CE and 2) MAC-CE + Stage-2 SCI. Therefore, we propose the following split of FG 32-5a: - FG 32-5a1: Support of request-based IUC feedback using MAC-CE - FG 32-5a2: Support of condition-based IUC feedback using MAC-CE - FG 32-5a3: Support of request-based IUC feedback using MAC-CE and Stage-2 SCI - FG 32-5a4: Support of condition-based IUC feedback using MAC-CE and Stage-2 SCI - In addition, at the last meeting optional support of MAC-CE and Stage-2 SCI for request of inter-UE coordination information was received. - Split FG 32-5a to the following FGs: - FG 32-5a1: Support of request-based IUC using MAC-CE - TX of request for IUC information using MAC CE - RX of request for IUC information using MAC CE - TX of feedback with IUC information using MAC CE - RX of feedback with IUC information using MAC CE - FG 32-5a2: Support of condition-based IUC using MAC-CE - TX of condition-based feedback with IUC information using MAC CE - RX of condition-based feedback with IUC information using MAC CE - FG 32-5a3: Support of request-based IUC feedback using MAC-CE and SCI format 2C - TX of feedback with IUC information using MAC CE and SCI format 2C - RX of feedback with IUC information using MAC CE and SCI format 2C - FG 32-5a4: Support of condition-based IUC feedback using MAC-CE and SCI format 2C - TX of condition-based feedback with IUC information using MAC CE and SCI format 2C - RX of condition-based feedback with IUC information using MAC CE and SCI format 2C - FG 32-5a5: Support of request for IUC feedback using MAC-CE and SCI format 2C - TX of request for IUC information using MAC CE and SCI format 2C - RX of request for IUC information using MAC CE and SCI format 2C For IUC scheme-2, the pre-requisites for FGs 32-5b1(transmission of scheme-2 feedback) and 32-5b2 (reception of scheme-2 feedback) are still under discussion. In our view, reception of the scheme 2 feedback requires at least support of the FG 15-11 (PSFCH Format 0) or its components and FG 15-3 or its sensing related components Transmission of the scheme 2 feedback additionally requires support of the FG 15-3 (TX M2) or FG 32-4 (TX M2 w/partial sensing) - Reception of the scheme 2 feedback (FG 32-5b2) requires support of the at least following FGs - FG 15-11 (PSFCH Format 0) or its PSFCH reception components - FG 15-3 (TX M2) or its sensing related components - Transmission of the scheme 2 feedback (FG 32-5b1) additionally requires support of the one of the following FGs - FG 15-11 (PSFCH Format 0) or its PSFCH transmission components - FG 15-3 (TX M2) - FG 32-4 (TX M2 w/partial sensing) |
| [9] | Apple | One RAN1 objective in Release 17 NR sidelink enhancement is to specify inter-UE coordination. Two inter-UE coordination schemes are supported. In inter-UE coordination scheme 1, the coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission; In inter-UE coordination scheme 2, the coordination information sent from UE-A to UE-B is the presence of expected/potential detected resource conflict on the resources indicated by UE-B’s SCI. It was agreed of three feature groups, where feature 32-5a is inter-UE coordination scheme 1, feature 32-5b-1 is transmitting inter-UE coordination scheme 2 and feature 32-5b-2 is receiving inter-UE coordination scheme 2. One open issue is whether/how to split feature 32-5a into multiple feature groups. In our view, UE-A, which sends inter-UE coordination scheme 1, needs to perform a new resource selection procedure to determine a set of non-preferred resources. Specifically, in UE-A’s determination of a set of non-preferred resources, one option is the reserved resource by another UE whose RSRP measurement is larger than a threshold, and the other option is the reserved resource by another UE whose RSRP measurement is smaller than a threshold and UE-A is a destination corresponding to that reservation. Both options are different from the legacy resource selection procedure. Hence, it is preferred to split the feature 32-5a, depending on whether a set of preferred resources or a set of non-preferred resources are transmitted. On the other hand, UE-B, which receives inter-UE coordination scheme 1, needs to utilize the received inter-UE coordination in its resource selection procedure. Unlike UE-A, UE-B does not have to perform/support sensing/resource exclusion. For example, it is supported that UE-B’s resource selection is based only on the received coordination information with a set of preferred resources. To support this type of UEs, we need to split the feature 32-5a, depending on the transmission or reception of inter-UE coordination scheme 1. For a set of non-preferred resources, UE-B’s PHY layer will exclude them in the resource selection procedure. For a set of preferred resources, UE-B’s MAC layer will utilize them depending on whether UE-B has its own sensing results or not. Hence, it is preferred to split the feature 32-5a, depending on whether a set of preferred resources or a set of non-preferred resources are received. Overall, we think feature 32-5a should be split according to transmitting inter-UE coordination scheme 1 with preferred resource set, transmitting inter-UE coordination scheme 1 with non-preferred resource set, receiving inter-UE coordination scheme 1 with preferred resource set, and receiving inter-UE coordination scheme 1 with non-preferred resource set. Proposal 5: Split feature 32-5a to the following four features - transmitting inter-UE coordination scheme 1 with preferred resource set, where - UE can transmit inter-UE coordination of preferred resource set, - UE can receive explicit request for inter-UE coordination. - transmitting inter-UE coordination scheme 1 with non-preferred resource set, where - UE can transmit inter-UE coordination of non-preferred resource set, - UE can receive explicit request for inter-UE coordination. - receiving inter-UE coordination scheme 1 with preferred resource set, where - UE can receive inter-UE coordination of preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2, - UE can transmit explicit request for inter-UE coordination. - receiving inter-UE coordination scheme 1 with non-preferred resource set, where - UE can receive inter-UE coordination of non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2, - UE can transmit explicit request for inter-UE coordination. In last RAN1 meeting, there is a proposal of new feature 32-7 “Determination of expected conflict in Scheme 2 based on RSRP difference”, where UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations. In our view, this feature is aligned with RAN1 agreement that supporting differential RSRP-based scheme is subject to UE capability. This feature is based on the existing feature 32-5b-1 and hence its pre-requisite feature group is 32-5b-1. Proposal 6: Introduce a new feature 32-7 “Determination of expected conflict in Scheme 2 based on RSRP difference”, - with the component “UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations” - with the pre-requisite feature group as 32-5b-1. In last RAN1 meeting, there is a proposal of new feature 32-6 “Reception of Scheme 1 inter-UE coordination information over 2nd SCI”, where UE can receiver Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. In our view, this feature is aligned with RAN1 agreement that 2nd SCI is UE Rx optional. Hence, we have the following proposal. Proposal 7: Introduce a new feature 32-6 “Reception of Scheme 1 inter-UE coordination information over 2nd SCI”, - with the component “UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission”. In last RAN1 meeting, we have a working assumption that MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A, where SCI 2-C is UE Rx optional. Hence, similar to feature 32-6, we propose to introduce a new feature of “Reception of Scheme 1 explicit request over 2nd SCI, with the component of “UE can receive Scheme 1 explicit request over 2nd SCI that is used in addition to the MAC-CE carrying the same explicit request in the same transmission”. Proposal 8: Introduce a new feature “Reception of Scheme 1 explicit request over 2nd SCI”, - with the component “UE can receive Scheme 1 explicit request over 2nd SCI that is used in addition to the MAC-CE carrying the same explicit request in the same transmission”. |
| [12] | Samsung | Regarding the splitting of FG 32-5a, the Moderator suggested in RAN1#107b that this issue can be further discussed after some progress is made in AI 8.11.1.2. Given the progress in the AI in RAN1#107b, we believe there is a strong need to split the UE features of transmitting and receiving inter-UE coordination information of Scheme 1. This is because, some UEs might be able to receive and implement the assistance information but not necessarily capable of providing assistance due to their limited sensing capabilities and power restrictions. Hence, similar to FG 32-5b, FG 32-5a can be simply split into 32-5a-1 with only transmitting capability and 32-5a-2 with only receiving capability. Proposal 2: The UE feature of transmitting and receiving Scheme 1 inter-UE coordination information is separated. |
| [13] | MediaTek Inc. | For inter-UE coordination, the following changes are preferred considering the different behaviors and support for UE-A and UE-B. Moreover, it is also related to the reception capability in SL power saving session. Thus it is not necessary for UE to receive the explicit request if the UE can’t receive PSSCH/PSCCH or condition based scheme 1 is applied. Proposal 5: FG 32-5a is split as following Table below. Features; Index; Feature group; Components. 32. NR_SL_enh; 32-5a-1; Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-B w/o transmission of the explicit request for scheme 1); 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2.. 32. NR_SL_enh; 32-5a-2; Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-B w/ transmission of the explicit request for scheme 1); 1) UE can transmit explicit request. 2) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2.. 32. NR_SL_enh; 32-5a-3; Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-A w/o reception of the explicit request); 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set. 32. NR_SL_enh; 32-5a-4; Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-A w/ reception of the explicit request); 1) UE can receive explicit request 2) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set.. |
| [14] | Qualcomm Incorporated | In general, many of the new features are impacted by the currently supported featureset. For example, the ability to process the additional information required for inter-UE coordination could be impacted by support of operations in other bands that would increase the baseband processing effort. Similarly transmitting or receiving such information would be impacted by other reception and transmission. Another example is sensing and partial sensing, which both require baseband processing taking away processing resources from other operations. Therefore, we propose that the FGs be defined per featureset. Proposal 1: UE features for sidelink enhancements are defined per featureset. RAN1 still needs to decide how to report the total number of supported PSFCHs transmissions and receptions and whether that value should combine PSFCH for the purpose of feedback and inter-UE coordination. The limitation on the maximum number of simulataneous PSFCH transmissions is an RF limitation, unrelated to why any PFSCH is transmitted. Similarly, the UE’s ability to support a maximum number of simultaneous PSFCH receptions is unrelated to the purpose of a PSFCH. Hence, the number of reported simultaneous PSFCH receptions and/or transmissions should be considered a maximum for the UE and the UE is not expected to support the sum of the reported M (or N) values. Moreover, there is no separate allocation for PFSCH for a specific purpose. Proposal 6: The number of reported simultaneous PSFCH receptions and/or transmissions is considered a maximum, and not the sum, across the FGs, regardless of whether it is reported jointly or separately. The reported number(s) applies to the total of PFSCH operations for feedback and Scheme 2. It was agreed to use separate inter-UE coordination UE capabilities into at least two FGs: one for Scheme 1 and one for Scheme 2. The latter was further split into reception and transmission FGs; however, Scheme 1 still has a pending FFS on further splitting. The ability to utilize preferred resource information is unrelated to the ability to utilize non-preferred resources and target different scenario that need not be supported by the same device. Therefore, the FGs for Scheme 1 with preferred resource indication, for Scheme 1 with non-preferred resource indication should be separated. Moreover, a UE that utilizes coordination information does not necessarily need to be able to generate coordination information, for example, a low power device can utilize the preferred resource set to select resources but might not be performing sensing and would not be able to generate such a set. Hence, those two aspects are listed in separate FGs for each variant. Proposal 7: Scheme 1 with preferred resource is a separate FG from Scheme 1 with non-preferred resources. Proposal 8: The capability to receive inter-UE coordination information is separate from the capability to generate and transmit inter-UE coordination information, at least for Scheme 1 with preferred resources. RAN1 agreed to support reception of Scheme 1 inter-UE coordination using SCI-2 as an optional feature. We propose a new FG, FG 32-6, to incorporate this agreement into the UE feature list. Similarly, RAN1 agreed that reception of a request for inter-UE coordination over SCI-2 is optional. We introduce FG 32-7 to capture this agreement. Lastly, RAN1 agreed to support the following condition for determining a conflict in Scheme 2 inter-UE coordination information as an optional feature. A new FG 32-8 is added to the table in the following section capturing this decision: 32. NR_SL_enh; 32-6; Reception of Scheme 1 inter-UE coordination information over 2nd SCI; 1) UE can receive transmissions with SCI format 2-C that include inter-UE coordination information.; None; No; No; Per FS; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. 32. NR_SL_enh; 32-7; Reception of Scheme 1 inter-UE coordination request over 2nd SCI; 1) UE can receive transmissions with SCI format 2-C that include a request for inter-UE coordination information.; None; No; No; Per FS; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. 32. NR_SL_enh; 32-8; Determination of expected conflcit in Scheme 2 based on RSRP difference; 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations; None; No; No; Per FS; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. |
| [15] | LG Electronics | For whether a UE jointly reports PSFCH Tx/Rx capability for Inter-UE Coordination (IUC) and SL HARQ-ACK, we think that there would be no problem in defining it as a joint report form if a note to clarify the relationship between FG 32-5b-1 and FG 15-11 or between FG 32-5b-2 and FG 15-11/FG 32-2 is added. For example, the value of the joint report of PSFCH Tx capability is greater than or equal to the value of M reported in FG 15-11. Also the value of the joint report of PSFCH Rx capability is greater than or equal to the value of N reported in FG 15-11 or the value reported in FG 32-2. The candidate values that can be used for the joint report of PSFCH Tx capability or PSFCH Rx capability are the same as M (i.e., 4, 8, 16) or N (i.e., 5, 15, 25, 32, 35, 45, 50, 64) values of FG 15-11, respectively. Proposal 3: For FG 32-5b-1, PSFCH Tx capability is jointly reported for inter-UE coordination and SL HARQ-ACK. - The value range for this capability is {4, 8, 16}. - The value reported in this capability is greater than or equal to the value of M reported in FG 15-11. Proposal 4: For FG 32-5b-2, PSFCH Rx capability is jointly reported for inter-UE coordination and SL HARQ-ACK. - The value range for this capability is {5, 15, 25, 32, 35, 45, 50, 64}. - The value reported in this capability is greater than or equal to the value of N reported in FG 15-11 or the value reported in FG 32-2. Considering the cyan marked part in the following working assumption, it would be desirable to define a new FG where a UE can determine an expected/potential resource conflict for overlapping resource reservations between UE-B and another UE based on RSRP measurement difference of the two reservations. Also it needs to have FG 32-5b-1 as the prerequisite FG. Proposal 5: Define a new FG for Scheme 2: “UE can determine an expected/potential resource conflict for overlapping resource reservations between UE-B and another UE based on RSRP measurement difference of the two reservations”. - FG 32-5b-1 is a prerequisite FG of this FG. In case of Scheme 1, even though the preferred and non-preferred resource sets may be beneficial in different scenarios, it does not necessarily mean that the FG for each resource set should be defined separately. Note that there is no significant difference in the fundamental processing required to derive/determine different types of resource sets, which is a factor that should be considered more important when defining a FG. Proposal 6: No need to define separate FGs between the preferred and non-preferred resource sets. Considering the following agreement made in the last meeting, it is clear that both preferred resource set and non-preferred resource set need to be supported for the case when the inter-UE coordination information is triggered by UE-B’s request. So, it would be desirable to Component 2 of FG 32-5a is updated as “UE can transmit and received an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set”. Proposal 7: Update Component 2 of FG 32-5a as “2) UE can transmit and received an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set”. For Scheme 2, the Tx/Rx capabilities of conflict indication were separated in consideration of reducing the power consumption of conflict indication transmission and the implementation complexity required for the conflict indication transmission. We think that the same principle can be applied to Scheme 1. In other words, FG 32-5a can be split into Tx and Rx capabilities. Proposal 8: Split FG 32-5a into Tx and Rx capabilities. As per the following agreements on the container of IUC information/explicit request transmission (see the grey marked parts), we can define a new FG where a UE can receive Scheme 1 IUC information/explicit request transmitted over SCI 2-C that is used in addition to MAC-CE carrying the same IUC information/explicit request in the same transmission. Proposal 9: Define a new FG for Scheme 1: “UE can receive Scheme 1 inter-UE coordination information/explicit request transmitted over SCI 2-C that is used in addition to MAC-CE carrying the same inter-UE coordination information/explicit request in the same transmission”. |
| [16] | Ericsson | In our view, a UE which is performing Inter-UE coordination scheme 1 should be able to transmit (and receive) both types of resource sets, i.e., preferred and non-preferred set. The motivation is that based on the information contained on an enquiry from a Tx UE, the Rx UE may need to perform the operation for a preferred or a non-preferred set. Moreover, the UE operation to obtain a preferred or non-preferred set of resources is really similar. Therefore, we support that both are included in the FG definition. A UE implementing FG 32-5a supports both preferred and non-preferred set of resources. Regarding the prerequisite feature groups, for the FG 32-5a, the UE needs to support the feature groups which are related to the transmissions and reception of SL transmissions. In this case, we support to include the FG 15-1 and FG 15-3 as defined for Rel-16 UEs. Moreover, it is also possible that a UE involved in the inter-UE coordination scheme 1 can be performing partial sensing mechanism, and therefore, we propose to include it too. Include in the pre-requisite feature groups field for the FG 32-5a the following FGs: 15-1, 15-3 and 32-4. For the remaining unstable fields of 32-5a, i.e., whether the gNB needs to know about the feature, signalling to other UEs about the feature and the granularity of the feature, we propose the following: - Need for the gNB to know if the feature is supported: Yes - Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: Yes - Granularity of the feature: Per band For the FG 32-5a, the remaining unstable fields are completed as follows: - Need for the gNB to know if the feature is supported: Yes - Applicable to the capability signalling exchange between UEs (Sidelink WI only)”: Yes - Granularity of the feature: Per band Regarding the reception of the 2nd stage SCI which contains the inter-UE coordination information as indicated in the agreement above, a new FG indicating this new optional UE optional is needed. The proposed FG description is included in the table below. Introduce a new optional FG to indicate that a UE is capable of receiving a 2nd stage SCI containing the inter-UE coordination information. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 32. NR_SL_enh; 32-5a1; Reception of Inter-UE coordination contained in 2nd stage SCI for Inter-UE coordination scheme 1 in NR sidelink mode 2; UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set in the 2nd stage SCI in its own resource (re-)selection in NR sidelink mode 2.; [32-5a]; [No]; [Yes]; UE does not support receiving inter-UE coordination information in 2nd stage SCI for scheme 1 in NR sidelink mode 2.; [Per UE]; N.A.; N.A.; N.A.; Optional with capability signalling.. For the FG32-5b-1 and FG32-5b-1, in our view, both FGs need as pre-requisite the FGs which are related to the transmission and reception capability for the UEs (similar to the pre-requisites defined for the inter-UE coordination scheme 1). Therefore, we propose to include the FG 15-1 and FG 15-11 as defined for Rel-16 UEs. Moreover, it has not been disregarded that a UE involved in the inter-UE coordination scheme 2 can be performing partial sensing mechanism, and therefore, we propose to include it too. The rationale behind including these FGs is that the UEs involved in the inter-UE coordination scheme 2 shall be able to perform sensing, i.e., full sensing (FG 15-1) or partial sensing (FG 32-4a), in order to detect a collision and they should also be able to transmit PSFCH (FG 15-11). Include in the pre-requisite feature groups field for the FG 32-5b-1 and FG 32-5b-2 the following FGs: 15-1, 15-11 and 32-4, along with the already included pre-requisite FGs. |
| [17] | ZTE, Sanechips | The working assumption on scheme 2 from RAN1#107bis-e as above says that UE-B shall indicate via SCI 1-A its capability and feasibility of being UE-B or not if enabled. This implies the scheme 2 capability itself should be of type per UE. Likewise, for scheme 1, to avoid the signaling storming issue due to excessive IUC request or report information under either request based or condition other than request based solution, it's equally reasonable that the Tx and Rx capability of scheme 1 IUC is per UE. The feature type for 32-5a ,32-5b-1 and 32-5b-2 is per UE as shown below. Similar to the split of scheme 2 IUC, a UE can support scheme 1 IUC reception without having to support transmission. However, reception should be prerequisite to transmission as well. In the meantime, the procedure and capability of determining preferred and non-preferred resource set is quite similar for scheme 1 IUC, with which UE-A can determine either up to implementation or the request from UE-B the resource type to be reported, thus there is no need to differentiate from the resource type perspective. Thus 32-5-a is proposed to split into Tx and Rx similar to 32-5b-1 and 32-5b-2. To clarify the capability for condition other than request based and request based IUC do not have to be simultaneously supported, a note can be captured by saying at least one component can be supported when this FG is indicated. Scheme 1 IUC 32-5-a is split into Tx and Rx capability with Tx using Rx as prerequisite 32-5a-1; Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set.; 32-5a-2; [Yes]; [Yes]; UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2.; [Per band] Per UE; N.A.; N.A.; N.A.; At least one component can be supported if UE indicates the support of this FG.; Optional with capability signalling.. 32-5a-2; Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set.; [TBD]; [Yes]; [Yes]; UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2.; [Per band] Per UE; N.A.; N.A.; N.A.; At least one component can be supported if UE indicates the support of this FG.; Optional with capability signalling.. For 32-5a IUC,15-1 and 15-4 should be the prerequisite of IUC reception 32-5a-2. For 32-5a-1, in addition to 32-5a-2, either 32-4 or 15-3 needs to be supported as prerequisite for the transmission of IUC information. For 32-5a-2 with the reception of IUC request, the UE is supposed to feedback IUC information, this point is clarified as a note. For 32-5b IUC, the reception or transmission of PSFCH can be added as components to 32-5b-2 and 32-5b-1 respectively, it should be noted that the number of PSFCH Rx/Tx occasions should be shared with 15-11, a note in Rel-17 FG is needed for clarification. To reflect the above changes, the following proposal is made, 32-5a-1; Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set.; 32-5a-2, At least one of 32-4 or 15-3; [Yes]; [Yes]; UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2.; [Per band] Per UE; N.A.; N.A.; N.A.; At least one component can be supported if UE indicates the support of this FG.; Optional with capability signalling.. 32-5a-2; Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set.; 15-1,15-4; [Yes]; [Yes]; UE does not support inter-UE coordination scheme 1 in NR sidelink mode 2.; [Per band] Per UE; N.A.; N.A.; N.A.; At least one component can be supported if UE indicates the support of this FG. If component 2 is supported, at least one of 32-4 or 15-3 shall be prerequisite.; Optional with capability signalling.. 32-5b-1; Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2; 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. 2) UE can transmit up to M PSFCHs resources in a slot; 32-5b-2, At least one of 32-4 or 15-3; [Yes]; [Yes]; UE does not support transmitting inter-UE coordination scheme 2 in NR sidelink mode 2.; [Per band] Per UE; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for M are {4, 8, 16} Note: When both 32-5b-1 and 15-11 are supported, the total number of PSFCH that a UE can transmit shall be up to M; Optional with capability signalling.. 32-5b-2; Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 2) UE can receive up to N PSFCH(s) resources in a slot.; 15-1,15-4; [Yes]; [Yes]; UE does not support receiving inter-UE coordination scheme 2 in NR sidelink mode 2.; [Per band] Per UE; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Candidate values for N are {5, 15, 25, 32, 35, 45, 50, 64} Note: When both 32-5b-1 and 15-11 are supported, the total number of PSFCH that a UE can receive shall be up to N; Optional with capability signalling.. |

Discussion

[FL1] High priority proposal 5-1:

- FG 32-5a is split into one FG for Tx capability and another for Rx capability as follows

  - FFS whether/how the FGs are split further

| 32. NR_SL_enh | 32-5a-1 | Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-1 into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support transmitting inter-UE coordination scheme 1 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a-2 | Receiving Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-1 into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support receiving inter-UE coordination scheme 1 in NR sidelink mode 2. | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - TX vs RX: HW, vivo, OPPO, CATT, Nokia, Apple, SS, MTK, LGE, ZTE - HW, vivo, CATT, Apple: a UE may only receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection, meanwhile it may not need to have the capabilities to transmit inter-UE coordination information for some power efficient UEs. - Preferred vs non-preferred: vivo. Intel, Apple, MTK - vivo: These sub-schemes are targeting different scenarios and device types. A power-limited or low-end SL UE can only support one sub-scheme (e.g., scheme 1 with preferred resource set), while a normal VUE can support both. - Apple: Specifically, in UE-A’s determination of a set of non-preferred resources, one option is the reserved resource by another UE whose RSRP measurement is larger than a threshold, and the other option is the reserved resource by another UE whose RSRP measurement is smaller than a threshold and UE-A is a destination corresponding to that reservation. Both options are different from the legacy resource selection procedure. - Condition-based vs request-based: Intel - MAC-CE vs MAC-CE+SCI-2C: Intel - No: FW, DCM - FW, DCM: a Scheme 1 Rx UE needs to detect the PSCCH/PSSCH to receive coordination information as MAC-CE is always used for coordination transmission (only coordination information in SCI is Rx optional). And even for the RX only UE, it also may need to send the coordination request or some info via RRC signaling to UE-A which is related to Tx functionality. Similarly for Scheme 1 Tx UE, i.e., UE-A, UE-A needs the reception capability of PSCCH for sensing to generate coordination information, i.e., again Tx and Rx mixed. For preferred vs non-preferred, many unified designs, e.g., container, content format, were agreed. Given majority companies prefer to at least split FG 32-5a into one FG for Tx capability and another for Rx capability, proposal 5-1 is set for the 1st round discussion. |
| NTT DOCOMO | Considering majority view, we can be OK with the proposal’s direction. From the motivation, however, it seems that it is better to include 32-5a-2 as prerequisite of 32-5a-1. |
| vivo | We are supportive to Tx/Rx based splitting, but we still support the preferred/non-preferred splitting. According the current spec/agreements, the UE behaviors are becoming further different between them. For example, the resource selection procedure enhancements are performed at different layers between these two schemes as below. Consequently, the hardware implementations are quite different – it is clearly not reasonable to enforce a UE to always support both schemes. Separate FGs for preferred/non-preferred should be supported. Preferred case: Agreement For Scheme 1 with preferred resource set Option A, - MAC layer selects resources using S_A and the received preferred resource set - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16. - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A under the constraint defined in Rel-16. Agreement For Scheme 1 with preferred resource set Option B, - MAC layer selects resources belonging to the received preferred resource set under the constraint defined in Rel-16 - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE Non-preferred case: Agreement For Scheme 1 with non-preferred resource set, - Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set |
| ZTE,Sanechips | Ok with the direction |
| Qualcomm | We are ok with the proposal and share vivo’s view to further split into preferred/non-preferred. |
| Apple | We support the proposal at least for Tx/Rx IUC separation. Here, a UE receiving IUC scheme 1 does not have to support the transmission of IUC scheme 1, since this UE may not perform sensing. We support to further split the FG based on preferred or non-preferred resource set. Note that UE-A’s behavior is quite different to generate a set of preferred resource set and to generate a set of non-preferred resource set. For preferred resource set, the legacy resource selection procedure is used by UE-A. For non-preferred resource set, UE-A needs to implement a new procedure. Hence, transmitting preferred resource set and non-preferred resource set should be separated for UE-A. Similarly, UE-B’s behavior is different when receiving a set of preferred resource set and when receiving a set of non-preferred resource set. For example, UE-B needs to have sensing result when receiving a set of non-preferred resource set, while this is not required when UE-B receiving a set of preferred resource set. For a set of non-preferred resources, UE-B’s PHY layer will exclude them in the resource selection procedure. For a set of preferred resources, UE-B’s MAC layer will utilize them depending on whether UE-B has its own sensing results or not. Hence, receiving preferred resource set and non-preferred resource set should be separated for UE-B. |
| OPPO | We support splitting the feature per Tx/Rx, however, seems there are typos in the components (2) of the 2 new FGs as below, as the explicit request is transmitted/received by the UE receiving/transmitting the IUC information. For 32-5a-1, it should be “2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set].” For 32-5a-2, it should be “2) UE can receive transmit an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set].” |
| Samsung | O.K with the proposal. |
| Huawei, HiSilicon | We are OK with the direction on FG 32-5a splitting. And we have the same view with OPPO it seems there are typos in the components (2) of the 2 new FGs For 32-5a-1, it stands for UE-A, it will transmit inter-UE coordination information, meanwhile, it will receive an explicit request from UE-B. For 32-5a-2, it stands for UE-B, it will UE receive inter-UE coordination information use the received information in its own resource (re-)selection, meanwhile it will transmit an explicit request to trigger UE-A to transmit inter-UE coordination information, the related word should be modified as following: For 32-5a-1, it should be “2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set].” For 32-5a-2, it should be “2) UE can receive transmit an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set].” For the first FFS in FG 32-5a-1 and FG 32-5a-2, we think it should be applicable to both preferred resource set and non-preferred resource set. There is no necessity to further split FGs between preferred and non-preferred resource set. UE-A has to perform sensing and resource exclusion procedure specified in 8.1.4 of TS 38.214 to determine the IUC resource set, regardless of preferred and non-preferred, i.e. the UE-A is capable of finding preferred resource, and is also capable of non-preferred resource, or vice versa, given that the sensing result includes both preferred and non-preferred resources. Thus, we don’t need see the technical justification, processing limitation or hardware constraint that a UE is only capable of handling one of the preferred and non-preferred resource set in scheme 1. For the second FFS in FG 32-5a-1 and FG 32-5a-2, further split is unnecessary. |
| Panasonic | We support the direction to split per Tx and Rx. |
| CATT, GOHIGH | We support to split the feature per Tx/Rx capability. |
| Ericsson | We prefer not to have these split. However, based on majority of companies we can accept to divide the Inter-UE coordination scheme 1 regarding the Tx and Rx capabilities. For the FG 32-5a-1, we propose to include the following as pre-requisite FG: 15-1, 15-3 and 32-4. For the FG 32-5a-2, we propose to include the following as pre-requisite FG: 32-5a-1. |
| Spreadtrum | We support to split into one FG for Tx capability and another for Rx capability. |
| Futurewei | We do not prefer further splitting FG 32-5a. But if majority want to split it into Tx/Rx coordination information FG, we are ok with this split provided that 32-5a-2 receiving coordination is made the prerequisite for 32-5a-1. We do not support further splitting in other directions as listed: preferred vs non-preferred, condition-based vs request-based, or MAC-CE vs MAC-CE+SCI-2C. For preferred vs non-preferred, many unified designs, e.g., container, content format, were agreed. We do not think it is necessary to split them. Similarly for request vs condition- based coordination, further splitting the FG is unnecessary. |
| Moderator | Summary of companies view - Support: DCM, ZTE, QC, Apple, OPPO, SS, HW, Pana, CATT, [E///]. SPRD, [FW] - Prerequisite of 32-5a-1: 32-5a-2 (DCM), 15-1, 15-3 and 32-4 (E///) - Prerequisite of 32-5a-2: 32-5a-1 (E///) - Preferred vs non-preferred: vivo, QC, Apple - Not support: HW, E///, FW Given all companies are fine with proposal 5-1 in principle, a updated proposal is set for GTW session [GTW1] High priority proposal 5-1: - FG 32-5a is split into one FG for Tx capability and another for Rx capability as follows - FFS whether/how the FGs are split further 32. NR_SL_enh; 32-5a-1; Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-1 into multiple FGs; [TBD]; [Yes]; [Yes]; UE does not support transmitting inter-UE coordination scheme 1 in NR sidelink mode 2.; [Per band]; N.A.; N.A.; N.A.; Optional with capability signalling.. 32. NR_SL_enh; 32-5a-2; Receiving Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can receive transmit an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-1 into multiple FGs; [TBD]; [Yes]; [Yes]; UE does not support receiving inter-UE coordination scheme 1 in NR sidelink mode 2.; [Per band]; N.A.; N.A.; N.A.; Optional with capability signalling.. |
| FL2 | Following was agreed in the GTW session on Feb 22. Agreement - FG 32-5a is split into one FG for Tx capability and another for Rx capability as follows - FFS whether/how the FGs are split further 32. NR_SL_enh; 32-5a-1; Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-1 into multiple FGs; [TBD]; [Yes]; [Yes]; UE does not support transmitting inter-UE coordination scheme 1 in NR sidelink mode 2. UE does not receive an explicit request for inter-UE coordination information; [Per band]; N.A.; N.A.; N.A.; Optional with capability signalling.. 32. NR_SL_enh; 32-5a-2; Receiving Inter-UE coordination scheme 1 in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can receive transmit an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-2 into multiple FGs; [TBD]; [Yes]; [Yes]; UE does not support receiving inter-UE coordination scheme 1 in NR sidelink mode 2. UE does not transmit an explicit request for inter-UE coordination information; [Per band]; N.A.; N.A.; N.A.; Optional with capability signalling.. 1) Regarding the FFS whether/how the FGs are split further, following is the summary of companies view - Preferred vs non-preferred - Support: vivo, QC, Apple - Not support: HW, E///, FW [FL2] High priority question 5-1a: - Companies are encouraged to provide view whether FGs 32-5a-1/32-5a-2 are split into one for preferred resource set and another for non-preferred resource set 2) Regarding other FFS parts which have RAN2 signaling impact, they will be discussed in questions 5-5/5-6/5-7 3) Regarding prerequisite FGs, following is the summary of companies view. - Prerequisite of 32-5a-1: 32-5a-2 (DCM), 15-1, 15-3 and 32-4 (E///) - Prerequisite of 32-5a-2: 32-5a-1 (E///) Since it does not have RAN2 signaling impact, it can be discussed with low priority (i.e., companies are not required to provide view for now but are free to share the view for further discussion) Low priority question 5-1b: - Companies are invited to provide view on the prerequisite FGs for FGs 32-5a-1/32-5a-2 |
| NTT DOCOMO | For question 5-1a, we do not think further split is necessary since there are a lot of unified behaviors. Only if majority really wants further split, we can accept it. For question 5-1b, we see that the motivation of split mentioned by companies is the same as for FGs 32-5b-1/32-5b-2. Thus we think it is reasonable as in FGs 32-5b-1/32-5b-2 that 32-5a-2 is prerequisite of 32-5a-1. |
| Apple | For question 5-1a, we think the split of FG 32-5a-1 is needed. The resource selection procedures of UE-A are quite different when determining a set of preferred resources vs. when determining a set of non-preferred resources. - For a set of preferred resources, UE-A applies the legacy resource selection procedure. - For a set of preferred resources, one option is the reserved resource by another UE whose RSRP measurement is larger than a threshold, and the other option is the reserved resource by another UE whose RSRP measurement is smaller than a threshold and UE-A is a destination corresponding to that reservation. Both options are different from the legacy resource selection procedure. Hence, it is preferred to split the feature 32-5a-1, depending on whether a set of preferred resources or a set of non-preferred resources are transmitted. We think the split of FG 32-5a-2 is needed. The UE-B’s behavior is quite different when receiving a set of preferred resources vs. when receiving a set of non-preferred resources. - For a set of preferred resources, UE-B’s MAC layer will utilize them depending on whether UE-B has its own sensing results or not. - For a set of non-preferred resources, UE-B’s PHY layer will exclude them in the resource selection procedure. - For a set of preferred resources, UE-B has an option of not performing sensing at all and relying only on the set of preferred resources for its resource selection. - For a set of non-preferred resources, UE-B needs to have its own sensing results to work together with the set of non-preferred resources for its resource selection. Hence, it is preferred to split the feature 32-5a-1, depending on whether a set of preferred resources or a set of non-preferred resources are received. |
| CATT, GOHIGH | From our understanding, since the resource set type can be carried by inter-UE coordination information, it means the UE should support to perform resource selection based on both preferred and non-preferred resource set, therefore the preferred/non-preferred resource set should not be separated as different FGs. Regarding the pre-requisite issue, we think the same principle of scheme 2 can be used, i.e. 23-5a-2 is the pre-requisite of 32-5a-1. |
| OPPO | We think 32-5a-2 should be split into one for preferred resource set and another for non-preferred resource set, considering that some UE in power saving mode can only use preferred resource set. But we do not think it is necessary to further split FGs 32-5a-1. |
| ZTE,Sanechips | Similar view as CATT to Q1, a UE should support both capabilities and determine the relevanat operations with the corresponding resource type set. To Q3, the prerequisites are as follows. 32-5b-1; Transmitting Inter-UE coordination scheme 2 in NR sidelink mode 2; 1) UE can transmit inter-UE coordination information of presence of expected/potential resource conflict in NR sidelink mode 2. 2) UE can transmit up to M PSFCHs resources in a slot; 32-5b-2, At least one of 32-4 or 15-3. 32-5b-2; Receiving Inter-UE coordination scheme 2 in NR sidelink mode 2; 1) UE can receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 2) UE can receive up to N PSFCH(s) resources in a slot.; 15-1,15-4. |
| vivo | We share the view with Apple. Regarding the comments from CATT/ZTE, the IUC signaling can indicate the resource set type, has no relevant to the discussion here. There are lots of DCI/SCI fields, and obviously we cannot say the UE should support all the features that may presented in DCI/SCI. Note that these FGs are exchanged between UEs, so the TX UE knows which resource type the Rx UE supports, thus it can send the correct type. |
| Qualcomm | We share the same view as Apple and vivo to split the two FGs into preferred/non-preferred. |
| Futurewei | We do not support further splitting 32-5a-1/32-5a-2 FGs. For preferred vs non-preferred, many unified designs, e.g., container, content format, were agreed. We do not think it is necessary to split them. Also similar view as CATT, the resource type indication is carried in the request and coordination information and UE can use both resource sets for resource selection. The two types of resource sets should be supported together. Regarding the prerequisite, we prefer following scheme 2, 32-5a-2 receiving capability is the prerequisite of the 32-5a-1. If the other way around, it may have an issue as the UE receiving coordination information may not have sensing capability. |
| Huawei, HiSilicon | 5-1a: We think there is no necessity to further split FGs between preferred and non-preferred resource set. As we mentioned in previous round, we don’t need see the technical justification, processing limitation or hardware constraint that a UE is only capable of handling one of the preferred and non-preferred resource set in scheme 1. 5-1b: as proposed in our paper we suggest the prerequisite group as following: Prerequisite of 32-5a-1: 15-1, 15-3, 15-4, 15-5, 15-11, 15-23 Prerequisite of 32-5a-2: 15-1, 15-4, 15-5, 15-11, 15-23 and at least one of 15-3, 32-4 |
| Ericsson | We do not support further splitting of the FG into preferred/non-preferred. In our view, the functionality and behavior for both preferred and non-preferred is quite unified so we do not see the need to create two separate FG. |
| Moderator | 1) Regarding the FFS whether/how the FGs are split further, following is the summary of companies view - Preferred vs non-preferred - Support: vivo, QC, Apple, OPPO (32-5a-2) - Not support: HW, E///, FW, DCM, CATT, ZTE [GTW2] Companies still have different view whether to split to preferred and non-preferred. More discussion is necessary Low priority question 5-1b: will be discussed later |
| FL3 | Regarding whether to split to Preferred vs non-preferred, it could not be discussed in the GTW session on Feb 24. Companies are encouraged to check the comments from other companies and provide further comments @Companies who still prefer to split to Preferred vs non-preferred: Please explain why you still think the separation is necessary. @Companies who still prefer NOT to split to Preferred vs non-preferred: Please explain why you still think the separation is NOT necessary. |
| vivo | The following reasons are observed from companies against the splitting: - The behavior is quite unified between the two schemes. - Inter-UE coordination information carries the resource set type of these two schemes. - Do not see the technical justification, processing limitation or hardware constraint that a UE is only capable of handling one of the preferred and non-preferred resource set in scheme 1. Regarding the 1st point, although there are some commonalities between them, there are also lots of differences as already demonstrated by Apple and many others, e.g., the resource selection behaviors are quite different, the resource determination based on IUC is performed at different layers (MAC vs PHY) between these two schemes, etc. Regarding the 2nd point, the IUC signaling can indicate the resource set type, does not mean that the UE is mandatory to support these two schemes. The TX UE can know which resource type the Rx UE supports, then it can send the correct type. There is no issue here. For example, the SCI 1-A has the “Second UE flag” indication of whether UE scheduling a conflict TB can be UE-B for scheme 2, but does of course does not mandate the UE to support this feature, nor preventing us to introduce the UE FG. Regarding the 3rd point, as commented above the resource determination based on IUC is performed at different layers, the hardware implementations are different for these two schemes. This is the obvious reason why these two schemes should be separated. A UE supporting preferred source only can avoid to have PHY layer impacts (in DSP/FPGA/ASIC). Again, considering that the use cases and chipsets are quite different between these two schemes, further splitting based on preferred/non-preferred are needed. |
| Apple | Agree with vivo. |
| Futurewei | We do not support further splitting of preferred and nonpreferred resource set. For preferred vs non-preferred, many unified designs, e.g., container, content format, were agreed. We do not think it is necessary to split them. As the resource type indication is carried in the request and coordination information and UE can use both resource sets for resource selection. The two types of resource sets should be supported together. We do not think the difference between preferred and nonpreferred set in the agreement warrant the splitting of the two. For scheme 1 coordination Tx, UE-A performs the same sensing process to form the prefer or nonpreferred set. For preferred set and nonpreferred set, as in conditions 1-A-1 and 1-B-1, a RSRP threshold is introduced. So the resource selection processes are the same for the two and slightly different from Rel-16 process. The half-duplex condition is also the same for forming the two resource set. Therefore, we do not think they need to be in separate features. For scheme 1 coordination Rx, although the non-preferred set is processed PHY, it still needs the MAC-CE decoding. Therefore, the change over PHY exclusion process is very minor. None of UE-B behaviors are very difficult to implement and there are many design choices. |
| NTT DOCOMO | We do not think split based on preferred vs non-preferred is unnecessary. Is there any reasonable motivation other than UE implementation perspective? Basically from system perspective, both should be supported. For example, if there are a lot of reservations, UE-A wants to send preferred resources since non-preferred resources lead to quite large payload size. But if UE-B transmitting the explicit request to this UE-A does not support non-preferred type, the UE-A shall transmit preferred type with quite large payload size. This is undesirable situation from system perspective, in our understanding. |
| CATT, GOHIGH | We don’t see the necessary of further splitting between preferred and non-preferred resource set. Even there is some difference for UE processing between preferred and non-preferred resource set, but from system perspective, if a UE is only capable of one of resource set, but other UE cannot know its capability, it is not clear for UE-A to determine which type of resource set will be used. |
| Ericsson | We do not support further splitting of the FG into preferred/non-preferred. In our view, the functionality and behavior for both preferred and non-preferred is quite unified so we do not see the need to create two separate FG. |
| Huawei, HiSilicon | We still see no strong motivation to further splitting. Although there are minor procedural differences on whether to include preferred or exclude non-preferred during resource selection, this does not necessarily lead to different UE capabilities, since we do not see limitations on hardware, e.g. processing time, buffer size, etc. Rather that this is more likely algorithmic differences such as whether it is addition or subtraction operations to process preferred or non-preferred resources on top of the sensing results or S_A. Moreover, for both preferred and non-preferred, MAC CE has to be used to carry resource set indication, i.e. the MAC layer has to be always involved on processing IUC information, regardless of preferred or non-preferred. |
| Moderator | Companies position has not changed. In addition to the UE implementation aspect, DCM and CATT raised system impact if these capabilities are separated. Given more companies prefer not to split Preferred vs non-preferred, following proposal is made for further discussion on the GTW [GTW3] High priority proposal 5-1a: - FGs 32-5a-1/32-5a-2 are not split into FGs for preferred resource set and another FGs for non-preferred resource set |
| FL4 | Following was agreed in the GTW session on Feb 28. Agreement - FG 32-5a-1 is not split into FGs for preferred resource set and another FGs for non-preferred resource set - FG 32-5a-2 is split into FGs for preferred resource set and another FGs for non-preferred resource set - At least one of the split FG 32-5a-2 is added as a prerequisite FG for FG 32-5a-1, FFS which one or both Let’s further discuss the FFS on prerequisite FG [FL4] Low priority proposal 5-1b: - Companies are encouraged to provide view which one or both of the following FGs is/are added as a prerequisite FG for FG 32-5a-1 - FG 32-5a-2: Receiving inter-UE coordination information of preferred resource set in NR sidelink mode 2 - FG 32-5a-3: Receiving inter-UE coordination information of non-preferred resource set in NR sidelink mode 2 |
| Futurewei | Following the scheme 2 Tx/Rx agreement, we think both Scheme 1 Rx features, i.e., FG 32-5a-2 and FG 32-5a-3 should be the prerequisite of Scheme 1 Tx FG 32-5a-1. Further, the scheme 1 coordination Rx FG was split because the coordination receiver of preferred resource set may not support sensing, i.e., not implementing the resource selection procedure in PHY which is required for the reception of non-preferred resource set. Therefore, the FG 32-5a-2 receiving preferred set should be the prerequisite of FG 32-5a-3 receiving non-preferred set. |

[FL1] High priority proposal 5-2:

- Add FG 32-6: Reception of Scheme 1 inter-UE coordination information over 2nd SCI

| 32. NR_SL_enh | 32-6 | Reception of Scheme 1 inter-UE coordination information over 2nd SCI | 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request |  | No | No |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Support: HW, FW, vivo, OPPO, Nokia, DCM, Apple, QC, LGE, E - Separate capabilities for preferred/non-preferred resource set and explicit request: HW, Apple, QC |
| NTT DOCOMO | OK |
| vivo | We are supportive to this proposal. Besides, this FG should be exchanged between UEs. |
| ZTE,Sanechips | Ok or can be put as component to 32-5a-2 |
| Nokia, NSB | Support |
| Qualcomm | We are ok with the direction of the proposal. The wording needs to be updated so that it is clear that the UE is not required to decode SCI 2-C if it does not support this FG. vivo raised a valid point that this FG needs to be reported to other UEs. We note that a similar FG is needed for the request over SCI-2 |
| Apple | Support. If the Tx/Rx of IUC scheme 1 are separate FGs (i.e., Proposal 5-1 is agreed), then it is natural to split 32-6 for IUC information reception and for explicit request reception. |
| OPPO | OK |
| Samsung | OK |
| Huawei, HiSilicon | We support this proposal as we proposed in our paper. |
| Panasonic | Support |
| CATT, GOHIGH | OK |
| Ericsson | Support |
| Spreadtrum | Support |
| Futurewei | We support this proposal based on the agreement. |
| Moderator | Summary of companies view - Support: HW, FW, vivo, OPPO, Nokia, DCM, Apple, QC, LGE, E, ZTE, SS, Pana, CATT, SPRD - Separate capabilities for preferred/non-preferred resource set and explicit request: HW, Apple, QC All companies are fine with the proposal. Regarding FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request, it can be discussed after some progress is made in Proposal 5-1 Based on the comments provided so far, the proposal is updated as follows [GTW1] High priority proposal 5-2: - Add FG 32-6: Reception of Scheme 1 inter-UE coordination information over 2nd SCI 32. NR_SL_enh; 32-6; Reception of Scheme 1 inter-UE coordination information over 2nd SCI; 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request; No; NoYes; UE is not required to decode SCI 2-C; Per FS; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. |
| FL2 | Following was agreed in the GTW session on Feb 22. Agreement - Add FG 32-6: Reception of Scheme 1 inter-UE coordination information over 2nd SCI 32. NR_SL_enh; 32-6; Reception of Scheme 1 inter-UE coordination information over 2nd SCI; 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request; No; NoYes; UE is not required to decode SCI 2-C for inter-UE coordination information; Per FS; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. 1) Regarding the FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request, [FL2] High priority question 5-2a: - Companies are encouraged to provide view whether following capability is added as a new FG or as a component in 32-6 - UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission 2) Regarding other FFS parts which have RAN2 signaling impact, [FL2] Medium priority question 5-2b: - Companies are encouraged to provide views on whether the type of FG 32-6 should be per UE, per band, or per FS [FL2] Medium priority question 5-2c: - Companies are encouraged to provide views on whether the column of “Need for the gNB to know if the feature is supported” for FG 32-6 should be “Yes” or “No” |
| NTT DOCOMO | For question 5-2a, we think separate FG is better since the component in FG32-6 and the discussed capability are related to different FG (FG 32-5a-1 vs FG 32-5a-2). For question 5-2b, it should be aligned with that for FGs 32-5a-1/32-5a-2. For question 5-2c, our feeling is “No”, but we would like to hear other companies’ views. |
| Apple | For question 5-2a, we support to add a new FG for receiving explicit request via 2nd SCI. Since we already agreed of FG 32-5a-1 for transmitting IUC and receiving explicit request and FG 32-5a-2 for receiving IUC and transmitting explicit request. The pre-requisite of FG 32-6 is FG 32-5a-2. The pre-requisite of the new FG for receiving explicit request via 2nd SCI is FG 32-5a-1. Hence, we do not think receiving explicit request via 2nd SCI is a component of 32-6, since otherwise, UE supporting 32-6 has to support both FG 32-5a-1 and FG 32-5a-2. For question 5-2b, we think FG 32-6 has the same granularity as FG 32-5a-2. For question 5-2c, we think gNB may need to know since it is related to resource selection reliability, which may affect gNB’s resource pool configuration. |
| CATT, GOHIGH | For 5-2a, we prefer to make it as a component, since both is for 2nd SCI format 2C reception, no need to further split it. And the name of FG32-6 can be change to “Reception of Scheme 1 inter-UE coordination information or request information over 2nd SCI”. |
| OPPO | 5-2a : Given transmitting/receiving IUC information have been split into 2 FGs, the capability for explicit request should be added, as it is for the UE transmitting the IUC information. 5-2b: per band. 5-2c : slightly prefer “yes”. |
| ZTE,Sanechips | 5-2a: Similar view as OPPO and CATT, can be added as component 5-2b: per UE. We think this capability should be UE wise to be aligned with agreed WA on indicating the capability via SCI 1-A - Working assumption: - For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not. - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication 5-2c: yes |
| vivo | 5-2a: We are OK to have separate FGs. 5-2b: Per band. We may also be OK to per FS. 5-2c: Yes |
| Qualcomm | 5-2a: In our view, receiving the explicit request over SCI-2 is unrelated to receiving inter-UE coordination information over SCI-2 and the two should be separate FGs. These operations also correspond to different UEs, similar to how 32-5a was split. 5-2b: per FS since it impacts baseband processing. 5-2c: There’s no need for the gNB to know support for this feature. We don’t see how the gNB would utilize this information. |
| Futurewei | 5-2a: We prefer to include the capability of receiving explicit request via 2nd SCI+MAC-CE as the component due to the same SCI-2C reception. But we are also ok to define it as a new separated FG, since Scheme 1 Tx/Rx capabilities are separated. 5-2b: per band 5-2c: We prefer Yes. |
| Huawei, HiSilicon | 5-2a: We prefer to have separate FGs since the prerequisite feature groups are different. 5-2b: Per band is sufficient, but we are open to per FS if necessary. 5-2c: Yes. It can give gNB a certain amount of information to use, if it wants to, in resource dimensioning due to IUC. |
| Nokia, NSB | 5-2b: Per band 5-2c: Slight preference to yes |
| Ericsson | 5-2a: We think that is reasonable to have separate FGs. 5-2b: Per band 5-2c: No |
| Moderator | High priority question 5-2a: - as a new FG: DCM, Apple, OPPO, vivo, QC, [FW], HW/HiSi, E/// - as a component in 32-6: CATT, ZTE, FW Given majority companies support it as a new FG, following proposal is made together with question 5-2b and question 5-2c: [GTW2] High priority proposal 5-2a: - FG 32-6 is split to two FGs as follows 32. NR_SL_enh; 32-6-1; Reception of Scheme 1 inter-UE coordination information over 2nd SCI; 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request; 32-5a-2; No Yes; Yes; UE is not required to decode SCI 2-C for inter-UE coordination information; Per FS Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. 32. NR_SL_enh; 32-6-2; Reception of Scheme 1 explicit request over 2nd SCI; 1) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission; 32-5a-1; Yes; Yes; UE is not required to decode SCI 2-C for explicit request; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. question 5-2b - Same type as FGs 32-5a-1/32-5a-2: DCM - Same type as FG 32-5a-2: Apple - Per band: OPPO, vivo, FW, HW/HiSi, Nokia/NSB, E/// - Per UE: ZTE, - Per FS: vivo, QC, [HW/HiSi] Majority companies support per band. The above proposal is made question 5-2c: - Yes: DCM, Apple, OPPO, ZTE, FW, HW/HiSi, Nokia/NSB - No: QC, E/// Majority companies support ”Yes”. The above proposal is made |
| FL3 | This proposal could not be discussed in the GTW session on Feb 24. No further input is necessary unless you have concern on this proposal [FL3] High priority proposal 5-2a: - FG 32-6 is split to two FGs as follows 32. NR_SL_enh; 32-6-1; Reception of Scheme 1 inter-UE coordination information over 2nd SCI; 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request; 32-5a-2; No Yes; Yes; UE is not required to decode SCI 2-C for inter-UE coordination information; Per FS Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. 32. NR_SL_enh; 32-6-2; Reception of Scheme 1 explicit request over 2nd SCI; 1) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission; 32-5a-1; Yes; Yes; UE is not required to decode SCI 2-C for explicit request; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. |
| Apple | Support. |
| CATT, GOHIGH | We still think it is unnecessary to further split the reception capability of 2nd SCI between request and IUC message, since they use same 2nd SCI format. There is no difference for UE hardware of processing between them. |
| Moderator | Only one company showed their concern on splitting FG 32-6 into one for IUC information and another for explicit request. Further discuss in the GTW [GTW3] High priority proposal 5-2a: - FG 32-6 is split to two FGs as follows 32. NR_SL_enh; 32-6-1; Reception of Scheme 1 inter-UE coordination information over 2nd SCI; 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request; 32-5a-2; No Yes; Yes; UE is not required to decode SCI 2-C for inter-UE coordination information; Per FS Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. 32. NR_SL_enh; 32-6-2; Reception of Scheme 1 explicit request over 2nd SCI; 1) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission; 32-5a-1; Yes; Yes; UE is not required to decode SCI 2-C for explicit request; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. |
| FL4 | This proposal could not be discussed in the GTW on Feb 28. Let’s further discuss directly over the reflector. |
| Moderator | Following was agreed via email endorsement Agreement - FG 32-6 is split to two FGs as follows 32. NR_SL_enh; 32-6-1; Reception of Scheme 1 inter-UE coordination information over 2nd SCI; 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request; FFS; No Yes; Yes; UE is not required to decode SCI 2-C and the associated PSSCH for inter-UE coordination information; Per FS [Per band]; [N.A].; [N.A].; [N.A.]; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. 32. NR_SL_enh; 32-6-2; Reception of Scheme 1 explicit request over 2nd SCI; 1) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission; FFS; Yes; Yes; UE is not required to decode SCI 2-C and the associated PSSCH for explicit request; [Per band]; [N.A.]; [N.A.]; [N.A.]; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. |

[FL1] High priority proposal 5-3:

- Add FG 32-7: Determination of expected conflict in Scheme 2 based on RSRP difference

| 32. NR_SL_enh | 32-7 | Determination of expected conflict in Scheme 2 based on RSRP difference | 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations | 32-5b-1 | No | No |  | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Support: FW, vivo, CATT, DCM, Apple, QC, LGE |
| NTT DOCOMO | OK |
| vivo | We are supportive to this proposal. Besides, this FG should be exchanged between UEs. |
| ZTE,Sanechips | Ok |
| Qualcomm | We support the proposal. |
| Apple | Support |
| OPPO | OK |
| Samsung | OK |
| Huawei, HiSilicon | OK |
| Panasonic | OK |
| CATT, GOHIGH | Support |
| Ericsson | Support |
| Spreadtrum | Support |
| Futurewei | We support this proposal based on the agreement. |
| Moderator | Summary of companies view - Support: FW, vivo, CATT, DCM, Apple, QC, LGE, ZTE,OPPO, SS, HW, Pana, E/// All companies are fine with the proposal. Based on the comments provided so far, the proposal is updated as follows [GTW1] High priority proposal 5-3: - Add FG 32-7: Determination of expected conflict in Scheme 2 based on RSRP difference 32. NR_SL_enh; 32-7; Determination of expected conflict in Scheme 2 based on RSRP difference; 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations; 32-5b-1; No; NoYes; Per FS; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. |
| FL2 | This proposal could not be discussed in the GTW session on Feb 22. Assuming all companies are fine with the proposal, in this round remaining FFSs are discussed. Also, if you have concern on adding FG 32-7, please express it asap. [FL2] Medium priority question 5-3a: - Companies are encouraged to provide views on whether the type of FG 32-7 should be per UE, per band, or per FS [FL2] Medium priority question 5-3b: - Companies are encouraged to provide views on whether the column of “Need for the gNB to know if the feature is supported” for FG 32-7 should be “Yes” or “No” |
| NTT DOCOMO | For question 5-3a, it should be aligned with that for FGs 32-5a-1/32-5a-2. For question 5-3b, our feeling is “No”, but we would like to hear other companies’ views. |
| Apple | For question 5-3a, FG 32-7 has the same signaling granularity as FG 32-5a-1. For question 5-3b, we think gNB may need to know since it is related to resource selection reliability, which may affect gNB’s resource pool configuration. |
| OPPO | 5-3a: per band. 5-3b: slightly prefer “yes”. |
| ZTE,Sanechips | 5-3a: Same signalling granularity as 32-5a-1, thus per UE is preferred. 5-3b:yes |
| vivo | 5-3a: Per band. We may also be OK to per FS. 5-3b: Yes |
| Qualcomm | We support introducing FG 32-7 5-3a: we prefer per FS. 5-3b: No, we don’t see how the gNB would utilize this information. |
| Futurewei | 5-3a: per band 5-3b: prefer Yes |
| Huawei, HiSilicon | We are OK to introduce FG 32-7 5-3a Per band is sufficient, but we are open to per FS if necessary. 5-3b: Yes. |
| Nokia, NSB | 5-3a: per band 5-3b: slight preference to yes |
| Ericsson | 5-3a: Per band 5-3b: No |
| Moderator | No companies showed concern on proposal 5-3 question 5-3a - Same as 32-5a-1/32-5a-2: DCM - Same as FG 32-5a-1: Apple, ZTE - Per band: OPPO, vivo, FW, HW/HiSi, Nokia/NSB, E/// - Per FS: QC, [HW/HiSi] Majority companies prefer per band question 5-3b - Yes: Apple, OPPO, vivo, FW, HW/HiSi, Nokia/NSB - No: DCM, QC, E/// Majority companies prefer Yes. Following proposal is made [GTW2] High priority proposal 5-3: - Add FG 32-7: Determination of expected conflict in Scheme 2 based on RSRP difference 32. NR_SL_enh; 32-7; Determination of expected conflict in Scheme 2 based on RSRP difference; 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations; 32-5b-1; Yes; Yes; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. |
| FL3 | Following was agreed in the GTW session on Feb 24 Agreement - Add FG 32-7: Determination of expected conflict in Scheme 2 based on RSRP difference 32. NR_SL_enh; 32-7; Determination of expected conflict in Scheme 2 based on RSRP difference; 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations; 32-5b-1; Yes; Yes; Per band; N.A.; N.A.; N.A.; Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1; Optional with capability signalling. 1) Need for the gNB to know if the feature is supported In the GTW session, some companies were not OK with “Yes” due to the lack of motivation. Companies are encouraged to provide the view why this FG is (not) necessary to be reported to gNB 2) Type Companies had different view in the GTW session. Companies are encouraged to provide the view why your supporting option is enough/necessary |
| Apple | 1). We still think it is better gNB knows this feature. If more UEs supporting this feature, this implies the resource collision chance is smaller. This can affect gNB’s resource pool configuration. 2). We prefer per band but are open for per FS. |
| Futurewei | - We prefer Yes. SIR(two RSRP measurement) based conflict detection is a new behavior. If would be better that gNB knows such capability of some UEs to oversee the UE’s with different capability for appropriate pool configuration and RA. - We think per band would be enough. |
| NTT DOCOMO | - Even when “YES”, how to use the reported information at gNB is unclear for us. But we are fine with “YES” if majority want. - Should be aligned with 32-5b-1. |
| CATT, GOHIGH | - Yes, this will impact the resource pool configuration from gNB. |
| Ericsson | We do not know what is the use for the gNB to have this feature reported and what will be the advantage. In our view, this is something the UE can support without further indication to the gNB. |
| Huawei, HiSilicon | - We still think “Yes” for the gNB to know, for inter-UE coordination, scheme 2 can be enabled or disabled or controlled by (pre-)configuration, and therefore gNB should be informed with UE capability. - With reference to the granularity of sidelink features in Rel-16 and considering that Rel-17 sidelink features are not RF-related, per band is sufficient. Open to per FS if necessary. |
| Moderator | 1) Need for the gNB to know if the feature is supported - Yes: Apple, OPPO, vivo, FW, HW/HiSi, Nokia/NSB, [DCM], CATT - No: QC, E/// 2) Type - Same as 32-5b-1: DCM - Same as FG 32-5a-1: ZTE - Per band: OPPO, vivo, FW, HW/HiSi, Nokia/NSB, E///, Apple - Per FS: QC, [HW/HiSi], [Apple] Following proposal is discussed after the type for FG 32-5b-1 is decided. [GTW3] Medium priority proposal 5-3a: - Type of FG 32-7 is per band [GTW3] Medium priority proposal 5-3b: - The column of “Need for the gNB to know if the feature is supported” for FG 32-7 is “Yes” |
| FL4 | These proposals could not be discussed in the GTW on Feb 28. Assuming proposal 5-5 is agreeable, the same proposal is set. No further input is necessary unless you have concern on the proposal [FL4] Medium priority proposal 5-3a: - Type of FG 32-7 is per band Also, some companies showed their flexibility to live with “Yes” for proposal 5-3b. No further input is necessary unless you have concern on the proposal [FL4] Medium priority proposal 5-3b: - The column of “Need for the gNB to know if the feature is supported” for FG 32-7 is “Yes” |
| Moderator | Let’s further discuss directly over the reflector. |
| Moderator | Following was agreed via email endorsement Agreement - The column of “Need for the gNB to know if the feature is supported” for FG 32-7 is “No” |

[FL1] Medium priority proposal 5-4:

- PSFCH Tx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-1

  - Value range for M is {4, 8, 16}

  - Add one of the following notes in FG 32-5b-1

    - Option 1: If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number. UE is not required to support the sum of each reported value M

    - Option 2: If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in FG 32-5b-1 is the total number, which is equal or greater than the value M reported in FG 15-11

- PSFCH Rx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-2

  - Value range for N is {5, 15, 25, 32, 35, 45, 50, 64}

  - Add one of the following notes in FG 32-5b-2

    - Option 1: If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number. UE is not required to support the sum of each reported value N

    - Option 2: If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in FG 32-5b-2 is the total number, which is equal or greater than the value N reported in FG 15-11

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view on whether/how to report PSFCH separately or jointly for inter-UE coordination and HARQ-ACK reporting - Separate: FW - Joint: HW, FW, vivo, CATT, DCM, LGE - Option 1: HW, CATT, DCM, QC - Option 2: FW, LGE - FW: Since a non-destination of a TB from UE-B can be UE-A, UE-A needs to transmit more PSFCHs than a Rel-16 UE for HARQ-ACK purpose only. Also, since UE-B needs to receive a PSFCH for the conflict report without transmitting a PSSCH, UE-B needs to receive more PSFCHs than a Rel-16 UE for HARQ-ACK only. Therefore, the number of simultaneous PSFCH receptions and/or transmissions shall be increased. |
| NTT DOCOMO | Option 1 for both M and N. If a large value can be reported in FG32-5b-1, the UE should report the same large value also in FG 15-11. There is no reason to report different values from actual UE implementation perspective. |
| vivo | We are supportive to this proposal. |
| ZTE,Sanechips | Option 1 for both. |
| Qualcomm | We support Option 1 in both cases. FG 32-2 need to be also included in the Rx capability part since the UE also reports an N value there. |
| Apple | Fine with Option 1 for both M and N. |
| OPPO | Similar view as DCM, PSFCH tx/rx capability reported for HARQ-ACK and inter-UE should be the same, and the reported values should be viewed as the total number (i.e. Optoin1). |
| Samsung | O.K for the direction. Prefer Option 1 for both cases. |
| Huawei, HiSilicon | Ok with the proposal and we support Option 1 for both Tx capability and Rx capability, and the value of M/N shall be the same in both FG reports (otherwise the UE seems to have an error). |
| Panasonic | We think option 1 for both. |
| CATT, GOHIGH | Option 1 for both. |
| Ericsson | Support the joint report. For both N and M, we support Option 1. |
| Spreadtrum | We support option 1 for both M and N. |
| Futurewei | We prefer separate report particularly when most conflict reports are on different PRBs (all if different PRBs are agreed) which is different from PSFCH HARQ-ARQ. For joint report, we support option 2 for both cases. With joint report and option 1, we are enhancing HARQ-ACK capability on configured PRB for PSFCH HARQ-ARQ now, which is not in our work item. |
| FL2 | Summary of companies view on whether/how to report PSFCH separately or jointly for inter-UE coordination and HARQ-ACK reporting - Separate: FW - Joint: HW, FW, vivo, CATT, DCM, LGE - Option 1: HW, CATT, DCM, QC, ZTE, Apple, OPPO, SS, Pana, E///, SPRD - Option 2: FW, LGE - FW: Since a non-destination of a TB from UE-B can be UE-A, UE-A needs to transmit more PSFCHs than a Rel-16 UE for HARQ-ACK purpose only. Also, since UE-B needs to receive a PSFCH for the conflict report without transmitting a PSSCH, UE-B needs to receive more PSFCHs than a Rel-16 UE for HARQ-ACK only. Therefore, the number of simultaneous PSFCH receptions and/or transmissions shall be increased. Given most companies prefer Option 1, the proposal is updated as follows [FL2] Medium priority proposal 5-4: - PSFCH Tx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-1 - Value range for M is {4, 8, 16} - Add following note in FG 32-5b-1 - If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number. UE is not required to support the sum of each reported value M - PSFCH Rx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-2 - Value range for N is {5, 15, 25, 32, 35, 45, 50, 64} - Add following note in FG 32-5b-2 - If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number. UE is not required to support the sum of each reported value N |
| NTT DOCOMO | Support. |
| Apple | Support |
| CATT, GOHIGH | Support |
| OPPO | Support |
| ZTE,Sanchips | Can we delete UE is not required to support the sum of each reported value N to avoid the confusion that N should be reported for each FG in case of support of more than one FGs is done. |
| vivo | OK |
| Qualcomm | Support |
| Futurewei | Since majority prefer the same M and N, we can accept the proposal provided that no new values of M or N are introduced. |
| Huawei, HiSilicon | We are Ok with the proposal. But we would like to confirm it is common understanding that if UE reports more than one of FGs 15-11 32-5b-1 and 32-5b-2, the reported values of M/N are the same. |
| Ericsson | Support |
| Moderator | Most companies are fine with the proposal in general. The proposal is updated to clarify that the reported values of M/N in FGs 15-11 32-5b-1 and 32-5b-2 are the same [GTW2] Medium priority proposal 5-4: - PSFCH Tx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-1 - Value range for M is {4, 8, 16} - Add following note in FG 32-5b-1 - If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number and the same among those FGs. UE is not required to support the sum of each reported value M - PSFCH Rx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-2 - Value range for N is {5, 15, 25, 32, 35, 45, 50, 64} - Add following note in FG 32-5b-2 - If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs. UE is not required to support the sum of each reported value N |
| FL3 | This proposal could not be discussed in the GTW session. No further input is necessary unless you have concern on this proposal [FL3] Medium priority proposal 5-4: - PSFCH Tx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-1 - Value range for M is {4, 8, 16} - Add following note in FG 32-5b-1 - If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number and the same among those FGs. UE is not required to support the sum of each reported value M - PSFCH Rx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-2 - Value range for N is {5, 15, 25, 32, 35, 45, 50, 64} - Add following note in FG 32-5b-2 - If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs. UE is not required to support the sum of each reported value N |
| Moderator | Following proposal is stable for more than 24 hours. We can quickly agree on this proposal either via email endorsement [email1] Medium priority proposal 5-4: - PSFCH Tx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-1 - Value range for M is {4, 8, 16} - Add following note in FG 32-5b-1 - If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number and the same among those FGs. UE is not required to support the sum of each reported value M - PSFCH Rx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-2 - Value range for N is {5, 15, 25, 32, 35, 45, 50, 64} - Add following note in FG 32-5b-2 - If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs. UE is not required to support the sum of each reported value N |
| Moderator | Following was agreed via email endorsement on Mar 1. Agreement - PSFCH Tx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-1 - Value range for M is {4, 8, 16} - Add following note in FG 32-5b-1 - If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number and the same among those FGs. - PSFCH Rx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-2 - Value range for N is {5, 15, 25, 32, 35, 45, 50, 64} - Add following note in FG 32-5b-2 - If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs. |

[FL1] Medium priority question 5-5:

- Companies are encouraged to provide views on whether the type of FGs 32-5a, 32-5b-1 and 32-5b-2 should be per UE, per band, or per FS

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Per band E/// - Per FS: QC - Per UE: ZTE |
| NTT DOCOMO | Slightly prefer per band. |
| vivo | We think per band would be enough, but we are open to others. |
| ZTE,Sanechips | Per UE given not all UEs within the same band shall support such capability |
| Nokia, NSB | Per band |
| Qualcomm | Per FS. Like in Q 4-4, these FGs Per FS require baseband processing and the UE might not be able to support them in the reported band(s) depending on what other features are also supported. |
| Apple | We think per UE is too rough. We are open between per band and per FS. |
| OPPO | Per band |
| Samsung | Prefer per band. |
| Huawei, HiSilicon | Like in Q 4-4, we support Per band but can also consider per FS. |
| Panasonic | Per band |
| Ericsson | Per band |
| Spreadtrum | Per band |
| Futurewei | We prefer per band. |
| Moderator | Summary of companies view - Per band E///, DCM, vivo, Nokia, [Apple], OPPO, SS, HW, Pana, E///, SPRD, FW - Per FS: QC, [Apple], [HW] - these FGs Per FS require baseband processing and the UE might not be able to support them in the reported band(s) depending on what other features are also supported. - Per UE: ZTE - not all UEs within the same band shall support such capability Given majority companies prefer to support as per band, following proposal is made assuming the same applies to the split FGs 32-5a-1/32-5a-2 [FL2] Medium priority proposal 5-5: - Type of FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are per band |
| NTT DOCOMO | Support. |
| Apple | Support. |
| OPPO | Support |
| ZTE,Sanechips | For the UEs operating in the same band, are they supposed to either support or not support any of the IUC scheme? If yes, then it should be per band. If not, per UE is more appropriate. |
| vivo | OK |
| Qualcomm | These features impact baseband processing requirements and are well suited for per FS reporting. |
| Futurewei | We are ok with the proposal. |
| Huawei, HiSilicon | We are Ok with the proposal. But we are open to consider per FS reporting. |
| Ericsson | Support |
| Moderator | Companies position has not changed. The same proposal is set for further discussion [GTW2] Medium priority proposal 5-5: - Type of FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are per band |
| FL3 | This proposal could not be discussed in the GTW session on Feb 24. Companies are encouraged to provide the view why your supporting option is enough/necessary taking the comments from companies into account. |
| Moderator | No further concern was received for the following proposal for more than 24 hours. We can quickly agree on this proposal via email endorsement [email1] Medium priority proposal 5-5: - Type of FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are per band |
| Moderator | This proposal could not be agreed via email endorsement over RAN1 reflector. Let’s further discuss directly over the reflector |

[FL1] Medium priority question 5-6:

- Companies are encouraged to provide views on whether the column of “Need for the gNB to know if the feature is supported” for FGs 32-5a, 32-5b-1 and 32-5b-2 should be “Yes” or “No”

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Yes: HW, DCM, E/// - No: |
| NTT DOCOMO | Yes |
| vivo | Yes |
| ZTE,Sanehcips | Yes |
| Nokia, NSB | Yes |
| Qualcomm | No It isn’t clear why the gNB needs this information and it would utilize it. |
| Apple | Yes |
| OPPO | yes |
| Samsung | Yes |
| Huawei, HiSilicon | Yes, to give gNB a certain amount of information to use, if it wants to, in resource dimensioning due to IUC. |
| Panasonic | Yes |
| Ericsson | Yes |
| Spreadtrum | Yes |
| Futurewei | Yes |
| Moderator | Summary of companies view - Yes: HW, DCM, E///, vivo, ZTE, Nokia, Apple, OPPO, SS, Pana, SPRD, FW - to give gNB a certain amount of information to use, if it wants to, in resource dimensioning due to IUC - No: QC - It isn’t clear why the gNB needs this information and it would utilize it Given majority companies prefer “Yes”, following proposal is made assuming the same applies to the split FGs 32-5a-1/32-5a-2 [FL2] Medium priority proposal 5-6: - The column of “Need for the gNB to know if the feature is supported” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes” |
| NTT DOCOMO | Support |
| Apple | Support |
| OPPO | Support |
| ZTE,Sanechips | Support |
| vivo | OK |
| Qualcomm | We’re still not clear how the gNB would utilize this information. However, we can go with the majority view in this case. |
| Futurewei | OK |
| Huawei, HiSilicon | Support |
| Nokia, NSB | Support |
| Ericsson | Support |
| Moderator | QC showed their flexibility to accept the proposal. The same proposal is set [GTW2] Medium priority proposal 5-6: - The column of “Need for the gNB to know if the feature is supported” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes” |
| FL3 | This proposal could not be discussed in the GTW session. No further input is necessary unless you have concern on this proposal [FL3] Medium priority proposal 5-6: - The column of “Need for the gNB to know if the feature is supported” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes” |
| Moderator | Following proposal is stable for more than 24 hours. We can quickly agree on this proposal via email endorsement [email1] Medium priority proposal 5-6: - The column of “Need for the gNB to know if the feature is supported” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes” |
| Moderator | Following was agreed via email endorsement on Mar 1. Agreement - The column of “Need for the gNB to know if the feature is supported” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes” |

[FL1] Medium priority question 5-7:

- Companies are encouraged to provide views on whether the column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-5a, 32-5b-1 and 32-5b-2 should be “Yes” or “No”

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Yes: DCM, E/// - No: |
| NTT DOCOMO | Yes |
| vivo | Yes |
| Nokia, NSB | Yes |
| Qualcomm | Yes |
| Apple | Yes |
| OPPO | Yes |
| Samsung | Yes |
| Huawei, HiSilicon | Yes |
| Panasonic | Yes |
| CATT, GOHIGH | Yes |
| Ericsson | Yes |
| Spreadtrum | Yes |
| Futurewei | Yes. One question, for 32-5b-2, we already have a working assumption to use 1 LSB of reserved bits of a SCI format 1-A to indicate of whether UE scheduling a conflict TB can be UE-B or not. Would that be sufficient for signaling exchange? |
| Moderator | Given all companies prefer “Yes”, following proposal is made assuming the same applies to the split FGs 32-5a-1/32-5a-2 [FL2] Medium priority proposal 5-7: - The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes” Also, companies are invited to answer the question from Futurewei as above. |
| NTT DOCOMO | Support at least for FGs 32-5a-1/32-5a-2. For 32-5b-1/32-5b-2 and FW’s question, we feel that still RRC signaling between UEs are beneficial e.g. UE-B can decide whether PSFCH for IUC-2 should be received or not, based on the signaling. But we would like to hear other companies’ views. |
| Apple | Support for FGs 32-5a-1/32-5a-2/32-5b-1/32-5b-2. Regarding FW’s question, we think the working assumption is based on resource pool configuration. If a resource pool is configured not to support using 1 LSB of reserved bit of SCI 1-A to indicate whether a UE can be UE-B or not, then RRC signaling between UEs will help. |
| OPPO | We are fine with proposal 5-7. As to Futurewei’s question, in our view SCI format 1-A is sufficient, there is no need to exchange 32-5b-2 via PC5-RRC. |
| ZTE,Sanechips | Yes |
| vivo | OK to the proposal. |
| Qualcomm | We support the proposal. |
| Futurewei | We appreciate companies’ inputs to our question. So as DCM suggested, UE-B can decide whether to receive IUC-2 dynamically with the 1-bit indication, but the reason for UE-B to do that is not very clear if RRC signaling is there (one possible case is that UE-B does not want to resource selection for some others as UE-A may be Rx of another UE). Regarding APPLE’s example, it seems that signaling of 1 LSB is unnecessary. We are generally fine with the proposal. Would like to hear more views. If no clear benefit, as OPPO suggested, SCI 1-A is sufficient for 32-5b-2. . |
| Huawei, HiSilicon | We support this proposal. |
| Nokia, NSB | Support FL2 5-7 |
| Ericsson | Support |
| Moderator | At least the proposal is stable for FGs 32-5a-1, 32-5a-2, 32-5b-1. For FG 32-5b-2, more discussion is necessary, as requested by FW. [GTW2] Medium priority proposal 5-7: - The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-5a-1, 32-5a-2, and 32-5b-1 and 32-5b-2 are “Yes” |
| FL3 | This proposal could not be discussed in the GTW session on Feb 24. For FG 32-5b-2, companies are encouraged to provide the view why your supporting option is enough/necessary taking the comments from companies into account. |
| Apple | If a resource pool configuration does not support SCI 1-A indication whether a UE can be UE-B, then the capability signaling exchange between UEs are useful. On the other hand, we do not think the signaling of 1 LSB in SCI 1-A’s reserved bits is unnecessary. This SCI indication is still useful for a UE-A, which does not have PC5-RRC connection with UE-B and does not know UE-B’s capability. |
| Futurewei | For FG 32-5b-2, we are ok with signaling. We would view that the RRC is for signaling the capacity, the 1-bit SCI-1A as a request, although we do not see much benefit that UE does not request in the pool that the 1-bit is enabled. |
| NTT DOCOMO | We are fine with either “Yes” or “No” for 32-5b-2. SCI 1A can includes 1 bit for the purpose, so maybe “No” is OK. |
| Huawei, HiSilicon | Considering that 1 LSB of reserved bits of a SCI format 1-A can be disabled, capability signaling exchange between UEs for FGs 32-5b-2 is still necessary. Working assumption - For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not. - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication |
| Moderator | FW is now fine with Yes for 32-5b-2. Following proposal is made, which can be agreed over the RAN1 reflector. [email1] Medium priority proposal 5-7: - The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes” |
| Moderator | Following was agreed via email endorsement on Mar 1. Agreement - The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes” |

Low priority question 5-8:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 32-5a, 32-5b-1 and 32-5b-2 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |

- 4-1 to 4-5 for LTE

In [2], FGs 4-1 to 4-5 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4. [NR_SL_enh] | 4-1 | [Receiving NR sidelink of PSCCH/PSSCHPSFCH/S-SSB] | 1) UE can receive NR PSCCH/PSSCH/PSFCH/S-SSB. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-2 | [Receiving NR sidelink of PSFCH/S-SSB only] | 1) UE can receive NR PSFCH/S-SSB only. | None | [Yes] | [No] |  | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-3 | Transmitting NR sidelink mode 2 with full sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with full sensing configured by LTE Uu. 2) UE supports the sensing and resource allocation operation as specified in Rel-16. | [4-1] | [Yes] | [No] | [UE can perfom random resource selection only] | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-4 | Transmitting NR sidelink mode 2 with partial sensing | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by LTE Uu. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. | [4-1], [4-3] | [Yes] | [No] | UE does not support trasmissoin according to the partial sensing and resource allocation | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |
|  | 4-5 | Inter-UE coordination in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive inter-UE coordination information of presence of expected/potential resource conflict and use the received information in its own resource re-selection in NR sidelink mode 2. 3) UE can transmit and received an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. | [4-1] | [Yes] | [Yes] | UE does not support inter-UE coordination in NR sidelink mode 2. | [Per band] | N.A. | N.A. |  | Optional with capability signalling. FFS: For UE supports LTE Uu configuring NR sidelink, UE must indicate this FG is supported. |

Following feedbacks are provided in contributions for the RAN1#108-e meeting.

| [1] | Huawei, HiSilicon | Cross-RAT configuration is supported in Rel-16 to allow an eNB to configure related parameters for NR sidelink to operate in LTE cell. A set of UE features were defined to allow reporting of whether UE supports such configurations. The purpose of cross-RAT configuration is to allow deployment of NR sidelink features while LTE networks are still present, to help smooth evolution between the 3GPP technologies without imposing tight constraints on operators, and to avoid losses of service, or inconsistent service provision, when a device moves between NR coverage and LTE coverage. If RAN1 does not define LTE UE features for support of Rel-17 NR sidelink enhancement, it is unlikely that RAN2 will define the cross-RAT RRC configurations to support the use of NR sidelink in LTE. Then, for in-coverage operation, a UE entering such a cell must abruptly terminate IUC operation, and, if it does not support full sensing, i.e. it only supports partial sensing or Rel-17 random selection, the UE may not be permitted to operate at all in the LTE cell. It is therefore desirable to define the LTE UE features by taking the NR feature list as the baseline, and this will trigger RAN2 to define the RRC signaling in support of it. Proposal 13: Define LTE Rel-17 UE feature list for NR sidelink enhancement based on NR Rel-17 feature list. |
| --- | --- | --- |
| [3] | vivo | The Rel-17 NR sidelink enhancement only targets NR specific enhancements. There is no impact to LTE sidelink, nor any LTE/NR sidelink interaction. Consequently, there should not be any impact to a Rel-17 UE supports LTE sidelink only. For a Rel-17 UE supports both LTE sidelink and enhanced NR sidelink, it should report the Rel-17 related UE features in NR sidelink only. Therefore, in our view the Rel-17 NR sidelink enhancement should have no impact to LTE UE feature. Observation 1: The Rel-17 NR sidelink enhancement WI has no impact to LTE UE feature. |
| [15] | LG Electronics | Further discussion is also necessary on whether to define LTE Rel-17 UE feature list for NR SL enhancement. We think that if this kind of feature is not introduced, the usability of Rel-17 NR SL enhancement mechanism will be lower compared to the case of Rel-16 NR V2X, which is not a desirable direction we should pursue. In this sense, our preference is to define LTE Rel-17 UE feature list for SL enhancement. Proposal 10: Define LTE Rel-17 UE feature list for NR SL enhancement (i.e., FG 4-1/4-2/4-3/4-4 in [2]). [2] R1-2112900, “Updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e,” Moderators (AT&T, NTT DOCOMO, INC.). |

Discussion

[FL1] High priority question 6-1:

- Companies are encouraged to provide views on whether to define Rel-17 LTE UE feature list for NR sidelink enhancement based on NR Rel-17 FGs

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Support: HW/HiSi, LGE - Not support: vivo |
| NTT DOCOMO | Our understanding is that LTE UE feature list for Rel-17 NR SL enh is necessary as in Rel-16 SL. |
| vivo | We don’t think it is needed to define (copy) these FGs for LTE – they are NR specific features. In the case of cross-RAT deployment, eNB can anyway acquire the NR features of the UE, via requiring the NR UE capability reporting. |
| Qualcomm | We don’t think it is necessary to include the FGs in the LTE feature list. |
| Apple | We think defining Rel-17 LTE UE feature list for NR sidelink enhancement may be needed to support the cross-RAT configuration. |
| OPPO | The FGs should be defined such that Rel-17 features can be used based on configurations of LTE network. |
| Huawei, HiSilicon | We suggest to define R17 LTE UE feature list to ensure smooth switch and consistent service when a UE moves between NR coverage and LTE coverage. |
| Ericsson | No. There set of features defined in NR SL Rel-17 do not have any impact on the LTE SL UE features. |
| Futurewei | Defining Rel-17 LTE UE feature for NR SL enhancement seems reasonable. We are ok with it. |
| Moderator | Summary of companies view - Support: HW/HiSi, LGE, DCM, Apple, OPPO, FW - Not support: vivo, QC, E/// Given more companies prefer to define LTE FG, following proposal is made [GTW1] High priority proposal 6-1: - Define Rel-17 LTE UE feature list for NR sidelink enhancement based on NR Rel-17 FGs |
| FL2 | This proposal could not be discussed in the GTW session on Feb 22. Companies not supporting the proposal are encouraged to check whether you can live with the proposal taking companies view into account. [FL2] High priority proposal 6-1: - Define Rel-17 LTE UE feature list for NR sidelink enhancement based on NR Rel-17 FGs |
| Apple | Support |
| vivo | We don’t support this proposal. The only reason provided is for eNB to acquire the R17 features for cross-RAT configuration. As we commented above, although this use case is valid, it can already be supported by eNB acquire the NR features of the UE, via requiring the NR UE capability reporting. This is a legacy mechanism supported in RAN2. No need to duplicate these NR FGs to LTE. |
| Futurewei | Support. |
| Huawei, HiSilicon | We support this proposal. Regarding vivo’s reply, NR features means sl-ParamterNR defined in TS 36.331, which directly refers to SidelinkParameterNR in TS38.331, it is the way RAN2 capture LTE UE feature list for NR sidelink, but they may not update the signaling content unless RAN1 agree such cross-RAT feature list should be supported, i.e. the signaling container would not be given the Rel-17 content. |
| Ericsson | In our view, the set of FG defined in NR SL Rel-17 should have no impact on the LTE SL features. |
| Moderator | Companies position has not changed. Further discussion is necessary. [GTW2] High priority proposal 6-1: - Define Rel-17 LTE UE feature list for NR sidelink enhancement based on NR Rel-17 FGs |
| FL3 | This proposal was discussed in the GTW session on Feb 24 but no consensus was achieved. No further input is necessary unless you have concern on this proposal. As discussed in the GTW session, companies are encouraged to check whether this proposal is necessary to support Rel-17 NR SL in LTE coverage etc. |
| vivo | After further checking with RAN2 colleagues, RAN2 is current discussing the similar issue (for the DRX related FGs). And the RAN1 LTE UE features for sidelink is only meaningful for RAN2 on whether cross-RAT configuration is supported, but not directly mean that UE capability signallings (e.g., sl-ParameterNR, or any additional container) are to be changed/added. In this sense, we are OK to this proposal. |
| Futurewei | Support |
| NTT DOCOMO | Support. RAN2 can decide whether the existing parameter sl-ParameterNR can cover Rel-17 features from RAN1 or not, i.e. up to RAN2 decision. What RAN1 should do is to send FGs for LTE-Uu. |
| Huawei, HiSilicon | We support this proposal. Define Rel-17 LTE UE feature list for NR sidelink enhancement means cross-RAT configuration is supported, it is depended on RAN2 to design the signalling details themselves. |
| Moderator | Thank you vivo for your careful check! Now all companies are fine with the proposal. This proposal can be agreed over the RAN1 reflector. [email1] High priority proposal 6-1: - Define Rel-17 LTE UE feature list for NR sidelink enhancement based on NR Rel-17 FGs |
| Moderator | Following was agreed via email endorsement on Mar 1. Agreement - Define Rel-17 LTE UE feature list for NR sidelink enhancement based on NR Rel-17 FGs |

- Conclusions

Following agreements were made in this meeting.

Agreement

- For the synchronization sources for NR sidelink transmission for FGs 32-4/4a,

  - Add a new FG 32-4b for synchronization sources for NR sidelink transmission as follows

    - eNB parts are included as another FG, FFS detail

| 32. NR_SL_enh | 32-4b | Synchronization sources for NR sidelink transmission | 1) UE supports GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 2) UE can transmit NR sidelink based on the synchronization to an gNB 3) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb if the UE supports Components 1 and 2 4) UE additionally supports gNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true if the UE supports Components 1 and 2. 5) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3 or 32-4 or 32-4a 6) UE supports SyncRef UE as the synchronization reference and can receive S-SSB in NR sidelink if it supports 15-1. |  | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Component 1 is only required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 Note: Components 2/3/4 are not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 32-5a is split into one FG for Tx capability and another for Rx capability as follows

  - FFS whether/how the FGs are split further

| 32. NR_SL_enh | 32-5a-1 | Transmitting Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can transmit and receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can transmit and receive an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-1 into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support transmitting inter-UE coordination scheme 1 in NR sidelink mode 2. UE does not receive an explicit request for inter-UE coordination information | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a-2 | Receiving Inter-UE coordination scheme 1 in NR sidelink mode 2 | 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. 2) UE can receive transmit an explicit request for inter-UE coordination information of [FFS: preferred resource set only or both preferred resource set and non-preferred resource set]. FFS whether/how to split FG 32-5a-2 into multiple FGs | [TBD] | [Yes] | [Yes] | UE does not support receiving inter-UE coordination scheme 1 in NR sidelink mode 2. UE does not transmit an explicit request for inter-UE coordination information | [Per band] | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

Agreement

- Add FG 32-6: Reception of Scheme 1 inter-UE coordination information over 2nd SCI

| 32. NR_SL_enh | 32-6 | Reception of Scheme 1 inter-UE coordination information over 2nd SCI | 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request |  | No | NoYes | UE is not required to decode SCI 2-C for inter-UE coordination information | Per FS | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Add FG 32-7: Determination of expected conflict in Scheme 2 based on RSRP difference

| 32. NR_SL_enh | 32-7 | Determination of expected conflict in Scheme 2 based on RSRP difference | 1) UE can determine a conflict for overlapping resource reservation between UE-B and another UE based on RSRP difference of the two reservations | 32-5b-1 | Yes | Yes |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 32-5a-1 is not split into FGs for preferred resource set and another FGs for non-preferred resource set

- FG 32-5a-2 is split into FGs for preferred resource set and another FGs for non-preferred resource set

  - At least one of the split FG 32-5a-2 is added as a prerequisite FG for FG 32-5a-1, FFS which one or both

Agreement

- For the synchronization sources for NR sidelink transmission for FGs 32-4/4a, add another FG 32-4c for eNB type synchronization sources for NR sidelink transmission with FG 32-4b as prerequisite

| 32. NR_SL_enh | 32-4c | eNB type synchronization sources for NR sidelink transmission | 1) UE can transmit NR sidelink based on the synchronization to an eNB. 2) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports component 1 in FG 32-4b, UE additionally supports eNB and GNSS as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | 32-4b | Yes | No |  | Per band | N.A. | N.A. | N.A. | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Type of FG 32-4b is per band

Agreement

- If UE reports more than one FGs of 15-3, 32-4 and 32-4a, the reported value B in each FG is the total number of SL processes and the same among those FGs.

Agreement

- The column of “Need for the gNB to know if the feature is supported” for FGs 32-4 and 32-4a are “Yes”

Agreement

- The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-4 and 32-4a are “No”

Agreement

- PSFCH Tx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-1

  - Value range for M is {4, 8, 16}

  - Add following note in FG 32-5b-1

    - If UE reports more than one FGs of 15-11 and 32-5b-1, the reported value M in each FG is the total number and the same among those FGs.

- PSFCH Rx capability is jointly reported for inter-UE coordination and HARQ-ACK reporting as a component in FG 32-5b-2

  - Value range for N is {5, 15, 25, 32, 35, 45, 50, 64}

  - Add following note in FG 32-5b-2

    - If UE reports more than one FGs of 15-11 and 32-5b-2, the reported value N in each FG is the total number and the same among those FGs.

Agreement

- The column of “Need for the gNB to know if the feature is supported” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes”

Agreement

- The column of “Applicable to the capability signalling exchange between UEs (Sidelink WI only)” for FGs 32-5a-1, 32-5a-2, 32-5b-1 and 32-5b-2 are “Yes”

Agreement

- Define Rel-17 LTE UE feature list for NR sidelink enhancement based on NR Rel-17 FGs

Agreement

- FG 32-6 is split to two FGs as follows

| 32. NR_SL_enh | 32-6-1 | Reception of Scheme 1 inter-UE coordination information over 2nd SCI | 1) UE can receive Scheme 1 inter-UE coordination transmission over 2nd SCI that is used in addition to the MAC-CE carrying the same inter-UE coordination information in the same transmission. FFS whether to separate capabilities for preferred/non-preferred resource set and explicit request | FFS | No Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH for inter-UE coordination information | Per FS [Per band] | [N.A]. | [N.A]. | [N.A.] | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_SL_enh | 32-6-2 | Reception of Scheme 1 explicit request over 2nd SCI | 1) UE can receive an explicit request for inter-UE coordination information of both preferred resource set and non-preferred resource set over 2nd SCI that is used in addition to the MAC-CE carrying the explicit request in the same transmission | FFS | Yes | Yes | UE is not required to decode SCI 2-C and the associated PSSCH for explicit request | [Per band] | [N.A.] | [N.A.] | [N.A.] | Note: configuration by NR Uu is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E.1-1 | Optional with capability signalling |

Agreement

- The column of “Need for the gNB to know if the feature is supported” for FG 32-7 is “No”

Agreement

- Type of FG 32-x is supported as follows

  - FG 32-4: Per FS

  - FG 32-4a: Per band

  - FG 32-4c: Per band

  - FG 32-5a-2: Per band

  - FG 32-5a-3: Per band

  - FG 32-5b-2: Per band

  - FG 32-6-1: Per band

References

[1]	R1-2200780	Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2112900	Updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e	Moderators (AT&T, NTT DOCOMO, INC.)

[3]	R1-2200965	Rel-17 UE features for NR sidelink enhancement	Huawei, HiSilicon

[4]	R1-2200984	UE features for NR sidelink enhancement	FUTUREWEI

[5]	R1-2201127	UE features for NR sidelink enhancement	vivo

[6]	R1-2201256	On UE feature list for NR sidelink enhancement	OPPO

[7]	R1-2201345	Remaining issues on Rel-17 UE features for sidelink enhancements	CATT, GOHIGH

[8]	R1-2201418	On UE features for NR sidelink enhancement	Nokia, Nokia Shanghai Bell

[9]	R1-2201510	Discussion on Rel.17 UE features for NR SL enhancement	NTT DOCOMO, INC.

[10]	R1-2201704	UE features for Rel.17 NR sidelink enhancements	Intel Corporation

[11]	R1-2201799	Views on Rel-17 NR Sidelink UE Features	Apple

[12]	R1-2201887	Discussion on UE features for NR sidelink enhancement	CMCC

[13]	R1-2201930	Discussion on Rel-17 UE features on sidelink enhancement	Xiaomi

[14]	R1-2202046	On UE features for NR sidelink enhancement	Samsung

[15]	R1-2202064	UE features for NR sidelink enhancements	MediaTek Inc.

[16]	R1-2202175	UE Features for Sidelink Enhancements	Qualcomm Incorporated

[17]	R1-2202259	Discussion on UE features for NR sidelink enhancement	LG Electronics

[18]	R1-2202265	UE features for NR sidelink enhancements	Ericsson

[19]	R1-2202381	Discussion on UE features for NR sidelink enhancement	ZTE, Sanechips