3GPP TSG RAN WG1 #108-e		R1-2201537

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:     8.1.2.3

Source:	Spreadtrum Communications

Title:	Discussion on enhancements on beam management for multi-TRP

Document for:	Discussion and decision

Introduction

In this paper, we will present our opinions on beam management related enhancement for multi-TRP transmission, and provide one text proposal.

Discussion

The first version of specifications reflecting R17 progress on this topic in RAN1 have been delivered in RAN#94e [1, 2]. After reviewing 38.213, we notice there exists one typo error in section 6 where there is  one additional word ‘the’.

Proposal 1: Suggest to adopt the following text proposal in 38.213.

------------------------------------------Start of Text Proposal#1 for TS 38.213--------------------------------------

- 6	Link recovery procedures

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the the set q1,1.

------------------------------------------End of Text Proposal#1 for TS 38.213--------------------------------------

Regarding to whether cell-specific and TRP-specific BFR can be configured on the same serving cell, previous meetings have some email discussions but no consensus. For one serving cell configured with M-TRP operation, generally there exists the following three beam failure cases as Fig.1 shows. For case 1 and case 2, beam failure happens in only either one of two TRP, where at least one link could work. TRP-

Figure 1 Beam failure cases for M-TRP

specific BFR is enough. For case 3, both links from TRP1 and TRP2 failed. In this case, generally gNB/UE could try to recover one TRP firstly, then perhaps try to recover another TRP. Or gNB/UE could try to recover both TRP independently. However, the essential still is TRP-specific BFR.

Observation 1:  It is not necessary to configure both cell-specific and TRP-specific BFR for one CC.

Conclusion

In this paper, we show our view on beam management for M-TRP, and have the following proposal and observation.

Proposal 1: Suggest to adopt the following text proposal in 38.213.

------------------------------------------Start of Text Proposal#1 for TS 38.213--------------------------------------

- 6	Link recovery procedures

A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes and corresponding two sets q1,0 and q1,1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList1 and candidateBeamRSList2, respectively, for radio link quality measurements on the BWP of the serving cell. The set q0,0 is associated with the set q1,0 and the set q0,1 is associated with the the set q1,1.

------------------------------------------End of Text Proposal#1 for TS 38.213--------------------------------------

Observation 1:  It is not necessary to configure both cell-specific and TRP-specific BFR for one CC.

Reference

- 3GPP TS 38.214: "NR; Physical layer procedures for data", h00

- 3GPP TS 38.213: "NR; Physical layer procedures for control", h00