3GPP TSG RAN WG1 #108-e                                	                   R1-2201541

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.2.1

Source:	Spreadtrum Communications

Title:	Discussion on initial access aspects for NR for 60GHz

Document for:	Discussion and decision

# Introduction

In the previous WID [1], the objective related to SSB and initial BWP is shown as follows.

- Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range. - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS(480kHz, 960kHz) for initial access related signals/channels in initial BWP. - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued.

According to WID objectives shown above, {120kHz, 120kHz} subcarrier spacing (SCS) combination for {SSB, initial BWP} should be supported as baseline.

In the newly updated WID [2], the objective related to SSB and initial BWP is revised and shown as follows.

- Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP. - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access. - Note: coverage enhancement for SSB is not pursued. - In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints: - Limited sync raster entry numbers - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz. - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS. - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: 480 kHz is an optional SSB numerology for initial access for the UE. A UE supporting a band in 52.6-71 GHz must at least support 120 kHz SCS (for initial access and after initial access) - Note: Dependency or lack thereof for a UE supporting 480kHz and/or 960kHz numerology for data and control to also support 480kHz SSB numerology for initial access is to be tackled as part of UE capability discussion. - Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting - Only 1 CORESET#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960). - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB. - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

According to WID objectives shown above, initial access using 480kHz SSB (in addition to 120kHz SSB) with some constrains should be supported in R17, and ANR/PCI confusion detection for 120/480/960kHz SSB should be supported in R17.

This contribution mainly focuses on initial access for NR from 52.6GHz to 71GHz. This contribution is revised from R1-2200273 in RAN1#107bis-e.

# SSB aspects

## Subcarrier spacing of SSB

### 120/480/960kHz SCS for channelization

Single SCS operation, or equivalently single numerology operation (ECP is not considered here), can reduce UE complexity, which has been strived to be supported in R16 NR-U. Which SCS is used in a carrier is mainly based on the maximum channel bandwidth for a given carrier. For instance, in R16 NR-U, 15kHz SCS can be used when the maximum channel bandwidth is 50MHz, and 30kHz SCS can be used when the maximum channel bandwidth is 100MHz. In RAN1#104e [3], the maximum channel bandwidth for 60GHz was agreed.

Agreement: - From RAN1 perspective, for NR operation in 52.6 GHz to 71 GHz, - The maximum channel bandwidth for 120 kHz SCS is 400 MHz - The maximum channel bandwidth for 480 kHz SCS is 1600 MHz - The maximum channel bandwidth for 960 kHz SCS is one of the following options - 2000 MHz - 2160 MHz - Send LS to RAN4 to inform about RAN1’s agreement of maximum channel bandwidth and ask RAN4 to decide and feedback the exact value of maximum channel bandwidth for 960 kHz SCS, the corresponding numbers of RBs for the maximum channel bandwidth of SCS(s) supported in 52.6 GHz to 71 GHz.

In general, 120kHz SCS can be used when the maximum channel bandwidth is 400MHz, and 480kHz SCS can be used when the maximum channel bandwidth is 1600MHz, 960kHz SCS can be used when the maximum channel bandwidth is 2000 or 2160MHz.

In 60GHz spectrum, there are plenty of spectrum resource, so 480/960kHz SCS is necessary to support the wide channel bandwidth. In addition, 120kHz SCS is necessary for in some cases, e.g. better coverage and narrowband operation [4].

### 120kHz SCS for SSB

UE can process with SSB with 120kHz SCS for initial access use cases, which center frequency of SSB is not provided to UE, e.g. initial cell selection and cell reselection. 120kHz SCS for SSB has benefit of low UE complexity, and thereby it is baseline.

### 240kHz SCS for SSB

Although 240kHz SCS for SSB is supported in R15/16 FR2 with acceptable UE complexity, the single SCS operation seems more important in most companies’ view. If 240kHz SCS for SSB is supported, the SCS of the associated Type0-PDCCH is still 120kHz, and thus the single SCS operation is not be achieved. We prefer 120kHz SCS for both SSB and Type0-PDCCH to enable the single SCS operation.

On the other hand, as mentioned in [5], 240kHz SCS for SSB in addition to 120kHz SCS will increase the number of frequency offsets of sync raster.

Therefore, in our view, 240kHz SCS for SSB can be down-prioritized.

### 480/960kHz SCS for SSB

In RAN1#104-e [3], 240kHz, 480kHz and 960kHz SCS were proposed. But there is no conclusion in RAN1#104-e.

Agreement: Whether or not to support 240 kHz, 480kHz and 960kHz SCS for SSB and the conditions under which SSB for 240 kHz, 480 kHz and 960 kHz may be supported will be decided no later than RAN1#104bis-e.

Time/frequency tracking for serving cell

First, UE does not have to switch between SSB-based measurement and data processing. This single SCS operation can ease UE implementation.

As another advantage, 480/960kHz SCS can provide finer time resolution than 120kHz SCS. When 120kHz SCS is applied for timing tracking, within the CP length of OFDM symbol with 960kHz SCS, there could be too small number of samples acquired by SSB with 120kHz SCS due to coarse time resolution. The poor number of sample within the CP length will potentially cause ISI issue.

In some companies’ view, SSB can be replaced by TRS/CSI-RS of serving cell, but in our view, validations of TRS/CSI-RS needs consuming lots of DCIs.

Serving cell RRM measurement

SSB with 480/960kHz SCS has benefit of single SCS operation for serving cell RRM measurement.

Similarly, if SSB can be replaced by TRS/CSI-RS of serving cell, validations of TRS/CSI-RS need to consume a large number of DCIs.

Observation 1: For time/frequency tracking and serving cell RRM measurement, 480/960kHz SCS for SSB has at least benefit of single SCS operation, and it is not a good way to replace SSB by TRS/CSI-RS of serving cell.

Neighboring cell RRM measurement

SSB with 480/960kHz SCS has benefit of single SCS operation for neighboring cell RRM measurement.

Observation 2: For neighboring cell RRM measurement, 480/960kHz SCS for SSB has at least benefit of single SCS operation.

Some companies proposed to using CSI-RS with 480/960kHz SCS to replace SSB. But, there are two drawbacks for using CSI-RS, when it is applied to the neighboring cell RRM measurement:

- Generally, CSI-RS RRM measurement for neighboring cell should base the timing on the SSB indicated by ssb-ToMeasure. So, CSI-RS RRM measurement needs the processing of SSB of neighboring cell, and thereby UE should switch between processing CSI-RS with 480/960kHz SCS and SSB with 120kHz SCS.

- In unlicensed band, CSI-RS cannot be validated since UE cannot receive DCI from the neighboring cell.

Observation 3: CSI-RS with 480/960kHz SCS cannot replace SSB with 480/960kHz for neighboring cell RRM measurement, because

- CSI-RS RRM measurement needs the processing of SSB of neighboring cell, and

- In unlicensed band, CSI-RS cannot be validated due to lack of DCI from neighboring cell.

Handover and SCell addition for CA/DC

Neighboring cell RRM measurement is prerequisite for handover and SCell addition for CA/DC.

After gNB commands UE to perform handover and SCell addition for CA/DC, UE should perform timing/frequency tracking for the neighboring cell (target cell) with center frequency provided by gNB. Similar to time/frequency tracking for the serving cell, SSB with 480/960kHz SCS can provide benefits like single SCS operation (deployed with the same SCS for mobility) and finer time resolution, which is desired for efficient handover and SCell addition. Because it is also neighboring cell processing, similar to neighboring cell RRM measurement, CSI-RS with 480/960kHz SCS cannot replace SSB with 480/960kHz SCS.

Observation 4: For handover and SCell addition for CA/DC for connected mode UE, 480/960kHz SCS for SSB has benefits of single SCS operation and finer time resolution. Similarly, CSI-RS with 480/960kHz SCS cannot replace SSB with 480/960kHz in these cases.

ANR and PCI confusion detection

In R16 NR-U, the off-raster SSB is introduced for ANR and PCI confusion detection, and gNB can provide the center frequency of the off-raster SSB to UE to enable MIB and SIB1 acquisition for ANR/CGI reporting. If this is applicable for NR from 52.6GHz to 71GHz, SSB with 480/960kHz SCS has benefit of single SCS operation. Because it is also neighboring cell processing, similar to neighboring cell RRM measurement, CSI-RS with 480/960kHz SCS cannot replace SSB with 480/960kHz SCS.

Observation 5: For ANR and PCI confusion detection for connected mode UE, 480/960kHz SCS for SSB has at least benefit of single SCS operation. Similarly, CSI-RS with 480/960kHz SCS cannot replace SSB with 480/960kHz in these cases.

Initial cell selection

It is questionable that whether SSB with 480/960kHz SCS can be applied to initial cell selection. SSB with 480/960kHz SCS will cause high UE complexity due to huge number of samples in 20ms window in blind detection for a candidate center frequency (a sync raster or a finer raster to mitigate frequency error) [6].

### Progress in RAN1#104bis-e

In RAN1#104bis-e [7], 480/960kHz SCS for SSB was agreed to be supported in a case, where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH.

Agreement: For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB - Note: Strive to minimize specification impact due to the new SCS for SSB

But whether to support 480/960kHz SCS for SSB in another cases, e.g. configuring Type0-PDCCH to support ANR/CGI reporting or defining sync raster for 480/960kHz SCS to support initial cell selection, is not agreed yet.

### Progress in RAN#92-e

As shown in Section 1, WID is updated to support 480kHz SSB for initial cell selection, and support 480/960kHz SSB for PCI/ANR confusion detection.

### ANR/PCI confusion detection with 120/480/960kHz SSB

The objectives for ANR/PCI confusion detection with 120/480/960kHz SSB is shown as follows.

- Support ANR and PCI confusion detection for 120, 480 and 960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 120, 480 and 960kHz SSB - FFS: additional method(s) to enable support to obtain neighbour cell SIB1 contents related to CGI reporting - Only 1 CORESET#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (120, 120), (480, 480) and (960, 960). - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis. - Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible - Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB. - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

It shows that the common SCS CORESET#0 is used.

In RAN1#105-e [8], it was agreed that configuring CORESET#0/Type0-PDCCH can be down selected from two alternatives.

Agreement: For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access) - Support configuring CORESET#0/Type0-PDCCH for the purpose of ANR/PCI confusion detection by down selecting from the following two alternatives - Alt 1) Using dedicated signaling - Alt 2) Using configuration in MIB - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

In our view, the mechanism of two offsets in MIB defined for NR-U in TS 38.213, i.e. Alt 2), can be reused for UE to determine CORESET#0/Type0-PDCCH.

For operation with shared spectrum channel access, a UE determines an offset from a smallest RB index of the CORESET for Type0-PDCCH CSS set to a smallest RB index of the common RB overlapping with a first RB of the corresponding SS/PBCH block - according to the offset in Table 13-1A or Table 13-4A, if the frequency position of the SS/PBCH block corresponds to the GSCN of a synchronization raster entry as defined in [8-1, TS 38.101-1], and - according to a sum of a first offset and a second offset if the frequency position of the SS/PBCH block is provided by ssbFrequency in a measurement configuration associated with a reporting configuration providing reportCGI and does not correspond to the GSCN of a synchronization raster entry as defined in [8-1, TS 38.101-1], where - the first offset is provided in Table 13-1A or Table 13-4A, and - the second offset is determined as the offset from a smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block indicated in the measurement configuration to a smallest RB index of the common RB overlapping with the first RB of a SS/PBCH block hypothetically located at the GSCN of a synchronization raster entry, where the single synchronization raster entry is located in the same channel as the SS/PBCH block used for the shared spectrum channel access procedure, as described in [15, TS 37.213]

### Initial cell selection with 480kHz SSB

The objectives for Initial cell selection with 480kHz SSB (in addition to 120kHz SSB) is shown in Section 1.

It shows that the sync raster is limited for initial cell selection with 480kHz SSB. However, in our view, UE complexity is still high.

For one detection, 480kHz SSB needs 4 times of the number of samples in 20ms than 120kHz SSB. Specifically, if there is 1 sync raster to be detected for 480kHz SSB, UE should buffer N samples in 20ms window. However, if there are 4 sync raster to be detected for 120kHz SSB, UE should only buffer N/4 samples in 20ms window and try 4 times of detection. From perspective of the size of memory, UE complexity is still high for initial cell selection with 480kHz SSB.

We strongly suggest that supporting initial cell selection with 480kHz SSB should be an optional UE capability separately from other processing with 480/960kHz SCS, e.g. supporting RRM measurement with 480/960kHz SSB or control/data processing with 480/960kHz SCS. In other words, we do not think supporting initial cell selection with 480kHz SSB has the similar level of UE complexity with supporting other processing with 480/960kHz SCS.

Proposal 1: Supporting initial cell selection with 480kHz SSB should be an optional UE capability separately from supporting other processing with 480/960kHz SCS.

## SSB pattern

### 120kHz SCS for SSB

The legacy pattern for SSB with 120kHz SCS can be considered. In RAN1#104bis-e [7], it was agreed that the legacy Case D is reused.

Agreement: For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz, - 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame. - For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18. - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB

### 480/960kHz SCS for SSB

In RAN1#104e [3], it was agreed that the reserving symbol gap between SSB positions for beam switching and the reserving gap for DL/UL switching and DL/UL transmission is FFS, if 480/960kHz SCS for SSB is agreed to be supported.

Agreement: For 480 kHz and 960 kHz SSB SCS (if agreed) - Study further on reserving symbol gap between SSB positions with different SSB index (and possibly between SSB position and other signal/channels) - FFS: whether symbol gap is needed for only 960 kHz or both 480 and 960 kHz. - Study further on reserving gap for UL/DL switching within the pattern accounting possibility for reserving UL transmission occasions in the SSB pattern - Study should account for inputs from RAN4

In RAN1#104e [3], it was agreed that beam switching, DL/UL switching and DL/UL transmission need to be discussed in RAN4.

Agreement: Send an LS to RAN4 to get input on gap required for gNBs and UEs for beam switching and for UL/DL and DL/UL switching.

If the symbol gap between SSB positions is agreed to be supported, the SSB pattern of Case A/C can be considered, in which at least two symbols are reserved as gap between each two adjacent SSBs.

In RAN1#105-e [8], it was agreed that both Case B/D, i.e. ALT 2) no symbol gap b/w two SSBs in a slot, and Case A/C like, i.e. ALT 1) symbol gap b/w two SSBs in a slot, are listed as alternatives for the SSB pattern for 480/960kHz.

Agreement: For 480kHz/960kHz SSB, select one of the following alternatives: - ALT 1) First symbols of the candidate SSB have index {X, Y} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame - value of X and Y are identical for 480kHz and 960kHz - FFS: exact value of X and Y - ALT 2) First symbols of the candidate SSB have index {4, 8, 16,20} + 28*n, where index 0 corresponds to the first symbol of the first slot in a half-frame - Values of n for 480kHz and 960kHz for ALT 1 and 2 - FFS: whether number of values for ‘n’ depend on LBT operation (i.e. LBT vs no-LBT) - FFS: exact values of ‘n’ for each SCS - Values of ‘n’ for one mode of operation shall be strictly a subset of values for another mode of operation, if two mode of operation exist for number of candidate SSBs - FFS: whether values of ‘n’ shall not be all consecutive integer values (i.e. non-candidate SSB slots are positioned every few candidate SSB slots)

In RAN1#106-e [9], it was agreed Alt 1) with {2, 9} as starting symbol is supported for 480/960kHz SCS for SSB.

Agreement: - For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, X} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame. - Alt 1: X = 8 - Alt 2: X = 9 Agreement: For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

## Discovery burst transmission window

In RAN1#104-e [3], some agreements related to DB and DBTW was achieved. Some conditions to support DBTW were agreed, e.g. mechanism to inform DBTW is enabled/disabled, the same PBCH payload size and number of PBCH DMRS sequences as that of FR2, no greater than 5ms for length of DBTW. Some FFS points were also agreed.

Agreement: For an unlicensed band that requires LBT, further study whether/how to support discovery burst (DB) and discovery burst transmission window (DBTW) at least for 120 kHz SSB SCS - If DB supported - FFS: What signals/channels are included in DB other than SS/PBCH block - If DBTW is supported - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs - FFS: how to support UEs performing initial access that do not have any prior information on DBTW. - PBCH payload size is no greater than that for FR2 - Duration of DBTW is no greater than 5 ms - Number of PBCH DMRS sequences is the same as for FR2 - The following points are additionally FFS: - How to indicate candidate SSB indices and QCL relation without exceeding limit on PBCH payload size - Details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands - Whether or not to support DBTW for SSB SCS(s) other than 120 kHz if other SSB SCS(s) are supported

In RAN1#104bis-e [7], it was agreed to support DB in 60GHz unlicensed, but whether to support DBTW at least for SSB with 120kHz SCS is still FFS.

Agreement: - For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0 - FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements - PBCH payload size is no greater than that for FR2 - Duration of DBTW is no greater than 5 ms - Number of PBCH DMRS sequences is the same as for FR2 - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs - FFS: how to support UEs performing initial access that do not have any prior information on DBTW. - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands - FFS: details of how to inform UEs of the configuration of DBTW

### Supporting DBTW

DBTW in R16 NR-U is actually the SSB transmission window, and only SSBs should be confined in DBTW. It is general view that both LBT and no LBT is supported for SSB transmission. For DBTW, we should consider these two cases both.

UE complexity

LBT case

For LBT case, usually, DBTW can support candidate SSB position more than SSB beams. Thus it seems DBTW will increase UE complexity largely, but it is not true.

If DBTW is short enough, it can reduce UE complexity, since DBTW can reduce the number of SSB beams. If DBTW is long, UE complexity is increased, since there are at most three candidate SSB positions corresponding to on SSB beams. However, UE may only blindly detect up to two candidate SSB positions as RAN4 requirement. Therefore, the increase of UE complexity is acceptable.

No LBT case

For no LBT case, if DBTW is disabled, UE does not need to blindly detect more than one candidate SSB positions for a given SSB beam. It can also reduce UE complexity for no LBT case.

Observation 6: For LBT case, DBTW can reduce UE complexity or increase UE complexity slightly. For no LBT case, disabling DBTW can reduce UE complexity.

SSB with 120kHz SCS

LBT exempt operation may not be applicable in some cases. For example, duration of 64 SSB beams with 120kHz SCS exceeds 10ms within an observation period of 100ms duration, if the default 20ms SSB burst periodicity is used.

DBTW can reduce the number of SSB beams with 120kHz SCS to satisfy the condition of LBT exempt operation. Alternatively, the number of SSB beams with 120kHz SCS can be reduced by stringently limiting the number of candidate SSB positions in a predefined way, e.g. less than 64, but it is inflexible. As another alternative, if the number of candidate SSB positions is equal to that of SSB beams, the indication of ssb-PositionsInBurst may replace the functionality of DBTW to reduce the number of candidate SSB positions. Unfortunately, ssb-PositionsInBurst is just a 16 bits bitmap, which cannot precisely indicate the on/off of 64 SSB beams when the number of SSB beams is reduced.

Hence, DBTW can efficiently reduce the number of SSB beams with 120kHz SCS to satisfy the condition of LBT exempt operation.

Observation 7: DBTW can reduce the number of SSB beams with 120kHz SCS to satisfy the condition of LBT exempt operation.

In RAN1#105-e [8], supporting DBTW at least for 120kHz SCS is the FFS point.

Agreement: FFS: Support DBTW at least for 120kHz - FFS whether DBTW will be applicable for 480/960 kHz SSB SCS - If DBTW is supported for 480/960kHz SSB: - For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access), indication of DBTW configuration (e.g. enable/disable of DBTW, NSSBQCL, and DBTW length) are supported by dedicated signaling. - For 120kHz SSB, support mechanism to distinguish at least the following scenarios: - Case 1) (Unlicensed with LBT off) + DBTW disabled - Case 2) (Unlicensed with LBT on) + DBTW enabled - Case 3) (Unlicensed with LBT on) + DBTW disabled - Case 4) (Licensed) + DBTW disabled - FFS: Whether/how LBT on/off is indicated in MIB - If not indicated in MIB, then FFS whether/how the UE determines different sizes of DCI 1_0 with CRC scrambled by SI-RNTI - FFS: whether any case(s) can be combined for DBTW signaling design and how to handle implications to DCI 1_0 size ambiguity if is not distinguished in signaling - FFS: whether all above cases need an explicit indication - FFS: Whether a single indication can be used for combination of more than one cases - For 120 kHz SSB, enable/disable of DBTW is indicated by one or more of the following methods: - Option 1) signaling in MIB - Option 1-1) disabling DBTW is jointly coded with NSSBQCL - Option 1-2) indicated by other bit fields in MIB - FFS: among options 1-1 and 1-2 - Option 2) distinct GSCN used by the SSB - Option 3) By comparing the value of NSSBQCL in MIB and DBTW length after UE reads SIB1 or by comparing the value of NSSBQCL in MIB and default DBTW length of 5 ms before UE reads SIB1. - FFS: whether to support option 1, 2, 3, or any combination of the options. - Note: enable/disable signaling of DBTW by MIB or GSCN does not preclude other signaling methods Agreement: If DBTW is supported, - Working assumption: MIB signaling to support - Alt A) indication of NSSBQCL at least for 120kHz SSB - In this case, the total number of values of NSSBQCL to not exceed 4 - Alt B) Explicit indication of SSB index and/or SSB candidate location - FFS on the details of signaling - FFS between Alt A, or B, or supporting both - Supported DBTW lengths - Alt 1) 0.5, 1, 2, 3, 4, 5 msec - Note: same as Rel-16 FR1 NR-U - Alt 2) maximum 5 msec - FFS other values - FFS between Alt 1 and 2 - Number of candidate positions when DBTW is enabled - For 120kHz SSB - FFS between 64 or 80 - If DBTW is additionally supported for 480/960kHz SSB - FFS between 64 or 128

In RAN1#106bis-e [10], supporting DBTW for 120kHz was agreed to be a working assumption.

Working assumption: Support DBTW for 120 kHz. - FFS: Support for 480 kHz and 960 kHz

It was confirmed in RAN1#107-e [11].

Agreement Confirm the following working assumptions: - (From #106-bis-e) Support DBTW for 120 kHz.

SSB with 480/960kHz SCS

To balance between the more opportunities of SSB transmissions at gNB and the less complexity of SSB beam detection at UE, we also support DBTW for 480/960kHz SCS.

Considering contention exemption short control signalling, without knowledge of enabling the short control signalling (e.g. disabling DBTW), UE should blindly detect multiple candidate SSB positions for measurement. To solve this issue, some companies proposed the explicit signalling for disabling DBTW in MIB (by Q value) to inform UE not to detect multiple candidate SSB positions. We think it is a solution, and it was agreed for SSB with 120kHz SCS in RAN1#106bis-e [10].

However, this does not mean DBTW is not necessary. Actually, DBTW length can still inform UE the number of SSB beams is reduced. DBTW length is parameter of SIB, and thus removing it has negligible overhead reduction.

In RAN1#107-e [11], it was agreed to support DBTW with 480/960kHz SCS.

Agreement - Support DBTW with 480 and 960 kHz SCS.

### DBTW length

The DBTW length for 120kHz SCS was defined in RAN1#106-e [9] as follows.

Agreement: For DBTW with 120kHz SCS (if supported), support DBTW lengths {0.5, 1, 2, 3, 4, 5} msec - Note: this should be the same as Rel-16 NR-U DBTW lengths.

The DBTW length for 480/960kHz SCS was defined in RAN1#107-e [11] as follows.

Agreement For 480 and 960 kHz, supported DBTW lengths are: - {1.25, 1, 0.75, 0.5, 0.25, 0.125, X} ms, where X = 0.0625 if Q=8 is supported and X is removed if Q=8 is not supported.

The DBTW length for 120kHz SCS reuse that for 15/30kHz SCS in R16 NR-U. And then the DBTW length for 480kHz SCS reuse the slots number for the DBTW length for 120kHz SCS. The slots number for the DBTW length is shown in the following tables.

Table 1: The slots number for the DBTW length for 15kHz, 30kHz and 120kHz SCS

|  | 0.5ms | 1ms | 2ms | 3ms | 4ms | 5ms |
| --- | --- | --- | --- | --- | --- | --- |
| 15kHz SCS in R16 NR-U | 1/2 slot | 1 slot | 2 slots | 3 slots | 4 slots | 5 slots |
| 30kHz SCS in R16 NR-U | 1 slot | 2 slots | 4 slots | 6 slots | 8 slots | 10 slots |
| 120kHz SCS in R17 60GHz | 4 slots | 8 slots | 16 slots | 24 slots | 32 slots | 40 slots |

Table 2: The slots number for the DBTW length for 480kHz and 960kHz SCS

|  | [0.0625ms] | 0.125ms | 0.25ms | 0.5ms | 0.75ms | 1ms | 1.25ms |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 480kHz SCS in R17 60GHz | 2 slots | 4 slot | 8 slots | 16 slots | 24 slots | 32 slots | 40 slots |
| 960kHz SCS in R17 60GHz | 4 slots | 8 slots | 16 slots | 32 slots | 48 slots | 64 slots | 80 slots |

### Indication of Q values

In RAN1#107-e [11], it was agreed to use 2 bits for Q values and send LS to RAN2 for the use of the spare bit in MIB.

- Working assumption: Use 2 bits for Q: - SubcarrierSpacingCommon - spare bit in MIB - Send LS to RAN2 for confirming the use of the spare bit in MIB - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used

In the replied LS from RAN2 [R1-2200874], it was not agreed for the use of the spare bit in MIB. If only one bit is supported for Q values, there are only two Q values {32, 64}. However, Q=16 can be used for the case where the number of beams is small but the interference is large (more opportunities for beam repetitions). Therefore, we think the additional bit needs to be found for Q values.

Proposal 2: Confirm the working assumption to use 2 bits for Q values, including SbucarrierSpacingCommon and other additional bit.

### Enabling/disabling the DBTW

Enabling/disabling the DBTW by Q value implicitly was agreed for SSB with 120kHz SCS in RAN1#106bis-e [10].

Agreement: For 120kHz SCS, for values: - If 2 bits are available in MIB for , at least support {16, 32, 64} - If 1 bit is available in MIB for , support {32, 64} - FFS: methods to indicate more values without increasing used number of bits, e.g., {16, 32, 64} - Note: value < 64 indicates DBTW enabled/supported and operation with shared spectrum. - Note: For operation without shared spectrum channel access, a UE expects to be configured with = 64. Use of =64 in shared spectrum is not precluded. - FFS: 1 bit or 2 bits used for

Enabling/disabling the DBTW by Q value implicitly was also agreed for SSB with 480/960kHz SCS in RAN1#107-e [11]. It shows that Q value equal to 64 means to disable the DBTW.

Agreement - Same NSSBQCL values using the same set of signaling bits are supported for 120, 480, and 960 kHz. - Supported values of NSSBQCL: {16, 32, 64} - Note: - For operation with shared spectrum channel access, any supported value of NSSBQCL can be indicated and value < 64 indicates DBTW enabled - UE is expected to be configured with NSSBQCL=64 in licensed operations - For operation with and without shared spectrum channel access, NSSBQCL=64 indicates that the SS/PBCH block index and the candidate SS/PBCH block index have a one-to-one mapping relationship.

However, Q value equal to 64 has two meanings behind the disabling of the DBTW. One is the operation in licensed operation, another one is the operation with the short control signalling in unlicensed operation. To let UE know which meaning is used, the reserved codepoint for Q value indication can be used.

Proposal 3: The reserved codepoint for Q value indication can be used to distinguish the operation in licensed operation and the operation with the short control signalling in unlicensed operation.

### Interpretation of ssb-PositionsInBurst

Like R16 NR-U, the interpretation of ssb-PotionsInBurst should be discussed, e.g. relationship between Q value and ssb-PositionsInBurst. It was widely discussed in RAN1#107-e [11] and finally reached the following agreements.

Agreement - If NSSBQCL is indicated, the same interpretation of ssb-PositionsInBurst in SIB1 or ServingCellConfigCommon as in Rel-16 is supported, i.e.: - A bit set to 1 at position k∈{1..64} indicates SS/PBCH block index k-1 - The UE assumes that a bit at position k > NSSBQCL is set to 0 - For ssb-PositionsInBurst in SIB1, the UE assumes that a bit at groupPresence corresponding to a SS/PBCH block index ≥ NSSBQCL is set to 0 - Note: for ssb-PositionsInBurst in SIB1, position k corresponds to the SS/PBCH block index indicated by a bit in inOneGroup and a bit in groupPresence - In operation with shared spectrum in 60 GHz, for ssb-PositionsInBurst in ServingCellConfigCommonSIB, - for MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense of ssb-PositionsInBurst: - if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to ‘SSB index’ equal to k-1+(m-1)×8 may be transmitted; - if MSB k of inOneGroup or MSB m of groupPresense is set to 0, the UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to ‘SSB index’ equal to k-1+(m-1)×8 is not transmitted; - In operation with shared spectrum in 60 GHz, for ssb-PositionsInBurst in ServingCellConfigCommon, - ssb-PositionsInBurst bits correspond to supported ‘SSB indices’, - and UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to indicated bit(s) may be transmitted; - and UE assumes that SSB(s) within DBTW with ‘candidate SSB index(es)’ corresponding to not indicated bit(s) are not transmitted - Note to spec editor: The above three bullets maintain the same behavior as Rel-16 NR-U

### Candidate SSB positions

If DBTW is supported, the candidate SSB positions should be predefined.

SSB with 120kHz SCS

In RAN1#106e [9], it was a working assumption that the number of candidate SSBs with 120kHz SCS in a half frame is 64 like legacy.

Working assumption: For 120kHz SSB, the number of candidates SSBs in a half frame is 64.

It was confirmed in RAN1#107-e [11].

Agreement Confirm the following working assumptions: - (From #106-e) For 120kHz SSB, the number of candidates SSBs in a half frame is 64.

The slot indexes for the candidate SSB positions should be determined. It is common understanding that the 64 candidate SSB positions has the legacy slot indexes.

Proposal 4: The candidate SSB positions for 120kHz SCS follows the current spec.

SSB with 480/960kHz SCS

In RAN1#107-e [11], it was agreed to support only 64 candidate SSB positions for 480/960kHz SCS.

Agreement - Support DBTW with 480 and 960 kHz SCS. - For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame

The slot indexes for the candidate SSB positions should be determined. In RAN1#106bis-e [10], the three alternatives were listed for slot patterns for 480/960kHz SSB.

Agreement: Supported value of n for 480/960kHz SSB slot pattern: - ALT A) non-contiguous, N slot gap (slots that do not contain SSB) every M slots that contain SSB - same pattern will apply to 480kHz and 960kHz (i.e same N and M for 480 and 960 kHz) - N = 2, M = 8 - FFS: starting position of n - ALT B) non-contiguous, N slot gap (slots that do not contain SSB) every M slots that contain SSB - scaled version pattern will apply between 480 and 960 kHz (i.e. N and M for 480kHz, 2N and 2M for 960 kHz) - N = 2, M = 8 - FFS: starting position of n - ALT C) slots that do not contain SSB correspond to the slots that do not contain SSB in 120 kHz Case D. - Note: ALT 4 means that only slots 32-39 for 480 kHz SSB pattern are reserved for UL and 960 kHz SSB pattern is contiguous.

After discussion in RAN1#107-e, it was agreed that the slot indexes for the candidate SSB positions are contiguous. It is actually ALT C) with limit of 64 candidate SSB positions for 480kHz SSB.

Agreement - For 480 kHz, slot index, n, that contain SSB are: - n = {0,1,2,3,4,5,6,7, 8,9,10,11,12,13,14,15, 16,17,18,19,20,21,22,23, 24,25,26,27,28,29,30,31} - For 960 kHz, slot index, n, that contain SSB are: - n = {0,1,2,3,4,5,6,7, 8,9,10,11,12,13,14,15, 16,17,18,19,20,21,22,23, 24,25,26,27,28,29,30,31}

## SSB-based CSI-RS validation

SSB-based CSI-RS validation had been discussed in R16 NR-U, but did not agreed. But in our view it should be supported to fully utilize functionality of CSI-RS.

TRS

For connected mode, UE should perform T/F tracking based on TRS timely. In unlicensed operation, whether TRS is transmitted is unknown by UE, and it makes UE behaviour different from that in licensed operation. Since UE does not know the presence of TRS, with current CSI-RS validation methods, UE should detect a DCI format 2-0 to get the CO information covering CSI-RS or determine a resource scheduled by a DCI format containing CSI-RS. But UE may detect the DCI format with large T/F error and thus suffer from miss-detection, if the valid occasions of TRS is not enough. It will be chicken-and-egg problem. In our view, there could be some solutions in gNB implementation, e.g.

- gNB should configure the enough number of TRS occasions, and

- gNB should achieve TXOPs to construct COs as many as possible to cover the configured TRS occasions, and transmit a DCI format to let UE validate TRS.

The above solutions are not efficient from both network and UE perspectives.

Fortunately, SSB transmission can be with no-LBT if SSB belongs to contention exemption short control signalling, so the SSB-based TRS validation can make the T/F tracking based on SSB/TRS as reliable as possible in connected mode.

Observation 8: The SSB based TRS validation can make the T/F tracking based on SSB/TRS as reliable as possible.

Additional TRS for paging enhancement

For idle mode, in R17 the paging enhancement with additional TRS in idle mode is being defined. In the logic of the additional TRS, the idle-mode UE should perform T/F tracking based on TRS a prior to paging PDCCH. But this is not feasible for unlicensed operation, since UE in unlicensed operation should validate TRS by detecting PDCCH successfully. Due to this, the additional TRS in idle mode cannot be applied in unlicensed operation relying on the DCI-based CSI-RS validation. Similar to connected mode, the SSB-based TRS validation can make the additional TRS in idle mode feasible.

Observation 9: The SSB-based TRS validation can make the additional TRS in idle mode feasible.

CSI-RS for beam management

In 60GHz, CSI-RS for beam management is important, so the DCI-based CSI-RS validation is too limited to enable reliable beam management based on CSI-RS.

Therefore, the SSB-based TRS/CSI-RS validation can be supported.

Proposal 4: The SSB-based TRS/CSI-RS validation can be supported.

# Conclusion

In the contribution, we have the following proposals.

SCS of SSB

Proposal 1: Supporting initial cell selection with 480kHz SSB should be an optional UE capability separately from supporting other processing with 480/960kHz SCS.

DBTW

Proposal 2: Confirm the working assumption to use 2 bits for Q values, including SbucarrierSpacingCommon and other additional bit.

Proposal 3: The reserved codepoint for Q value indication can be used to distinguish the operation in licensed operation and the operation with the short control signalling in unlicensed operation.

Proposal 4: The candidate SSB positions for 120kHz SCS follows the current spec.

SSB-based CSI-RS validation

Proposal 4: The SSB-based TRS/CSI-RS validation can be supported.

# Reference

- RP-210862, “Revised WID:  Extending current NR operation to 71 GHz”, CMCC, Mar. 16th – 26th, 2020.

- RP-211584, “Revised WID:  Extending current NR operation to 71 GHz”, Qualcomm, Intel, June 14-18, 2021.

- 3GPP RAN1#104-e Chairman’s Notes, Jan. 25th - Feb. 5th, 2021.

- R1-2101306, “Initial Access Aspects”, Ericsson, RAN1#104e, Jan. 25th - Feb. 5th, 2021.

- R1-2103229, “Initial access aspects for NR from 52.6 GHz to 71 GHz”, Samsung, RAN1#104e, April 12th - 20th, 2021, 2021.

- R1-2101453, “Initial access aspects for NR to support operation between 52.6 GHz and 71 GHz”, Qualcomm, RAN1#104e, Jan. 25th - Feb. 5th, 2021.

- 3GPP RAN1#104bis-e Chairman’s Notes, April 12th - 20th, 2021.

- 3GPP RAN1#105-e Chairman’s Notes, May 10th - 27th, 2021.

- 3GPP RAN1#106-e Chairman’s Notes, August 16th - 27th, 2021.

- 3GPP RAN1#106bis-e Chairman’s Notes, Oct. 11th - 19th, 2021.

- 3GPP RAN1#107-e Chairman’s Notes, November 11th - 19th, 2021.