3GPP TSG RAN WG1 #108-e			R1- 2201544

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.3.1

Source: 	Spreadtrum Communications

Title: 	Discussion on HARQ-ACK feedback enhancements for Rel-17 URLLC

Document for:	Discussion and decision

Introduction

This contribution provides some thoughts on the remaining issues of physical Layer UE feedback enhancements for HARQ-ACK.

Interaction with Rel-17 Intra-UE multiplexing

SPS HARQ-ACK deferral and Rel-17 intra-UE multiplexing

It was achieved a conclusion that separate deferral can be done for different PHY priorities of SPS HARQ, when the UE is not configured with Rel-17 Intra-UE multiplexing, but configured with Rel-16 PHY prioritization.

Conclusion RAN1 106b-e If the UE is not configured with Rel-17 Intra-UE multiplexing, SPS HARQ for deferral of different PHY priorities can be separately deferred with the target PUCCHs separately determinated according to their respective PHY priorities. - FFS on the PHY priority handling for SPS HARQ deferral if the UE configured with Rel-17 Intra-UE multiplexing Conclusion RAN1 107e For SPS HARQ-ACK deferral, if a UE is not configured with Rel-17 intra-UE multiplexing but configured with Rel-16 PHY prioritization, the UE first performs Rel-16 UCI multiplexing and PHY prioritization in both initial slot and target slot and if a LP SPS HARQ-ACK PUCCH is deprioritized, the LP SPS HARQ-ACK is not deferred. - Note: If the SPS HARQ-ACK is deprioritized in any slot, no further deferral.

For UE is configured with Rel-17 intra-UE multiplexing case, we support to have the same handling for UE is configured with Rel-16 Intra-UE multiplexing, which is SPS HARQ for deferral of different PHY priorities can be separately deferred with the target PUCCHs separately determined according to their respective PHY priorities.

One commented case is when PUCCH1 with HP HARQ-ACK and PUCCH with LP HARQ-ACK are overlapping in a slot but none of them can be transmitted, then deferral is performed. The argument is joint deferral or separate deferral. First of all, joint deferral is not reasonable considering the different slot length. The slot length for LP and HP HARQ-ACK are separately configured, based on traffic delay requirements. It is not fair that LP share the same slot length with HP just because they are overlapping in one slot. Second, the PUCCH resource for Rel-17 Intra-UE multiplexing	use a resource in the second PUCCH-Config. It is also not proper to always use HP HARQ-ACK PUCCH resource in all the slots. So an easier and straightforward method is SPS HARQ for deferral of different PHY priorities can be separately deferred with the target PUCCHs separately determined according to their respective PHY priorities even Rel-17 intra-UE multiplexing is configured.

Figure 1: intra-UE multiplexing and SPS HARQ-ACK deferral

Another case is the in both initial slot and target slot, if a LP SPS HARQ-ACK PUCCH is dropped such as LP HARQ-ACK PUCCH with format 2/3/4 overlapping with HP SR etc., the LP SPS HARQ-ACK is not deferred.

- If the UE is configured with Rel-17 Intra-UE multiplexing, SPS HARQ for deferral of different PHY priorities can be separately deferred with the target PUCCHs separately be determinated according to their respective PHY priorities.

- If a LP SPS HARQ-ACK PUCCH is dropped according to Rel-17 intra-UE multiplexing, the LP SPS HARQ-ACK is not deferred.

PUCCH carrier switching

Semi-static PUCCH cell switching and PUCCH repetition

During the RAN1#107b-e, there was some discussion about semi-static PUCCH cell switching and PUCCH repetition. We support Mod Proposal 11, and do not support PUCCH cell switching per PUCCH repetition.

Question 6.5.3: After further consideration from your side, which one do you prefer: - Alt. 1: Mod Proposal 11 from the GTW session (i.e. no PUCCH cell switching within a repetition bundle) - Alt. 2: PUCCH cell switching per PUCCH repetition with clarification according to either Option 1 or Option 2 above Mod Proposal 11: For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and is not counted towards the total number of PUCCH repetitions / the repetition is post-poned as in Rel-16.

First, PUCCH repetition always have same frequency RB, and same starting symbol and length in a slot/sub-slot. However, for PUCCH cell switching, it already in different cells, so it cannot use the same time/frequency location among different repetitions.

Second, in PUCCH sSCell, the PUCCH resources are separately configured, so for same PUCCH resource ID on PCell and PUCCH sSCell, the PUCCH actual resource location is totally different. Thus, for a same PUCCH resource there is no way to do cross cells’ repetition.

Third, since PUCCH configuration is separately configured for PCell and PUCCH sSCell. Then there is a chance that for same priority, the slot length is different between the two PUCCH cells. From this understanding, the repetition time unit would be totally different. Therefore, it makes no sense to do repetitions between two cells.

- Support Mod Proposal 11:

For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported

A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and is not counted towards the total number of PUCCH repetitions / the repetition is post-poned as in Rel-16.

SPS operation with dynamic PUCCH cell switching

Another open issue is SPS operation with dynamic PUCCH cell switching, we support the Alt 1in following proposal.

Mod Proposal 5: For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement). - Alt. 1 (proposed by ZTE, Nokia online): - The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI. - Note: This is basically Alt. 3, but does not change the earlier agreement as the UE still applies the indicated cell (just the PUCCH sSCell cannot be indicated) - Alt. 2 (mentioned by some companies – based on current dynamic PUCCH restrictions): - The following conclusion should be sufficient to not requiring to multiplex SPS HARQ-ACK on from PCell /SPcell / PUCCH Scell with the HARQ for the first SPS PDSCH based on the activation DCI on the PUCCH sSCell.

According to the HARQ-ACK of SPS PDSCH without associated DCI, if the proposal 5 is adopted, the main concern is codebook generation for SPS PDSCH, especially considering when a SPS configuration is activated with indication for HARQ-ACK on the PUCCH-sSCell, while there are some SPS HARQ-ACK for SPS PDSCH without associated DCI on the PCell. Because the SPS HARQ-ACK CB and Type 1/2 HARQ-ACK CB are generated per slot, SPS HARQ-ACK bits are always appended/within the CB together if there are dynamic HARQ-ACK feedback. Thus, we agree that Alt 2 in proposal 5 do not resolve the problems. An easy and straight forward way is Alt 1, and it does not against the previous agreement a certain extent.

- Support Alt 1 in Mod proposal 5.

Mod Proposal 5: For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement).

- Alt. 1 (proposed by ZTE, Nokia online):

  - The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI.

Conclusion

In this contribution, we made the following proposals.

- If the UE is configured with Rel-17 Intra-UE multiplexing, SPS HARQ for deferral of different PHY priorities can be separately deferred with the target PUCCHs separately be determinated according to their respective PHY priorities.

- If a LP SPS HARQ-ACK PUCCH is dropped according to Rel-17 intra-UE multiplexing, the LP SPS HARQ-ACK is not deferred.

- Support Mod Proposal 11:

For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported

A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and is not counted towards the total number of PUCCH repetitions / the repetition is post-poned as in Rel-16.

- Support Alt 1 in Mod proposal 5.

Mod Proposal 5: For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement).

- Alt. 1 (proposed by ZTE, Nokia online):

  - The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI.

References

Draft Report of 3GPP TSG RAN WG1 #107b-e.

R1-2200776 Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT