3GPP TSG RAN WG1 #108-e                	                      R1-2201555

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:  8.8.1.3

Source:	Spreadtrum Communications

Title:	Discussion on joint channel estimation for PUSCH

Document for:	Discussion and decision

Introduction

In this contribution, we continue discussing the remaining issues in joint channel estimation for PUSCH.

TPC command

According to the discussion in last meeting, the power control procedure still needs further study.

Capture the following observations in Chair’s notes. Observations: - Clarification on whether absolute TPC command is supported for group common TPC with DCI format 2_2 for Rel-15/16 is needed. - If supported, whether the timeline of absolute TPC command follows the accumulate TPC command. - Clarification on the interpretation of the definition of KPUSCH(i) for DG-PUSCH in TS 38.214 for Rel-15/16 is needed. - Interpretation 1: KPUSCH(i) is defined as the number of OFDM symbols after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission occasion i. With this interpretation, value of KPUSCH(i) for a PUSCH transmission occasion is different from the one for another PUSCH transmission occasion among the same set of PUSCH repetitions for a TB. - Interpretation 2: KPUSCH(i) is defined as the number of OFDM symbols after a last symbol of a corresponding PDCCH reception and before a first symbol of the first PUSCH repetition for a TB. With this interpretation, value of KPUSCH(i) for all PUSCH transmission occasions are the same for the TB.

For the first observation in the above proposal, we agree with the FL that absolute TPC command is supported for group common TPC with DCI format 2_2 for Rel-15/16. For the second observation, we think Interpretation 1 is the right one. Considering this is Rel-17 item, those two observations are related with Rel-15, so it would be good to align our understandings first. And whether the explicit observation can be in chairman note, we are both fine with them.

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

Since companies’ viewpoints are not aligned for the timeline of absolute TPC command. We prefer agreeing the first sub-bullet, and discussing the second sub-bullet until reaching the consensus on absolute TPC command.

Proposal 1. Confirm the 1st sub-bullet in working assumption.

Dynamic & semi-static events

Two questions concerning dynamic & semi-static events have been discussed during last meeting.

- Case 1: A semi-static event is triggered after one or multiple dynamic events. Whether a new actual TDW is created after the semi-static event? - Case 2: A semi-static event overlaps with a dynamic event. Whether a new actual TDW is created after the semi-static event?

For case 1, since semi-static event can always be known with the knowledge of high layer signaling, UE can have an a priori preparation for creating a new TDW. Similarly, for case 2, UE can still create a new actual TDW after the semi-static event.

Proposal 2. For UE not capable of restarting DM-RS bundling，UE can create a new actual TDW for both cases:

- Case 1: A semi-static event is triggered after one or multiple dynamic events.

- Case 2: A semi-static event overlaps with a dynamic event.

Dynamic & semi-static events

For a reduced capability half-duplex UE in paired spectrum, a lot of discussions were made during RAN1 107b-e meeting.

We want to make more clarifications here:

- For HD-FDD RedCap UEs, an event is constituted if the gap between two consecutive PUSCH repetitions overlap with any symbol of an SS/PBCH block provided by ssb-PositionInBurst. We support this case. It is valid that when there are SSB overlapping or not overlapping with PUSCH repetition, there is an event. So we support proposal 10. Besides, PUCCH repetitions also should be included. It is fine to include in the following proposal, or separately discussed in PUCCH AI.

Proposal 10: - For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH repetitions overlaps with any symbol of an SS/PBCH block provided by ssb-PositionInBurst even if neither of the repetitions overlaps with it.

- For HD-FDD RedCap UEs, an event is constituted if the gap between two consecutive CG-PUSCH repetitions overlap with any symbol of CSI-RS or PDSCH provided by DCI. For this case, CG-PUSCH would be canceled on the conditions. As shown below in 38.213. Furthermore, an event is constituted for higher layer configured PUCCH repetitions.

38.213 clause 17.2 Half-Duplex UE in paired spectrum If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols and the UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the HD-UE does not expect to cancel the transmission of…

- For HD-FDD RedCap UEs, an event is constituted if the gap between two consecutive PUSCH repetitions overlap with any symbol of PDCCH, or PDSCH, or CSI-RS, or DL PRS configured by higher layers when the PUSCH repetitions do not overlap with it. For this case, HD-FDD UE would receive the DL and constitute an event. As shown below in 38.213. There would be no collision for DL and UL when they are both configured by higher layer. Furthermore, an event is also constituted for higher layer configured PUCCH repetitions.

38.213 clause 17.2 Half-Duplex UE in paired spectrum If a HD-UE is configured by higher layers to receive a PDCCH, or PDSCH, or CSI-RS, or DL PRS in a set of symbols, the HD-UE receives the PDCCH, or PDSCH, or CSI-RS, or DL PRS if the HD-UE does not detect a DCI format that indicates to the HD-UE to transmit a PUSCH, or PUCCH, or PRACH, or SRS in at least one symbol of the set of symbols; otherwise, the HD-UE does not receive the PDCCH, or PDSCH, or CSI-RS, or DL PRS in the set of symbols.

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH repetitions overlaps with any symbol of an SS/PBCH block provided by ssb-PositionInBurst even if neither of the repetitions overlaps with it.

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive CG-PUSCH repetitions overlaps with any symbol of a CSI-RS or PDSCH indicated by DCI even if neither of the repetitions overlaps with it.

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted if the gap between two consecutive PUSCH repetitions overlap with any symbol of PDCCH, or PDSCH, or CSI-RS, or DL PRS configured by higher layers when the PUSCH repetitions do not overlap with it.

We are fine with the general description from Ericsson:

For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH repetitions overlaps with any symbol of downlink reception or downlink monitoring an SS/PBCH block provided by ssb-PositionInBurst even if neither of the repetitions overlaps with it.

Proposal 3.For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH repetitions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it.

So beside the events capture in the 38214, a downlink reception or downlink monitoring based on higher layer singling or DCI for a reduced capability half-duplex UE in paired spectrum also need to treat as an event. Furthermore, it belongs to Rel-17 collision rules. Then a reference to this section should be added.

6.1.7	UE procedure for determining time domain windows for bundling DM-RS

< Unchanged part is omitted >

Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are:

< Unchanged part is omitted >

-	For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1, clause 11.2A and clause 17.2 of [6, TS 38.213].

-	For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9.2.6, clause 11.1 and clause 17.2 of [6, TS 38.213].

< Unchanged part is omitted >

Conclusions

In this contribution, we discuss joint channel estimation over multi-PUSCH with the following observations and proposals:

Observation 1.	For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH repetitions overlaps with any symbol of an SS/PBCH block provided by ssb-PositionInBurst even if neither of the repetitions overlaps with it.

Observation 2.	For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive CG-PUSCH repetitions overlaps with any symbol of a CSI-RS or PDSCH indicated by DCI even if neither of the repetitions overlaps with it.

Observation 3.	For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted if the gap between two consecutive PUSCH repetitions overlap with any symbol of PDCCH, or PDSCH, or CSI-RS, or DL PRS configured by higher layers when the PUSCH repetitions do not overlap with it.

Proposal 1. Confirm the 1st sub-bullet in working assumption.

Proposal 2. For UE not capable of restarting DM-RS bundling，UE can create a new actual TDW for both cases:

- Case 1: A semi-static event is triggered after one or multiple dynamic events.

- Case 2: A semi-static event overlaps with a dynamic event.

Proposal 3.For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH repetitions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it.

References

Draft Report of 3GPP TSG RAN WG1 #107b-e

R1-2200790	FL Summary#3 of joint channel estimation for PUSCH	Moderator (China Telecom)