3GPP TSG RAN WG1 Meeting #108-e	R1-2201564

e-Meeting, February 21th – March 3rd, 2022

Title:	Draft reply LS on UE capability for supporting single DCI transmission schemes for multi-TRP

Response to:	R1-2200872

Release:	Release 16

Work Item:	NR_eMIMO-Core

Source:	LG Electronics [RAN1]

To:	RAN4

Contact Person:

Name:

E-mail Address:

Attachments:	None

Introduction

RAN1 would like to thank RAN4 for the question on UE capability for supporting single DCI transmission schemes for multi-TRP in Rel-16, and would like to provide a response as below.

Response to the question regarding UE capability for supporting single DCI transmission schemes for multi-TRP in Rel-16

R1-2200872: RAN4 requests RAN1 to clarify if explicit signaling of maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes or the UE capability of supportFDM-SchemeA-r16 or singleDCI-SDM-scheme-r16 is sufficient.

Response: Explicit signaling of maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes. The reason that maxNumberTCI-states-r16 was defined for single DCI based inter-slot TDM was to consider both single TRP based TDM repetition and multi-TRP based TDM repetition. However, unlike single DCI based inter-slot TDM, single TRP transmission was not a target scenario of single DCI FDM and SDM transmission schemes. Consequently, maxNumberActiveTCI-PerBWP other than n1 is required for single DCI FDM and SDM transmission schemes.

Actions to RAN4

RAN1 respectfully requests RAN4 to take the above information into account.

Date of next TSG-RAN WG1 meetings:

TSG-RAN WG1 Meeting#109-e,				May 	16th – 27th, 2022