3GPP TSG RAN WG1 #108-e		R1-2201579

e-Meeting, February 21st – March 3rd, 2022

Agenda Item: 	8.3.1

Source: 	Sony

Title: 	Remaining issues on HARQ-ACK enhancements for URLLC

Document for: 	Discussion / decision

# Introduction

We have the following conclusion in RAN1#107e:

Conclusion

For SPS HARQ-ACK deferral, if a UE is not configured with Rel-17 intra-UE multiplexing but configured with Rel-16 PHY prioritization, the UE first performs Rel-16 UCI multiplexing and PHY prioritization in both initial slot and target slot and if a LP SPS HARQ-ACK PUCCH is deprioritized, the LP SPS HARQ-ACK is not deferred.

- Note: If the SPS HARQ-ACK is deprioritized in any slot, no further deferral.

In this contribution, we discuss the joint operations of SPS HARQ-ACK deferral and intra-UE multiplexing of different L1 priorities.

# Discussions

It was concluded that HARQ-ACK SPS of different L1 priorities are independently deferred to target PUCCH of the same L1 priority when Rel-17 intra-UE UCI multiplexing is not enabled.  However, we have not concluded whether or not joint operation of SPS HARQ-ACK deferral and Rel-17 intra-UE UCI multiplexing is supported.  SPS HARQ-ACK deferral is introduced to avoid dropping of SPS HARQ-ACK due to semi-static slot configurations which can happen regardless of whether Rel-17 intra-UE UCI multiplexing is enabled or not.  Hence, we should support joint operations of SPS HARQ-ACK deferral and Rel-17 intra-UE UCI multiplexing.

Proposal 1: Support joint operations of SPS HARQ-ACK deferral and Rel-17 intra-UE UCI multiplexing of different L1 priorities.

For the case where the deferred SPS HARQ-ACKs contain only LP HARQ-ACKs, then if Rel-17 intra-UE UCI multiplexing is enabled, these deferred LP HARQ-ACKs can have a target PUCCH that is LP or HP.  On the other hand, if the deferred SPS HARQ-ACKs contain HP HARQ-ACKs, a target PUCCH that is LP may not provide the reliability required for URLLC.  The target PUCCH is the first available PUCCH and avoiding multiplexing deferred HP HARQ-ACK into a target LP PUCCH increases its latency which may not be beneficial for URLLC operations.

Observation 1: When Rel-17 intra-UE UCI multiplexing is enabled, if the deferred SPS HARQ-ACKs contain HP HARQ-ACKs, a first available PUCCH that is LP may not provide the required reliability for the HP HARQ-ACKs.  However, avoiding the first available LP PUCCH leads to increase in latency.

When two PUCCHs of different L1 priorities collide and Rel-17 intra-UE UCI multiplexing is enabled, the UCIs of different L1 priorities are multiplexed into a HP PUCCH, i.e. the PUCCH resource is selected from the “2nd PUCCH Config”, to ensure the reliability of the multiplexed UCIs.  Hence to ensure the reliability is maintained, if the deferred SPS HARQ-ACKs contain HP HARQ-ACKs, then the UE selects a PUCCH resource for the target PUCCH from the “2nd PUCCH Config”, even if the target PUCCH is originally scheduled as LP-PUCCH.

Proposal 2: When Rel-17 intra-UE UCI multiplexing is enabled and if the deferred SPS HARQ-ACKs contain HP HARQ-ACKs, the resource for the target PUCCH is selected from the 2nd PUCCH Config, regardless of the L1 priority of the originally scheduled target PUCCH.

# Conclusion

In this contribution, we discuss some remaining issues on HARQ-ACK enhancements, and we observe the following:

Observation 1: When Rel-17 intra-UE UCI multiplexing is enabled, if the deferred SPS HARQ-ACKs contain HP HARQ-ACKs, a first available PUCCH that is LP may not provide the required reliability for the HP HARQ-ACKs.  However, avoiding the first available LP PUCCH leads to increase in latency.

We therefore propose the following:

Proposal 1: Support joint operations of SPS HARQ-ACK deferral and Rel-17 intra-UE UCI multiplexing of different L1 priorities.

Proposal 2: When Rel-17 intra-UE UCI multiplexing is enabled and if the deferred SPS HARQ-ACKs contain HP HARQ-ACKs, the resource for the target PUCCH is selected from the 2nd PUCCH Config, regardless of the L1 priority of the originally scheduled target PUCCH.

# References

[1] R1-2112758, “Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT,” 	Moderator (Nokia), RAN1#107e