3GPP TSG RAN WG1 Meeting#108-e		R1-2201592

e-Meeting, Feb. 21st – Mar. 3rd, 2022

Agenda item:	8.12.1

Source:	TD Tech, Chengdu TD Tech

Title:	Discussion on RAN2 LS on MBS SPS

Document for:	Discussion and Approval

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e to provide MBS in a cell. In the RAN2#116bis-e meeting, an LS from RAN2 to RAN1 is sent in R2-2202016. In the LS, the following two questions are provided.

Q1:RAN2 respectfully asks RAN1 to confirm RAN2’s understanding and the maximal number of G-CS-RNTI configured for UE? Can multiple G-CS-RNTIs be mapped to same MBS SPS-config and if so how that would work?

Q2: Clarify the handling of PTM retransmissions.

In the contribution, the answers to the two questions are suggested.

- Question 1

The multiple-to-one mapping between G-CS-RNTI and MBS SPS-config is supported in RAN1. But RAN2 doesn't understand how multiple G-CS-RNTIs can be mapped to a MBS SPS-config. From RAN2 side, an MBS SPS-config can only be used by one G-CS-RNTI. But in RAN1, multiple G-CS-RNTIs can use a same MBS SPS-config in TDM mode. That is, different G-CS-RNTIs can use a same MBS SPS-config during different time intervals which are not overlapped with each other. At some time starting point, an MBS SPS-config can only be activated by a G-CS-RNTI. Then it is used by the G-CS-RNTI until it’s deactivated by the G-CS-RNTI. After it’s deactivated by the G-CS-RNTI, it can be reactivated by the G-CS-RNTI or activated by a second G-CS-RNTI. If it’s reactivated by the G-CS-RNTI, it is reused by the G-CS-RNTI again until it’s deactivated again by the G-CS-RNTI. If it’s activated by a second G-CS-RNTI, it is used by the second G-CS-RNTI until it’s deactivated by the second G-CS-RNTI. Therefore, the following proposal is suggested to clarify the multiple-to-one mapping between G-CS-RNTI and MBS SPS-config.

Proposal 1: RAN1 confirms the multiple-to-one mapping between G-CS-RNTI and MBS SPS-config. The maximum number of G-CS-RNTIs configured for UE can be set as a UE’s capability. For example, the maximum number of G-CS-RNTIs can be 2, 4 or 8, where only one G-CS-RNTI is counted if the multiple-to-one mapping between G-CS-RNTI and MBS SPS-config is applied for a set of G-CS-RNTIs. RAN1 further clarifies the multiple-to-one mapping between G-CS-RNTI and MBS SPS-config works in the following way.

Multiple G-CS-RNTIs can be mapped to a same MBS SPS-config in TDM mode. Different G-CS-RNTIs use a same MBS SPS-config in different time intervals which are not overlapped with each other. At some time starting point, a same MBS SPS-config can only be activated by a G-CS-RNTI and used by the G-CS-RNTI until it’s deactivated by the G-CS-RNTI. After the deactivation, it can be re-activated by the G-CS-RNTI or activated by a second G-CS-RNTI. When it’s activated by a second G-CS-RNTI, it’s used by the second G-CS-RNTI until it's deactivated by the second G-CS-RNTI.

- Question 2

Based on the agreement in the past RAN1 meetings, a TB of an MBS session can be retransmitted with PTM and/or PTP. The PTP mode can be configured per UE by the RRC signalling. If the PTP mode is configured for a UE, whether or not the PTP mode is used to retransmit a TB to the UE is up to the gNB’s decision. If a TB is retransmitted with PTP, CS-RNTI of the UE is used to scramble the PDCCH/PDSCH carrying the scheduling information/TB. If the UE is configured with the PTP mode for multiple G-CS-RNTIs, a same CS-RNTI of the UE is used for the retransmissions of all G-CS-RNTIs configured with the PTP mode.

If the PTM mode is configured for the MBS session by the RRC signalling, each UE can receive the MBS session with PTM mode. But whether or not the PTM mode is used to retransmit a TB is up to the gNB’ decision.

Based on the above discussion, the following proposal is suggested to answer question 2.

Proposal 2: The PTM mode can be configured by RRC signalling per MBS session. The PTP mode can be configured by RRC signalling per UE per MBS session. If the PTM/PTP mode is configured, the mode for the retransmission of a TB can be changed per TB or per TB per retransmission. For the retransmission of a TB with PTP, CS-RNTI of UE is used to scramble the PDCCH/PDSCH carrying the scheduling information/TB. If the PTP mode is configured for multiple G-CS-RNTIs monitored by UE, a same CS-RNTI is used for the PTP retransmission of all these G-CS-RNTIs.

- Conclusion

Based on the discussion on the questions in the RAN2 LS in R2-2202016, the response LS is suggested to include the following proposals:

Proposal 1: RAN1 confirms the multiple-to-one mapping between G-CS-RNTI and MBS SPS-config. The maximum number of G-CS-RNTIs configured for UE can be set as a UE’s capability. For example, the maximum number of G-CS-RNTIs can be 2, 4 or 8, where only one G-CS-RNTI is counted if the multiple-to-one mapping between G-CS-RNTI and MBS SPS-config is applied for a set of G-CS-RNTIs. RAN1 further clarifies the multiple-to-one mapping between G-CS-RNTI and MBS SPS-config works in the following way.

Multiple G-CS-RNTIs can be mapped to a same MBS SPS-config in TDM mode. Different G-CS-RNTIs use a same MBS SPS-config in different time intervals which are not overlapped with each other. At some time starting point, a same MBS SPS-config can only be activated by a G-CS-RNTI and used by the G-CS-RNTI until it’s deactivated by the G-CS-RNTI. After the deactivation, it can be re-activated by the G-CS-RNTI or activated by a second G-CS-RNTI. When it’s activated by a second G-CS-RNTI, it’s used by the second G-CS-RNTI until it's deactivated by the second G-CS-RNTI.

Proposal 2: The PTM mode can be configured by RRC signalling per MBS session. The PTP mode can be configured by RRC signalling per UE per MBS session. If the PTM/PTP mode is configured, the mode for the retransmission of a TB can be changed per TB or per TB per retransmission. For the retransmission of a TB with PTP, CS-RNTI of UE is used to scramble the PDCCH/PDSCH carrying the scheduling information/TB. If the PTP mode is configured for multiple G-CS-RNTIs monitored by UE, a same CS-RNTI is used for the PTP retransmission of all these G-CS-RNTIs.

- References

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e

- R2-2202016, LS on MBS SPS, RAN2 Meeting#116bis-e