3GPP TSG RAN WG1 #108-e		      		   	   R1- 2201612

e-Meeting, February 21th – March 3rd, 2022

Source: 	ETRI

Title: 	Intra-UE Multiplexing/Prioritization

Agenda Item:	8.3.3 Intra-UE Multiplexing/Prioritization

Document for:	Discussion

- Introduction

In this contribution, we address our views about details of both PUCCH and PUSCH focusing on the multiplexing framework.

- Discussion

PUCCH multiplexing in Step 2.1

Agreement For resolving collision of PUCCHs of different priorities without repetition within a time unit, down-select from the following options: - Option 1: - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration - In step 2.1-2, select up to one PUCCH resource overlapping with the reference PUCCH resource according to Rel-15 pseudo code - Option 2: - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code - Option 3: - The reference PUCCH resource is determined by prioritizing HP PUCCH over LP PUCCH on top of Rel-15 rules - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code - Option 4: - The reference PUCCH resource is determined by prioritizing LP PUCCH carrying HARQ-ACK on top of Rel-15 rules - In step 2.1-2, If a LP PUCCH carrying HARQ-ACK overlaps with multiple HP PUCCHs and one of the HP PUCCH includes HARQ-ACK, only select the HP PUCCH including HARQ-ACK in step 2.1-2; otherwise, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code FFS: Details on time units for all options

Above options has different approaches to choose a reference resource and to collect overlapped resources. We would like to focus on whether UCI type is dependent on this agreement. In our understanding, the legacy behavior would depend on time resource of a PUCCH with a single priority index, and the UE PHY may not need to distinguish UCI payload in terms of UCI type. In this perspective, option 4 may need additional implementation.

Regarding pairwise/all resource for resolution, a result PUCCH of both HP and LP UCI does not collide with other HP PUCCH and HP PUSCH, following the agreement/conclusion in the last meeting. We understand either way has a different resultant HP PUCCH but its performance difference can be minimal. Thus, we would prefer the option 3.

Working Assumption For resolving collision of PUCCHs of different priorities without repetition within a time unit, the time unit of HP HARQ-ACK is used. For a LP PUCCH overlapping with multiple time units, down-select from: - Alt. 1: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s) - Alt. 2: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH with HARQ-ACK if any. Otherwise, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s). - Alt. 3: the LP PUCCH is associated with the last time unit with overlapping HP PUCCH(s)

Regarding the above working assumption, we suggest to agree Alt 1 because of its simplicity. Alt 2 has to check UCI type during Step 2.2. As far as we understand, the Rel-15 also check UCI type, i.e., positive SR PUCCH is considered to form the set Q. This legacy behaviour is feasible because UE MAC layer determines to deliver or not positive/negative SR. Within the timeline constraint, the UE PHY is given all UL resources for further processing.

However, the above Alt 2 seems to require UE PHY to distinguish UCI payload in terms of UCI type. In our understanding, it impacts the implementation, whereas Alt 1 or Alt 3 may simply choose the first or the last time unit. Though we also note that LP HARQ may be dropped in Alt 1/3 but it may be overcome by the gNB, e.g., by retransmitting the LP HARQ codebook. Between Alt 1 and Alt 3, we prefer Alt 1 because the timeline condition is satisfied for all channels and LP HARQ can be fed back faster.

Agreement To apply Rel-17 intra-UE multiplexing/dropping rules to resolve overlapping among the reference PUCCH resource and O PUCCH resource(s) in step 2.1-3, LP PUCCH(s) without HARQ ACK are dropped before multiplexing if multiplexing is to be performed. Proposal 1.3.18: from the feature lead summary R1-2200772 To resolve overlapping between a HP PUCCH and a LP PUCCH with HARQ-ACK and CSI/SR, - If the HP PUCCH includes HP HARQ-ACK, LP CSI/SR in LP PUCCH is dropped and LP HARQ-ACK is multiplexed with HP HARQ-ACK according to Rel-17 rule; - If the HP PUCCH includes HP SR only, LP PUCCH with PF2/3/4 is dropped; - FFS LP PUCCH with LP HARQ-ACK and LP SR with PF 0/1

The above agreement clarifies to drop LP CSI/LP SR in Step 2.1-3, and the proposal 1.3.18 also clarifies in its first bullet. A resultant LP PUCCH after Step 1 may have many UCI types, and if a UE is able to drop some UCI types then the UE should keep track of UCI types during Step 1 and Step 2.1 at least for LP UCI.

For the second bullet, since the negative SR if overlapped with other UCIs may not be considered from Step 1, the positive HP SR is included in HP PUCCH and can drop LP PUCCH of format 2/3/4. The positive HP SR is in format 0/1 and we would suggest to reformulate based on the format, i.e., if the HP PUCCH is format 0/1, then LP PUCCH with format 2/3/4 is dropped. This could simplify the proposal because the UCI type is not dependent. Also, in perspective of using UCI types during Step 2.1, the modified texts may hold irrespective of HP UCI types.

Proposal 1: Determine to allow the Step 2.1 to depend on UCI types or not.

LP PUSCH overlapping with HP PUCCH including positive SR

Working Assumption For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH which carries positive SR are dropped before UCI multiplexing. - Step 1.2 behavior is not affected by the above

The above working assumption focuses on HP PUCCH with positive SR. One clarification may be HARQ-ACK bits with multiple SR bits, i.e., L bits. According to the working assumption, when the UE MAC delivers at most one positive HP SR in HP PUCCH, the LP PUSCHs would be dropped. Since the HP SR is not multiplexed on LP PUSCH, the working assumption may be a natural consequence.

Considering negative SR cases where UE MAC delivers all zero bits for the SR payload, the LP PUSCH may or may not be dropped. If LP PUSCH is transmitted, then this implies that the SR payload in the HP PUCCH determines whether to multiplex/drop LP PUSCH, which is not desirable in our view. We prefer to describe the behavior in terms of format because the legacy specification is usually dependent on format to multiplex HARQ-ACK/CSI/SR.

Proposal 2: If HP SR is included in HP PUCCH of format 2/3/4, then the LP PUSCH(s) overlapping with HP PUCCH are dropped before UCI multiplexing.

Interaction with PUCCH repetitions

Proposed Conclusion 1.8.1: from the feature lead summary R1-2200772 For resolving collision of two overlapping channels with different priorities in Step 2, - If a LP PUCCH overlaps with only one HP PUCCH and the LP or HP PUCCH is with repetitions, the LP PUCCH is dropped. - If a LP PUSCH overlaps with a HP PUCCH with repetitions, the LP PUSCH is dropped. - If a HP PUSCH overlaps with a LP PUCCH with repetitions, the LP PUCCH is dropped. Note: the dropping of LP PUCCH/PUSCH is performed per repetition.

In the previous meeting, UCI repetition is discussed but not reached agreements. The legacy specification allows only one UCI type to repeat, and in Rel-17 the repetition factor is associated with a PUCCH resource and this implies some UCI types are repeated together from the first repetition.

We agree with the second and the third bullet, and we want to clarify that at the first bullet. We think that a PUCCH with repetition has been already transmitted and the Step 2 is considered for its second repetition or later. In our understanding, the resultant PUCCH after Step 2 would have the repetition factor and a resultant PUCCH resource after Step 1 may have a repetition factor but it is not effective yet.

Proposal 3: Determine whether to distinguish the first repetition or the later repetition if some of overlapped PUCCH is repeated. If a LP PUCCH overlaps with only one HP PUCCH and the LP or HP PUCCH is with repetitions and is not the first repetition, the LP PUCCH is dropped.

Proposal 1.8.2: from the feature lead summary R1-2200772 For resolving collision of PUCCHs and/or PUSCHs with different priorities in Step 2.1, first resolve collision of PUCCHs without repetitions and then resolve collision of PUCCHs (with or without repetitions). Proposal 1.8.3: from the feature lead summary R1-2200772 For resolving collision of PUCCHs and/or PUSCHs with different priorities in Step 2.2, first resolve collision of PUCCHs with repetitions and PUSCHs, and then resolve collision of PUCCHs without repetitions and PUSCHs.

Regarding above proposals, the similar consideration can be applied. Depending on whether the first repetition or second (or later) repetitions, the multiplexing behavior can be different. In this perspective, the proposal 1.8.3 can be modified to put ‘if the repetition is not the first repetition’ in appropriate positions.

Interaction with ULCI:

We can further consider ULCI in the DCI format 2_4, which can drop/stop PUSCH/SRS transmissions provided that the timeline is satisfied. In the Rel-16 discussion, the PUCCH is not affected by the ULCI but PUSCH can be affected regardless of presence/absence of UCI bits, assuming that uplinkCancellationPriority is present.

The LP PUSCH with HP UCI can be cancelled because the ULCI does not distinguish LP UCI or HP UCI. Frequent dropping HP UCI with LP PUSCH by DCI format 2_4 can be relieved by disabling this type of multiplexing by DCI format 0_1/0_2. This approach is valid but may not be reasonable or not efficient because LP PUSCH is not scheduled even if the gNB cannot predict the ULCI transmission.

The alternative approach is to consider one more case if HP UCI is included in LP PUSCH. We suggest that when HP UCI is present, the LP PUSCH may not be affected by the ULCI.

Proposal 4: If HP UCI is included in LP PUSCH, the ULCI may not cancel the PUSCH transmission.

- Conclusion

In this contribution, we would like to propose the followings:

Proposal 1: Determine to allow the Step 2.1 to depend on UCI types or not.

Proposal 2: If HP SR is included in HP PUCCH of format 2/3/4, then the LP PUSCH(s) overlapping with HP PUCCH are dropped before UCI multiplexing.

Proposal 3: Determine whether to distinguish the first repetition or the later repetition if some of overlapped PUCCH is repeated. If a LP PUCCH overlaps with only one HP PUCCH and the LP or HP PUCCH is with repetitions and is not the first repetition, the LP PUCCH is dropped.

Proposal 4: If HP UCI is included in LP PUSCH, the ULCI may not cancel the PUSCH transmission.