3GPP TSG RAN WG1 Meeting #108-e					            R1-2201642

e-Meeting , February 21st – March 3rd, 2022

Agenda Item:	8.7.1.2

Source:	Panasonic

Title:	Remaining issues on enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs

Document for:	Discussion/Decision

# Introduction

For TRS enhancement for IDLE/INACTVE UEs, the conclusions/agreements in the appendix were achieved in RAN1#107bis-e meeting [1]:

This contribution provides our views on the remaining details of enhancement for TRS/CSI-RS occasion(s) for idle/inactive UEs.

# Discussion

On UE default assumption of TRS availability

In the previous meeting, it was discussed whether/how to specify UE default assumption of TRS availability in the below cases:

- A UE just camps on a cell or received an updated TRS configuration, before the UE receives a L1 TRS availability indication.

- During the system modification period and before the new SIBs become valid, a UE can not acquire the up to date TRS configuration but still stores previous TRS configuration.

In our view, although the TRS resource should be regarded not reliable for AGC and T/F synchronization, it is not necessary to specify the UE behaviour on whether to assume the TRS is available or not. The reasons are:

- The above cases take small portion of the whole UE operation time and thus the gain for power saving is neglectable.

- To mandatory specify the UE behaviour would prevent UE from possible smart implementation to handle these cases to achieve better performance. For example, if gNB shares the TRS in a more static or long term way, UE may detect by using previous configuration/information and estimate the TRS actually availability. Such possibility should be up to UE implementation. UE may choose whether to assume the TRS is available by its own strategy.

Therefore,

Proposal 1: UE default assumption of TRS availability should be described as "may" instaed of "shall", if either of SIB configuration or L1 indication is temporarily absent or unreliable in below cases:

- A UE just camps on a cell or received an updated TRS configuration, before the UE receives a L1 TRS availability indication.

- During the system modification period and before the new SIBs become valid, a UE can not acquire the up to date TRS configuration but still stores previous TRS configuration.

On number of configured TRS resource set

In the below agreement [2], it is still FFS whether the number of configured TRS resource set should be limited by ssb-PositionsInBurst in SIB1.

Agreement For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 - FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

In our understanding, the FFS bullet is not so clear.

- It potentially implies an intention that for each SSB beam index configured in ssb-PositionsInBurst, only one TRS resource set is configured.

- Or if more than one TRS resource set are allowed to be configured QCLed with an SSB index, the total number of TRS resource set can still meet the condition in the FFS bullet. But just for some SSB index, there is no TRS resource set configured.

In our opinion, as the TRS is shared from RRC CONNECTED mode UEs, how many TRS resource is configured to QCLed with an SSB index is up to gNB implementation. With the maximum number of TRS resource sets already defined, the system works fine.

Proposal 2: The maximum number of configured TRS resource sets is 64 and no need to limit by the number of actual transmitted SSBs in ssb-PositionsInBurst.

On whether/how to align the reference point for paging and PEI based TRS availability indication

It has been agreed that paging and PEI based TRS availability indication would both be utilized if SIB configures additional TRS occasions for IDLE/INACTIVE UE. So based on the below agreement [2], it remains open on how to define the reference point as the start of the validity period.

Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration Note: Qualcomm and Huawei have concern with above agreement

For the first PF of a paging cycle, it is highly possible that the associated PEI-O locates in the previous paging cycle, considering the frame-level offset is defined as 0~16 radio frame. So if it happens, the resulted reference point based on the above agreement would be different for the PEI and paging DCI, which are the SFN of the first PF of paging cycle #N and the SFN of the first PF of paging cycle #N+1.

From the network perspective, it is possible that a certain TRS resource set group is ON in paging cycle #N and it is turned OFF in paging cycle #N+1, as this paging cycle boundary may be also a boundary of validity period based on SIB configuration. Based on the agreed mechanism, gNB has two choices:

- gNB does not transmit PEI

- gNB transmits PEI indicating ‘0’ for this TRS resource set group. UE should also be fine even it only receives PEI and skips receiving paging

In our observation, both options may work but the first option would lose some power saving gain.

On the other hand, if a certain TRS resource set group is OFF in paging cycle #N and it is turned ON in paging cycle #N+1, what gNB can do are basically the same:

- gNB does not transmit PEI. Or

- gNB transmits PEI indicating ‘0’ for this TRS resource set group and in the next paging cycle, indicates ‘1’ in the associated PO. However, this may further lead to two issues:

  - This misalignment of the reference point of the L1 availability indications in the PEI and paging DCI leads to inconsistency between paging DCI and associated PEI based TRS availability indication. This would need further standardization efforts to deal with this aspect, e.g. to decide whether this is allowed and what is the UE behaviour in case UE detects inconsistent indication.

  - It is also possible that UE may only detects PEI without receiving paging. This may lead to an undesirable consequence that UE is not able to use TRS even it is transmitted by gNB, which is similar to miss detection. How long this consequence would last depends on whether and when gNB repeat the L1 indication within the validity duration.

Observation 1: In the case that PEI and associated POs locate in different paging cycle, if reference point is not aligned for the L1 availability indications in the PEI and paging DCI, below issues happen:

- gNB may skip the PEI transmission, which is harmful for UE power saving gain.

- Inconsistency between paging DCI and associated PEI based TRS availability indication may be required, which needs additional standardization efforts and potential specification impact.

- The UEs whose POs are within the first POnumPerPEI POs may lose power saving gain due to not being able to utilize the TRS.

Therefore, from the above analysis and to avoid the potential issues as mentioned, we propose to align the reference point between PEI and paging DCI based L1 TRS availability indication with a straightforward manner as below:

Proposal 3: For both PEI and associated paging based TRS availability indication, the reference point for start of the validity duration is SFN of the first PF of the current default DRX cycle corresponding to the first MO of PO.

For the consistency between PEI and paging DCI based indication, as we explained also above,

Proposal 4: UE assumes the TRS availability indication consistent between PEI and associated paging DCI. If UE detects inconsistent indication, this is treated as error case and no need to specify.

# Conclusion

- Based on the discussion, the following proposals are highlighted:

Proposal 1: UE default assumption of TRS availability should be described as "may" instaed of "shall", if either of SIB configuration or L1 indication is temporarily absent or unreliable in below cases:

- A UE just camps on a cell or received an updated TRS configuration, before the UE receives a L1 TRS availability indication.

- During the system modification period and before the new SIBs become valid, a UE can not acquire the up to date TRS configuration but still stores previous TRS configuration.

Proposal 2: The maximum number of configured TRS resource sets is 64 and no need to limit by the number of actual transmitted SSBs in ssb-PositionsInBurst.

Observation 1: In the case that PEI and associated POs locate in different paging cycle, if reference point is not aligned for the L1 availability indications in the PEI and paging DCI, below issues happen:

- gNB may skip the PEI transmission, which is harmful for UE power saving gain.

- Inconsistency between paging DCI and associated PEI based TRS availability indication may be required, which needs additional standardization efforts and potential specification impact.

- The UEs whose POs are within the first POnumPerPEI POs may lose power saving gain due to not being able to utilize the TRS.

Proposal 3: For both PEI and associated paging based TRS availability indication, the reference point for start of the validity duration is SFN of the first PF of the current default DRX cycle corresponding to the first MO of PO.

Proposal 4: UE assumes the TRS availability indication consistent between PEI and associated paging DCI. If UE detects inconsistent indication, this is treated as error case and no need to specify.

# Reference

[1] RAN1 Chair’s Notes of 3GPP RAN1#107bis-e

[2] RAN1 Chair’s Notes of 3GPP RAN1#107-e

# Appendix

Agreements achieved in RAN1#107bis-e meeting

Agreement

Support a configuration parameter for the number of, X, TRS resources for a TRS resource set at least for FR2

- Applicable values for X: {2, 4}

- For FR1, X is based on configuration parameter with applicable value {2, 4}

Note: Configuration follows restriction specified in sub-clause 5.1.6.1.1 in TS38.214 for connected mode TRS

Agreement

One or more scrambling IDs is configured for a TRS resource set.

- If a single scrambling ID is configured, it applies to all the TRS resources.

- Otherwise, each TRS resource is provided with a scrambling ID.

Agreement

The applicable values:{64, 128, 256, 512} are supported for the validity duration configured by higher layer,

- Note: If UE is provided a configuration of validity duration longer than 10.24s, and the UE does not support eDRX, it is up to UE implementation whether to assume the validity duration length is no larger than 10.24s

Agreement

UE can receive L1 based signaling for TRS availability indication before the expiration/end of validity duration associated with previous L1 based signaling for TRS availability indication

- For each bit indicated as ‘1’ in the availability indication field of the current L1 based signaling, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 based signaling.

- For each bit indicated as ‘0’ in the availability indication field of the current L1 based signaling, the UE keeps the existing assumption on the availability or unavailability of the corresponding TRS resource set(s).

Note: the validity duration for different group of TRS resources sets correspond to different bits in the availability indication field can be different and are maintained independently.

Note: CATT has concern on the power saving gain based on this agreement.

Agreement

Confirm the following working assumption

Working Assumption

If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration.

Agreement

The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set.

Agreement

If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI

Note: Huawei, MTK and ZTE have concern on the agreement.