3GPP TSG-RAN WG1 Meeting #108-e	R1-2201667

e-Meeting, 21st February – 3rd March 2022

Agenda Item:	5.2

Source:	Ericsson

Title:	RAN1 aspects for NR small data transmissions in INACTIVE state

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the following aspects related to small data transmission (SDT) from UEs in RRC inactive state:

- SDT on the RedCap-specific separate initial BWP

- Periodicities for CG-SDT

- Non-fallback DCI formats on

- Power control updates for CG PUSCH

- SSB to CG PUSCH mapping and CG PUSCH resource configuration

- TA validation and TA offset for CG-SDT

# 2	Discussions

## 2.1 	SDT on the RedCap-specific separate initial BWP

During RAN2#116-bis-e, RAN2 has sent an LS to RAN1 with the following question [1]:

Question to RAN1: RAN2 would like RAN1 to confirm that the separate BWP in case of REDCAP may still be considered as the initial BWP and SDT resources can hence be configured on this BWP for REDCAP UEs.

Regarding the idle/inactive mode operation of RedCap UEs, the following agreements were made in RAN#94-e and RAN2#116-bis-e:

RAN#94-e - Scheme 1 (i.e., UE in IDLE and INACTIVE monitors paging in an initial BWP associated with CD-SSB) is adopted for further work in Rel-17. Scheme 2 (i.e. UE in IDLE and INACTIVE monitors paging in an initial BWP associated with NCD-SSB) is not considered further in Rel-17. RAN2#116bis-e - A RedCap UE in idle/inactive mode monitors paging only in an initial BWP (default or RedCap specific) associated with CD-SSB and performs cell (re-)selection and measurements on the CD-SSB - If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH. - If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, measurements are based on CD-SSB for initial RACH resource selection. - From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission. - RedCap-specific two-step RACH, if configured, and four-step RACH are always configured in the same BWP.

Furthermore, the following agreement has been made by RAN1 in RAN1#107-e in the RedCap WI (a similar agreement was also made for FR2 but is not copied below):

Agreement: - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - […] - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - […]

From the above agreements, it is clear that:

- In idle/inactive states, a RedCap UE can be configured with a separate RedCap-specific initial DL BWP, where it monitors paging and performs cell (re-)selection and measurements. This initial BWP will contain CD-SSB (and hence also CORESET#0).

- A separate RedCap-specific initial DL/UL BWP can be configured for RedCap UEs to perform random access (RA). The separate initial DL BWP where the UE performs RA may not contain CORESET#0, and if it is configured for random-access only, it may not contain also SSB (CD or NCD).

For CG-SDT, the initial DL BWP (separate or same as non-RedCap) paired with the initial UL BWP where the UE is configured with CG PUSCH occasions will contain CD-SSB. In other words, the UE will need to retune for monitoring paging and/or for performing cell (re-)selection and measurements. Therefore, in our view, there are no concerns on configuring the CG-SDT on the separate RedCap-specific initial DL/UL BWP

For RA-SDT, the separate initial DL BWP where the UE executes RA-SDT may not contain SSB and CORESET#0 (see an exemplary configuration in the figure below). However, this is not expected to cause problems as the UE operates on this BWP only for a limited period of time, at least when subsequent transmissions are not needed. When, for example, multiple subsequent transmissions are needed, the NW can choose to move the UE to connected mode so as to avoid interruption due to retuning to SSB/CORESET#0 BW for performing measurements and/or monitoring paging.

Figure 1: An exemplary network configuration with separate RedCap-specific initial UL/DL BWP.

Based on the above considerations, we propose the following:

RAN1 confirms that the separate initial UL/DL BWP for RedCap UEs may still be considered as the initial BWP and SDT resources can hence be configured on this BWP for RedCap UEs.

Send an LS to inform RAN2 and RAN4 that CG/RA-SDT can be configured on the RedCap-specific separate initial UL/DL BWP.

## 2.2 	Periodicities for CG-SDT

In NR connected mode, the maximum periodicity configurable for CG Type 1 is 640 ms. In LTE-PUR, considerable larger periodicities are possible, as can be seen from the following excerpt from 36.331:

PUR-PeriodicityAndOffset-r16 ::= CHOICE {

periodicity8		INTEGER (1..7),

periodicity16		INTEGER (1..15),

periodicity32		INTEGER (1..31),

periodicity64		INTEGER (1..63),

periodicity128		INTEGER (1..127),

periodicity256		INTEGER (1..255),

periodicity512		INTEGER (1..511),

periodicity1024		INTEGER (1..1023),

periodicity2048		INTEGER (1..2047),

periodicity4096		INTEGER (1..4095),

periodicity8192		INTEGER (1..8191)

where the value of periodicity is in the unit of H-SFN duration (i.e., 10.24s). This means that the periodicities vary between 82 seconds and 23 hours (23.3), i.e. an entirely different time scale compared to CG in NR connected mode. Since the use cases for LTE-PUR and CG-SDT are similar and it would be sufficient to rely on CG in connected mode for the shortest periodicities. In fact, in the case of shorter periodicities, it would be more efficient for the network to move the UE to connected mode and configure the UE with CG Type 1/2, with more advanced configuration parameters (e.g., link adaptation, higher modulation order, larger BW, etc.). Note that CG-SDT can only be configured in an initial BWP which in typical configuration is of limited BW, whereas CG Type 1 can be configured on a dedicated BWP which can be much larger. Therefore, it seems reasonable to somewhat align the periodicities for CG-SDT by including longer periodicities.

Based on the above considerations, we propose the following:

Periodicites up to 20.48 seconds should be configurable for CG-SDT

RAN1 to send an LS to RAN2 capturing the allowed periodicities for CG-SDT and indicating that it is up to RAN2 to decide how to define the new periodicities.

## 2.3 	Non-fallback DCI formats on initial BWP for CG-SDT

Based on the updated TS 38.213 specification, a USS set for CG-SDT is configured by sdt-CG-SearchSpace for DCI formats with CRC scrambled by C-RNTI or CS-RNTI.

a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI(s), SL-RNTI, SL-CS-RNTI, or SL Semi-Persistent Scheduling V-RNTI, or configured by sdt-CG-SearchSpace for DCI formats with CRC scrambled by C-RNTI or CS-RNTI as described in clause 19.1.

Currently, it is not possible to schedule UEs using non-fallback DCI formats (0_1/1_1) on an initial DL BWP (for BWP#0 configuration option 1). This is clearly captured in TS 38.331, Annex B.2:

With the first option (illustrated by figure B2-1 below), the BWP#0 is not considered to be an RRC-configured BWP, i.e. UE only supporting one BWP can still be configured with BWP#1 in addition to BWP#0 when using this configuration. The BWP#0 can still be used even if it does not have the dedicated configuration, albeit in a more limited manner since only the SIB1-defined configurations are available. For example, only DCI format 1_0 can be used with BWP#0 without dedicated configuration, so changing to another BWP requires RRCReconfiguration since DCI format 1_0 doesn't support DCI-based switching.

The implication of this is that retransmissions and subsequent transmissions, which are scheduled dynamically, cannot be scheduled using non-fallback DCI formats (0_1/1_1) and only using fallback DCI formats (0_0/1_0). Therefore, for e.g., Type 0 FDRA, precoding, etc., cannot be used for retransmissions and subsequent transmissions.

Discuss whether non-fallback DCI formats can be used to schedule retransmissions and subsequent transmissions for CG-SDT in an initial BWP.

## 2.4 	Power control updates for CG-PUSCH

In RAN1#107-e, it was agreed that the UE specific power control parameters P0 and alpha are configured for the initial UL transmissions.

Agreement - UE specific power control parameters P0 and alpha should be configured for initial UL transmission for CG-SDT - Existing closed loop power control mechanism can be reused for re-transmission and subsequent data transmission.

These parameters can be provided in the RRCRelease message. For possible re-transmissions of the initial UL transmissions and subsequent UL transmissions, legacy closed loop power control mechanism will be re-used. That is, during a re-transmission or subsequent transmission, the UE will receive a TPC update from the gNB, and the UE transmit power is adjusted accordingly.  However, for new transmissions on CG resources, there is no possibility to update the values of P0 and alpha for a CG-SDT configuration while the UE is in inactive state. Since the channel conditions may change during the ongoing CG-SDT procedure, especially in cases where the procedure would be ongoing for a long time, it would be advantageous to be able to update these without sending a new RRCRelease message with updated parameters. An attractive option is to define a new MAC CE to enable updates of P0 and alpha. The MAC CE could be defined in different ways as long as the essential information of new settings of P0 and alpha is included. The procedure is illustrated in Figure 1.

Figure 2 New MAC CE to update P0 and alpha for CG-SDT.

Therefore, we propose the following:

Introduce mechanism to allow semi-static updates of the power control parameters P0 and alpha for CG-SDT. How to provide the semi-static updates (e.g., via MAC-CE) is up to RAN2.

## 2.5 	SSB to CG PUSCH mapping and CG PUSCH resource configuration

### 2.5.1	SSB to CG PUSCH mapping

During the previous RAN1 meetings, the following agreements have been made on SSB to CG PUSCH resource mapping:

Agreement (RAN1 #104-e): - CG resources per CG configuration are associated with a set of SSB(s) configured by explicit signalling. - FFS how to define an SSB-to-PUSCH resource mapping within the CG configuration. - FFS specific changes to the CG configuration to support the additional SSB-to-PUSCH mapping, if any. Agreement (RAN1 #105-e): - The SSB-to-PUSCH resource mapping within the CG configuration is implicitly defined. - The ordering of the SSB and CG PUSCH resources are to be captured in RAN1 spec. - A PUSCH resource refers to a transmission occasion and a DMRS resource used for PUSCH transmission - The ordering of the SSB can reuse from the SSB-to-RO mapping - The ordering of CG PUSCH resources can reuse from that of MsgA PUSCH as much as possible - FFS determination of mapping ratio and association period, e.g., explicitly signaled or implicitly derived - FFS any limitation on the combination of the parameters for CG resources Agreement (RAN1 #106-e): - Each N of consecutive SSB indexes associated to one CG configuration are mapped to valid CG PUSCH resources - first, in increasing order of DMRS resource indexes, where a DMRS resource index DMRSid is determined first in an ascending order of a DMRS port index and second in an ascending order of a DMRS sequence index - second, in increasing order of CG period indexes in the association period - The mapping ratio N is explicitly signalled and the association period is implicitly derived - FFS candidate value set of mapping ratio, and whether it is configured per CG configuration or per cell - The SSB to CG PUSCH association period is the duration of multiple of CG periods depending the smallest time duration in the set determined by the CG period such that all SSBs associated with the CG configuration are mapped at least once to CG PUSCH resources. - An association pattern period includes one or more association periods and is determined so that a pattern between CG PUSCH occasions and SS/PBCH block indexes associated with the CG configuration repeats at most every 640 msec. - Note: The mapping ordering and steps may be revisited if multiple CG PUSCH occasions in one CG period is supported Agreement (RAN1 #106-bis-e): When SSB set indication is absent, UE assumes the SSB set includes all actually transmitted SSBs configured by SIB1. - Mapping ratio of SSB to CG PUSCH is configured per CG configuration. - FFS whether to restrict the same value for all CG configuration and/or allow different value for different CG configurations. - For the candidate value set of SSB to CG PUSCH mapping ratio, support at least {1, 2, 4, 8, 16} - FFS {1/8,1/4,1/2} Conclusion (RAN1 #107-e): No need to restrict the same value of mapping ratio for all CG configurations

One open issue is related to SSB to CG PUSCH association period candidate values, which is defined as follows based on the agreement in RAN1#106-e:

- The SSB to CG PUSCH association period is the duration of multiple of CG periods depending the smallest time duration in the set determined by the CG period such that all SSBs associated with the CG configuration are mapped at least once to CG PUSCH resources.

In Table 8.1-1 of TS 38.213 (copied below [2]), the SSB to RO association period candidate values are a multiple of PRACH configuration periods. Similarly, a mapping between CG period and SSB to CG PUSCH association period needs to be defined in a table, such that the SSB to CG PUSCH association period candidate values can be determined based on CG periods.

Table 8.1-1 in 38.213: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period

| PRACH configuration period (msec) | Association period (number of PRACH configuration periods) |
| --- | --- |
| 10 | {1, 2, 4, 8, 16} |
| 20 | {1, 2, 4, 8} |
| 40 | {1, 2, 4} |
| 80 | {1, 2} |
| 160 | {1} |

In current specification, candidate CG period values for CG Type 1 PUSCH are considerably more than the candidate PRACH configuration period values. For less than 5 ms, the CG period values (given by the parameter periodicity [3]) can be 0.125, 0.25, 0.5, 0.625, 1, 2, 4 ms or less than 1 slot (1/6 slot, 1/7slot, or 1/2 slot) depending on the subcarrier spacing. However, it is not clear to us the need for CG-SDT period to be very small values, e.g., less than 5 ms. If the traffic profile demands such configuration of CG-SDT period, it is better that UE is configured with CG Type 1/Type 2 PUSCH in connected state, rather that CG-SDT in inactive state.

periodicity Periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: - 15 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} - 30 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} - 60 kHz with normal CP: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} - 60 kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} - 120 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120}

According to the current agreements for CG-SDT, the association pattern period should not exceed 640 ms. Also, the SSB to CG PUSCH association period should be no less than the minimum SSB period (5 ms). Based on these considerations, the set of candidate values for SSB to CG PUSCH association period can be defined as in Table 1 below. However, if a longer CG period than 640 ms (c.f. Section 2.2) is agreed in RAN1/RAN2, there should be updates to the existing agreements, as well to the different rows in Table 1.

Table 1: Mapping between CG period and SS/PBCH block to CG PUSCH resource association period

| CG period Tcg (msec) | Association period (number of CG periods except when CG period is less than 5 ms) |
| --- | --- |
| <5 | FFS |
| 5 | {1, 2, 4, 8,16, 32, 64, 128} |
| 8 | {1, 2, 4, 5, 8, 10, 16, 20, 40, 80} |
| 10 | {1, 2, 4, 8,16, 32, 64} |
| 16 | {1, 2, 4, 5, 8,10,20,40} |
| 20 | {1, 2, 4, 8,16, 32} |
| 32 | {1, 2, 4, 5, 10, 20} |
| 40 | {1, 2, 4, 8, 16} |
| 64 | {1, 2, 5, 10} |
| 80 | {1, 2, 4, 8} |
| 128 | {1, 5} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

If the CG period values for SDT are the same as those defined for CG Type 1 PUSCH, the candidate values of SSB to CG PUSCH association period is defined similar to the candidate values of SSB to RO association period according to Table 1. However, if a longer CG period than 640 ms is agreed in RAN1/RAN2, there should be updates to the existing agreements for CG-SDT, as well to the different rows in Table 1.

RAN1 to design the SSB to CG PUSCH association period for CG-SDT based on the CG period values that will be agreed in RAN1/RAN2.

When multiple CG configurations are provided, different SSB to CG PUSCH association periods may be derived, for e.g., when different CG configurations have different CG periods. The SSB to CG PUSCH mapping is performed for each CG configuration independently. In this case, if the different CG configurations are associated with the same PUSCH resource, the UL data associated with the CG configuration with the highest priority can be transmitted on the PUSCH resource, while the UL data associated with the other CG configurations can be discarded/postponed. There can also be cases where the same PUSCH resource is associated with different SSBs for different CG configurations. Therefore, additional information may be required for the gNB to determine the SSB associated with the UL data received on this PUSCH resource. Therefore, together with the data payload, additional information (e.g., service type, CG configuration ID of the associated UL data transmission, SSB ID) can be also transmitted from the UE to help the gNB with the information on the SSB associated with this UL data transmission.

When multiple CG PUSCH configurations are provided, additional information from the UE can be provided to the gNB to determine the SSB associated with the UL data received on an overlapping PUSCH resource associated with multiple CG configurations.

### 2.5.2	CG PUSCH configuration and resource validation

With regards to whether multiple CG PUSCH occasions can be configured per CG period, the following conclusion has been reached during RAN1#107-e:

Conclusion RAN1 cannot reach consensus on whether to support multiple CG occasions per CG period - Note that the CG PUSCH with multiple DMRS is considered as one CG occasion.

However, in our view, it should be possible to configure multiple CG PUSCH occasions per CG period for the sake of flexibility at the network side. The multiple CG PUSCH occasions can be TDM-ed and/or FDM-ed with the first PUSCH occasion. Furthermore, if is not possible to TDM/FDM CG PUSCH occasions per CG period, the association period will span multiple CG periods even with a few number SSBs associated with a CG configuration. This will impact the latency with which UL data can be delivered to the network. Note that for the Rel-16 2-step RACH, multiple PUSCH occasions can be configured for MsgA PUSCH.

Multiple CG PUSCH occasions in time and/or frequency domain can be configured per CG period for CG-SDT in RRC inactive state.

The CG PUSCH resource allocation in RRC inactive can be similar to CG Type 1 PUSCH resource allocation in RRC connected state, except that the TDRA list in the dedicated RRC signaling is not available for the UE and it needs to be configured in the RRC release message.

When multiple TDRA tables are defined, which one to select for CG PUSCH transmission in RRC inactive state can be determined based on a predetermined rule. More specifically, if a TDRA table is provided in the RRC release message, the UE should select the table provided in the RRC release message, else if a TDRA table is provided in the pusch-ConfigCommon message, the UE selects the table defined in pusch-ConfigCommon, else the UE selects the default table.

A UE specific TDRA list for CG PUSCH resource allocation in RRC inactive state should be configured in the RRC release message. Which TDRA list or table to select for CG-SDT can be based on predetermined rules when multiple TDRA lists, or tables are available.

In RAN1 #106-e and RAN1#106-bis-e, following agreements were made with regards to the DMRS configuration and with regards to whether multiple layer PUSCH transmission is supported for CG-SDT:

Agreement Support multiple DMRS resources per CG configuration when single layer PUSCH transmission is assumed, and each DMRS resource could be mapped to the same or different SSB(s) - FFS if multi-layer PUSCH transmission is supported for CG-SDT - FFS any limitation on the DMRS configuration if multiple CG PUSCH occasions per CG period is supported Agreement For CG-SDT, the UE can assume the PDCCH carrying the DCI has the same DM-RS antenna port quasi co-location properties as for a SSB associated to the CG PUSCH transmission e.g. for detection of retransmission DCI in response to a CG PUSCH transmission. Agreement Multi-layer PUSCH transmission is not supported for CG-SDT.

Multiple DMRS resources can be configured per CG configuration in the RRC release message, and each DMRS resource can be mapped to one or multiple SSBs. The DMRS configuration can be independent from (multiple) CG PUSCH occasion configurations in each CG period, and it can be up to the network to decide how many PUSCH occasions and/or DMRS resources should be configured.

DMRS configuration can be independent from the configurations of multiple CG PUSCH occasions.

Another open issue is regarding the validation of CG PUSCH resource, related to which following agreements have been made in RAN1:

Agreement - The following PUSCH occasion validation rule is applied for CG-SDT - for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon - if a UE is provided tdd-UL-DL-ConfigurationCommon, the valid PO is the PO in UL part in a slot, or at least Ngap symbols after the end of the DL part in a slot or after the end of the SSB in a slot - if a UE is not provided tdd-UL-DL-ConfigurationCommon, the valid PO does not precede a SS/PBCH block in the PUSCH slot, starts at least Ngap symbols after a last SS/PBCH block symbol - Ngap is provided in Table 8.1-2 in TS 38.213 - FFS if any validation rule following the CG-PUSCH in RRC connected state is applicable, and whether and how to handle the overlapping between CG-PUSCH occasions for CG-SDT and any valid PRACH occasion or MsgA PUSCH occasion. - FFS the rule for paired spectrum, and whether/how to support CG-SDT for UEs operating in Type-A HD-FDD. Agreement A CG PUSCH occasion is not valid if it overlaps with any valid PRACH occasion. - FFS overlapping between CG PUSCH occasions and MsgA PUSCH occasion

With regards to “FFS overlapping between CG PUSCH occasions and MsgA PUSCH occasion”, our view is that CG PUSCH occasion is not valid if it overlaps with MsgA PUSCH at least for CBRA of 2-step RA type.

A CG PUSCH occasion is not valid if it overlaps with MsgA PUSCH occasion at least for CBRA.

Unlike CG Type 1 PUSCH transmission in RRC connected state, CG-SDT PUSCH transmission in RRC inactive state has neither dedicated TDD uplink-downlink configuration nor dynamic SFI. So, CG-SDT PUSCH doesn’t have to follow the validation rules of CG Type 1 PUSCH. In Rel-15, only the uplink symbols of CG PUSCH in RRC connected state are allowed for transmission, and the flexible symbols must be changed to be uplink. In Rel-16, however, if enableConfiguredUL is provided, CG PUSCH can be transmitted directly on flexible symbols.

If CG-SDT is only allowed in uplink symbols/slots, additional TDD uplink-downlink configuration specifically for the UE may be needed for the case when all slots are flexible, e.g., when the cell specific TDD uplink-downlink configuration is not provided. To solve this issue, CG-SDT can be allowed to be transmitted on both flexible and uplink slots and/or symbols. For example, enableConfiguredUL can be configured in the RRC release message so that CG-SDT can be allowed in the flexible symbols.

Further discuss in RAN1 on whether CG-SDT in RRC inactive state is allowed on flexible symbols.

If CG-SDT is only allowed in uplink symbols, additional UE specific TDD uplink-downlink configuration should be supported in the RRC release message.

To support CG-SDT in flexible symbols, enableConfiguredUL can be configured in the RRC release message.

## 2.6	TA validation and TA offset for CG based SDT

### 2.6.1	TDOA based TA validation

The main issue with RSRP based TA validation is that it is not very robust. One reason for this is that RAN4 requirements on absolute accuracy of RSRP measurements is rather relaxed (cf. Section 10.1.2/10.1.3 in TS 38.133), which would impact the reliability of RSRP based TA validation.  Therefore, on top of the TA validation mechanisms based on RSRP and TAT, another TA validation mechanism based on the received time difference of arrival (TDOA) would be beneficial. This makes the TA validation more reliable, which is especially important in FR2 where the RSRP (or SSB beam) change may not reflect the actual movement of the UE (e.g., only the orientation of the UE may be changed).

Compared to RSRP based TA validation, the TDOA based TA validation is more reliable. This is because TDOA change (based on the first detected path) better reflects the actual change in the propagation delay than the RSRP change. Furthermore, TDOA based TA validation may be less complex and more power efficient as it is based on the timing detection of DL reference signals. If TDOA based TA validation is used, the UE may not need to perform dedicated RSRP measurements on the subset of SSBs configured for RSRP based TA validation, which makes the TA validation procedure both faster and power efficient. Note that power efficiency is crucial especially for RedCap devices, which are more likely to be configured with CG-SDT. It is also advantageous to configure the UE with both RSRP based and TDOA based TA validation mechanisms as it improves the reliability of the TA validation even more.

The TDOA based TA validation can be enabled by using the SFN and frame timing difference (SFTD) measurements, which is already supported in Rel-16 and is used to determine the time offset between the serving cell and other network nodes (e.g., serving neighboring cells). In more detail, the UE would perform SFTD measurements for the configured cell pairs at multiple time instances (e.g., at the time instant when the UE receives the latest TAC and at the time instant when UE determines TA validation) and calculate the maximum change in SFTD detected across the measured cell pairs. If the SFTD change is greater than X µs, the UE would determine the TA as invalid, and otherwise, as valid. Here, X is the SFTD change threshold, which can be configured by the network. The value of X may be provided in the CG-SDT configuration.

Based on the above considerations, we propose the following:

In addition to the RSRP and TAT based TA validation mechanisms, support TDOA based TA validation based on SFTD measurements for CG- SDT in RRC inactive state.

### 2.6.2	TA offset

With regards to the TA offset, the TA offset in a serving cell may not be available after the UE enters RRC inactive state, if the TA offset is configured UE-specifically for the serving cell (e.g., in the case of carrier aggregation). In this case, whether the default TA offset should be used or a TA offset should be provided in the RRC release message needs to be resolved in order to avoid misunderstanding between the UE and the gNB. In our view, the TA offset can be optionally configured in the CG-SDT configuration contained in the RRC release message. If the TA offset is configured, the UE applies this TA offset for CG PUSCH transmissions on this serving cell. If this field is absent, the UE applies the default value defined for the duplex mode and frequency range of this serving cell.

TA offset can be optionally configured in the RRC release message for CG-SDT. If the TA offset is configured, the UE applies this TA offset for CG PUSCH transmissions on this serving cell. If this field is absent, the UE applies the default value defined for the duplex mode and frequency range of this serving cell.

# 3	Conclusion

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN1 confirms that the separate initial UL/DL BWP for RedCap UEs may still be considered as the initial BWP and SDT resources can hence be configured on this BWP for RedCap UEs.

Proposal 2	Send an LS to inform RAN2 and RAN4 that CG/RA-SDT can be configured on the RedCap-specific separate initial UL/DL BWP.

Proposal 3	Periodicites up to 20.48 seconds should be configurable for CG-SDT

Proposal 4	RAN1 to send an LS to RAN2 capturing the allowed periodicities for CG-SDT and indicating that it is up to RAN2 to decide how to define the new periodicities.

Proposal 5	Discuss whether non-fallback DCI formats can be used to schedule retransmissions and subsequent transmissions for CG-SDT in an initial BWP.

Proposal 6	Introduce mechanism to allow semi-static updates of the power control parameters P0 and alpha for CG-SDT. How to provide the semi-static updates (e.g., via MAC-CE) is up to RAN2.

Proposal 7	If the CG period values for SDT are the same as those defined for CG Type 1 PUSCH, the candidate values of SSB to CG PUSCH association period is defined similar to the candidate values of SSB to RO association period according to Table 1. However, if a longer CG period than 640 ms is agreed in RAN1/RAN2, there should be updates to the existing agreements for CG-SDT, as well to the different rows in Table 1.

Proposal 8	RAN1 to design the SSB to CG PUSCH association period for CG-SDT based on the CG period values that will be agreed in RAN1/RAN2.

Proposal 9	When multiple CG PUSCH configurations are provided, additional information from the UE can be provided to the gNB to determine the SSB associated with the UL data received on an overlapping PUSCH resource associated with multiple CG configurations.

Proposal 10	Multiple CG PUSCH occasions in time and/or frequency domain can be configured per CG period for CG-SDT in RRC inactive state.

Proposal 11	A UE specific TDRA list for CG PUSCH resource allocation in RRC inactive state should be configured in the RRC release message. Which TDRA list or table to select for CG-SDT can be based on predetermined rules when multiple TDRA lists, or tables are available.

Proposal 12	DMRS configuration can be independent from the configurations of multiple CG PUSCH occasions.

Proposal 13	A CG PUSCH occasion is not valid if it overlaps with MsgA PUSCH occasion at least for CBRA.

Proposal 14	Further discuss in RAN1 on whether CG-SDT in RRC inactive state is allowed on flexible symbols.

Proposal 15	If CG-SDT is only allowed in uplink symbols, additional UE specific TDD uplink-downlink configuration should be supported in the RRC release message.

Proposal 16	To support CG-SDT in flexible symbols, enableConfiguredUL can be configured in the RRC release message.

Proposal 17	In addition to the RSRP and TAT based TA validation mechanisms, support TDOA based TA validation based on SFTD measurements for CG- SDT in RRC inactive state.

Proposal 18	TA offset can be optionally configured in the RRC release message for CG-SDT. If the TA offset is configured, the UE applies this TA offset for CG PUSCH transmissions on this serving cell. If this field is absent, the UE applies the default value defined for the duplex mode and frequency range of this serving cell.

# References

R1-2200881, “Reply LS on the L1 aspects of small data transmission”, RAN2, 3GPP TSG RAN WG1 #108-e, February 2022.

3GPP TS 38.213, V17.0.0, “NR; Physical layer procedures for control”, January 2022.

3GPP TS 38.331, V16.7.0, “NR; Radio Resource Control (RRC) protocol specification”, December 2021.