3GPP TSG RAN WG1 #108-e	R1-2201674

e-Meeting, February 21st – March 3rd, 2021

Agenda item:		8.10.2

Source:	Nokia, Nokia Shanghai Bell

Title:		Other enhancements for simultaneous operation of IAB-node’s child and parent links

Document for:		Discussion and Decision

# 1	Introduction

IAB Rel.17 WID [1] has following objectives led by RAN1:

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In RAN1 #102-e/103-e/104-e/105-e/106-e/106b-e/107-e meetings [3-25], several agreements related to timing enhancements, CLI mitigation, and power control were made, and they are captured in Annex I. Further considerations on timing enhancements and power control are included in this contribution.

# 2	Timing Options

## 2.1	Case #6 Timing

The following agreement was made in RAN1 #106bis-e regarding OTA synchronization during operation in Case 6 timing mode:

Agreement

Select Alt 2 from the aforementioned RAN1#106b-e agreement without specification impact other than the following:

- Alt A: the T_delta range is updated to support Case 6 timing.

FFS: Update of one-way delay estimation equation in TS38.213 subclause 14

On the FFS issue, we note that the specification for one way propagation delay estimation in Clause TS 38.213 was written for Timing Case 1 and applies as such for Timing Case 7 but it should be modified to better cover the propagation delay estimation in Timing Case 6. Obtaining NTA the way it is described for UE in the referred Clause 4.2 of TS38.213 does not tell how NTA is obtained for Case 6. As MT UL TX timing is not explicitly controlled by NTA signaling in Case 6 it should be defined that IAB node calculates NTA as NTA=TTA/Tc-NTA,offset, where TTA is the timing advance of the IAB-node MT transmission relative to the IAB-node MT reception. A difference between Case 1/7 and Case 6 is that while the accuracy of the propagation delay estimation in Case 1/7 does not depend on the granularity of N_TA, in Case 6 granularity would directly affect the accuracy.

The following modification of TS 38.213 is proposed

# 14 Integrated access-backhaul operation < Unchanged parts omitted > If an IAB-node is provided an index Tdelta in a Timing Delta MAC CE [11, TS 38.321] from a serving cell, the IAB-node may assume that NTA/2+Ndelta+Tdelta⋅Gstep⋅Tc is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-MT when NTA/2+Ndelta+Tdelta⋅Gstep>0, where NTA is obtained as for a "UE" in clause 4.2 for the TAG containing the serving cell and Ndelta and Gstep are determined as - Ndelta=-70528 and Gstep=64, if the serving cell providing the Timing Delta MAC CE operates in FR1, - Ndelta=-17664 and Gstep=32, if the serving cell providing the Timing Delta MAC CE operates in FR2 The IAB-node may use the time difference to determine a DU transmission time. For a serving cell of an IAB-MT, the IAB-MT can be provided by Timing Case Indication MAC CE [11, TS 38.321] an indication of the IAB-MT transmission timing mode in a slot. Upon reception of the Timing Case Indication for a serving cell in a TAG, the IAB-MT applies a same IAB-MT transmission timing mode in a slot on all serving cells in the TAG. If the indicated IAB-MT transmission timing mode in a slot is set to 'Case1', the IAB-MT transmission time is determined as for a "UE" in clause 4.2. If the indicated IAB-MT transmission timing mode in a slot is set to 'Case6', the IAB-node sets the IAB-MT transmission time to the transmission time of the IAB-DU. If the indicated IAB-MT transmission timing mode in a slot is set to 'Case7', the IAB-MT is provided a timing advance offset value NTA,offset,2 for a serving cell by Case7 Timing Offset MAC CE [11, TS 38.321]. The IAB-MT determines its uplink transmission timing as NTA+ NTA,offset+ NTA,offset,2 ⋅Tc where NTA and NTA,offset are obtained as for a "UE" in clause 4.2 and NTA,offset,2=Toffset,2⋅16⋅642μ where Toffset,2 is provided by the Absolute Time Offset MAC CE [11, TS 38.321]. If an IAB-node is provided an index Tdelta in a Timing Delta MAC CE [11, TS 38.321] from a serving cell, the IAB-node may assume that NTA/2+Ndelta+Tdelta⋅Gstep⋅Tc is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-MT when NTA/2+Ndelta+Tdelta⋅Gstep>0. For timing Case 1 and 7, NTA is obtained as for a "UE" in clause 4.2 for the TAG containing the serving cell. For timing Case 6, NTA=TTA/Tc-NTA,offset, where TTA is the timing advance of the IAB-node MT transmission relative to the IAB-node MT reception. Ndelta and Gstep are determined as - Ndelta=-70528 and Gstep=64, if the serving cell providing the Timing Delta MAC CE operates in FR1, - Ndelta=-17664 and Gstep=32, if the serving cell providing the Timing Delta MAC CE operates in FR2 The IAB-node may use the time difference to determine a DU transmission time. < Unchanged parts are omitted >

Proposal 2.1:	Update the specification of propagation delay estimation with description of NTA determination for Case 6 as    NTA=TTA/Tc-NTA,offset by adopting the text proposal provided above.

## 2.2 Case #7 Timing

The following agreements were made in RAN1 #106bis-e regarding support for case #7 timing.

Agreement

Case 7 UL timing offset is indicated by the parent-node via MAC-CE.

Agreement

The granularity of Case 7 UL timing offset is the same as the UL TA granularity.

Additionally, the following agreement was made in RAN1#107-e regarding enhanced case #7 timing indication:

Agreement

The dynamic range of the MAC CE case #7 timing offset indication is 12 bits.

- FFS the numerical values of the endpoints of the range

Based on the agreement, the additional timing offset signal for case #7 timing, N_TA,offset2, can be understood to have the following calculation

NTA,offset2= -0,…,4096⋅16⋅642μ+Δ⋅Tc

As indicated in the agreement from RAN1#107-e the value of Δ is still FFS.  If the value of Δ≥0,NTA,offset2 strictly delays the UL Tx timing of the child node.  This would ensure that NTA,offset2 is only intended to reduce the slot timing offset between DU Rx and MT Rx at the IAB node.  Since there is no requirement to maintain slot-level alignment while IAB is operating in case #7 timing mode this isn’t a necessary constraint but increasing the offset has the potential to create larger network coordination issues and should therefore be avoided.  Additionally, a minimum offset of 0 is desirable as it avoids potential issues introducing a minimum supportable timing offset.

Proposal 2.2:	The range of possible values for N_TA,offset2 indicating timing offset for a child node supporting an IAB node operating in case 7 timing mode is 0,…,4096.

# 3	Power Control

The following agreement was made in RAN1#107-e regarding indicated power control adjustments:

Agreement

Support optionally indicating “slot index” in the provided DL TX power adjustment indication, that comprises indicating a list of one or multiple slot indices for which the associated DL power adjustment is applied.

- FFS:  support of “slot index” indication in the desired DL TX power adjustment

- FFS:  support of “slot index” indication in the desired UL PSD range indication

As with beam restriction there is no ambiguity regarding which resources the power adjustments are intended for since UL Tx power adjustment is intended for power balancing while operating in case A multiplexing and DL Tx power adjustment is intended for power balancing while operating in case B multiplexing.  Since there is no ambiguity in which resources the power allocations are associated with, slot indication is redundant and therefore unnecessary.

Proposal 3.1:	Indication of both desired DL Tx power adjustment and UL PSD range does not include an explicit association with slot index.

Regarding the suitable range of power adjustment indications RAN4 sent a reply LS to RUN 1 (R1-220906) providing guidance.

The guidance from RAN4 indicates that the IAB-MT must support a minimum UL PSD dynamic range of 10dB for local area-IAB and 5dB for wide area-IAB in FR1 and no restriction exists for FR2.  To be consistent with RAN4 requirements the indication for desired UL TX PSD range should be consistent with minimum requirements IAB-MT

Proposal 3.2:	Indication of desired UL PSD range should fall within than range of values between maximum MT Tx power and maximum MT Tx power – 5dB.

# 4	Conclusions

Our observations and proposals on timing options are:

Proposal 2.1:	Update the specification of propagation delay estimation with description of NTA determination for Case 6 as NTA=TTA/Tc by adopting the text proposal provided above.

Proposal 2.2:	The range of possible values for N_TA,offset2 indicating timing offset for a child node supporting an IAB node operating in case 7 timing mode is 0,…,4096.

Our observations and proposals on power control enhancements are:

Proposal 3.1:	Indication of both desired DL Tx power adjustment and UL PSD range does not include an explicit association with slot index.

Proposal 3.2:	Indication of desired UL PSD range should fall within than range of values between maximum MT Tx power and maximum MT Tx power – 5dB.

# References

- RP-193251, New WID on Enhancements to Integrated Access and Backhaul.

- TR 38.874, Study on Integrated Access and Backhaul

- Chairman’s notes RAN1#102-e

- R1-2007241, Summary #1 of [102-e-NR-eIAB-01], Moderator (AT&T)

- R1-2007322, Summary #2 of [102-e-NR-eIAB-01], Moderator (AT&T)

- R1-2007287, Summary of [102-e-NR-eIAB-02], Moderator (Qualcomm)

- R1-2007352, Summary #3 of [102-e-NR-eIAB-02], Moderator (Qualcomm)

- Chairman’s notes RAN1#103-e

- R1-2009604 Summary #1 of [103-e-NR-eIAB-01] Moderator (AT&T)

- R1-2009567 Summary #1 of [103-e-NR-eIAB-02] Moderator (Qualcomm Incorporated)

- R1-2101878 Summary #1 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links Moderator (Qualcomm)

- R1-2101889 Summary of [104-e-NR-eIAB-01] – 1st Checkpoint Moderator (AT&T)

- R1-2101890 Summary of [104-e-NR-eIAB-01] – 2nd Checkpoint Moderator (AT&T)

- R1-2101891 Summary of [104-e-NR-eIAB-01] – 3rd Checkpoint Moderator (AT&T)

- R1-2101909 Summary #2 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links Moderator (Qualcomm)

- R1-2102031 Summary #3 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links Moderator (Qualcomm)

- R1-2102172 Summary #4 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links Moderator (Qualcomm)

- TS 38.213, NR Physical layer procedures for control

- R1-2106009	Feature lead summary#1 on enhancements for simultaneous operation of IAB-node’s child and parent links	Moderator (Qualcomm Inc.)

- R1-2106083	Feature lead summary#2 on enhancements for simultaneous operation of IAB-node’s child and parent links	Moderator (Qualcomm Inc.)

- R1-2106173	Feature lead summary#3 on enhancements for simultaneous operation of IAB-node’s child and parent links	Moderator (Qualcomm Inc.)

- R1-2108339	FL summary#1 on other enhancements for simultaneous operation of IAB-node’s child and parent links [106-e-NR-eIAB-04]	Moderator (Qualcomm)

- R1-2108479	FL summary#2 on other enhancements for simultaneous operation of IAB-node’s child and parent links [106-e-NR-eIAB-04]	Moderator (Qualcomm)

- R1-2108568  FL summary#3 on other enhancements for simultaneous operation of IAB-node’s child and parent links [106-e-NR-eIAB-04]	Moderator (Qualcomm)

- R1-2110497	Summary#1 [106bis-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links	Moderator (Qualcomm Incorporated)

- R1-2112636	Summary #1 for [107-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node's child and parent links	Moderator (Qualcomm Incorporated)

# Annex I

## RAN1 #107-e

Agreement

The dynamic range of the MAC CE case #7 timing offset indication is 12 bits.

- FFS the numerical values of the endpoints of the range

Agreement

Select Alt 2 from the aforementioned RAN1#106b-e agreement without specification impact other than the following:

- Alt A: the T_delta range is updated to support Case 6 timing.

FFS: Update of one way delay estimation equation in TS38.213 subclause 14

Conclusion

In Rel-17 there is no specification support for the indication of “DL signal/channel type” in the provided DL TX power adjustment indication.

Agreement

Send an LS to RAN4 to specify the range of values for the indication of the desired/provided DL TX power adjustment, as well as the range of values for the indication of the desired MT UL PSD range.

LS is endorsed in R1-2112973.

Agreement

The provided DL TX power adjustment is applied only to PDSCH and its associated DMRS and PTRS.

Agreement

The indicated desired/provided DL TX power adjustment is in terms of a relative offset to the PDSCH a CSI-RS TX power that is RRC configured.

Agreement

Support optionally indicating “slot index” in the provided DL TX power adjustment indication, that comprises indicating a list of one or multiple slot indices for which the associated DL power adjustment is applied.

- Support of “slot index” indication in the desired DL TX power adjustment indication

Agreement

TCI state ID and RS ID (SSB ID and/or CSI-RS ID) is used to indicate IAB-MT’s DL beam for the desired/provided DL TX power adjustment indication by the IAB-node/the parent-node.

In case the desired/provided DL TX power adjustment indication does not include information about the associated IAB-MT’s DL beams, the adjustment is applied to all MT’s DL beams.

Agreement

Support optionally indicating “slot index” in the provided DL TX power adjustment indication, that comprises indicating a list of one or multiple slot indices for which the associated DL power adjustment is applied.

- FFS:  support of “slot index” indication in the desired DL TX power adjustment

- FFS:  support of “slot index” indication in the desired UL PSD range indication

Conclusion

In Rel-17 there is no specification support for the indication of “timing mode” in the desired/provided DL TX power adjustment or the desired UL PSD range indication.

- Note: This does not prevent the indication of case 6 timing mode and UL PSD range as separate conditions to enable a given multiplexing case.

Conclusion

For Rel-17, no further discussion in RAN1 on eIAB supporting new triggering conditions for sending an updated PHR.

Agreement

SRI is used to indicate IAB-MT’s UL beam for the desired UL PSD range indication.

In case the desired UL PSD range indication does not include information about the associated IAB-MT’s UL beams, the PSD range is applied to all MT’s UL beams.

Agreement

The indication of the desired/provided DL TX power adjustment and desired UL PSD range can further include:

- An indication of whether a desired/provided power configuration or adjustment is applied on FDM resources where the simultaneous MT’s and DU’s signals are non-overlapping in the frequency-domain and/or on non-FDM resources where the simultaneous MT’s and DU’s signals may overlap in the frequency-domain, for a given (MT CC, DU cell).

## RAN1 #106bis-e

Agreement

The following alternative is selected for the association between the indicated parent-node’s DL TX power adjustment, provided by an IAB-MT to its parent-node, and IAB-node’s resources and/or configurations:

- Alt 2. The desired DL TX power adjustment is indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

  - Multiplexing mode

  - MT’s DL beam (e.g. TCI state id)

  - (MT CC, DU cell) pair

  - DU resource configuration

  - FFS: Slot index

  - FFS: timing mode (e.g., Case-7 timing)

Agreement

Case 7 UL timing offset is indicated by the parent-node via MAC-CE.

Agreement

The granularity of Case 7 UL timing offset is the same as the UL TA granularity.

Conclusion

RAN1 does not discuss further proposals for IAB interference management enhancements in 8.10.2.

Agreement

The desired parent-node’s DL TX power adjustment, provided by an IAB-MT to its parent-node, is indicated via MAC-CE.

- The indication further includes the associated configurations and/or resources for which the indicated power adjustment is applicable.

- The indicated adjustment is in terms of a relative offset to a reference DL TX power.

  - FFS: the reference power (e.g., an RS such as CSI-RS, etc) for the indication of desired adjustment.

  - FFS: the range of values for the indicated adjustment.

Agreement

The desired IAB-MT’s UL PSD range, provided by the IAB-MT to its parent-node, is indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- Multiplexing mode,

- MT’s UL beam (e.g., SRI id),

- (MT CC, DU cell) pair,

- DU resource configuration

- FFS: slot index

- FFS: timing mode (e.g., Case-6 timing)

Agreement

The desired IAB-MT’s UL PSD range, provided by an IAB-MT to its parent-node, is indicated via a new MAC-CE.

- The indication further includes the associated configurations for which the indicated PSD range is applicable.

- FFS: the range of values for the indicated PSD range and whether RAN4 input is needed.

- FFS: IAB-MT’s behaviour in case the configured/indicated UL TX power is outside the indicated desired PSD range and whether RAN4 input is needed.

Conclusion

RAN1 to further discuss, under 8.10.2, whether to support new triggering conditions to send an updated PHR (e.g., upon change of multiplexing mode, or applying a desired DL TX power adjustment indication from a child-node).

Agreement

The DL TX power adjustment, provided by the parent-node to IAB-MT, is indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- Multiplexing mode

- MT’s DL beam (e.g., TCI state id, RS id)

- (MT CC, DU cell) pair

- DU resource configuration

- FFS: DL signal/channel type

- FFS: slot index

- FFS: timing mode (e.g., Case-7 timing)

Agreement

The DL TX power adjustment, provided by the parent-node to the IAB-MT, is indicated via MAC-CE.

- The indication further includes the associated configurations and/or resources for which the indicated power adjustment is applicable.

- The indicated adjustment is in terms of a relative offset to a reference DL TX power.

  - FFS: the reference power (e.g., an RS such as CSI-RS, etc) for the indication of DL Tx power adjustment.

  - FFS: the range of values for the indicated adjustment.

Agreement

The indicated DL TX power adjustment is not applied to SSBs.

- FFS: any other cell-specific/semi-static DL signal to be exempted.

- FFS: applicability of the indicated TX power adjustment to other RS/channel which share the same QCL Type-D assumption.

Agreement

RAN1 to downselect in RAN1#107-e one of the following for an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode:

- Alt 1: no change or enhancement to the Rel-16 OTA synchronization specification is supported in Rel-17 for Case 6 timing.

- Alt 2: in Rel-17 the Rel-16 OTA synchronization specification is updated to support OTA synchronization for an IAB-node operating solely in Case 6 timing during IAB-MT Tx.

  - FFS range of T_delta.

NOTE: this is to provide a feasible solution to the RAN1#103-e agreement: “An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode”

Agreement

RAN1 to down select in RAN1#107-e one of the following for an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode:

- Alt 1: no change or enhancement to the Rel-16 OTA synchronization specification is supported in Rel-17 for Case 6 timing.

- Alt 2: in Rel-17 the Rel-16 OTA synchronization specification is updated to support OTA synchronization for an IAB-node operating solely in Case 6 timing during IAB-MT Tx.

  - FFS range of T_delta.

NOTE: this is to provide a feasible solution to the RAN1#103-e agreement: “An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode”

## RAN1 #106-e

Agreement

For Case 6 timing at a given IAB-node, the IAB-MT Tx timing is set by the node to the timing obtained for the node’s DL Tx.

- FFS: Need for additional details with reference to support of OTA synchronization (e.g. T_delta)

Agreement

For Case 7 timing at a parent node, the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

- FFS range, granularity, and signaling details of the offset.

Agreement

For the support of DU-to-DU measurement and report:

- For DU-to-DU CLI measurement:

  - Option 1.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

- For DU-to-DU CLI report:

  - Option 2.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

Agreement

Support the exchange of semi-static Rel-16 IAB-DU H/S/NA resource configuration information and Rel-17 frequency domain IAB-DU H/S/NA resource configuration information among neighbouring IAB-nodes/IAB-donors

Agreement

An IAB-node is explicitly indicated by the parent node when Case 6 timing is performed at the IAB-node at least for specific time resources.

- FFS: whether the indication should be associated with another dimensions, e.g. multiplexing cases

- FFS whether an IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the IAB-node.

Conclusion

Details on the design of the indication of when Case 6 timing (and Case 7 timing, if agreed) is performed at the IAB-node are to be discussed under 8.10.1.

Agreement

An IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the parent node.

- FFS for signalling details

Agreement

The desired DL TX power adjustment, indicated by the IAB-MT to its parent-node to assist with the parent-node’s DL TX power allocation, is provided at least for specific time resources.

The desired DL TX power adjustment can further be associated with spatial configuration. (e.g., MT’s DL RX beams).

- FFS: signalling details, e.g. indication via MAC-CE, PUCCH, or legacy CSI framework.

Agreement

Support an IAB-node indicating adjustment to its DL TX power to a child node (e.g., in response to receiving the DL TX power assistance information from the child node) at least for specific time resources.

The DL TX power adjustment indication can further be associated with spatial configuration. (e.g., MT’s DL RX beams).

- FFS: signalling details.

Conclusion

Discuss under 8.10.1 whether CLI measurements should be included in the beam report from a node to its parent node.

Agreement

Support an IAB-node indicating its desired IAB-MT PSD range to help with its MT’s UL TX power control.

- This information is provided to the parent node

FFS: Applicability of assistance information, e.g., per multiplexing scenario, per resource, etc.

FFS: Signalling details, including the possibility to extend PHR.

Conclusion

In Rel-17 the following may be up to network implementation, no specific specification work required:

- Differentiating access and backhaul slots.

- Restricting simultaneous operation of MT and DU to DL slots.

## RAN1 #105-e

Agreement

Rel-16 CLI coordination signalling (Intended TDD DL-UL Configuration) is extended to support IAB specific UFD patterns.

- FFS: Support the exchange of IAB-DU H/S/NA resource configuration information among neighbouring IAB-nodes/IAB-donors for CLI management purposes.

Agreement

Decide in RAN1#106-e whether to support an IAB-node indicating assistance information to help with its MT’s UL TX power control. The assistance information can be:

- FFS: Desired TX power

- FFS: Offset to a baseline PHR

- FFS: Desired dynamic range

FFS: whether this information is provided to the parent-node, the CU, or both.

FFS: whether the MT’s UL TX power control formula needs to be changed

Agreement

The information to assist DL power allocation of the parent-node is indicated by the IAB-MT to the parent node DU in terms of desired power adjustment.

- FFS applicability of assistance information, e.g. per multiplexing scenario, per resource, etc.

Agreement

RAN1 to downselect how the IAB-MT Tx timing is set for Case 6 timing at a given IAB-node:

- Alt1: the IAB-MT Tx timing is obtained by the node via the legacy TA loop plus an offset from the parent node.

  - FFS details of the required offset.

- Alt2: the IAB-MT Tx timing is set by the node to the timing obtained for the node’s DL Tx.

- Alt3: the IAB-MT Tx timing is obtained by the node jointly with the IAB-DU Tx timing via a common offset from the parent node.

Downselection to consider at least the following aspects:

- Dependency of DL synchronization schemes at the IAB-DU

- Potential additional signaling overhead.

- Achievable DU Tx / MT Tx alignment error tolerance.

- Suitability for switching between timing modes.

Agreement

RAN1 to downselect how the IAB-MT Tx timing is set at an IAB-node for Case 7 timing at the parent node:

- Alt1: the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

  - FFS details of the required offset

- Alt2: the IAB-MT Tx timing of the node is obtained via the legacy TA loop from the parent node.

- Alt3: the IAB-MT Tx timing of the node is obtained via a Case 7 specific TA loop from the parent node.

Downselection to consider at least the following aspects:

- Potential impact to OTA synchronization availability for DU Tx at the IAB-node.

- Potential additional signaling overhead.

- Suitability for switching between timing modes.

Agreement

An IAB-node is indicated when Case 6 timing is performed at the IAB-node.

- FFS details of the indication (e.g. semi-static and/or dynamic, implicit and/or explicit, linkage to multiplexing capability, etc.).

FFS whether an IAB-node is also indicated when Case 7 timing is performed at the IAB-node.

## RAN1 #104-e

Agreement

Case 7 timing is supported with symbol level alignment without explicit support for slot level alignment.

Agreement

Switching between Case 1, Case 6, and Case 7 timing is supported.

- FFS whether Case 6 and Case 7 timing shall be restricted to certain resources, e.g., excluding resources used for access or TDM backhaul.

- FFS details on switching including the switching conditions.

- FFS relationship between switching timing modes with the usage/indication of different resource multiplexing modes.

- FFS whether Rel-16 OTA synchronization shall be enhanced to support switching timing modes.

Agreement

RAN1 to further study whether the legacy UL power control mechanism (including PHR) is sufficient for an IAB-node operating in an enhanced multiplexing mode.

- FFS: if not (i.e., the legacy mechanism is not sufficient), support an IAB-node indicating information to assist with its UL power control.

Agreement

Support an IAB-node indicating information to assist with the DL power control of its parent-node towards the IAB-node without mandating an expected behavior at the parent node.

- Note: At least the assistance information is for supporting the simultaneous operation within the IAB-node to avoid power imbalance

- FFS: type of assistance information (e.g., desired received power, power adjustment, preferred CSI-RS resource)

- FFS: whether this information is provided to the parent-node, the CU, or both.

- FFS: applicability of the assistance information (e.g., relation to beams or multiplexing modes)

- FFS: the channel carrying this assistance information

Conclusion

In Rel-17, RAN1 will not specify specific mechanisms for intra-IAB-node interference (self-interference) management.

- Self-interference can be handled by the implementation or via using the available techniques defined, or to be defined in Rel-17, that can commonly be used for other interference scenarios as well.

Agreement

RAN1 to select among the following options to support DU-to-DU measurement and report.

- For DU-to-DU CLI measurement:

  - Option 1.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 1.2. enhanced legacy DU-based measurement procedures (e.g., enhanced Rel-16 RIM)

  - Option 1.3. enhanced MT-based measurements (e.g., MT-based CLI, MT RRM measurements)

- For DU-to-DU CLI report:

  - Option 2.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

  - Option 2.2. enhanced legacy DU-based report (e.g., enhanced Rel-16 RIM)

  - Option 2.3. enhanced MT-based report (e.g., MT-based CLI, MT RRM measurements)

Agreement

RAN1 to decide whether to enhance interference mitigation through information exchange to support beam-management at the parent or child node in RAN1#104bis-e

  - FFS: reporting of desired beams for reception in DL or desired beams for transmission in UL by the IAB node for a given multiplexing mode

  - FFS: indicating applicable beams in DL or beams in UL for a given multiplexing mode.

## RAN1 #103-e

Agreement

Select one or both of the following modes of operation for Case 7 timing in RAN1#104-e:

- symbol level alignment without slot level alignment

- slot level alignment

Agreement

Case 6 timing mode operation at an IAB-node is controlled by the parent node to which the UL transmission is intended for.

Agreement

Use the Rel-16 interference management frameworks (e.g., CLI, RIM) to handle IAB interference scenarios, and discuss if any of the following enhancements are needed (not an exhaustive list):

- FFS: extend the information exchange (e.g., the resource configuration, result of CLI measurements, etc.) among different entities (e.g., between parent-child nodes, adjacent IAB nodes, between network and IAB-node, etc.)

- FFS: required enhancements on CLI measurement accuracy (e.g., via timing adjustment, etc.)

- FFS: required enhancements on CLI measurements (e.g., introducing short-term measurements, multi-beam measurements, etc.)

Agreement

Further study requirement of enhanced DL and UL Tx power control mechanism considering the following:

- DL/UL power control with assistance information from the child node.

- DL/UL power control with assistance information from the parent node.

- Central (e.g., by CU) power control coordination (e.g., semi-static max DL/UL Tx power limits).

- Coexistence of different power control mechanisms within an IAB node and in the network.

Note. Any power control mechanism should consider the following aspects:

- Existing base station design principles (e.g., power control and dynamic range capability, etc.) related to transmission power.

- Network constraints in regard to transmitted reference signals with constant power.

Agreement

Interference management for the following IAB interference scenarios should be discussed:

- Inter-IAB scenarios, including:

  - MT to MT, DU to DU, DU to MT, and MT to DU.

- Interference to non-IAB nodes, including:

  - IAB-DU to non-IAB-DU

  - IAB-MT to non-IAB-DU

- Intra-IAB-node (self-interference) scenarios (Interference between a DU and MT of an IAB-node).

This agreement does not necessarily mean that specification support is needed for any of the scenarios.

Agreement

Consider resource and beam coordination techniques to mitigate/avoid interference, including (not an exhaustive list):

- FFS: whether or not to support IAB‐node (MT) transmissions in DL access slots

  - FFS: if this has RAN1 impact or it can be handled by implementation.

  - FFS: network coordination impact

- FFS: whether Rel-16 resource management framework is sufficient.

Agreement

An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode.

- FFS whether the Rel-16 OTA synchronization mechanism is sufficient, or enhancements are required.

  - If required, details of enhancements including the uplink timing(s) required to support different timing alignment cases.

Agreement

An IAB-node, when operating in Case 7 timing mode, can enable a child node to set its DL Tx timing based on Rel-16 OTA timing synchronization mechanism.

- FFS whether Rel-16 OTA synchronization mechanism enhancements are required.

  - FFS details of enhancements, if required

## RAN1 #102-e

Agreement

- Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 2 (simultaneous MT-Rx/DU-Rx)

- Case 6 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 1 (simultaneous MT-Tx/DU-Tx)

  - RAN1 should strive to minimize specification impact due to this feature.

- FFS: Whether Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 4 (simultaneous MT-Tx/DU-Rx)