3GPP TSG RAN WG1 Meeting #108-e                                                                   R1-2201694

e-Meeting, February 21st – March 3rd, 2022

Source: 	Intel Corporation

Title: 	Remaining Clarifications to Enable URLLC/IIoT in Unlicensed Band

Agenda item: 	8.3.2

Document for: 	Discussion/Decision

# Introduction

In 3GPP TSG RAN Meeting #88 [1], a new WID related to enhancements to industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) was approved. As part of the objectives of this working item (WI), the following aspects were included:

- Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]: - Specify support for UE-initiated COT for FBE with minimum specification effort - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

In this contribution, in the context of enhancements to enable URLLC to operate in the sub-6 GHz unlicensed band, based on the agreements and conclusions made during the previous RAN1 meetings along this WI [2-9], we will be discussing in the context of semi-static channel access mode the Rel.16 LBT failure reporting procedure for the case when a UE operates as an initiating device.

# LBT Failure Reporting Procedure

In Rel.16, an LBT failure reporting procedure has been defined, which allows a UE to determine whether a systematic/consistent listen before talk (LBT) failure occurs by counting over time the number of channel access failures that may occur and notify accordingly the higher layers, as described in Sec. 4.3.1 in TS 37.213:

### 4.3.1 Channel access procedures to initiate a channel occupancy For semi-static channel occupancy, the procedures in Clause 4.3.1.1 are followed if ue-SemiStaticChannelAccessConfig is absent. Otherwise, the procedures in Clause 4.3.1.2 are applicable. If a UE fails to access the channel(s) prior to an intended UL transmission to a gNB, Layer 1 notifies higher layers about the channel access failure.

While this procedure applies to both semi-static and dynamic channel access mode, it was designed having in mind and assuming that every LBT failure at the UE may have the same effect and impact on the overall system performance. However, while this is true for the dynamic channel access procedure, and for semi-static channel access mode when only a gNB can operate as an initiating device, since a UE has multiple occasions to reperform LBT at any time, the impact of an LBT failure may be different for semi-static channel access mode when a UE operates as an initiating device depending on when this may occur. In fact, for the latter case, if a UE fails to succeed LBT at a u-FFP when the UE operates as an initiating device, that UE is forbidden from transmitting and performing LBT for the entire duration of that u-FFP or until the UE may be able to operate as a responding device. With that said, it is quite intuitive to realize that the effect of the LBT failure in this case is more detrimental and a way to weight more or indicate this type of LBT failure is needed, so that to timely inform the network about this type of events which may introduce large latencies and delays.

In order to minimize changes to the existing LBT failure reporting, and specification impact, but also constrain changes within RAN1 perspective, the reporting of the channel access failures could be modified so that for the case when a UE fails to access the channel(s) prior to an intended UL transmission which aligns with a u-FPP and the UE operates as an initiating device, the UE reports a channel access failure to the higher layers not only when the UE fails to perform the channel access procedure at the beginning of that u-FFP, but also for each UL burst that the UE was intended to transmit within that u-FFP as if the UE operates as an initiating device. This minimal change guarantees that the UE may weight an LBT failure at the u-FFP when that UE operates as an initiating device more than LBT failures occurring in all other cases. This will allow to detect systematic/consistent LBT failure in a timelier manner and not only help the network identify this scenario, but also timely allow it to make countermeasures.

Proposal 1: For the case when a UE operates as an initiating device, and the UE fails to access the channel(s) prior to an intended UL transmission which aligns with a u-FFP, Layer 1 notifies higher layer about a channel access failure for each burst that the UE was intended to transmit as an initiating device within that u-FFP.

In order to capture the above intent, the following text should be captured in Sec. 4.3.1 of TS 37.213:

------------------------------ TP#1: TS 37.213 Sec. 4.3.1----------------------------------- *** Unchanged text is omitted *** ### 4.3.1 Channel access procedures to initiate a channel occupancy For semi-static channel occupancy, the procedures in Clause 4.3.1.1 are followed if ue-SemiStaticChannelAccessConfig is absent. Otherwise, the procedures in Clause 4.3.1.2 are applicable. If a UE fails to access the channel(s) prior to an intended UL transmission to a gNB, Layer 1 notifies higher layers about the channel access failure. Additionally, if a UE fails to initiate a channel occupancy in a period of duration Tu, Layer 1 notifies higher layers about a channel access failure for each intended UL transmission burst starting after the beginning of that period and ending before the start of the idle duration of that period which is associated with a channel occupancy that is initiated by that UE as described in Clause 4.3.1.2.3 and 4.3.1.2.4. #### 4.3.1.1 Channel occupancy initiated only by gNB *** Unchanged text is omitted ***

Proposal 2: Support TP#1.

# Conclusion

In this contribution, we discussed the last aspects related to the UL enhancements for URLLC operating in unlicensed spectrum, and made the following proposals:

Proposal 1: For the case when a UE operates as an initiating device, and the UE fails to access the channel(s) prior to an intended UL transmission which aligns with a u-FFP, Layer 1 notifies higher layer about a channel access failure for each burst that the UE was intended to transmit as an initiating device within that u-FFP.

Proposal 2: Support TP#1.

# References

- RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR,” Nokia, Nokia Shanghai Bell.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #102 e-Meeting, August, 2020.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #103 e-Meeting, November, 2020.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #104 e-Meeting, February, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #105 e-Meeting, May, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #106 e-Meeting, August, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #106bis e-Meeting, October, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #107 e-Meeting, November, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #107bis e-Meeting, January, 2022.