3GPP TSG RAN WG1 #108-e																		    R1-2201712

e-Meeting, Februar 17th – 25th, 2022

Source: 	Intel Corporation

Title:	Remaining details on Msg3 PUSCH repetition

Agenda item:	8.8.3

Document for:	Discussion and Decision

# Introduction

At the RAN1#107b-e meeting, the following agreements and conclusions were made regarding Msg3 PUSCH repetition [1]:

Agreement

Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported.

- When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

Agreement

The 3 LSB bits of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI is used to indicate one value from 8 candidate MCS indexes for Msg3 retransmission.

- The 8 candidate MCS indexes can be configured by SIB1, MCS 0~7 are applied if the configuration is absent. The first 4 indexes of the 8 candidate MCS indexes are used for initial PUSCH transmission scheduled by RAR UL grant.

Agreement

For the number of repetitions configured by numberOfMsg3Repetitions, support {1, 2, 3, 4, 7, 8, 12, 16}.

Conclusion

For Rel-17 CE WI, Issue (4~7) in Section 2.6 of R1-2200712 will not be discussed in RAN1 in future meetings, and issue (1~3) in Section 2.6 of R1-2200712 can only be discussed in RAN1 if requested by other WGs.

Agreement

All slots are considered as available slots for Msg3 repetition for both FD-FDD UEs and HD-FDD RedCap UEs.

Agreement

Introduce the following the RRC parameter for indication of the number of repetitions for PUSCH repetition scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | numberOfMsg3Repetitions | new | The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | SEQUENCE (SIZE (4)) OF INTEGER (1,2, 3, 4, 7, 8, 12, 16) | {1, 2, 3, 4} | RACH-ConfigCommon | Cell-specific | 38.331 | Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field. Agreements - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH. Agreement Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported. - When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication. When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. Agreement For the number of repetitions configured by numberOfMsg3Repetitions, support {1, 2, 3, 4, 7, 8, 12, 16}. |

Agreement

Introduce the following the RRC parameter for indication of the MCS index for PUSCH repetition scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_cov_enh-Core | Type A PUSCH repetitions for Msg3 | mcs-Msg3Repetition | new | Configure eight candidate MCS indexes for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. Only the first 4 configured or default MCS indexes are used for PUSCH transmission scheduled by RAR UL grant. | SEQUENCE (SIZE (8)) OF INTEGER (0..31) | {0, 1, 2, 3, 4, 5, 6, 7} | RACH-ConfigCommon | Cell-specific | 38.331 | Agreements - For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted. - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent. If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}. Agreement For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted. FFS: [8] MCS index to be used for Msg3 re-transmission Working assumption : support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH. - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions. - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced. - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition. - No additional RAN1 specification impact Note: UE reports Msg3 repetition capability after initial access. Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH. Agreement The 3 LSB bits of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI is used to indicate one value from 8 candidate MCS indexes for Msg3 retransmission. The 8 candidate MCS indexes can be configured by SIB1, MCS 0~7 are applied if the configuration is absent. The first 4 indexes of the 8 candidate MCS indexes are used for initial PUSCH transmission scheduled by RAR UL grant. |

Conclusion

- Same Tx beam is applied for all repetitions of Msg3 initial transmission scheduled by a RAR UL grant.

  - It is up to UE implementation how the Tx beam forms.

- Same Tx beam is applied for all repetitions of Msg3 re-transmission scheduled by a DCI scrambled with TC-RNTI.

  - It is up to UE implementation how the Tx beam forms.

Agreement

Adopt the following text proposal for Clause 7.3.1.1.1 in TS 38.212 h00.

##### 7.3.1.1.1 Format 0_0 **Unchanged parts are omitted** The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – number of bits determined by the following: - bits if the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is not configured, where - is the size of the initial UL bandwidth part. - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Clause 8.3 of [5, TS 38.213], where if and otherwise - bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provide the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214] - If the higher layer parameter useInterlacePUCCH-PUSCH in BWP-UplinkCommon is configured - 5 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz - Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214] - If the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], 5 bits as defined in Clause 6.1.2.1 and Clause 6.1.4.1 of [6, TS 38.214], - else, 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]. - New data indicator – 1 bit, reserved - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits, reserved - TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213] - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise - Padding bits, if required. - UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB **Unchanged parts are omitted**

Agreement

Adopt the following text proposal for Clause 6.1.4.1 in TS 38.214 h00

#### 6.1.4.1 Modulation order and target code rate determination For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a fallbackRAR UL grant or for a MsgA PUSCH transmission, or for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI, and if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3 - if mcs-TableDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel; - elseif the UE is not configured with MCS-C-RNTI, mcs-TableDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-Table in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-Table in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter msgA-MCS for IMCS and Table 5.1.3.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - elseif for PUSCH repetition type A the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-4, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. else - if mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoderDCI-0-2 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoder in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam256', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam64LowSE', - if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or - if PUSCH is transmitted with configured grant, - the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif for a MsgA PUSCH transmission, - the UE shall use higher layer parameter MsgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel. - the UE shall use q=2 for determining modulation order Qm in Table 6.1.4.1-1. - elseif for PUSCH repetition type A the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], when transmitting PUSCH scheduled by RAR UL grant, - the 2 LSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-3, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - elseif the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by the TC-RNTI, - the 3 LSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by the TC-RNTI provide a codepoint to determine the MCS index IMCS according to Table 6.1.4.1-4, based on whether or not the higher layer parameter mcs-Msg3Repetition is configured. The UE shall use the determined IMCS and Table 6.1.4.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. - else - the UE shall use IMCS and Table 6.1.4.1-1to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel. end <Unchanged parts are omitted> Table 6.1.4.1-3: MCS index IMCS as a function of 2 LSBs of MCS information field in RAR UL grant mcs-Msg3Repetition is configured; mcs-Msg3Repetitions is not configured. Codepoint; IMCS; Codepoint; IMCS. 00; First value of mcs-Msg3Repetition; 00; 0. 01; Second value of mcs-Msg3Repetition; 01; 1. 10; Third value of mcs-Msg3Repetition; 10; 2. 11; Fourth value of mcs-Msg3Repetition; 11; 3. Table 6.1.4.1-4: MCS index IMCS as a function of 3 LSBs of MCS information field in DCI format 0_0 with CRC scrambled by the TC-RNTI mcs-Msg3Repetition is configured; mcs-Msg3Repetitions is not configured. Codepoint; IMCS; Codepoint; IMCS. 000; First value of mcs-Msg3Repetition; 000; 0. 001; Second value of mcs-Msg3Repetition; 001; 1. 010; Third value of mcs-Msg3Repetition; 010; 2. 011; Fourth value of mcs-Msg3Repetition; 011; 3. 100; Fifth value of mcs-Msg3Repetition; 100; 4. 101; Sixth value of mcs-Msg3Repetition; 101; 5. 110; Seventh value of mcs-Msg3Repetition; 110; 6. 111; Eighth value of mcs-Msg3Repetition; 111; 7. <Unchanged parts are omitted>

Agreement

Adopt the following text proposal for Clause 8.3 in TS38.213 h00

## 8.3 PUSCH scheduled by RAR UL grant <Unchanged parts are omitted> A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. If the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321], Tthe UE transmits repeats the PUSCH transmission over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0 with CRC scrambled by the TC-RNTI, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <Unchanged parts are omitted>

In the contribution, we discuss remaining issues for Msg3 PUSCH repetition for coverage enhancement. Our views on enhancements on PUSCH repetition type A, TB processing over multi-slot PUSCH and joint channel estimation for PUSCH are described in our companion contributions [2], [3] and [4], respectively.

# Discussion on response to RAN2 LS

In the RAN2 LS, the following question was asked regarding UL BWP with PRACH resources only for RACH with Msg3 repetition [5]:

1. Overall Description: RAN2 discussed the issue whether network can configure PRACH resources only for RACH with Msg3 repetition in a dedicated UL BWP (i.e. for use in RRC_CONNECTED) and made the following working assumption: - “Working assumption: From RAN2’s perspective, a dedicated UL BWP can be configured with only CE RACH resources. Its feasibility is to be confirmed by RAN1.” However, RAN2 did not reach consensus on whether such a configuration is feasible from RAN1’s perspective. 2. Actions: To RAN1 ACTION: RAN2 respectfully asks RAN1 to discuss the above working assumption and then inform RAN2 whether such a configuration for RACH with Msg3 repetition is feasible or not.

At the RAN1#105-e meeting, the following agreement was made that separate preambles with shared ROs can be configured to request Msg3 PUSCH repetition [6]. Further, it is FFS whether Option 2 or Option 3 can be additionally supported for requesting Msg3 PUSCH repetition.

Agreement: - For requesting Msg3 PUSCH repetition, support the following: - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs. - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE. - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH). - FFS whether or not to additionally support one (& only one) more option: - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs - E.g., Option 3: Use separate RO, which include - the separate RO configured by a separate RACH configuration index from legacy UE, and - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

In our view, when number of preambles associated with an SSB is limited in the network, separate ROs can be configured for enhanced UEs, which can help in minimizing the impact on legacy system due to congestions on PRACH resources. Further, when separate ROs are configured, different PRACH formats may be configured for enhanced UEs and legacy UEs. For instance, PRACH format 0 may be configured for legacy UEs, while PRACH format 1 may be configured for enhanced UEs that request Msg3 PUSCH repetition. This may help in improving the PRACH coverage performance for cell edge UEs by using PRACH format 1.

Based on the discussions above, separate ROs can be additionally supported for requesting Msg3 PUSCH repetition. Note that when separate ROs are configured for requesting Msg3 PUSCH repetition, the working assumption from RAN2 can be confirmed, i.e., a dedicated UL BWP can be configured with only CE RACH resources.

Proposal 1

- Separate ROs can be additionally supported for requesting Msg3 PUSCH repetition.

- RAN2 working assumption that a dedicated UL BWP can be configured with only CE RACH resources is confirmed if separate ROs can be additionally supported for requesting Msg3 PUSCH repetition.

# Text proposal for repetition factor determination for Msg3

At the RAN1#107b-e meeting, the TP for repetition factor determination for Msg3 initial transmission and retransmission was agreed [1]. However, the current description on repetition factor determination for Msg3 repetition is not accurate.

The following TP is proposed for repetition factor determination for Msg3 initial transmission and retransmission.

------------------------------ TP#1: TS 38.213----------------------------------- ## 8.3 PUSCH scheduled by RAR UL grant < Unchanged text omitted > A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. The UE determines whether or not the RAR UL grant or the DCI format 0_0 indicates a number of NPUSCHrepeat repetitions for the PUSCH transmission based on a set of PRACH preambles that includes the PRACH preamble associated with the PUSCH transmission [11, TS 38.321]. When the UE requests repetition of PUSCH scheduled by RAR UL grant [11, TS 38.321] and if If the RAR UL grant or the DCI format 0_0 indicates NPUSCHrepeat repetitions for the PUSCH transmission, the UE transmits the PUSCH over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0. The UE determines a MCS for the PUSCH transmission by the 2 LSBs of the MCS field in the RAR UL grant or by the 3 LSBs of the MCS field in the DCI format 1_0, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. < Unchanged text omitted >

Proposal 2

- Agree on TP#1 for repetition factor determination for Msg3 initial transmission and retransmission.

# Text proposal for Tx beam for Msg3 repetitions

At the RAN1#107b-e meeting, it was concluded that same Tx beam is applied for all repetitions of Msg3 initial transmission and same Tx beam is applied for all repetitions of Msg3 re-transmission. In addition, it is up to UE implementation how the Tx beam forms [1]. In our view, this conclusion should be clearly captured in the specification. Otherwise, UE behaviour for the Tx beam determination during Msg3 repetitions is undefined and gNB may not know which corresponding Rx beams are employed for Msg3 reception during repetitions.

The following TP is proposed for Tx beam determination for Msg3 repetitions.

------------------------------ TP#2: TS 38.213----------------------------------- ## 8.3 PUSCH scheduled by RAR UL grant < Unchanged text omitted > A UE can be provided in RACH-ConfigCommon a set of numbers of repetitions for a PUSCH transmission with PUSCH repetition Type A that is scheduled by a RAR UL grant or by a DCI format 0_0 with CRC scrambled by a TC-RNTI. The UE determines whether or not the RAR UL grant or the DCI format 0_0 indicates a number of NPUSCHrepeat repetitions for the PUSCH transmission based on a set of PRACH preambles that includes the PRACH preamble associated with the PUSCH transmission [11, TS 38.321]. If the RAR UL grant or the DCI format 0_0 indicates NPUSCHrepeat repetitions for the PUSCH transmission, the UE transmits the PUSCH over NPUSCHrepeat slots, where NPUSCHrepeat is indicated by the 2 MSBs of the MCS field in the RAR UL grant or in the DCI format 0_0. The UE determines a MCS for the PUSCH transmission by the 2 LSBs of the MCS field in the RAR UL grant or by the 3 LSBs of the MCS field in the DCI format 1_0, and determines a redundancy version and RBs for each repetition as described in [6, TS 38.214]. For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆ where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. If NPUSCHrepeat>1, PUSCH repetitions scheduled by a RAR UL grant use a same spatial filter, and PUSCH repetitions scheduled by a DCI format 0_0 with CRC scrambled by a TC-RNTI use a same spatial filter. < Unchanged text omitted >

Proposal 3

- Agree on TP#2 for Tx beam determination for Msg3 repetitions.

# Conclusions

In this contribution, we discussed remaining issues for Msg3 PUSCH repetition for coverage enhancement. Further, we summarize the proposals as follows:

Proposal 1

- Separate ROs can be additionally supported for requesting Msg3 PUSCH repetition.

- RAN2 working assumption that a dedicated UL BWP can be configured with only CE RACH resources is confirmed if separate ROs can be additionally supported for requesting Msg3 PUSCH repetition.

Proposal 2

- Agree on TP#1 for repetition factor determination for Msg3 initial transmission and retransmission.

Proposal 3

- Agree on TP#2 for Tx beam determination for Msg3 repetitions.

# References

- Chairman’s notes, RAN1 #107b e-Meeting, January 2022

- R1-2201708, “Remaining details of enhancements on PUSCH repetition type A”, Intel Corporation, e-Meeting, February 2022

- R1-2201709, “Remaining details on TB processing over multi-slot PUSCH”, Intel Corporation, e-Meeting, February 2022

- R1-2201710, “Remaining details on joint channel estimation for PUSCH”, Intel Corporation, e-Meeting, February 2022

- R1-2200885, “LS on UL BWP with PRACH resources only for RACH with Msg3 repetition”, February 2022

- Chairman’s notes, RAN1 #105 e-Meeting, May 2021