3GPP TSG RAN WG1 #108-e			    R1-2201722

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	UE Features for NR MBS

Agenda item:	8.16.12

Document for:	Discussion and Decision

# Introduction

In this paper, we provide our views on the latest version of RAN1 UE features for Rel-17 NR MBS [2].

# Discussion

The initial version of NR MBS UE feature list provides a set of feature groups based on current design of NR MBS. We have provided our initial feedback in Table 1. The proposed changes are marked in red in the table. In summary we propose the following updates to the initial version of the UE Feature list:

- FG 33-1

  - Support of DCI indicated slot level repetition can be separated into a dependent FG 33-1a

- FG 33-2

  - Separate support of DCI 4_2 into a dependent FG

  - The PTP and PTM retransmission for MBS should be separated from 33-2 and added to dependent FG on ACK/NACK based HARQ feedback

- FG 33-2a

  - Merge 33-2c into FG 33-2a

- FG 33-4

  - Add component for support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signaling

- FG 33-5-1

  - Separate HARQ-ACK feedback and slot repetition support into separate dependent FGs

- FG 33-7:

  - Add NACK-only feedback also to this FG

The changes proposed are shown in red text in the table below from [2].

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH FFS DCI indicated slot-level repetition up to [8 or 16] for MTCH is defined as another FG |  |
| 33. NR_MBS | 33-1a | Dynamic repetition indication for broadcast | - Support DCI indicated slot-level repetition up to [8 or 16] for MTCH | 33-1 |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast FFS whether to separate the capability for support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast |  |
| 33. NR_MBS | 33-2x | Support of DCI format 4_2 for dynamic scheduling | - Support of DCI format 1_1 with CRC scrambled with G-RNTI for multicast | 33-2 |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling - Support of PTM retransmission for multicast At least 33-2c or 33-2d is merged, FFS which one or both | 33-2 |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a |
| 33. NR_MBS | 33-2c | PTM retransmission for multicast | Support of PTM retransmission for multicast FFS whether to merge with 33-2a | 33-2a |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast FFS whether to merge with 33-2a | 33-2a |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | 33-2 |
| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. - Support TDM-ed Type-2 HARQ-ACK codebook for multicast. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 |
| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | TBD |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot | 33-2b |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | Support NACK-only based HARQ-ACK feedback and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling | 33-2 |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-4 |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. - Support slot-level repetition for SPS group-common PDSCH FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 |
|  | 33-5-1a | HARQ-ACK Feedback for SPS group-common PDSCH for multicast | - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. | 33-5-1 |
|  | 33-5-1b | Slot level repetition for SPS group-common PDSCH for multicast | - Support slot-level repetition for SPS group-common PDSCH | 33-5-1 |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling [ACK/NACK and NACK-only based] HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK and NACK-only based] HARQ-ACK feedback | 33-2 |
| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1 |

# Conclusions

In this paper, we provided feedback on the RAN1 UE feature list for NR MBS.

# References

- RP-193248, New Work Item on NR Support of Multicast and Broadcast Services, Huawei, RAN#86, Sitges, Spain, December 2019.

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, RAN1#107bis-e, Jan 2022

- 3GPP TS 38.211 v17.0.0, NR Physical Channels and Modulation (Release 17)

- 3GPP TS 38.212 v17.0.0, NR Multiplexing and Channel Coding (Release 17)

- 3GPP TS 38.213 v17.0.0, NR Physical Layer Procedures for Control (Release 17)

- 3GPP TS 38.214 v17.0.0, NR Physical Layer Procedures for Data (Release 17)