3GPP TSG RAN WG1 #108-e					        													R1-2201729

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	On UE features for NR NTN

Agenda item:	8.16.4

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide discussion on UE features for NR NTN based on the latest proposal in [1]. Table with the details on UE features for NR NTN is captured in the Appendix (copied from [1] with proposed changes highlighted by red font).

# Discussion

We have the following comments for the UE feature list for NR NTN in [1].

- Words Satellite and HAPS can be removed since it is already clear that operation with satellite or HAPS is assumed for NTN.

- UE capability for NTN can be indicated per band to allow more flexibility for UE implementation.

- For Component 2 in FG 26-1, it is not necessary to describe UE behavior when common TA is not provided. So, we propose to delete the text in yellow.

- Component 4 in FG 26-1 is not needed considering that other components in this FG cover support of UE-specific TA, Common TA and combination of open-loop and closed-loop TA.

- FG 23-3 can be merged with FG 23-1 since timing relationship enhancements are essential part of NTN operation.

  - For Components 1 and 2 in FG 23-3, FFS can be removed.

  - Component 3 in FG 23-3 can be removed.

- For FG 26-4, many aspects for UE reporting of information about the UE specific TA pre-compensation were defined in RAN2 including reporting of TA during initial access and in RRC_CONNECTED state and reporting of UE location information. Thus, in our view RAN2 can define the details on the UE capability signaling for the exact content of UE reporting of information about the UE specific TA pre-compensation.

- For FG 26-6 and FG 26-6a, Component 2 should be supported since HARQ disabling feature have impact not only for the HARQ codebook construction but also for the restrictions to receive the PDSCH with the same HARQ process ID before the corresponding ACK-NACK feedback.

- For Component 1 in FG 26-6 and FG 26-6a, the HARQ codebook enhancements are supported only for disabled HARQ processes, so it can be stated in the component description.

- For FG 26-6a, in our view HARQ disabling can be an optional feature for UE supporting NTN. So, there is no need to merge it with FG 26-1.

- Enhancements for Type-3 HARQ codebook with disabled HARQ feedback were introduced at the last RAN1 meeting. Thus, new FG 23-6b should be introduced.

- FG 26-7 can be deleted since enhancements for the PDSCH repetition are not supported for Rel-17 NR NTN.

- For Component 2 in FG 26-8, there is no UE behavior specified by RAN1 for indication of polarization in SI. So, the corresponding component in RAN1 UE feature list is not needed. In our view the indication of polarization supported by the UE is not mandatory for NTN UE. Regarding the consequence if the feature is not supported by the UE, there is no need to specify particular assumption on UE polarization in case this FG is not reported.

- For Component 2 in FG 26-9, UE-specific K_offset for PDCCH ordered PRACH is not supported.

# Conclusion

In this contribution, we provide discussion on UE features for Rel-17 NR NTN. The following proposal is made.

Proposal:

- Consider changes to the UE feature list for NR NTN marked below using red font

# References

- R1-2112902, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e

# Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network [(UE considers common TA as 0 if the parameter is not provided)] - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ARTG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset - FFS: delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | FFS: whether this FG gets merged with FG 26-1 Merge this FG with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | Support UE reporting of information about the UE specific TA pre-compensation - [The UE capability signalling for Tthe exact content of UE reporting of information about the UE specific TA pre-compensation is up to RAN2 e.g., frequency of the reports, granularity of the reported conten, etc.] | [26-1] | Yes | No |  | [Per UE/Per band] | No | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | - The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported | [Per band or per FSPC or per UE] | No | No |  | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook Enhancement | - Support of type-2 HARQ codebook enhancements [for feedback-disabled HARQ processes] - FFS: UE supports HARQ disabling |  | Yes | No |  | [Per UE/per band] | No | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | [26-6a] | [Type-1 HARQ codebook enhancement] | - Enhancement on Type-1 HARQ codebook in NTN for feedback-disabled HARQ processes - FFS: UE supports HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6b | Type-3 HARQ codebook enhancement | - Enhancement on Type-3 HARQ codebook in NTN for feedback-disabled HARQ processes - UE supports HARQ disabling | [26-1, 26-2] | Yes | No |  | per band | No | No | [support mixture of FDD/TDD and/or FR1/FR2] |  | Optional with capability signalling Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported |
| 26. NR_NTN_solutions | [26-7] | [NTN Performance enhancement] | - The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 16 or 32 |  | Yes | No |  | [Per UE/per band/Per BC] | [No] | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network |  | No | No | [It is assumed by the network that UE supports at least linear polarization] | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - Determining the timing of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-3, 26-4 | Yes | No |  | FFS | No | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |