3GPP TSG RAN WG1 Meeting #108-e			                       R1- 2201732

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	Discussion on UE features related to UE Power Saving

Agenda item:	8.16.7

Document for:	Discussion and Decision

Introduction

In this contribution, we express our views on UE features regarding paging enhancements, TRS/CSI-RS occasions in idle/inactive mode, and PDCCH monitoring adaptation in active time based on [1].

# Discussion on UE features for UE power saving

Paging enhancements

For the FG 29-1, we have the following suggestions:

- Since UE sub-grouping information is only carried via PEI, then it makes sense to group support of PEI and UE subgrouping indication under a common FG.

  - We expect that RAN2 will provide a decision soon on the issue following the LS sent in RAN1 107bis-e.

  - Moreover, we suggest to update the component description as follows:

1. Support receiving paging early indication via DCI format 2_7 2. Support receiving UE subgroup indication via DCI format 2_7

- “Note” column should capture the following:

  - maximum number of sub-groups per PO can be eight

  - Behv-A for PEI detection

- Although it has been left to RAN2 to decide, we think this FG should be optional with capability signaling. Signaling is needed since gNB needs to select appropriate UE sub-grouping methods. It also seems to be already implied from RAN2’s agreement below:

UE’s capability of supporting the UE ID-based subgrouping is reported to RAN by AS UE capability signalling while R2 assumes that UE’s capability of supporting the CN-assigned subgrouping is reported to CN by NAS signalling. Introduce a UERadioPagingInfo IE in the UECapabilityInformation message in NR in Rel-17.

- It seems companies’ views are divided regarding whether the reporting will be per UE or per band. Some companies raised concern on signaling overhead if Per band is used. Per band capability signaling was mostly motivated considering licensed/unlicensed band differentiation. However, licensed/unlicensed band differentiation can also be achieved using separate bit in per UE capability signaling. Hence, as compromise, we are supportive of per UE with separate bits to indicated support for licensed and unlicensed bands, such as follows:

PagingEnhancement-r17 SEQUENCE { licensedBand-r17 ENUMERATED {supported} OPTIONAL, unlicensedBand-r17 ENUMERATED {supported} OPTIONAL } OPTIONAL,

Proposal 1: Support of PEI and UE sub-grouping can be a common FG 29-1.

- Support of this FG can be Per UE with licensed/unlicensed band differentiation.

Proposal 2: Support of FG 29-1 should be optional with capability signalling.

Proposal 3: Update FG 29-1 component description as follows:

1. Support receiving paging early indication via DCI format 2_7 2. Support receiving UE subgroup indication via DCI format 2_7

TRS/CSI-RS occasions

For the FG 29-2, we have the following suggestions

- Update component description as “Support receiving L1 indication for TRS availability via paging DCI”. It is expected that paging DCI based indication would be default.

  - If PEI is configured and include TRS availability indication field, a separate FG can be created such as FG 29-2A where FG 29-2 and FG 29-1 can be prerequisite.

  - If there is no convergence to agree separate FG for PEI based TRS availability indication, we suggest to at least revise component description of FG 29-2 to make it more clear how the feature works. Supporting PEI based TRS availability indication would require the UE to support FG 29-1. Without support of FG 29-1, it implies that UE can only receive TRS availability from paging DCI. To this end, we suggest following revisions by explicitly capturing the candidate L1 signaling options for clarity. Support of FG 29-2 should not imply UE supports FG 29-1.

TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI format 1_0 3. Support receiving L1 indication for TRS availability via DCI format 2_7 if the UE supports FG 29-1

- This FG can be optional without capability signaling. We are OK to leave it to RAN2 for decision.

  - If optional with capability signaling is selected by RAN2, then reporting can be Per UE with licensed/unlicensed band differentiation, similar to FG 29-1. If optional without capability signaling is adopted, then Per UE seems sufficient.

- Update description under  “Consequence if the feature is not supported by the UE”, as UE does not support TRS occasions for idle/inactive UEs

Observation 1: Current component description of FG 29-2 does not clearly describe how the feature works, such as whether support of L1 indication mandates support of receiving PEI as well when TRS is configured.

Proposal 4: Support one of the following regarding FG 29-2:

- If PEI based TRS availability indication is supported, a separate FG can be created such as FG 29-2A where FG 29-1 and FG 29-2 can be prerequisite. In that case, FG 29-2 indicates support of receiving TRS availability via DCI format 1_0 only.

- If separate FG for PEI based availability indication cannot be agreed, then at least update component description of FG 29-2 as follows to make it more clear.

TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI format 1_0 3. Support receiving L1 indication for TRS availability via DCI format 2_7 if the UE supports FG 29-1

Proposal 5: RAN2 can confirm optional with or without capability signalling for FG 29-2. Based on that, reporting type can be finalized.

PDCCH monitoring adaptation in active time

For PDCCH monitoring adaptation FG 29-3, we can accept reporting type per UE with licensed/unlicensed band differentiation.

Proposal 6: Support of FG 29-3 can be per UE with licensed/unlicensed band differentiation.

# Conclusions

In summary, we have following observations and suggestions to update the feature lists table for FGs 29-1/2/3.

Observation 1: Current component description of FG 29-2 does not clearly describe how the feature works, such as whether support of L1 indication mandates support of receiving PEI as well when TRS is configured.

Proposal 1: Support of PEI and UE sub-grouping can be a common FG 29-1.

- Support of this FG can be Per UE with licensed/unlicensed band differentiation.

Proposal 2: Support of FG 29-1 should be optional with capability signalling.

Proposal 3: Update FG 29-1 component description as follows:

1. Support receiving paging early indication via DCI format 2_7 2. Support receiving UE subgroup indication via DCI format 2_7

Proposal 4: Support one of the following regarding FG 29-2:

- If PEI based TRS availability indication is supported, a separate FG can be created such as FG 29-2A where FG 29-1 and FG 29-2 can be prerequisite. In that case, FG 29-2 indicates support of receiving TRS availability via DCI format 1_0 only.

- If separate FG for PEI based availability indication cannot be agreed, then at least update component description of FG 29-2 as follows to make it more clear.

TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI format 1_0 3. Support receiving L1 indication for TRS availability via DCI format 2_7 if the UE supports FG 29-1

Proposal 5: RAN2 can confirm optional with or without capability signalling for FG 29-2. Based on that, reporting type can be finalized.

Proposal 6: Support of FG 29-3 can be per UE with licensed/unlicensed band differentiation.

References

[1]    R1-2200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e”, Moderators (AT&T, NTT DOCOMO, INC.)