3GPP TSG RAN WG1 #108-e																		    R1-2201733

e-Meeting, February 21st – March 3rd, 2022

Source: 	Intel Corporation

Title:	Discussion on UE features for NR coverage enhancement

Agenda item:	8.16.8

Document for:	Discussion and Decision

# Introduction

At the RAN1#107b-e meeting, the following agreements were made regarding UE features for NR coverage enhancement [1].

Agreement

- Further discuss whether/how to separate/merge FGs 30-1 and 30-1a from following options

  - Option 1: Keep current structure, i.e. FG 30-1 for DG, 30-1a for type 1 and 2 CG

  - Option 3: Merge FGs 30-1 and 30-1a into an FG

Agreement

- Further discuss whether/how to separate/merge FGs 30-2 and 30-2a from following options

  - Option 1: Keep current structure, i.e., FGs 30-2 for DG, 30-2a for type 1 and 2 CG

  - Option 3: Merge FGs 30-2 and 30-2a into an FG

Agreement

- Further discuss whether/how to separate FG 30-3 from following options

  - Option 1: Keep current structure

  - Option 3: Split 30-3 into 2 separate FGs: 1st one for DG, 2nd one for CG

Agreement

- FG 30-3 and FG30-3a are not merged

Agreement

- FG 30-4x are split to back-to-back transmission and non-back-to-back transmission

  - FFS whether to introduce as many FG as FG30-4x or one FG for all FG30-4x for non-back-to-back transmission

Agreement

- FG for intra-slot frequency hopping and inter-slot frequency hopping for TBoMS is not introduced

Agreement

- Introduce one FG for all FG30-4x for non-back-to-back transmission as follows

| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) | 30-4a, 30-4b, 30-4c, or 30-4d | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FGs 30-4 to 30-4d are not merged

  - FFS: whether to Add a note in FG 30-4c: “Note: If a UE reports support of FG [30-4a], 30-4c, [30-3] and/or 30-3a, the UE supports DMRS bundling for the repetitions of TBoMS”

Agreement

- FGs 30-4e and 30-4f are not merged

Agreement

- Component of FG 30-4d is updated as “Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4”

In the contribution, we discuss UE features for NR coverage enhancement. The updated UE features for NR coverage enhancement after RAN1#107b-e meeting was captured in [2] and are listed in the Appendix.

# Discussion on UE features for NR coverage enhancement

## UE features for PUSCH repetition type A enhancement

At the RAN1#107b-e meeting, it was agreed to further discuss whether/how to separate/merge FGs 30-1 and 30-1a, FGs 30-2 and 30-2a for PUSCH reception type A enhancement, respectively [1]. In our view, it is more appropriate to divide the UE feature group into DG-PUSCH and CG-PUSCH for both increased maximum number of repetitions and PUSCH repetition type A with counting based on available slots. In this regard, current structure needs to be kept and merging FG 30-1a with FG 30-1, and FG 30-2a with FG 30-2 is not necessary.

At the RAN1#107-e meeting, it was agreed that increased maximum number of repetitions is supported for Type 1 and Type 2 CG-PUSCH. In particular, repK in ConfiguredGrantConfig supports up to 32 repetitions [1]. Based on this agreement, Type 1 CG-PUSCH needs to be included as part of FG30-1a.

Based on the discussions above, Table 1 illustrates suggested update for UE feature groups for PUSCH repetition type A enhancement.

Table 1. UE feature groups for PUSCH repetition type A enhancement

| Index | Feature group | Components |
| --- | --- | --- |
| 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. |
| 30-1a | Increased maximum number of Type 2 configured grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV indicated in a TDRA list and by repK-r17. A row index of the TDRA list is indicated by a Type 1 and Type 12 configured grant configuration. |
| 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. |
| 30-2a | Configured grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for Type 1 and Type 2 configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. |

Proposal 1

- For UE feature groups of PUSCH repetition type A enhancement,

  - Option 1 is adopted, i.e., keep current structure with FG 30-1 and FG 30-2 for DG, 30-1a and 30-2a for type 1 and 2 CG.

  - UE features for PUSCH repetition type A enhancement are defined per UE.

  - FDD/TDD differentiation is not necessary.

  - Consider Table 1 for UE feature groups of PUSCH repetition type A enhancement.

## UE features for TBoMS

At the RAN1#107b-e meeting, it was agreed to further discuss whether/how to separate FG 30-3 [1]. Note that as agreed in the RAN1#106-e meeting, TB processing over multiple slots (TBoMS) is supported for both configured grant and dynamic grant [4]. Similar to PUSCH repetition type A enhancement, it is more appropriate to differentiate DG-PUSCH and CG-PUSCH for the UE feature group.

Table 2 illustrates suggested update for UE feature groups for TBoMS.

Table 2. UE feature groups for TBoMS

| Index | Feature group | Components |
| --- | --- | --- |
| 30-3 | TB processing over multi-slot PUSCH for DG-PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode for DG-PUSCH. |
| 30-3a | TB processing over multi-slot PUSCH for CG-PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode for CG-PUSCH. |

Proposal 2

- For UE feature groups of TBoMS,

  - Option 3 is adopted, i.e., split 30-3 into 2 separate FGs: 1st one for DG, 2nd one for CG.

  - UE features for TBoMS are defined per UE.

  - FDD/TDD differentiation is not necessary.

  - Consider Table 2 for UE feature groups of TBoMS.

## UE features for DMRS bundling for PUSCH and PUCCH

At the RAN1#107b-e meeting, the following agreement was made for DMRS bundling restart during a nominal TDW [1].

Agreement Update the description of the RRC parameters PUSCH-Window-Restart and PUCCH-Window-Restart as follows. - UE bundles PUSCH DM-RS remaining in a nominal time domain window after event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements - UE bundles PUCCH DM-RS remaining in a nominal time domain window after event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.

Based on this, Table 3 illustrates suggested update for UE feature groups for DMRS bundling.

Table 3. UE feature groups for DMRS bundling

| 30-4g | Restart DM-RS bundling after the event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity | Support restarting DM-RS bundling after the event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g., frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded. | [30-4] 30-4a, 30-4b, 30-4c, or 30-4d |
| --- | --- | --- | --- |
| 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d), respectively, | 30-4a, 30-4b, 30-4c, or 30-4d |

Proposal 3

- Consider Table 3 for UE feature groups of DMRS bundling.

## UE features for dynamic repetition factor indication for PUCCH

At the RAN1#106b-e meeting, the following agreement was made for dynamic PUCCH repetition indication [3].

Agreement Support dynamic PUCCH repetition factor indication for all PUCCH formats including format 0, 1, 2, 3, 4 with a unified mechanism as agreed in RAN1#106e under agenda 8.8.2. Note: it does not impact the discussion of slot level or sub-slot level repetition

Based on this and considering that slot based PUCCH repetition is mainly targeted for coverage enhancement, Table 4 illustrates suggested update for UE feature groups for dynamic PUCCH repetition factor indication.

Table 4. UE feature groups for dynamic PUCCH repetition factor indication

| 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 | 4-23 and/or 25-2 |
| --- | --- | --- | --- |

Proposal 4

- For UE feature groups of dynamic PUCCH repetition factor indication

  - FG 30-5 is not separated to multiple FG

  - UE features for dynamic PUCCH repetition factor indication are defined per UE.

  - FDD/TDD differentiation is not necessary.

  - Consider Table 4 for UE feature groups of dynamic PUCCH repetition factor indication.

# Conclusions

In this contribution, we discussed UE features for NR coverage enhancement. Further, we summarize the proposals as follows:

Proposal 1

- For UE feature groups of PUSCH repetition type A enhancement,

  - Option 1 is adopted, i.e., keep current structure with FG 30-1 for DG, 30-1a for type 1 and 2 CG and FGs 30-2 for DG, 30-2a for type 1 and 2 CG.

  - UE features for PUSCH repetition type A enhancement are defined per UE.

  - FDD/TDD differentiation is not necessary.

Proposal 2

- For UE feature groups of TBoMS,

  - Option 3 is adopted, i.e., split 30-3 into 2 separate FGs: 1st one for DG, 2nd one for CG.

  - UE features for TBoMS are defined per UE.

  - FDD/TDD differentiation is not necessary.

  - Consider Table 2 for UE feature groups of TBoMS.

Proposal 3

- Consider Table 3 for UE feature groups of DMRS bundling.

Proposal 4

- For UE feature groups of dynamic PUCCH repetition factor indication

  - FG 30-5 is not separated to multiple FG

  - UE features for dynamic PUCCH repetition factor indication are defined per UE.

  - FDD/TDD differentiation is not necessary.

  - Consider Table 4 for UE feature groups of dynamic PUCCH repetition factor indication.

# References

- Chairman’s notes, RAN1 #107b e-Meeting, January 2022

- R1-200780, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e”, January 2022

- Chairman’s notes, RAN1 #106b e-Meeting, October 2021

# Appendix:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions for dynamic grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. | [5-17] | Yes | N/A | UE does not support dynamic grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD | Yes | N/A |  | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A | FFS whether to Add a note in FG 30-4c: “Note: If a UE reports support of FG [30-4a], 30-4c, [30-3] and/or 30-3a, the UE supports DMRS bundling for the repetitions of TBoMS” | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) | 30-4a, 30-4b, 30-4c, or 30-4d | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 | 4-23 and/or 25-2 | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |