3GPP TSG RAN WG1 #108-e		R1- 2201755

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	7.1

Source:	Apple Inc.

Title:	On Remaining Issues for PUSCH UCI Multiplexing without HARQ-ACK PUCCH

Document for:	Discussion

# Introduction

When multiplexing UCI on the PUSCH, the specification is clear on the UE behavior when there are multiple overlapping PUSCHs each overlapping with a PUCCH carrying HARQ-ACK. However, when there are multiple overlapping PUSCHs with no overlapping PUCCH carrying HARQ-ACK e.g., in the case the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release on any serving cell, the UE behavior is unclear especially if the DCI scheduling the PUSCH(s) includes a TDAI.

In RAN1 #106-e, the behavior for Rel-15 UEs was clarified in this scenario. In RAN1 #107-e, the behavior for Rel-16 UEs in some specific scenarios was clarified. In this contribution, we seek to clarify the behavior for Rel-16 UEs with the remaining scenario i.e. more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if for at least one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or  equal to 1 (for Type 1 codebook).

# Discussion

## Scenarios

In the case of multiple overlapping PUSCHs with overlapping PUCCH, the original Rel-15. understanding is that the UE uses PUSCH prioritization rules to select a PUSCH and multiplexes HARQ-ACK information in the PUSCH according to the indicated value of DAI field in DCI format 0_1. This means that the UE would multiplex on at most one PUSCH. However, if the UE misses the DL DCI, then the UE behavior needs to be clarified. To assist in the discussion, the following example could be used. In the example, the UE misses the DL DCI and its associated PUCCH. On CC1, the UL TDAI can be set to X where X = 4 or X = {1, 2 or 3} while on CC2, UL DCI2 is set to 1. We identify this as case 0 (See Figure 1).

Figure 1: HARQ-ACK Transmission with overlapping PUSCH and no PUCCH

In addition to the original case, four new cases were defined in the discussions as follows (see Figure 2):

- Case 1: In uplink CA, there is a single standalone PUSCH, while there are other PUSCHs in the slot, but the standalone PUSCH does not overlap with other PUSCHs.

- Case 2: In uplink CA, there is a single PUSCH in a slot on a CC, and there are other PUSCH on other CCs in the same slot.

- Case 3: In uplink CA, PCC is FR1(30Khz), SCC is FR2 (120Khz). On SCC, each slot has a PUSCH. The missing PUCCH can overlap with 4 PUSCHs across 4 slots on SCC

- Case 4: No uplink CA. In one slot, UE only receives one PUSCH

Figure 2: Additional Cases

In RAN1 #106-e [5], the following conclusions and agreements were made for Rel-15 UEs:

| Scenario | Agreements/Conclusions/Status |
| --- | --- |
| Case 0: multiple overlapping PUSCHs with no overlapping PUCCH | For Rel-15, in the case of multiple overlapping PUSCHs with no overlapping PUCCH and if any UL-TDAI n.e. 4 (for Type 2 codebook) or UL-TDAI e.q. 1 (for Type 1 codebook) the UE behavior is left to UE implementation |
| Case 1, Case 2, Case 3 : more than one non-overlapping PUSCH and no overlapping PUCCH within a span on one slot (both single carrier and UL CA) | For Rel-15 with more than one non-overlapping PUSCH and no overlapping PUCCH within a span on one slot (both single carrier and UL CA) and if the UL-TDAI for the PUSCH UL-TDAI not equal to 4 (for Type 2 codebook) or UL-TDAI equal to 1 (for Type 1 codebook), the UE behavior is up to the UE implementation |
| Case 4 : one PUSCH and no overlapping PUCCH within a span of one slot | For Rel-15 with one PUSCH and no overlapping PUCCH within a span of one slot and if the UL-TDAI for the PUSCH UL-TDAI not equal to 4 (for Type 2 codebook) or UL-TDAI equal to 1 (for Type 1 codebook), there is no consensus for any conclusion on one aligned UE behavior. |

In RAN1 #107-e [6], the following conclusions and agreements were made for Rel-16 UEs:

| Scenario | Agreements/Conclusions/Status |
| --- | --- |
| Case 0: multiple overlapping PUSCHs with no overlapping PUCCH | For Rel-16 with multiple overlapping PUSCHs with no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot, if the UL-TDAI n.e. 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook) there is no consensus in RAN1 on Rel-16 UE behavior |
| Case 1, Case 2, Case 3 : more than one non-overlapping PUSCH and no overlapping PUCCH within a span on one slot (both single carrier and UL CA) | Continue discussion on UE behavior with respect to multiplexing HARQ-ACK in PUSCH for the following case in Rel-16: More than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if for at least one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook) |
| Case 4 : one PUSCH and no overlapping PUCCH within a span of one slot | For Rel-16 with one PUSCH and no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot (both single carrier and UL CA), if the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE multiplexes HARQ-ACK following the UL-TDAI into the PUSCH. |

For case 1, case 2 and case 3 with Rel-16 UEs, we make the following proposals:

We assume that the decisions are made at time t1 = time before PUSCH preparation timeline.

Proposal 1:

- Scenario 1: For Rel-16 UEs with more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if for at MOST one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or  equal to 1 (for Type 1 codebook)

  - In this case, the UE can multiplex on the PUSCH with UL-TDAI is not equal to 4 (for Type 2 codebook) or  equal to 1 (for Type 1 codebook)

- Scenario 2: For Rel-16 UEs with more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if MORE THAN one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or  equal to 1 (for Type 1 codebook) :

  - In this case, the UE behavior is similar to case 0.

- We assume that the decisions are made at time t1 = time before PUSCH preparation timeline

# Conclusion

In this contribution, would like to clarify the UE specification behavior when there is more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA) and propose to include the following behavior in the specification:

Proposal 1:

- Scenario 1: For Rel-16 UEs with more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if for at MOST one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or  equal to 1 (for Type 1 codebook)

  - In this case, the UE can multiplex on the PUSCH with UL-TDAI is not equal to 4 (for Type 2 codebook) or  equal to 1 (for Type 1 codebook)

- Scenario 2: For Rel-16 UEs with more than one non-overlapping PUSCH and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), if MORE THAN one of the PUSCHs the UL-TDAI is not equal to 4 (for Type 2 codebook) or  equal to 1 (for Type 1 codebook) :

  - In this case, the UE behavior is similar to case 0.

- We assume that the decisions are made at time t1 = time before PUSCH preparation timeline

# References

- 3GPP TS 38.213 V16.4.0 (2020-12), NR; Physical layer procedures for control, (Release 16)

- Chairman’s Notes, RAN1 #104-e

- Chairman’s Notes, RAN1 #97

- R1-2106317, Summary for [105-e-NR-7.1CRs-02] Discussions on PUSCH UCI Multiplexing without HARQ-ACK PUCCH        Moderator (Apple)

- R1-2108647 Summary for [106-e-NR-7.1CRs-07] Discussion on HARQ-ACK multiplexing on PUSCH without PUCCH Moderator (Apple)

- R1-2112859, Summary for [107-e-NR-7.1CRs-06] Discussion on HARQ-ACK multiplexing on PUSCH Moderator (Apple)

# Appendix

## Existing Specification

The UE behavior for UCI multiplexing with multiple overlapping PUSCHs is specified in TS 38.213 [3]:

If a UE transmits multiple PUSCHs in a slot on respective serving cells and the UE would multiplex UCI in one of the multiple PUSCHs and the UE does not multiplex aperiodic CSI in any of the multiple PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in Clause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in Clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot.

If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in Clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH.

This behavior assumes an overlapping PUCCH as captured in the conclusion in RAN1 #97 [3]:

conclusion For the issue raised in the draft CR R1-1906302, the intended UE behavior per specification is commonly understood as follows: - For UCI multiplexing, within a PUCCH group, on PUSCH, the following two steps are performed with step 1 first, then followed by step 2: - Step 1: UCI in overlapped PUCCH transmissions is multiplexed into one PUCCH resource (resource Z) on PCC. This step is done per PUCCH slot. - Step 2: UCI, that doesn’t include SR, in Z is multiplexed into one PUSCH, if Z overlaps with at least one PUSCH, following the priorities (sequentially from high to low) as listed below. - First priority: PUSCH with A-CSI as long as it overlaps with Z - Second priority: earliest PUSCH slot(s) based on the start of the slot(s) - If there are still multiple PUSCHs overlap with Z in the earliest PUSCH slot(s), follow the following priorities (sequentially from high to low) - Third priority: Dynamic grant PUSCHs > PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH - Fourth priority: PUSCHs on CC serving cell with smaller CC serving cell index > PUSCHs on CC serving cell with larger CC serving cell index - Fifth priority: Earlier PUSCH transmission > later PUSCH transmission Note: The clarification applies to both cases with the same (except the second priority part) and different numerologies among PUCCH and PUSCHs.

Once the PUSCH has been identified, then the behavior in the conclusion to the email discussion 104-e-NR-7.1CRs-15 held during RAN1 #104-e becomes applicable as follows [2]:

Conclusion For both Rel-15 and Rel-16, - If a UE is not provided PDSCH-CodeBlockGroupTransmission, for a type-2 HARQ-ACK codebook transmission in a PUSCH scheduled by DCI format 0_1 with V_UL_TDAI , - If UE has HARQ-ACK for SPS PDSCH only with B bit(s) to be transmitted in the PUSCH, - UE multiplexes 2*A+B bits HARQ-ACK in the PUSCH if UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks and harq-ACK-SpatialBundlingPUSCH is not provided. - UE multiplexes A+B bits HARQ-ACK in the PUSCH otherwise. - If a UE is provided PDSCH-CodeBlockGroupTransmission, for a type-2 HARQ-ACK codebook transmission in a PUSCH scheduled by DCI format 0_1 with first DAI field value , - If UE has HARQ-ACK for SPS PDSCH only in the first sub-codebook to be transmitted in the PUSCH, - UE multiplexes 2*A+B bits HARQ-ACK in the first sub-codebook in the PUSCH if UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks and harq-ACK-SpatialBundlingPUSCH is not provided. - UE multiplexes A+B bits HARQ-ACK in the first sub-codebook in the PUSCH otherwise.

However, there may be scenarios in which the PUSCH TDAI indicates HARQ-ACK bits are present but there is no DL DCI received by the UE that contains a  PRI indicating a PUCCH resource. As such, there is no PUCCH overlapping or colliding with the PUSCH(s). This discussion seeks to clarify the UE behavior in this case for both Rel-15 UEs and Rel-16 UEs.