3GPP TSG RAN WG1 #108-e	                                         	      R1-2201762

e-Meeting, Feb 21st – Mar 3rd, 2022

Agenda Item:	8.1.3

Source:	Apple Inc.

Title:	Views on Rel-17 SRS enhancement

Document for:	Discussion/Decision

# Introduction

This contribution addresses some maintenance issues regarding the Rel-17 SRS enhancement.

- Include SRS repetition with {10, 14} consecutive OFDM symbols in 38.211

- Introduce UE capability of support d N=4 for 1T4R and N=2 for 1T2R/2T4R, N is the number of aperiodic SRS resource sets for antenna switching

# SRS repetition with {10, 14} symbols

In Rel-17 SRS enhancement, we additionally support SRS repetition with {10, 14} symbols. After reviewing TS38.211, we think this enhancement is not captured in TS38.211. To resolve this, we have the following proposal

Proposal 1 Consider the following TP to TS38.211 to include  SRS repetition with {10, 14} consecutive OFDM symbols

#### 6.4.1.4 Sounding reference signal ##### 6.4.1.4.1 SRS resource An SRS resource is configured by the SRS-Resource IE or the SRS-PosResource IE and consists of - NapSRS∈1,2,4 antenna ports pii=0NapSRS-1, where the number of antenna ports is given by the higher layer parameter nrofSRS-Ports if configured, otherwise NapSRS=1, and pi=1000+i when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet not set to 'nonCodebook', or determined according to [6, TS 38.214] when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet set to 'nonCodebook' - NsymbSRS∈1,2,4,10,8,12,14 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping - l0, the starting position in the time domain given by l0=Nsymbslot-1-loffset where the offset loffset∈0,1,…,13 counts symbols backwards from the end of the slot and is given by the field startPosition contained in the higher layer parameter resourceMapping and loffset≥NsymbSRS-1 - k0, the frequency-domain starting position of the sounding reference signal

# Missing UE capability

We have the following agreement

Agreement For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional

This agreement is also implemented in TS38.214 in brackets as “[extension of aperiodic antenna switching SRS configuration]”. However, current UE feature list does not have this FG defined

Proposal 2 Ensure UE capability of “[extension of aperiodic antenna switching SRS configuration]” is introduced

# Conclusion

This contribution addresses some maintenance issues regarding the Rel-17 SRS enhancement.

- Include SRS repetition with {10, 14} consecutive OFDM symbols in 38.211

- Introduce UE capability of support d N=4 for 1T4R and N=2 for 1T2R/2T4R, N is the number of aperiodic SRS resource sets for antenna switching

We have the following proposals

Proposal 1 Consider the following TP to TS38.211 to include  SRS repetition with {10, 14} consecutive OFDM symbols

#### 6.4.1.4 Sounding reference signal ##### 6.4.1.4.1 SRS resource An SRS resource is configured by the SRS-Resource IE or the SRS-PosResource IE and consists of - NapSRS∈1,2,4 antenna ports pii=0NapSRS-1, where the number of antenna ports is given by the higher layer parameter nrofSRS-Ports if configured, otherwise NapSRS=1, and pi=1000+i when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet not set to 'nonCodebook', or determined according to [6, TS 38.214] when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet set to 'nonCodebook' - NsymbSRS∈1,2,4,10,8,12,14 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping - l0, the starting position in the time domain given by l0=Nsymbslot-1-loffset where the offset loffset∈0,1,…,13 counts symbols backwards from the end of the slot and is given by the field startPosition contained in the higher layer parameter resourceMapping and loffset≥NsymbSRS-1 - k0, the frequency-domain starting position of the sounding reference signal

Proposal 2 Ensure UE capability of “[extension of aperiodic antenna switching SRS configuration]” in TS38.214 is introduced. Consider the following TP

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differe-ntiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23-8-9 | Extension of aperiodic antenna switching SRS configurations for <=4Rx | Support of N=4 for 1T4R and N=2 for 1T2R/2T4R. N is the number of aperiodic SRS resource sets for antenna switching |  | Yes | N/A |  | Per FS | No | No |  |  | Optional with capability signaling |