3GPP TSG RAN WG1 #108-e		                               R1-2201771

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.4.1

Source:	Apple

Title:	Remaining Issues of Timing Relationship Enhancements for NR NTN

Document for:	Discussion/Decision

# Introduction

It was concluded [1] that release 17 RAN1 work on NR NTN is completed. The maintenance on solutions for NR NTN is conducted in RAN1 #108-e meeting.

One of the RAN1-lead objectives in release 17 NR NTN is timing relationship enhancements. In this contribution, we provide our views on some remaining issues related to timing relationship enhancements for NR NTN.

# Discussion

## Update of cell specific Koffset and Kmac

It was agreed in RAN1 that the information of cell specific Koffset and the information of Kmac are carried in system information. In a recent RAN2 LS, it is mentioned [2] that a new NTN-specific SIB at least carries a list of parameters for serving cell: ephemeris, common TA parameters, validity duration for UL sync information, t-Service (the timing information on when the serving cell is going to stop serving the area), cell reference location, epoch time, Kmac, cell-specific Koffset, indication for network enabled/disabled TA report.

In the NTN-specific SIB, ephemeris, common TA parameters and epoch time are used for uplink synchronization, while Kmac and cell-specific Koffset are used for timing relationship enhancement.

The possible validity duration values for uplink synchronization information are {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240} seconds. However, these validity duration values may not be appropriate for Kmac and cell-specific Koffset.

Consider the case of LEO satellite with moving speed of 7.8 km/s. After a validity duration of 5 or 240 seconds, the LEO satellite moves 39 km or 1872 km, which implies the RTT between satellite and UE or the RTT between satellite and gateway/gNB changes as large as  39000⋅23⋅108m/s=0.26 ms or 1872000⋅23⋅108m/s=12 ms, respectively. Considering the granularity of cell-specific Koffset or Kmac is in milliseconds for FR1, the validity duration for uplink synchronization information is either too long or too short for cell-specific Koffset or Kmac.

In summary, the timing relationship enhancement parameters (i.e., cell-specific Koffset or Kmac) generally have a validity duration from that for uplink synchronization information (i.e., ephemeris, common TA parameters and epoch time) in NTN-specific SIB.

Observation 1: The validity duration of timing relationship enhancement parameters (cell-specific Koffset and Kmac) in NTN-specific SIB is generally different from the validity duration of uplink synchronization parameters (ephemeris, common TA parameters and epoch time) in NTN-specific SIB.

The accuracy requirement for uplink synchronization is much stricter than that for timing relationship enhancement. Considering fast moving satellite, the uplink synchronization parameters generally change in successive NTN-specific SIB transmissions. With this update, the validity duration of uplink synchronization parameters may remain unchanged. This is illustrated in Figure 1, where in successive NTN-specific SIB transmissions, different ephemeris parameters are provided with a constant validity duration.

Observation 2: The values of uplink synchronization parameters (ephemeris, common TA parameters and epoch time) generally change in successive NTN-specific SIB transmissions.

On the other hand, Kmac and cell-specific Koffset are in the unit of milliseconds for FR1. The values of these timing relationship enhancement parameters generally do not change between successive NTN-specific SIB transmissions. The update of these timing relationship enhancement parameters may occur once in several NTN-specific SIB transmission periods. Subsequently, the ending time of validity duration for timing relationship enhancement parameters in successive NTN-specific SIB transmissions are the same. This implies that the validity duration for timing relationship enhancement parameters in one NTN-specific SIB transmission is different from that in another NTN-specific SIB transmission. This is illustrated in Figure 1, where in successive NTN-specific SIB transmissions, the same cell-specific Koffset value is provided, but with reduced validity durations.

Observation 3: The validity durations of timing relationship enhancement parameters generally change in successive NTN-specific SIB transmissions.

Figure 1: Illustration of uplink synchronization parameters and timing relationship enhancement parameters in NTN-specific SIB

Based on the above observations, additional procedure is needed to ensure UE prompt updating with the latest timing relationship enhancement parameters.

In one approach, NTN-specific SIB additionally indicates the validity duration of timing relationship enhancement parameters, besides the validity duration of uplink synchronization parameters. The validity duration in an earlier NTN-specific SIB is generally larger than the validity duration in a later NTN-specific SIB, unless at the time when the timing relationship enhancement parameters are updated.

In another approach, NTN-specific SIB does not additionally indicate the validity duration of timing relationship enhancement parameters. Update of cell-specific Koffset or Kmac in NTN-specific SIB affects the value tag and triggers SI modification procedure. The advantage of this approach is the reduction of the NTN-specific SIB overhead. The disadvantage of this approach is that all UEs in RRC idle or inactive state will need to read NTN-specific SIB when cell-specific Koffset or Kmac is updated, which may not be necessary and wastes power.

Proposal 1: For UEs updating the timing relationship enhancement parameters, down select one of the following two alternatives:

- Alt 1: Update of cell-specific Koffset or Kmac in NTN-specific SIB affects the value tag and triggers SI modification procedure.

- Alt 2: Introduce additional validity duration for timing relationship enhancement parameters in NTN-specific SIB.

## K1 range extension

It was agreed that the value range of K1 is extended to 31. It is open whether this value range extension has impact on the size of PDSCH-to-HARQ_feedback timing indicator field in DCI.

In our view, this field size should remain unchanged in DCI. In DCI formats 1_1 or 1_2, this field size is upper bounded by 3 bits, where the actual field size depends on the number of entries in a high layer configured table. The similar upper bound could be used for extended K1 values. Only the value range of the entries in the configured table (i.e., dl-DataToUL-ACK) needs to be updated.

In DCI format 1_0, the PDSCH-to-HARQ_feedback timing indicator field size is fixed to 3 bits to indicate the value of K1 from 1 to 8 slots. It is beneficial to extend the K1 range indicated by DCI format 1_0 to beyond 8 slots. One way is to increase the PDSCH-to-HARQ_feedback timing indicator field size in DCI format 1_0. However, this increases DCI format 1_0 payload size and is not preferred. Another way is to introduce a scaling factor (S) when determining the K1 value for DCI format 1_0. For example, the K1 value indicated in DCI format 1_0 is given by S⋅A for unpaired spectrum, where A is the value directly interpreted from the PDSCH-to-HARQ_feedback timing indicator field, and S is the configured scaling factor.

Proposal 2: The K1 range extension in unpaired spectrum does not change the PDSCH-to-HARQ_feedback timing indicator field size in DCI.

- For non-fallback DCI, only extend the value range of entries in the configured dl-DataToUL-ACK table.

- For fallback DCI, introduce a scaling factor when determining K1 value.

## Timing relationship enhancement for beam failure recovery

The timing relationship enhancement for beam failure recovery (BFR) was agreed in RAN1 #107-e meeting [3]. Specifically, Kmac is introduced to enhance the timing relationship of BFR response monitoring window. One remaining open issue is whether the timing relationship of UE’s applying new PUCCH transmission beam needs to be enhanced in NTN.

In NR terrestrial network, when a UE receives a first PDCCH in the dedicated search space set configured for BFR, it resets its PUCCH transmission beam based on the newly identified beam after 28 symbols after the last symbol of the PDCCH. This also implies the time from which gNB adjusts its reception beam for the PUCCH.

TS 38.213 Section 6 For the PCell or the PSCell, after 28 symbols from a last symbol of a first PDCCH reception in a search space set provided by recoverySearchSpaceId for which the UE detects a DCI format with CRC scrambled by C-RNTI or MCS- C-RNTI and until the UE receives an activation command for PUCCH-SpatialRelationInfo [11, TS 38.321] or is provided PUCCH-SpatialRelationInfo for PUCCH resource(s), the UE transmits a PUCCH on a same cell as the PRACH transmission using

In NTN, due to large propagation delay, the time from when gNB adjusts its reception beam for the PUCCH (with new beam) needs to be clarified. This is illustrated in Figure 2.

Figure 2: Timing relationship of PUCCH with new beam

The issue of PUCCH transmission with new beam in BFR is a typical downlink to uplink timing interaction problem. It is similar to downlink configuration MAC CE activation timing issue. For the latter issue, the activation time is the first (downlink) slot shat is after (uplink) slot n+3Nslotsubframe, μ+Kmac, where n is the slot when UE transmits ACK for the MAC CE. This implies that the transition between downlink slot and uplink slot is conducted with the assumption of TA being 0. However, in the current specification for PUCCH transmission with new beam in BFR, the transition between downlink timing and uplink timing takes the effect of TA into account. Hence, the current specification needs to be modified.

Like other timing relationship enhancements (e.g., transmission timing of HARQ-ACK on PUCCH), we could introduce Koffset to enhance the timing relationship on PUCCH. Specifically, the PUCCH with new beam is transmitted after uplink symbol n+K'offset+28, where n is the last downlink symbol of PDCCH reception with BFR response and K'offset is Koffset converted to symbols. This is shown in Figure 3.

Figure 3: Illustration of introducing Koffset to enhance PUCCH transmission time

Proposal 3: RAN1 to introduce Koffset to enhance the timing relationship on the PUCCH transmission with new beam in the beam failure recovery procedure.

# Conclusion

In this contribution, we provided our views on the remaining issues of timing relationship enhancement for NR NTN. Our observations and proposals are as follows:

Observation 1: The validity duration of timing relationship enhancement parameters (cell-specific Koffset and Kmac) in NTN-specific SIB is generally different from the validity duration of uplink synchronization parameters (ephemeris, common TA parameters and epoch time) in NTN-specific SIB.

Observation 2: The values of uplink synchronization parameters (ephemeris, common TA parameters and epoch time) generally change in successive NTN-specific SIB transmissions.

Observation 3: The validity durations of timing relationship enhancement parameters generally change in successive NTN-specific SIB transmissions.

Proposal 1: For UEs updating the timing relationship enhancement parameters, down select one of the following two alternatives:

- Alt 1: Update of cell-specific Koffset or Kmac in NTN-specific SIB affects the value tag and triggers SI modification procedure.

- Alt 2: Introduce additional validity duration for timing relationship enhancement parameters in NTN-specific SIB.

Proposal 2: The K1 range extension in unpaired spectrum does not change the PDSCH-to-HARQ_feedback timing indicator field size in DCI.

- For non-fallback DCI, only extend the value range of entries in the configured dl-DataToUL-ACK table.

- For fallback DCI, introduce a scaling factor when determining K1 value.

Proposal 3: RAN1 to introduce Koffset to enhance the timing relationship on the PUCCH transmission with new beam in the beam failure recovery procedure.

# References

- RP-212803, “Status report for WI: NR-NTN-solutions,” Dec. 2021.

- R1-2200875, “LS on NTN-specific SIB,” Feb. 2022.

- R1-2112716, “Feature lead summary #5 on timing relationship enhancements,” Nov. 2021.