3GPP TSG RAN WG1 #108-e	                                         	               R1-2201801

e-Meeting, Feb 21st – Mar 3rd, 2022

Agenda item:	8.16.13

Source: 	Apple Inc.

Title: 	Views on Rel-17 eDSS UE features

Document for:	Discussion/Decision

- Views on Rel-17 eDSS UE features

Based on the updated UE feature list for Rel-17 NR provided by the moderators in [1], we provided our views on the Rel-17 DSS enhancement UE features as below

- We propose to introduce a new FG34-1a to cover another simplified Type A UE, i.e., the USS can only be configured on sSCell, but cannot be configured on PCell/PSCell. This is based on the following Working Assumption

Working Assumption - When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 that schedule PDSCH/PUSCH on PCell/PSCell on PCell/PSCell USS set(s), and/or on sSCell USS set(s) - The WA to be confirmed after agreements are made on PDCCH BD/CCE handling and PDCCH overbooking handling for CCS from sSCell to PCell/PSCell - Specs also allow UEs supporting functionality of only Alt-1. Capability signaling details, if any, can be handled during the UE capability discussion for Rel17 - FFS: Whether the UE can monitor PDCCH from both cells in the same slot.

Even though we agree to support both the sSCell activation/deactivation and sSCell dormancy. Its operation is in contrary to the objective of this WID, i.e., to enable sSCell scheduling SpCell to improve DSS performance. From UE performance perspective, we would prefer to ensure reliable and good performance on the primary cell as much as possible, therefore

- We propose UE feature FG34-3 on whether UE support sSCell deactivation/activation

- We propose UE feature FG34-4 on whether UE support sSCell dormancy

The updated UE feature list with our proposal and with track change on is provided in Appendix in this contribution.

- Reference

[1] R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, 3GPP TSG RAN WG1 #107bis-e,e-Meeting, January 17th – 25th, 2022

- Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | [Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B)] - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - FFS: #unicast DCI limits for PCell/PSCell scheduling - Processing one unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing one unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of REG-bundle size when CCS from sSCell to PCell/PSCell is configured Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | 6-5 [, 34-1] | Yes |  |  | Per BC | No | Applicable to FR1 only |  | [Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60) for N=4, {30,30}, {30,60},{60,60}) Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Note: The CCS from sSCell to Pcell is applicable to FR1 only but there can be other Scells in FR2 configured for the UE | Optional with capability signalling |
| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell [with search space restrictions] (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell [with search space restrictions] (Type A) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - FFS: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters and UE reporting of any associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling - Processing one unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing one unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 configured on sSCell for CCS from sSCell to Pcell/PSCell - FFS: sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of Pcell/PSCell and sSCell. FFS overlap handling - FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s) - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) per BWP are 1 and 3, respectively - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of REG-bundle size when CCS from sSCell to PCell/PSCell is configured | 6-5 | Yes |  |  | Per BC | No | Applicable to FR1 only |  | [Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60) for N=4, {30,30}, {30,60},{60,60}) Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Note: The CCS from sSCell to PCell is applicable to FR1 only but there can be other SCells in FR2 configured for the UE | Optional with capability signalling |
| 34. NR-DSS | 34-1a | Further restriction to Type A | Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), | 34-1 | Yes | N/A |  | Per BC | N | N |  |  | Optional with capability signalling |
| 34. NR-DSS | 34-3 | Support of sSCell deactivation/activation | Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured | 34-1/34-2 | Yes | N/A |  | Per BC | N | N |  |  | Optional with capability signalling |
| 34. NR-DSS | 34-4 | Support of sSCell dormancy | Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured | 34-1/34-2 | Yes | N/A |  | Per BC | N | N |  |  | Optional with capability signalling |