3GPP TSG RAN WG1 #108-e		R1-2201817

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:     8.12.3

Source:	Spreadtrum Communications

Title:	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs

Document for:	Discussion and decision

Introduction

In this contribution, we provide our analysis and views on MBS reception for RRC_IDLE/RRC_INACTIVE UEs, and one text proposal is being presented.

Discussion

For Rel-15/Rel-16, for single cell operation or for operation with carrier aggregation in a same frequency band, when the QCL-TypeD property of PDCCH in Type-0/0A/2/3 CSS or USS set are different from the QCL-TypeD property of PDCCH in Type-1 CSS and, and both PDCCHs or associated PDSCH are overlapping or partially overlapping in time, a UE does not expect to monitor the PDCCH in a Type-0/0A/2/3 CSS or USS set.

In Rel-17 MBS, in addition to common search space Type-0, common search space Type-0B has been introduced for broadcast MBS in latest specification. During previous meetings, the principle that MBS can be the same or even lower priority than unicast seems to be common understanding among people. For example, we have already agreed that for PDCCH overbooking, the monitoring priority of Type-0B CSS is determined based on the search space set indexes of the Type-0B CSS set for MBS and USS sets.  Thus, in our understanding, the above solution to collision issue of the QCL-Type D property also applied to the case when the QCL-TypeD property of PDCCH in Type-0B CSS are different from the QCL-TypeD property of PDCCH in Type-1 CSS and, and both PDCCHs or associated PDSCH are overlapping or partially overlapping in time. Thus, we have the following proposal:

Proposal 1: Suggest to adopt the following text proposal in 38.213.

------------------------------------------Start of Text Proposal#1 for TS 38.213--------------------------------------

- 10.1	UE procedure for determining physical downlink control channel assignment

-----------------------------Unchanged part omitted--------------------------

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/0B/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set is not configured with same qcl-Type set to 'typeD' properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/0B/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

------------------------------------------End of Text Proposal#1 for TS 38.213--------------------------------------

Conclusion

In this paper, we provide our opinions on MBS reception for RRC idle/inactive UEs with the following proposal:

Proposal 1: Suggest to adopt the following text proposal in 38.213.

------------------------------------------Start of Text Proposal#1 for TS 38.213--------------------------------------

- 10.1	UE procedure for determining physical downlink control channel assignment

-----------------------------Unchanged part omitted--------------------------

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/0B/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set is not configured with same qcl-Type set to 'typeD' properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/0B/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

------------------------------------------End of Text Proposal#1 for TS 38.213--------------------------------------

- References

- 3GPP TS 38.213: "NR; Physical layer procedures for control", h00