3GPP TSG RAN WG1 #108-e	R1-2201830

e-Meeting, February 21st – March 3rd, 2022

Title:	[Draft] Reply LS on MBS SPS

Response to:	R1-2200888/R2-2202016

Release:	Release 17

Work Item:	NR_MBS-Core

Source:	RAN1

To:	RAN2

Contact person:	Tuo Yang

yangtuo@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

1. Overall Description:

RAN1 would like to thank RAN2 for the LS (R1-2200888/R2-2202016) on MBS SPS.

Please find RAN1 answers on the questions below.

Q1:RAN2 respectfully asks RAN1 to confirm RAN2’s understanding and the maximal number of G-CS-RNTI configured for UE? Can multiple G-CS-RNTIs be mapped to same MBS SPS-config and if so how that would work?

A1:

- RAN1 confirms RAN2’s understanding about multiple MBS SPS-configs and multiple G-CS-RNTIs and the association between a G-CS-RNTI and a MBS SPS-config is indicated by DCI scrambled with G-CS-RNTI.

- The maximal number of G-CS-RNTI configured for UE is left to RAN2 design without RAN1 spec impact.

- Multiple G-CS-RNTIs can be mapped to same MBS SPS-config, but for one MBS SPS-config it can only be activated by one G-CS-RNTI at a time. Regarding how multiple G-CS-RNTIs mapped to same MBS SPS-config, there are two possible alternatives:

  - Alt 1. For a given MBS SPS-config which has been activated by one G-CS-RNTI, it can only be activated again by another G-CS-RNTI after the MBS SPS-config has been deactivated using the previous G-CS-RNTI or CS-RNTI.

  - Alt 2. For a given MBS SPS-config which has been activated by one G-CS-RNTI, it can be activated again by another G-CS-RNTI without the SPS deactivation signalling.

Q2: Clarify the handling of PTM retransmissions.

A2:

- Retransmission scheme (i.e. via PTM or PTP) can be changed per TB or per TB per transmission.

- A single CS-RNTI is used for PTP retransmissions of all G-CS-RNTIs.

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

3. Date of Next TSG-WG1 Meetings:

TSG-WG1 Meeting #109-e	16th – 27th May 2022			Online Meeting